3GPP TSG-RAN WG3 Meeting #127bis	R3-252075

Wuhan, China, 7 – 11 April, 2025

Agenda item:	9.2

Source:	Nokia, Huawei, Deutsche Telekom, Jio Platforms, FiberCop, Qualcomm, BT

Title:	Discussion on Optional IEs in the Data Collection Reporting Initiation procedure

Document for:	Discussion and Decision

# 1	Introduction

In RAN3 #127, some contributions brought up the issue that some optional IEs in the DATA COLLECTION REQUEST message, from a functional point of view, have to be included in case certain measurement objects are requested. In particular, this is the case for the following IEs:

- Cell To Report List for Data Collection IE is an optional IE but, from a functional point of view, this information is expected to be present if the Report Characteristics for Data Collection IE (which lists all the measurement objects that can be requested) is present and when cell-specific information is requested from a neighbouring node.

- Requested Prediction Time IE is an optional IE but, from a functional point of view, this information is expected to be present if the Report Characteristics for Data Collection IE is present and when predictions are requested from a neighbouring node.

- UE Performance Collection Configuration IE is an optional IE but, from a functional point of view, this information is expected to be present if the Report Characteristics for Data Collection IE is present and when UE Performance measurements are requested from a neighbouring node.

- UE Trajectory Collection Configuration IE is an optional IE but, from a functional point of view, this information is expected to be present if the Report Characteristics for Data Collection IE is present and when measured UE Trajectory is requested from the target node.

In the last meeting some companies brought solutions adding procedural text to address this issue in the Data Collection Reporting Initiation procedure but RAN3 could not converge on the actual solution. However, we acknowledged the existence of this issue and the need to resolve it. In this contribution, we provide a list of possible solutions and our views on the preferred way forward.

# 2	Discussion

In the last meeting, the solutions brought by some companies considered enhancing the existing procedural text of some of the above listed IEs in the DATA COLLECTION REQUEST message which, however, still remains specified from the sender perspective. During the offline discussions, several alternatives were proposed. Next, we list possible approaches to address the acknowledged issue and our proposed way forward.

## 2.1 Enhance the currently specified procedural text of optional IEs

This was the solution proposed in the last meeting by some companies. The proposed text under this approach would enhance the currently specified procedural text of affected IEs by clarifying under which conditions such IEs should be included in the DATA COLLECTION REQUEST message, specified from sender’s perspective. This can be seen in the following:

This approach creates requirements on the NG-RAN node that sends the DATA COLLECTION REQUEST message. However, the preferred documentation approach in RAN3 is to avoid such requirements on the sender side in the procedural text of optional IEs, and rather describe the receiving node behaviour. It is true that examples of the former approach may exist in our specifications, but in our view we should strive to avoid reinforcing this practice, especially when dealing with the introduction of new IEs (or most recent ones, as for the ones in Rel-18).

Observation 1: Specifying the sender behaviour should be avoided in our specifications.

## 2.2 Introduce abnormal conditions for optional IEs

Abnormal conditions are used to address scenarios that shall lead to an unsuccessful outcome of the signalling procedure. An example of addressing the issue through abnormal conditions is illustrated next:

We think addressing this issue through abnormal conditions is cumbersome, as there are other means to solve the issue in a much simpler way such as signalling enhancements. Moreover, it also reduces the readability of the specification, since we are defining the “negative cases”, that is, when the procedure should fail. Therefore, we believe that using abnormal conditions should be avoided as well.

Observation 2: Using abnormal conditions to clarify the presence of Cell To Report List for Data Collection IE, Requested Prediction Time IE, UE Performance Collection Configuration IE and UE Trajectory Collection Configuration IE should be avoided since this scenario can be addressed by other means such as via signalling enhancements.

## 2.3 Change the presence of the IEs from Optional to Conditional

In our view, this is the simplest solution, which would only require to change the presence of the IEs under discussion from optional to conditional. This would require the following updates in the DATA COLLECTION REQUEST message for the UE Trajectory Collection Configuration IE and UE Performance Collection Configuration IE.

Similarly, Cell To Report List for Data Collection IE and Requested Prediction Time IE can be made conditional. But we see two possible solutions for that.

In the first solution (Solution A), we can introduce a single condition ifCellPredictions valid for both IEs and based on whether the first, second or third bit of Report Characteristics for Data Collection IE is set to ‘1’. These bits are related to cell-level information which, however, in Rel-18 is only associated to predictions. Hence, it would make sense to have a single condition for the presence of the Cell To Report List for Data Collection IE, pending the request of cell-level information in the Report Characteristics for Data Collection IE (which coincidentally in Rel-18 is only associated to predictions), and for the presence of the Requested Prediction Time IE, pending the request of predicted information in the Report Characteristics for Data Collection IE (which coincidentally in Rel-18 is only at cell level). This solution is shown below:

Another solution (Solution B) is to be more granular and introduces separate conditions for cell-level measurements (that may not be only predictions, even if this is not the case in Rel-18) and for predictions (that may not be only at cell level, even if this is not the case in Rel-18). This would be more future proof to account for optional IEs that pertain to cell-level measurements (that are not predictions) that may be introduced in future releases or optional IEs that are predictions but of different granularity (e.g., node level) and that may be introduced in future releases. This alternative is shown next:

Proposal 1: Change the presence of Cell To Report List for Data Collection IE, Requested Prediction Time IE, UE Trajectory Collection Configuration IE and UE Performance Collection Configuration IE from Optional to Conditional.

Proposal 2: RAN3 to discuss and select one solution between Solution A and Solution B for the conditional presence of Cell To Report List for Data Collection IE and Requested Prediction Time IE.

# 3	Conclusion

In this paper we provide the following observations and proposals:

Observation 1: Specifying the sender behaviour should be avoided in our specifications.

Observation 2: Using abnormal conditions to clarify the presence of Cell To Report List for Data Collection IE, Requested Prediction Time IE, UE Performance Collection Configuration IE and UE Trajectory Collection Configuration IE should be avoided since this scenario can be addressed by other means such as via signalling enhancements.

Proposal 1: Change the presence of Cell To Report List for Data Collection IE, Requested Prediction Time IE, UE Trajectory Collection Configuration IE and UE Performance Collection Configuration IE from Optional to Conditional.

Proposal 2: RAN3 to discuss and select one solution between Solution A and Solution B for the conditional presence of Cell To Report List for Data Collection IE and Requested Prediction Time IE.

We also provide a set of CRs that reflect our proposals in R3-251987, R3-251988 and R3-252076.