3GPP TSG-RAN WG3 Meeting #127bis	R3-252079

Wuhan, China, 7 – 11 April, 2025

Agenda item:	11.4

Source:	Nokia, Deutsche Telekom, FiberCop

Title:	Energy Saving in Split Architecture

Document for:	Discussion and Decision

# 1	Introduction

RAN3 in Rel-19 considers split architecture support for the Rel-18 use cases. In this context, RAN3 discussed how to measure the Energy Cost of a split gNB and in RAN3 #126 the following agreement was made:

In split architecture, the Energy Cost of gNB is the sum of the Energy Cost of its gNB-DUs.

Mapping rule is only configured at gNB-DU and shall be unified for all the gNB-DUs within a defined area.

Towards this end, RAN3 sent an LS (R3-247907) to SA5 asking SA5 to update their specifications to support the configuration of the Energy Cost mapping rule towards a gNB-DU with the mapping rule being unified for all gNB-DUs within a defined area. In this meeting, SA5 responded to RAN3 in R3-251529 including a CR to TS 28.541 enabling gNB-DUs within a defined area to be configured with the same Energy Cost mapping rule.

In this contribution we provide some observations related to the Energy Cost index calculation in the gNB-DU, based on the existing measurements provided by SA5.

# 2	Discussion

Despite the introduced support by SA5 to enable configuration of a mapping rule towards a gNB-DU that is unified within an area of gNB-DUs, it is still unclear how the mapping rule will map an Energy Consumption to an Energy Cost value. This is because currently there are no standardized means enabling a gNB-DU to measure its Energy Consumption. In TS 28.554 [1] different Energy Consumption measurements are provided including NF Energy Consumption and Energy Consumption of a gNB in non-split NG-RAN architecture. However, there is no measurement to describe the Energy Consumption of a gNB-DU (split NG-RAN architecture). It is therefore currently unclear how a gNB-DU can map its Energy Consumption to an Energy Cost measurement in a standardized manner.

Proposal: Introduce a new measurement for the Energy Consumption of a gNB-DU.

If RAN3 confirms the need to introduce a new Energy Consumption measurement of a gNB-DU, RAN3 needs to send a reply LS to SA5 to request SA5 to introduce a new measurement in TS 28.554.

We provide a draft reply LS to SA5 in [2].

# 3	Conclusion

In this paper we provide the following proposal:

Proposal: Introduce a new measurement for the Energy Consumption of a gNB-DU.

# References

[1]		3GPP TS 28.554, “5G end to end Key Performance Indicators (KPI)” (Rel.19)

[2]		R3-252080, “[Draft] Reply LS on LS reply to LS on energy saving for split gNB”, Nokia