3GPP TSG-RAN WG3 #127bis	                                        R3-252110

Wuhan, China, 7th – 11th April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | XXXX | Rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE Context Suspend/Resume procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Jio Platforms Ltd (JPL) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The S1AP specification lacks procedures to temporarily suspend a UE’s S1 context and later resume it. In Non-Terrestrial Network (NTN) store-and-forward scenarios (e.g., satellite IoT devices with intermittent connectivity), the network currently must release and re-establish the context, incurring significant signaling overhead. Introducing explicit Suspend/Resume procedures enables more efficient context handling during planned connectivity outages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Two new elementary procedures, UE Context Suspend and UE Context Resume, are added to S1AP. New S1AP messages (UE CONTEXT SUSPEND REQUEST/RESPONSE and UE CONTEXT RESUME REQUEST/RESPONSE/FAILURE) and Information Elements (IEs) are defined. S1 SETUP REQUEST/RESPONSE are extended with an IE to indicate support for this optional feature. Corresponding cause values and ASN.1 definitions are introduced. These changes preserve backward compatibility, as the new procedures are only used if both eNB and MME signal support during S1 Setup. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In NTN use cases, eNBs and MMEs will lack an efficient mechanism to handle UEs that temporarily leave coverage. UEs will be forced to perform full detach and reattach cycles, leading to unnecessary signaling, higher latency, and potential service disruption. This would undermine support for IoT devices over NTN, impeding the Work Item objectives. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.7, 8.3.8, 9.1.4.15–9.1.4.19, 9.1.8.4, 9.1.8.5, 9.2.1, 10 (ASN.1) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

-------------------------------------------Start of changes-------------------------------------------

### 8.3.7	UE Context Suspend

#### 8.3.7.1	General

The purpose of the UE Context Suspend procedure is to suspend the UE context, the UE-associated logical S1-connection and the related bearer contexts in the E-UTRAN and the EPC.

#### 8.3.7.2	Successful Operation

Figure 8.3.7.2-1: UE Context Suspend procedure. Successful operation.

The eNB initiates the procedure by sending the UE CONTEXT SUSPEND REQUEST message to the MME.

Upon receipt of the UE CONTEXT SUSPEND REQUEST the MME shall act as defined in TS 23.401 [11].

Upon receipt of the UE CONTEXT SUSPEND RESPONSE message, the eNB shall suspend the UE context, the UE-associated logical S1-connection and the related bearer contexts and send the UE to RRC_IDLE.

If the Information on Recommended Cells and eNBs for Paging IE is included in the UE CONTEXT SUSPEND REQUEST message, the MME shall, if supported, store it and may use it for subsequent paging.

If the Cell Identifier and Coverage Enhancement Level IE is included in the UE CONTEXT SUSPEND REQUEST message, the MME shall, if supported, store it and use it for subsequent paging.

If the Secondary RAT Usage Report List IE is included in the UE CONTEXT SUSPEND REQUEST message, the MME shall handle this information as specified in TS 23.401 [11].

If the Security Context IE is included in the UE CONTEXT SUSPEND RESPONSE message, the eNB shall store the received Security Context IE in the UE context and remove any existing unused stored {NH, NCC} as specified in TS 33.401 [15].

The eNB shall, if supported, report in the UE CONTEXT SUSPEND REQUEST message location information of the UE in the User Location Information IE. If the PSCell Information IE is included in the User Location Information IE, it indicates the UE was configured with EN-DC radio resources at the eNB. Also, if the Time Since Secondary Node Release IE is included in the UE CONTEXT SUSPEND REQUEST message, it indicates the time elapsed since EN-DC operation in the eNB was stopped for the UE.

The eNB initiates the procedure by sending a UE CONTEXT SUSPEND REQUEST message to the MME.

The MME, if it supports the Suspend procedure, shall suspend the UE’s S1 context (preserving the UE context and any associated EPS bearers in a dormant state) and reply with a UE CONTEXT SUSPEND RESPONSE message to the eNB, confirming that the UE context is now suspended.

At this point, the MME may cease paging the UE and may buffer downlink data for the UE. The UE-associated logical S1-connection remains allocated in the MME and eNB, but is marked as “suspended.”

#### 8.3.7.2	Abnormal Condition

If the MME does not respond to the UE CONTEXT SUSPEND REQUEST (e.g., due to lack of support or communication failure) within the applicable timer, the eNB shall consider the Suspend procedure failed.

In this case, the eNB shall handle the situation by either initiating a UE Context Release procedure or maintaining the context as active as per implementation, to ensure the UE does not remain in an indeterminate state.

The Suspend procedure shall not be initiated if the eNB has not received indication of MME support for it during S1 Setup. If the MME receives a UE CONTEXT SUSPEND REQUEST for a UE that it does not recognize (e.g., unknown MME UE S1AP ID), the MME shall ignore the request or respond with an appropriate failure (via ERROR INDICATION), and the UE context shall remain unchanged.

### 8.3.8	UE Context Resume

#### 8.3.8.1	General

The purpose of the UE Context Resume procedure is to indicate to the MME that the UE has resumed the suspended RRC connection or accesses for early data transmission and to request the MME to resume the UE context, UE-associated logical S1-connection and the related bearer contexts in the EPC.

The UE Context Resume procedure is used to restore a previously suspended UE context over S1. The eNB initiates this procedure when the UE that was suspended becomes reachable again (for example, when an NTN UE returns to coverage or exits a power-saving state).

The procedure allows the eNB and MME to resume normal S1-connected operations for the UE without requiring a full context re-establishment. The UE Context Resume procedure shall only be used if both eNB and MME have indicated support for Suspend/Resume during S1 Setup, and only for a context that was previously suspended.

#### 8.3.8.2	Successful Operation

Figure 8.3.8.2-1: UE Context Resume procedure. Successful operation.

The eNB initiates the procedure by sending the UE CONTEXT RESUME REQUEST message to the MME. If the eNB is not able to admit all suspended E-RABs the eNB shall indicate this in the E-RABs Failed To Resume List IE.Upon receipt of the UE CONTEXT RESUME REQUEST message the MME shall act as defined in TS 23.401 [11] and respond with the UE CONTEXT RESUME RESPONSE. If the MME is not able to admit all suspended E-RABs the MME shall indicate this in the E-RABs Failed To Resume List IE.7

The eNB shall release resources for each E-RAB failed to resume and shall assume that the EPC has released respective resources as well.

If the Security Context IE is included in the UE CONTEXT RESUME RESPONSE message, the eNB shall store the received Security Context IE in the UE context and the eNB shall use it for the next suspend/resume or X2 handover or Intra eNB handovers as specified in TS 33.401 [15].

If the Pending Data Indication IE is included in the UE CONTEXT RESUME RESPONSE message, the eNB shall use it as defined in TS 23.401 [11].

If the UE Context Resume procedure was initiated as a result of an EDT session as described in TS 36.300 [14] and the Pending Data Indication IE is received in the UE CONTEXT RESUME RESPONSE message, the eNB shall, if supported, use it to decide whether to proceed to set up an RRC connection for the UE.

When the eNB determines that a suspended UE has reappeared (e.g., the UE initiates RRC signaling to the eNB), the eNB sends a UE CONTEXT RESUME REQUEST message to the MME. This message references the suspended UE context (by including the previously assigned MME UE S1AP ID and the eNB’s UE S1AP ID). Upon receiving this request, the MME validates that it has a suspended context for the UE. If the context is found and the MME is prepared to resume service for the UE, the MME shall reactivate the UE’s context (e.g., resume any buffered data delivery, reset context timers) and respond with a UE CONTEXT RESUME RESPONSE message. Reception of the UE CONTEXT RESUME RESPONSE by the eNB confirms that the S1 UE context is resumed; normal uplink and downlink communications may proceed on the re-established S1 connection.

#### 8.3.8.3	Unsuccessful Operation

Figure 8.3.8.3-1: UE Context Resume procedure. Unsuccessful operation.

If the MME is not able to resume a single E-RAB it releases the UE-associated logical S1-connection by sending the UE CONTEXT RESUME FAILURE message to the eNB. Upon reception of the UE CONTEXT RESUME FAILURE message, the eNB shall release the RRC connection as specified in TS 36.331 [16] and release all related signalling and user data transport resources.

If the MME cannot resume the UE context (e.g., the context no longer exists, or resources are not available), the MME shall send a UE CONTEXT RESUME FAILURE message to the eNB, indicating the resume attempt was unsuccessful along with an appropriate cause (such as “unknown context”).

Upon receiving UE CONTEXT RESUME FAILURE, or if no response is received within the expected time, the eNB shall abort the resume attempt. In this case, the eNB shall remove or reset any locally stored context for the UE and may treat the UE as needing to reattach (e.g., triggering an NAS detach if applicable or initiating an initial context setup as if a new connection).

The MME, after sending a Resume Failure or timing out the procedure, shall continue to consider the UE context as suspended or may release it, depending on the failure cause. Both eNB and MME shall ensure that a failed resume does not result in a context inconsistency.

-------------------------------------------Next change-------------------------------------------

#### 9.1.4.15	UE CONTEXT SUSPEND REQUEST

This message is sent by the eNB to request the MME to suspend the UE context and the related bearer contexts.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| Cause | M |  | 9.2.1.3 |  | YES | ignore |
| Information on Recommended Cells and eNBs for Paging | O |  | 9.2.1.105 |  | YES | ignore |
| Cell Identifier and Coverage Enhancement Level | O |  | 9.2.1.109 |  | YES | ignore |
| Secondary RAT Usage Report List | O |  | 9.2.1.124 |  | Yes | ignore |
| User Location Information | O |  | 9.2.1.93 |  | YES | ignore |
| Time Since Secondary Node Release | O |  | 9.2.1.143 |  | Yes | ignore |

-------------------------------------------Next change-------------------------------------------

#### 9.1.8.4	S1 SETUP REQUEST

This message is sent by the eNB to transfer information for a TNL association.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| Global eNB ID | M |  | 9.2.1.37 |  | YES | reject |
| eNB Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| Supported TAs |  | 1..<maxnoofTACs> |  | Supported TAs in the eNB. | GLOBAL | reject |
| >TAC | M |  | 9.2.3.7 | Broadcast TAC. | - |  |
| >Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| >RAT-Type | O |  | 9.2.1.117 | RAT-Type associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | M |  | 9.2.1.16 |  | YES | ignore |
| CSG Id List |  | 0..1 |  |  | GLOBAL | reject |
| >CSG Id |  | 1 .. <maxnoofCSGIds> | 9.2.1.62 |  |  |  |
| UE Retention Information | O |  | 9.2.1.112 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.2.1.114 |  | YES | ignore |
| Connected en-gNB List |  | <0.. maxnoofConnecteden-gNBs> |  |  | GLOBAL | ignore |
| >en-gNB ID | M |  | BIT STRING (SIZE(22..32)) | The MME derives the Global en-gNB ID based on the en-gNB ID IE and the first PLMN Identity in the Supported TAs list for the en-gNB. |  |  |
| >Supported TAs |  | 1..<maxnoofTACs> |  | Supported (EPS) TAs in the en-gNB. | - |  |
| >>Configured TAC | M |  | TAC 9.2.3.7 | This information is used as specified in TS 36.300 [14]. | - |  |
| >>Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. | - |  |
| >>>PLMN Identity | M |  | 9.2.3.8 |  |  |  |
| SuspendResumeSupportIndicator | O |  | 9.2.1.x1 | Indicates eNB supports UE Context Suspend/Resume | - | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 6. |
| maxnoofCSGIds | Maximum no. of CSG Ids within the CSG Id List. Value is 256. |
| maxnoofConnecteden-gNBs | Maximum no. of en-gNBs connected to the eNB. Value is 256. |

#### 9.1.8.5	S1 SETUP RESPONSE

This message is sent by the MME to transfer information for a TNL association.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| Served GUMMEIs |  | 1..<maxnoofRATs> |  | The LTE related pool configuration is included on the first place in the list. | GLOBAL | reject |
| >Served PLMNs |  | 1..<maxnoofPLMNsPerMME> |  |  | - |  |
| >>PLMN Identity | M |  | 9.2.3.8 |  | - |  |
| >Served GroupIDs |  | 1..<maxnoofGroupIDs> |  |  | - |  |
| >>MME Group ID | M |  | OCTET STRING (SIZE(2)) |  | - |  |
| >Served MMECs |  | 1..<maxnoofMMECs> |  |  | - |  |
| >>MME Code | M |  | 9.2.3.12 |  | - |  |
| >GUMMEI Type | O |  | ENUMERATED (native, mapped, …, mappedFrom5G) |  | - | ignore |
| Relative MME Capacity | M |  | 9.2.3.17 |  | YES | ignore |
| MME Relay Support Indicator | O |  | 9.2.1.82 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |
| UE Retention Information | O |  | 9.2.1.112 |  | YES | ignore |
| Served DCNs |  | 0..<maxnoofDCNs> |  |  | GLOBAL | ignore |
| >Served DCNs Items | M |  | 9.2.1.121 |  | - |  |
| IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for IAB. | YES | ignore |
| SuspendResumeSupportIndicator | O |  | 9.2.1.x1 | Indicates MME supports UE Context Suspend/Resume |  | Ignore |

Editorial Note- The SuspendResumeSupportIndicator IE is added as optional in the S1 SETUP RESPONSE. If the MME supports the Suspend/Resume feature and the eNB indicated support, the MME shall include this IE to confirm its capability. If this IE is absent in the response, the eNB shall assume the MME does not support Suspend/Resume (even if the eNB requested it). Absence of the IE can also occur if the eNB did not indicate support in the request. The Criticality is ignore, so old eNBs will simply ignore this IE if present.

| Range bound | Explanation |
| --- | --- |
| maxnoofPLMNsPerMME | Maximum no. of PLMNs per MME. Value is 32. |
| maxnoofRATs | Maximum no. of RATs. Value is 8. |
| maxnoofGroupIDs | Maximum no. of GroupIDs per node per RAT. Value is 65535. |
| maxnoofMMECs | Maximum no. of MMECs per node per RAT. Value is 256. |
| maxnoofDCNs | Maximum no. of DCNs servered by one MME. Value is 32. |

-------------------------------------------Next changes-------------------------------------------

### 9.2.1	Radio Network Layer Related IEs

#### 9.2.1.x1	SuspendResumeSupportIndicator

This IE indicates support for the UE Context Suspend/Resume procedures on the S1 interface. It is used as an optional flag in the S1 SETUP REQUEST/RESPONSE me

The IE is of type ENUMERATED with value “supported” (value=0) indicating support for the feature. Presence of this IE shall be interpreted as “Suspend/Resume supported.” Its absence means “Suspend/Resume not supported.” No additional values are defined for this IE (the enumeration may be extended in the future if needed) ssages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SuspendResumeSupportIndicator | M |  | supported(value=0) |  |

#### 9.2.1.x2	Cause

In the Cause IE (clause 9.2.1.3), under the category “Radio Network Layer Cause,” add the following cause value:

SuspendProcedureRequested – This cause indicates that the action is triggered by a UE Context Suspend procedure. For example, the eNB uses this cause in the UE CONTEXT SUSPEND REQUEST to inform the MME that the UE’s S1 connection is being suspended (rather than released) intentionally (e.g., due to the UE moving out of coverage temporarily). In general, this cause value is used to distinguish a planned suspend event from an unexpected radio link loss.

The new cause value is added to the enumeration in the Radio Network layer causes (see ASN.1 in clause 10). This cause value may be used in any message that includes the Cause IE where applicable; specifically, eNB should set Cause = SuspendProcedureRequested in UE Context Suspend Request. The MME may include this cause in its logs or in any relevant failure responses if needed. This cause has the same meaning in both eNB-initiated and MME messages and is considered a radio network cause.

-------------------------------------------Next changes-------------------------------------------

# B.2	ASN.1 definition

-- Elementary Procedures

id-UEContextSuspend       ProcedureCode ::= 55

id-UEContextResume        ProcedureCode ::= 56

******

The S1AP Elementary Procedures definitions are updated as follows (new entries underlined)

uEContextSuspend S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE      UEContextSuspendRequest

SUCCESSFUL OUTCOME      UEContextSuspendResponse

PROCEDURE CODE          id-UEContextSuspend

CRITICALITY             reject

}

uEContextResume S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE      UEContextResumeRequest

SUCCESSFUL OUTCOME      UEContextResumeResponse

UNSUCCESSFUL OUTCOME    UEContextResumeFailure

PROCEDURE CODE          id-UEContextResume

CRITICALITY             reject

}

****

UEContextSuspendRequest ::= SEQUENCE {

protocolIEs    ProtocolIE-Container ::= {

{ ID id-MME-UE-S1AP-ID        CRITICALITY reject TYPE MME-UE-S1AP-ID        PRESENCE mandatory},

{ ID id-eNB-UE-S1AP-ID        CRITICALITY reject TYPE ENB-UE-S1AP-ID        PRESENCE mandatory},

{ ID id-InformationOnRecommendedCellsAndENBsForPaging

CRITICALITY ignore TYPE InformationOnRecommendedCellsAndENBsForPaging   PRESENCE optional},

{ ID id-CellIdentifierAndCELevel

CRITICALITY ignore TYPE CellIdentifierAndCELevel   PRESENCE optional}

}

}

UEContextSuspendResponse ::= SEQUENCE {

protocolIEs    ProtocolIE-Container ::= {

{ ID id-MME-UE-S1AP-ID        CRITICALITY ignore TYPE MME-UE-S1AP-ID        PRESENCE mandatory},

{ ID id-eNB-UE-S1AP-ID        CRITICALITY ignore TYPE ENB-UE-S1AP-ID        PRESENCE mandatory},

{ ID id-CriticalityDiagnostics

CRITICALITY ignore TYPE CriticalityDiagnostics   PRESENCE optional},

{ ID id-SecurityContext       CRITICALITY reject TYPE SecurityContext       PRESENCE optional}

}

}

UEContextResumeRequest ::= SEQUENCE {

protocolIEs    ProtocolIE-Container ::= {

{ ID id-MME-UE-S1AP-ID        CRITICALITY reject TYPE MME-UE-S1AP-ID        PRESENCE mandatory},

{ ID id-eNB-UE-S1AP-ID        CRITICALITY reject TYPE ENB-UE-S1AP-ID        PRESENCE mandatory},

{ ID id-E-RABFailedToResumeList

CRITICALITY reject TYPE E-RABFailedToResumeList-ResumeReq   PRESENCE optional},

{ ID id-RRCResumeCause        CRITICALITY ignore TYPE RRC-Establishment-Cause PRESENCE optional}

}

}

UEContextResumeResponse ::= SEQUENCE {

protocolIEs    ProtocolIE-Container ::= {

{ ID id-MME-UE-S1AP-ID        CRITICALITY ignore TYPE MME-UE-S1AP-ID        PRESENCE mandatory},

{ ID id-eNB-UE-S1AP-ID        CRITICALITY ignore TYPE ENB-UE-S1AP-ID        PRESENCE mandatory},

{ ID id-E-RABFailedToResumeList

CRITICALITY reject TYPE E-RABFailedToResumeList-ResumeRes   PRESENCE optional},

{ ID id-CriticalityDiagnostics

CRITICALITY ignore TYPE CriticalityDiagnostics   PRESENCE optional},

{ ID id-SecurityContext       CRITICALITY reject TYPE SecurityContext       PRESENCE optional}

}

}

UEContextResumeFailure ::= SEQUENCE {

protocolIEs    ProtocolIE-Container ::= {

{ ID id-MME-UE-S1AP-ID        CRITICALITY ignore TYPE MME-UE-S1AP-ID        PRESENCE mandatory},

{ ID id-eNB-UE-S1AP-ID        CRITICALITY ignore TYPE ENB-UE-S1AP-ID        PRESENCE mandatory},

{ ID id-Cause                CRITICALITY reject TYPE Cause                PRESENCE mandatory},

{ ID id-CriticalityDiagnostics

CRITICALITY ignore TYPE CriticalityDiagnostics   PRESENCE optional}

}

}

***

New IE definitions are added to S1AP ASN.1 for the feature support indicator:

SuspendResumeSupportIndicator ::= ENUMERATED { supported }

id-SuspendResumeSupportIndicator OBJECT IDENTIFIER ::= { id-IE 147 }   -- next available IE ID

***

Cause IE ASN.1 (CauseMisc enumerations) is extended with new values (underlined):

CauseMisc ::= ENUMERATED {

...,

unknownPLMN,

...,

<u>resumeProcedureNotSupported,</u>

<u>ueContextUnavailable</u>

}

-------------------------------------------End of changes-------------------------------------------