3GPP TSG-RAN WG3 #127bis	             		                           R3-252117

Wuhan, China, 7th – 11th April 2025

Agenda Item:	17.2

Source:	Jio Platforms Ltd (JPL)

Title:	XnAP Enhancements for UL WUS Configuration in On-Demand SIB1

Document for:	Discussion

# Introduction

Release-19 Network Energy Saving (NES) features include techniques for reducing unnecessary broadcasts, such as on-demand SIB1 transmission. The motivation is to let a cell avoid continuously broadcasting System Information Block Type 1 (SIB1) unless a UE actually needs it, thereby saving energy​. In the on-demand SIB1 Case 2 scenario, a UE connected in one cell (Cell A) is provided with an Uplink Wake-Up Signal (UL WUS) configuration for a neighboring “sleeping” cell (the NES cell). The UE then transmits a UL WUS on that NES cell to trigger that cell to wake up and broadcast its SIB1 on-demand​. This mechanism ensures the UE can obtain SIB1 from the target NES cell when needed, without the NES cell having to broadcast SIB1 continuously.

However, supporting this scenario requires coordination between the gNBs that control Cell A and the NES cell. Specifically, Cell A’s gNB must receive the UL WUS configuration of the NES cell (so it can forward it to the UE), and later they must agree when to cease using that configuration. The current Xn Application Protocol (XnAP) specification (TS 38.423 v18.5.0) has no procedures defined for exchanging or withdrawing UL WUS configurations between gNBs – a clear gap identified in recent discussions. At RAN3#126/#127, the Working Group noted the absence of any XnAP signaling to support on-demand SIB1 and agreed new procedures are needed​. In essence, to enable Case 2 operation in a multi-gNB network, new XnAP messages must be introduced to provision the UL WUS configuration info to a neighboring node and to stop that configuration when it’s no longer required.

This document proposes enhancements to XnAP to fill that gap. We focus on defining two new global procedures over Xn — one to provision (distribute) a UL WUS configuration for a given cell, and another to indicate the cessation of that configuration. We describe the envisioned triggers and behavior for each procedure, and outline message content (Information Elements, IE) aligning with existing specifications (reusing identifiers like NR-CGI, Cause, etc., and a new IE for the UL WUS parameters). The aim is to integrate these procedures cleanly into TS 38.423 without impacting other NES scenarios or overlapping with UE-specific mobility signaling.

# Discussion

Observation from RAN3 meetings: In the RAN3#126/#127 technical discussions for the Rel-19 NES work, it became evident that no XnAP support exists for the on-demand SIB1 Case 2 scenario. While RAN2 is handling the RRC mechanisms for UL WUS (how the UE is configured over the air) and on-demand SI behavior, RAN3 saw a clear spec gap on the inter-node side. Specifically, TS 38.423 (XnAP) does not define any procedure for a gNB to provision UL WUS configuration to a neighbor, nor to withdraw such configuration once provided. As a result, without new XnAP extensions, a multi-vendor or multi-cell deployment cannot utilize on-demand SIB1 Case 2 – the serving gNB would have no standardized way to obtain the NES cell’s UL WUS parameters or to coordinate the usage of UL WUS. This was highlighted in the RAN3 discussions: the consensus was that a new XnAP class 1 procedure is required to transfer UL WUS configuration between gNBs, and additionally a class 2 procedure is needed to signal when that UL WUS configuration should stop being used​

Gaps in current XnAP signaling: Table 8.1.1 of TS 38.423 lists all Class 1 procedures (e.g. Xn Setup, Configuration Update, Cell Activation, etc.) and Table 8.1.2 lists Class 2 procedures (e.g. RAN Paging, Xn-U Address Indication, etc.). None of these cover the exchange of UL WUS configuration. The existing Global procedures like NG-RAN Node Configuration Update allow nodes to exchange supported cell information, but they do not encompass dynamic UL WUS config provisioning or on-demand SI triggers. Moreover, the Cell Activation procedure (used e.g. in dual connectivity and IAB contexts) is aimed at activating a cell’s user-plane resources, not at delivering uplink wake-up signal parameters to a neighbor. Therefore, new dedicated procedures are justified. The absence of such procedures in TS 38.423 v18.5.0 means we must introduce them as part of Rel-19 updates​.

New Class 1 procedure – “UL WUS Config Provision”: We propose a new Class 1 global procedure to facilitate UL WUS configuration provisioning between gNBs. This would be a request/response style procedure (since it’s Class 1) initiated by the gNB that provides the UL WUS configuration (or alternatively by the gNB that needs it – see below for trigger options). The procedure’s purpose is to deliver the necessary UL WUS parameters of a specific NES cell to the neighboring node:

- Trigger: This procedure could be triggered in two main ways. (a) NES Cell initiated (push): when a gNB places one of its cells into an on-demand SIB1 state (e.g., entering a dormant broadcast mode), it can initiate UL WUS Config Provision toward neighboring gNBs, so they have the UL WUS config ready to distribute to UEs if needed. (b) Serving Cell initiated (pull): alternatively, when a serving gNB determines a UE may need to trigger SIB1 on a neighbor (for example, during measurement or mobility prep), it can request the UL WUS config from the neighbor gNB. In either case, the initiating message carries the identity of the concerned cell and the UL WUS configuration data. The peer node responds with a successful outcome if it can handle the config (or with failure if, say, the cell ID is unknown or it cannot apply the config).

- Message content: We anticipate the initiating message (call it UL WUS CONFIGURATION PROVISION message) will include at least two key IEs: NR-CGI (NR Cell Global Identifier of the NES cell whose WUS config is being provisioned) and a new UL-WUS-ConfigInfo IE containing the UL wake-up signal parameters for that cell. The NR-CGI IE identifies the cell unambiguously (PLMN + NR Cell ID), reusing the existing definition from XnAP​.

The UL-WUS-ConfigInfo IE would be defined in alignment with RAN2’s RRC specification and with any similar IE on F1AP, to ensure consistency. For example, UL-WUS-ConfigInfo might include the uplink resource (frequency, time slot or occasion, PRACH preamble or other signal type, etc.), the expected timing relative to SSB or paging, power or repetition info, and any identification pattern the NES cell expects from UEs. By aligning this IE with RRC’s UL WUS configuration (as defined in TS 38.331 and the NES study conclusions), we ensure that the info XnAP carries can be directly used to configure UEs over RRC​.

Besides those, the procedure can reuse standard IEs like Global gNB ID (to identify the source/destination node if needed, though in a point-to-point Xn interface it may not be strictly necessary in the message body) and Criticality Diagnostics (to report any errors in case of unsuccessful outcome). If the receiving gNB accepts the config, it sends a successful UL WUS CONFIGURATION PROVISION ACK (name TBD) with no additional payload except maybe an acknowledgement indication. If it rejects, an UL WUS Config Provision Failure could carry a Cause IE (e.g., unknown cell, not supported, etc.) to inform why.

- Behavior: On successful provisioning, the neighbor (receiving) gNB will store the UL WUS config for the identified NES cell. When a UE under that neighbor’s coverage needs on-demand SIB1 from the NES cell, the neighbor can now act – for instance, if the UE is in Connected mode on Cell A, Cell A’s gNB can send the UL WUS config to the UE via RRC. From the NES cell’s perspective, it now expects that UEs may transmit the UL WUS as configured. Importantly, this procedure is global (not UE-specific) – it sets up the configuration at the node level, potentially for any UEs that might require it. This avoids tying the exchange to a particular handover or UE context, which keeps it independent of mobility procedures. By making it a global class 1 procedure, multiple UEs can benefit once the config is provisioned (no need to request per UE). It also fits multi-vendor scenarios: e.g., an Ericsson gNB can obtain UL WUS config from a Nokia gNB via Xn, which was a key reason to standardize this exchange.

New Class 2 procedure – “UL WUS Config Stop”: We also propose a Class 2 global procedure to signal the termination or withdrawal of a previously provided UL WUS configuration. This is effectively a one-way notification (no response expected in normal conditions, since Class 2 procedures are unacknowledged) to inform the peer that the UL WUS config for a given cell should no longer be considered active.

- Trigger: This procedure would be initiated when the conditions for on-demand SIB1 on that cell no longer apply. For example, if the NES cell is being fully reactivated (it will resume normal SIB1 broadcasting), or if the feature is turned off, the gNB controlling that cell should inform neighbors to stop using the old UL WUS config. Another trigger could be the serving gNB’s decision based on its own policy – e.g., if a UE that needed the config has left and no others are expected, Cell A’s gNB might tell the NES cell’s gNB that it can cease offering the config (especially if the NES cell was broadcasting some info about UL WUS in its SIB1 for Case 1, which could be stopped to save overhead). Notably, during RAN3#127 it was discussed that “Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB”, indicating the serving side can trigger the stop when appropriate​.

In practice, either gNB might initiate this stop message depending on who has the knowledge that the config is no longer needed; but typically the provider of the config (NES cell’s gNB) would signal stop when the cell exits energy-saving mode, or the consumer (serving gNB) would signal if it no longer requires the config.

- Message content: The initiating message (e.g., UL WUS CONFIGURATION STOP indication) would carry the necessary identifiers to specify which configuration is being stopped. At minimum, an NR-CGI for the cell is needed (so the peer knows which cell’s UL WUS config to remove). Including a Cause IE is advisable as well, to indicate why the config is being stopped (normal operation, configuration change, etc.), although for a simple stop it might be optional. We can reuse the standard Cause IE from XnAP (with appropriate cause values such as normal release, unspecified, etc.). No response is expected; the receiving node, upon getting this message, should mark that UL WUS config as inactive and stop using it for any new UEs. If the receiving node had already provided that config to some UE in progress, how to handle that is out of scope of XnAP (that would be up to RRC/upper layers – presumably the UE will simply not get a response if it tries to use it after stop, or the network ensures no UE is mid-procedure when stopping). In case the receiving node does not recognize the cell in the message or encounters an error, the general XnAP error handling (via an Error Indication procedure with appropriate cause) can be utilized – but there is no direct response to a Class 2 procedure.

- Behavior: The UL WUS Config Stop procedure being Class 2 means it’s mainly an advisory/notification from one gNB to the other. On reception of UL WUS Config Stop, the neighbor gNB will purge or mark obsolete the stored UL WUS configuration for that cell. This prevents future UEs from being given an invalid config. If the NES cell is now active, UEs can obtain SIB1 through normal SI reading (no special UL WUS needed). The stop message thus helps synchronize the end of on-demand SIB1 operation between nodes. It ensures that one gNB doesn’t continue to send UEs to wake up a cell that is no longer in energy-saving state or that the neighbor knows to stop expecting UL WUS triggers. By defining an explicit stop procedure, we avoid overloading existing messages or leaving the behavior ambiguous. This procedure is also kept separate from any UE-specific context; it is a global network coordination message. We also avoid any conflict with Xn UE Context Release or handover cancellation messages – those deal with specific UE handovers, whereas UL WUS Config Stop is purely about cell configuration info and can occur independently of whether a UE context is active or not.

Information Elements and alignment: A new IE UL-WUS-ConfigInfo will need to be specified. We propose to base this on the RRC-defined structure for UL WUS configuration (as developed in RAN2) and any analogous F1AP signaling. For instance, RAN2’s ongoing work (per TR 38.864 and RAN2 contributions) likely defines parameters such as the UL WUS resource timing, a Wake-up Signal ID or pattern, periodicity or window for the UE to transmit, etc. These can be encapsulated in the UL-WUS-ConfigInfo IE so that XnAP conveys exactly what the target cell expects. This IE could mirror any similar IE on F1AP if the gNB-CU needs to configure a gNB-DU with the UL WUS details (ensuring consistency across interfaces). Additionally, we will reuse existing IEs wherever possible to avoid duplicate definitions. The use of Global gNB ID IE can identify the sending/receiving node in Xn Setup or error situations, but for the new procedures, the node identities are implicitly known from the Xn association (hence including it in every message may be redundant). The NR-CGI IE is already defined in XnAP (used in many contexts to identify cells, e.g. in Xn Configuration Update when exchanging served cells or in cell activation) – we will reuse that to specify the cell that the UL WUS config or stop pertains to. The Cause IE is already defined for many procedures and can be reused in the failure message of the Provision procedure and in the Stop (if we include cause in stop). CriticalityDiagnostics IE should be included in any unsuccessful outcome message, following normal XnAP error handling practice, to detail which IE or aspect was problematic if a message is rejected. By leveraging these existing IEs, we ensure the new messages integrate smoothly into the XnAP ASN.1 and semantics.

Compatibility and scope considerations: It is important that these new procedures be designed such that they do not interfere with other NES use cases or general mobility procedures. For NES Case 1 (where a UE camps on an NES cell and obtains the UL WUS config directly from that cell, possibly via broadcast or RRC if the cell provides it), no Xn signaling is needed – and our proposed solution does not affect that case. In fact, the NES cell could choose whether to broadcast UL WUS config in its SIB1 or to rely on neighbors to supply it; our proposal simply standardizes the neighbor-to-neighbor exchange for the case where that is needed (multi-cell coordination scenario). The procedures also do not overlap with any UE-associated procedures like Handover or SN Addition: those remain separate. Even during a handover, if an on-demand SIB1 is needed, the UL WUS config exchange would be done via the global provision procedure outside of the per-UE handover messaging (this avoids any impact on critical handover timing paths). We also ensure the naming and coding of the new procedures do not clash with existing ones: new unique Procedure Codes will be assigned in the ASN.1 for “UL WUS Config Provision” and “UL WUS Config Stop,” and new IE codes for UL-WUS-ConfigInfo, etc. This will be added in the ASN.1 definitions (likely in Clause 9 and Annex A of TS 38.423, which contain message and IE definitions).

Placement in the specification: To incorporate these changes cleanly, we suggest adding a new sub-clause in the XnAP spec where these procedures will be described. Since Section 8.4 of TS 38.423 covers “Global procedures” and already contains Xn Setup, Configuration Update, Cell Activation, etc., our new procedures could naturally be added as 8.4.x items (e.g., 8.4.7 and 8.4.8 if numbering continues) for “UL WUS Configuration Provision” and “UL WUS Configuration Stop.” Alternatively, if the editors prefer to group NES-related procedures in a dedicated section (to improve readability), we could introduce an Section 8.6 (or 8.7) specifically for “Network Energy Saving procedures,” under which these two procedures are defined. The exact placement can be decided for minimal editorial disruption – for instance, after the IAB-related procedures (which in Rel-17 occupy section 8.5, if any)​.

The key is that these procedures are global (node-to-node), not UE-specific, so they belong in the global procedures part of the spec. Corresponding message definitions will be added in Section 9 (likely under 9.1.x “Messages for Global Procedures”), including the structure of UL WUS Config Provision Request/Acknowledge/Failure and UL WUS Config Stop Indication. ASN.1 additions in Annex A will define new Procedure Codes for these procedures, and new IE definitions (UL-WUS-ConfigInfo) as well as add these IEs to the appropriate message ASN.1 structures.

By introducing these XnAP procedures, we ensure that on-demand SIB1 can be supported in a coordinated multi-node environment. This fills the gap identified in RAN3 and allows the UE-triggered SIB1 concept (which showed significant energy saving benefits in studies​) to be realized in practice. The proposals below summarize the recommended specification updates.

# Conclusion

In conclusion, we propose the following actions for 3GPP RAN3 to support UL WUS signaling over Xn for on-demand SIB1 (NES Case 2):

- Proposal 1: Introduce a new XnAP Class 1 global procedure “UL WUS Config Provision.”

- Proposal 2: Introduce a new XnAP Class 2 global procedure “UL WUS Config Stop.”

- Proposal 3: Define a new Information Element IE “UL-WUS-ConfigInfo.”

- Proposal 4: Update the XnAP ASN.1 (Annex A) to support the above: add new Procedure Codes, new message PDU definitions, and necessary IE container additions.

- Proposal 5: Incorporate the new procedures and messages into the appropriate sections of TS 38.423 (Rel-19).