3GPP TSG-RAN WG3 Meeting #127-bis	R3-252121

Wuhan, CN, 7-11 Apr, 2025

Title:	Discussion on SONMDT for Intra-NTN mobility

Agenda Item:	10.3.1

Source:	Huawei, CMCC, China Unicom

Document for:	Discussion

1	Introduction

This document continues to discuss the enhancements for SON and MDT for NTN on top of the progress of last meeting.

2	Discussion

2.1	Location based trigger event

In the previous meeting, the following open issues were identified

Revert the WA: There is no need to report any other info in the RLF report for the location based CHO.

For earth moving case, the accuracy and complexity issue of the network-based solution is acknowledged.

- Option 1: to introduce the absolute UTC time of when RLF/HOF happens in the RLF report.

- Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell (when RLF occurred).

FFS on option 1 or option 2.

In general, both option 1 and option 2 are workable. However, option 1 only omits the UTC time calculation step when RLF occurred (step 1 in the Figure below). It still needs the network to  perform step 2, 3 and 4. Which means that option 1 only reduces the uncertainty, but does not eliminate the complexity of the network based solution. It is also noted that the network needs to perform step 2, 3 and 4, not only for the serving cell, but also for the target cell.

While by option 2, there is no any additional calculation processing in the network.

On the other hand, as specified in TS 38.311, the distance is measured by the UE based on the serving cell referenceLocation, the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to the event for D2. And the epoch time is updated every sub frame as per its definition.

It may conclude that the UE needs to update the reference location and measure the distance every sub frame.

distanceThreshFromReference1, distanceThreshFromReference2 Distance from a fixed reference location configured with referenceLocation1 or referenceLocation2 for condEventD1. Distance from a moving reference location determined by the UE based on the serving cell movingReferenceLocation broadcast in SIB19 or referenceLocation and the corresponding epoch time and satellite ephemeris configured within the MeasObjectNR associated to the event for condEventD2. Each step represents 50m.

#### EpochTime The IE EpochTime is used to indicate the epoch time for the NTN assistance information, and it is defined as the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information. The reference point for EpochTime of the serving, target, or neighbour NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point when this field is provided in an NTN cell and the gNB when this field is provided in a TN cell. In case of handover or conditional handover, EpochTime is based on the timing of the target cell, i.e. the SFN and sub-frame number indicated refers to the SFN and sub-frame of the target cell. Otherwise, EpochTime is used based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated refers to the SFN and sub-frame of the serving cell.

Furthermore, the UE will anyway have to continuously evaluate the distance from the condEventD2 is configured to the handover execution (to evaluate the enter condition or leaving condition of the event). Therefore, there is no additional cost for the UE just to insert the latest available distance measurements into the RLF report.

- RAN3 to agree that for location based trigger event, the UE will include when RLF occurs

- the measured distances from the UE to the moving reference location of the serving cell

- the measured distances from the UE to the moving reference location of the candidate cell

- an indicator to indicate which threshold is fulfilled, the distanceThreshFromReference1, the distanceThreshFromReference2, or neither

2.3 Logged MDT enhancement for NTN

Logged MDT is mainly applicable for UEs in RRC_IDLE/INACTIVE. The UE measurement in idle/inactive mode in NTN follows the network configurations, such as SMTCs, and assistance information provided in SIB19. Specifically, in NTN the UE can perform time-based and location-based measurements on neighbour cells. For example, the location based measurement will be performed by the UE in following criteria as specified in TS 38.304:

#### 5.2.4.2 Measurement rules for cell re-selection Following rules are used by the UE to limit needed measurements: If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ: - If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN quasi-Earth-fixed system and has obtained its location information: - If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements; - Else, the UE shall perform intra-frequency measurements; - else if distanceThresh and movingReferenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN Earth-moving system and has obtained its location information: - if the distance between UE's location and the serving cell reference location determined based on movingReferenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements; - else, the UE shall perform intra-frequency measurements; - Else, the UE may not perform intra-frequency measurements; - Else, the UE shall perform intra-frequency measurements.

Similar measurement rules for inter-frequency and inter RAT frequency cell reselection can be found in the same section.

It can be shortly summarized that if the signal power/quality of the serving cell falls lower than the preconfigured thresholds, the measurement will be directly triggered. Else if the distance to the reference location of serving cell is larger than the preconfigured threshold, the measurement will be triggered.

According to TS 37.320 and TS 38.413, different to periodic logged MDT, event triggered logged MDT supports event trigger L1 Event to reduce the measurement data collected by logged MDT.

We think that it is beneficial from functional perspective to introduce a location based event trigger type for logged MDT in NTN cells so that the idle/inactive mode measurements can be collected when the UE distance to the reference location of serving cell is larger than a preconfigured threshold.

The main reason is that the legacy A2-like L1 event trigger type may not work so effectively. As the radio signal quality of NTN cells is expected quite good even in the border area due to the different near-far effect in TN and NTN as depicted in the Figure below.

Figure: near-far effect in TN and NTN

In order to enable the network to collect radio measurements from idle/inactive UEs at NTN cells border, it is proposed:

RAN3 to introduce a location based event trigger type for logged MDT in NTN cells and inform RAN2 and SA5 to design the corresponding protocol.

The event trigger logged MDT configuration currently is defined as a CHOICE IE as specified in TS 38.413. If the new location based event trigger is introduced as a new branch in the Event Trigger Logged MDT Configuration IE, it means that the L1 event and the location event cannot be configured to the UE simultaneously. It will disable the UE to log measurements for the two event trigger types in a single logged MDT task which may impact signalling based logged MDT negatively.

Therefore, it is proposed to introduce the location based event trigger type as a separate IE, as implemented below.

#### 9.3.1.180 Event Trigger Logged MDT Configuration This IE defines the event trigger logged MDT configuration. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE Event trigger type; M. >Out-of-coverage. >>Out-of-Coverage Configuration; M; ENUMERATED (true, ...). >L1 Event. >>CHOICE L1 Event Threshold; M. >>>RSRP. >>>>Threshold RSRP; M; INTEGER (0..127); Corresponds to the RSRP-Range IE as defined in TS 38.331 [18].. >>>RSRQ. >>>>Threshold RSRQ; M; INTEGER (0..127); Corresponds to the RSRQ-Range IE as defined in TS 38.331 [18].. >>Hysteresis; M; INTEGER (0..30); This parameter is used within the entry and leave condition of an event triggered reporting condition.. >>Time to Trigger; M; ENUMERATED (ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256, ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560, ms5120); Time during which specific criteria for the event needs to be met in order to trigger a measurement report.. Location Event Trigger Type; 0..1. >Threshold Distance; M; INTEGER(0.. 65535); The threshold distance from the reference location of the serving cell..

RAN3 to agree that the location based event trigger type can be configured simultaneously with the legacy event type.

3	Conclusion

This document proposes:

Proposal 1:	RAN3 to agree that for location based trigger event, the UE will include when RLF occurs

-	the measured distances from the UE to the moving reference location of the serving cell

-	the measured distances from the UE to the moving reference location of the candidate cell

-	an indicator to indicate which threshold is fulfilled, the distanceThreshFromReference1, the distanceThreshFromReference2, or neither

Proposal 2:	RAN3 to introduce a location based event trigger type for logged MDT in NTN cells and inform RAN2 and SA5 to design the corresponding protocol.

Proposal 3:	RAN3 to agree that the location based event trigger type can be configured simultaneously with the legacy event type

The corresponding TPs and draft LS are provided in [1] ~ [3].

The draft LS is provided in [4].

4 	Reference

[1] R3-252122	(TP for MDT BLCR for TS38.413): Logged MDT enhancement for NTN

[2] R3-252123	(TP for MDT BLCR for TS38.423): Logged MDT enhancement for NTN

[3] R3-252124	(TP for MDT BLCR for TS37.320): Logged MDT enhancement for NTN

[4] R3-252125	[DRAFT] LS on SONMDT for NTN