3GPP TSG-RAN WG3 #127bis								R3-252165

Wuhan, Greece, 7th – 11th April 2025

Agenda Item:	16.2

Source:	China Telecom

Title: 	Signalling Procedure for Inventory and Command

Document for:	Discussion

Introduction

In SI phase, the signalling and procedures for Topology 1 were agreed and captured into TR 38.769[1]. The work item on solutions for Ambient IoT (Internet of Things) in NR was agreed in RAN#106 meeting [2]. In RAN3#127 meeting, the following agreements are achieved:

About Inventory Procedures:

WA: Introduce a NGAP Class 1 Inventory Request procedure.

Introduce INVENTORY REQUEST message, which includes Inventory Request Transfer IE.

Introduce INVENTORY RESPONSE message, which includes Inventory Response Transfer IE.

Introduce INVENTORY FAILURE messages, which includes Inventory Failure Transfer IE.

Introduce a NGAP Class 2 Inventory Report procedure.

Introduce INVENTORY REPORT message, which includes Inventory Report Transfer IE.

The above Transfer IEs are transparent to AMF in case of indirect communication.

FFS: For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

About Command Procedure:

WA: Introduce a NGAP Class 1 Command Request procedure

Introduce COMMAND REQUEST message, which includes Command Request Transfer IE.

Introduce COMMAND RESPONSE message, which includes Command Response Transfer IE.

Introduce COMMAND FAILURE messages, which includes Command Failure Transfer IE.

The above Transfer IEs are transparent to AMF in case of indirect communication.

FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure?

FFS: Command request procedure is a per device procedure or to be used for multiple devices?

FFS: The need for a Command Report procedure?

FFS: Device association between gNB and AIOTF is used in Command procedure.

About Interface management procedures:

FFS: In indirect path communication, existing NG interface management procedures are used between the gNB and the AMF.

FFS: In direct path communication, existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used between the gNB and the AIOTF.

FFS: For interface management procedures, if reuse existing ones, how to handle the not applicable mandatory IEs, and how to handle A-IoT only support case.

Work on the stage2/3 details and open issues....

This contribution aims to continue discussing the procedure and content information for inventory and command.

Discussion

Content in Inventory Procedure

According to the conclusion in last meeting, the following information still need to be discussed:

FFS: For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

According to conclusions of TR23.700-13, the following information can be included in inventory request message:

- correlation identifier: As a single A-IoT RAN may connect to multiple A-IoT CNs, the correlation identifier allows the AIOTF to match inventory responses with the corresponding request. Therefore, the correlation identifier is required in inventory request, response, and failure message.

- AIOTF identifier: To route the request back to the originating AIOTF, the AMF additionally provides an AIOTF identifier when forwarding the request from the AIOTF. Therefore, the AIOTF identifier is necessary in inventory request, response, and failure messages.

- part of or full AIoT Device Identifier: Inventory requests can be sent by the A-IoT CN for a single device, a group of devices, or all devices. According to TR23.700-13, AIoT devices compare the AIoT Device Identification information with their own AIoT Device Identifier (partial or full) to determine whether they should respond to the paging message. Additionally, a partial AIoT Device Identifier can be used as filtering information to associate multiple AIoT devices.

Additionally, we propose to introducing a session ID, as the specifications must allow multiple inventory procedure instances to run in parallel over the RAN-CN interface. Given that the A-IoT RAN system may cover multiple regions in practical deployments, the inventory area list should be included in the inventory message. Therefore, the following information can be included in A-IoT Inventory Request message:

- [M] correlation identifier

- [O] AIOTF identifier

- [M] AIoT Device Identifier

- [M] Inventory Session ID

- [O] Inventory Area List

- [O] approximate number of AIoT devices

Proposal 1: the following information can be included in A-IoT Inventory Request message:

- [M] correlation identifier

- [O] AIOTF identifier

- [M] AIoT Device Identifier

- [M] Inventory Session ID

- [O] Inventory Area List

- [O] approximate number of AIoT devices

Content in Command Procedure

The following open issue need to be discussed in this meeting:

FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure?

FFS: Command request procedure is a per device procedure or to be used for multiple devices?

FFS: The need for a Command Report procedure?

FFS: Device association between gNB and AIOTF is used in Command procedure.

The Command Request message is performed after inventory message. According to conclusions of TR23.700-13, the following information can be included in Command Request message:

- correlation identifier

- CN UE ID and RAN UE ID: in legacy mechanism, both RAN and CN need to allocate a separate ID for UE. This mechanism can reused in A-IoT. With regard to the issue whether multiple devices is support in command message, we understand that multiple device IDs need to be introduced to support batch operations.

- Approximate D2R message size: approximate D2R message size based on AF request as assistance information to A-IoT RAN is provided by AIOTF

- IoT NAS PDU

- Command Type: Read, Write and Disable. The definition of three types is as follow:

  - Request to read information from an AIoT Device.

  - Write: Request to write information to an AIoT Device.

  - Disable: Request that an AIoT Device has its capability to transmit RF permanently disabled.

In addition, a separate command report also needs to be introduced to response the read information from an AIoT device.

Proposal 2: the following information can be included in A-IoT Command Request message:

- [M]correlation identifier

- [M] CN UE ID

- [M] RAN UE ID

- [O]Approximate D2R message size: approximate D2R message size based on AF request as assistance information to A-IoT RAN is provided by AIOTF

- [M]IoT NAS PDU

- [O]Command Type: Read, Write and Disable. The definition of three types is as follow:

  - Request to read information from an AIoT Device.

  - Write: Request to write information to an AIoT Device.

  - Disable: Request that an AIoT Device has its capability to transmit RF permanently disabled.

Proposal 3: Single Command request procedure supports single device and multiple devices case.

Proposal 4: Command Report is needed to response the information from an AIoT device.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: the following information can be included in A-IoT Inventory Request message:

- [M] correlation identifier

- [O] AIOTF identifier

- [M] AIoT Device Identifier

- [M] Inventory Session ID

- [O] Inventory Area List

- [O] approximate number of AIoT devices

Proposal 2: the following information can be included in A-IoT Command Request message:

- [M]correlation identifier

- [M] CN UE ID

- [M] RAN UE ID

- [O]Approximate D2R message size: approximate D2R message size based on AF request as assistance information to A-IoT RAN is provided by AIOTF

- [M]IoT NAS PDU

- [O]Command Type: Read, Write and Disable. The definition of three types is as follow:

  - Request to read information from an AIoT Device.

  - Write: Request to write information to an AIoT Device.

  - Disable: Request that an AIoT Device has its capability to transmit RF permanently disabled.

Proposal 3: Single Command request procedure supports single device and multiple devices case.

Proposal 4: Command Report is needed to response the information from an AIoT device.

Reference

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR