3GPP TSG-RAN WG3 Meeting #127bis	R3-252169

Wuhan, China, Apr 7th – 11th, 2025

Agenda Item:	11.4

Source:	CMCC

Title:	Discussion on AI/ML based Mobility Optimization for NR-DC

Document for:	Discussion

# 1 Introduction

In the previous RAN3 meetings, the following agreements were achieved for AI/ML based Mobility Optimization for NR-DC [1].

RAN3#123bis

Agree to take as baseline for the Mobility Optimization for NR-DC that the use case is studied assuming inference at the MN. The main use case is limited to dual connectivity only (e.g. no conditional procedures are in scope).

RAN3#124

Enhance the Dual Connectivity procedure (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance.

RAN3#125bis

As a starting point, SN Addition and MN-initiated SN Change procedures are considered to be enhanced to trigger the collection of UE performance from SN to MN. This assumes that the involved gNBs have an existing Data Collection Reporting Initiation procedure in place configuring the collection of UE performance.

RAN3#126

Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side.

Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

The open issue was captured in last meeting as below and need to be continued in next meeting:

RAN3#127

Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information?

In this contribution, we would continue the discussion on the solutions for NR-DC mobility optimization and provide some proposals.

# 2 Discussion

2.1 Prediction Information of Mobility Optimization for NR-DC

Introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information?

Regarding to AI/ML based Mobility Optimization, we think the benefit is mainly reflected in the prediction information provided by AI/ML model. However, there is still no consensus on the type of prediction information for NR-DC mobility optimization.

In the previous meeting, whether to introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) is the focus of discussion for AI/ML based NR-DC Mobility Optimization. From our point of view, in SN Addition and MN-initiated SN Change procedures, the key operations for the target SN are to choose the PSCell and configure the SCG cell(s). If the predicted PSCell ID could be introduced as one of assistance information, it could help MN select the appropriate target SN node and help the target SN choose a suitable PSCell, configure the SCG cell(s) and reserve resource in advance.

Observation 1: The predicted PSCell could help MN select the appropriate target SN node. And the target SN could further use the prediction information to choose a suitable PSCell, configure the SCG cell(s) and reserve resource in advance.

During the discussion in previous RAN3 meetings, a few companies concerned that AI/ML model deployed in MN could not generate the predicted PSCell within the target SN or it does not make sense for MN to provide the predicted PSCell since PSCell is selected by target SN.

We observe from the legacy SN Addition and MN-initiated SN Change procedures in TS 37.340 [2] that MN could provide measurement results of PSCells to the target SN for SN to choose and configure the SCG cell(s). That means the measurement results related to the target SN could be used as the input data for AI/ML model to generate the predicted PSCell(s). Furthermore, some other assistance information (e.g., resource status of neighbour nodes) maybe used by MN in the legacy target SN node selection. The assistance information in the legacy target SN node selection could also be used as the input data for AI/ML model. This means that MN has the enough input data for AI/ML model to generate the predicted PSCell(s).

Observation 2: It is feasible for MN to generate the predicted PSCell ID. In legacy SN Addition and MN-initiated SN Change procedures, MN could get the measurement results related to the target SN and other assistance information for target SN node selection. The assistance information could be used as the input data for AI/ML model to generate the predicted PSCell(s).

In Rel-18, for AI/ML based Mobility Optimization of single connectivity, the UE Trajectory Prediction is transferred from source node to target node via the HANDOVER REQUEST message. UE Trajectory Prediction is provided as a list of cells where the UE is predicted to connect after handover, each of which is indicated together with an expected time of stay into the cell (optional). The similar solution could also apply to NR-DC Mobility Optimization.

In NR-DC procedures, MN could generate the predicted PSCell ID and use the prediction to select the appropriate target SN node. And MN could also transfer the predicted PSCell ID as one of the assistance information to target SN which use it to choose and configuration the SCG cell(s).

Observation 3: In Rel-18, the UE Trajectory Prediction is transferred from source node to target node for AI/ML based Mobility Optimization of single connectivity. UE Trajectory Prediction is provided as a list of cells where the UE is predicted to connect after handover. The similar solution could also apply to NR-DC Mobility Optimization.

Based on the above analysis, we propose that the predicted PSCell ID should be considered as the baseline of NR-DC Mobility Optimization.

Proposal 1: Predicted PSCell ID should be introduced in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information for AI/ML based Mobility Optimization for NR-DC.

2.2 Measured information collection of Mobility Optimization for NR-DC

As per the agreement achieved in RAN3#124 meeting [3], the collection of measured UE performance shall be supported as the feedback in Rel-19.

Enhance the Dual Connectivity procedure (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance.

Besides that, we think the actual PSCell ID should be introduced into the measured information as the feedback for NR-DC Mobility Optimization. The PSCell ID is necessary for measured information of NR-DC, since it is helpful for MN to identify the correlation between the cell(s) and the measured information (e.g., UE performance) reported by target SN. Moreover, if the predicted PSCell ID in proposal 1 is agreed as the assistance information for NR-DC, the actual PSCell which target SN select could be used for AI/ML model performance testing and model retraining.

Observation 4: The actual PSCell ID is necessary for measured information of NR-DC, since it is helpful for MN to identify the correlation between the cell(s) and the measured information (e.g., UE performance) reported by target SN and also could be used for AI/ML model performance testing and model retraining.

Proposal 2: Besides measured UE performance, RAN3 to further consider to introduce the actual PSCell ID into the measured information collection as the feedback for NR-DC Mobility Optimization.

In Rel-18, the measured information collection starts from successful Handover Execution, and the following four conditions introduced to terminate the measured information collection:

- The time since UE was successfully handed over to the target node is equal to the value of the Collection Time Duration;

- The configured number of hand-overs within the same target node is reached;

- UE moves to RRC_INACTIVE or RRC_IDLE state;

- UE is handed over to a cell belonging to an NG-RAN node different from the target node.

From our point of view, the trigger conditions to start and terminate the measured information collection in target SN are also needed for Mobility Optimization for NR-DC. In RAN3#126 meeting, TP to TS 38.423 [4] was agreed to introduce the Data Collection ID IE into the S-NODE ADDITION REQUEST message for the Data Collection Reporting and the Data Collection Reporting Initiation procedures. And the target SN could start the measured information collection for NR-DC after successful SN addition for SN Addition and MN-initiated SN change.

In order to terminate the measured information collection of NR-DC in time and avoid the unnecessary measurements in target SN, the trigger conditions used to terminate the measured information collection are also needed for NR-DC. For example, the target SN could terminate the measured information collection when the time since the successful establishment of NR-DC with the target SN is equal to the value of the collection time duration of NR-DC, which is similar with the mechanism that the time since UE was successfully handed over to the target node is equal to the value of the Collection Time Duration in Rel-18. And the target SN could terminate the measured information collection when another cell becomes the PSCell or the target SN receives SN Release Request message, that means UE will move to single connectivity with MN. It is similar with the mechanism that UE is handed over to a cell belonging to an NG-RAN node different from the target node.

Proposal 3: Target SN could terminate the collection of measurement for NR-DC, e.g.,

- The time since the successful PSCell Addition or PSCell change is equal to the value of the collection time duration of NR-DC;

- PSCell change to another cell;

- The execution of Secondary Node Release procedure.

2.3 Signaling procedures

In the case that AI/ML model is deployed in MN, the MN could generate the prediction information of NR-DC in advance. So, we think on one hand, the prediction information generated by the MN could be used by MN itself as the assistance information for target SN selection for SN Addition and MN-initiated SN Change. On the other hand, the MN could send the prediction information to the target SN for resource reservation. Similar with the signaling procedure of AI/ML based mobility optimization in Rel-18 that the prediction is transferred to the target gNB via the HANDOVER REQUEST message, we think that MN could send the prediction information to target SN via S-NODE ADDITION REQUEST message for NR-DC.

Proposal 4: After MN generate the prediction information of NR-DC (e.g., the predicted PSCell ID),

- The MN could use the prediction information as the assistance information for target SN selection for SN Addition and MN-initiated SN Change.

- The MN could send the prediction information to the target SN for resource reservation via S-NODE ADDITION REQUEST message.

# 3 Conclusion

In this paper, we discuss on the solutions for NR-DC mobility optimization, we have the following observations and proposals:

Prediction Information of Mobility Optimization for NR-DC

Observation 1: The predicted PSCell could help MN select the appropriate target SN node. And the target SN could further use the prediction information to choose a suitable PSCell, configure the SCG cell(s) and reserve resource in advance.

Observation 2: It is feasible for MN to generate the predicted PSCell ID. In legacy SN Addition and MN-initiated SN Change procedures, MN could get the measurement results related to the target SN and other assistance information for target SN node selection. The assistance information could be used as the input data for AI/ML model to generate the predicted PSCell(s).

Observation 3: In Rel-18, the UE Trajectory Prediction is transferred from source node to target node for AI/ML based Mobility Optimization of single connectivity. UE Trajectory Prediction is provided as a list of cells where the UE is predicted to connect after handover. The similar solution could also apply to NR-DC Mobility Optimization.

Proposal 1: Predicted PSCell ID should be introduced in the DC procedure (e.g., SN Addition, MN-initiated SN change) as one of assistance information for AI/ML based Mobility Optimization for NR-DC.

Measured information collection of Mobility Optimization for NR-DC

Observation 4: The actual PSCell ID is necessary for measured information of NR-DC, since it is helpful for MN to identify the correlation between the cell(s) and the measured information (e.g., UE performance) reported by target SN and also could be used for AI/ML model performance testing and model retraining.

Proposal 2: Besides measured UE performance, RAN3 to further consider to introduce the actual PSCell ID into the measured information collection as the feedback for NR-DC Mobility Optimization.

Proposal 3: Target SN could terminate the collection of measurement for NR-DC, e.g.,

- The time since the successful PSCell Addition or PSCell change is equal to the value of the collection time duration of NR-DC;

- PSCell change to another cell;

- The execution of Secondary Node Release procedure.

Signaling procedures

Proposal 4: After MN generate the prediction information of NR-DC (e.g., the predicted PSCell ID),

- The MN could use the prediction information as the assistance information for target SN selection for SN Addition and MN-initiated SN Change.

- The MN could send the prediction information to the target SN for resource reservation via S-NODE ADDITION REQUEST message.

# Reference

- RAN3_127_agenda_20250221_1300_EOM, Feb 2025.

- 3GPP TS 37.340: NR; Multi-connectivity; Overall Description; Stage 2.

- RAN3_124_agenda_20240524_eom, May 2024.

- R3-247895, (TP to BLCR of 38.423) Mobility Optimization for NR-DC, Nov 2024.