3GPP TSG-RAN WG3 Meeting #127bis	                               R3-252171

Wuhan, China, April 7 - April 11, 2025

Agenda item:	19.2

Source:	CMCC

Title:	Discussion on supporting CLI handling in SBFD

Document for:	Discussion

Introduction

In RAN3#127 meeting, the following agreements are made on the enhancements for CLI handling in SBFD, shown as:

Exchange of SBFD configuration: gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Served Cell Information IE in Served Cell Information IE in gNB-CU Configuration Update message. The SBFD time and frequency configuration is encoded as container defined by RAN2. Exchange of measurement configuration: For exchange of SSB info over Xn, nothing to be done, already supported in RAN3 spec. For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition? Exchange of measurement result: For exchange of strongest DL beam information over Xn, SSB info is defined as SSB-Index INTEGER (0..63), NZP CSI-RS info is defined as NZP CSI-RS Resource Indicator INTEGER (1..64). Exchange of CLI-mitigation request: To define CLI-mitigation request as an optional IE. FFS on the details. How to exchange the strongest DL beam information and the CLI-mitigation request over Xn, e.g. to reuse existing procedures such as NG-RAN node Configuration Update procedure or Resource Status Reporting procedure, or, to introduce new procedures?

In this contribution, we would like to provide our views on the information exchange among gNBs for CLI handling in SBFD.

Discussion

Exchange of semi-static cell-specific SBFD time and frequency location configuration

In RAN3#126 meeting [1], the following agreements are made:

The SBFD time and frequency location configuration with cell granularity in Served Cell Information NR IE shall be introduced in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

Therefore, it is clear that the remaining issue is that IE design. For specific IE design, the time configuration of TDD cell is contained in TDD UL-DL Configuration Common NR IE in TDD Info of Served Cell Information NR currently. For TDD UL-DL Configuration Common NR IE, it is directly referred to tdd-UL-DL-ConfigurationCommon IE in TS 38.331. To this end, the design of semi-static cell-specific SBFD time and frequency location configuration in RAN3 is closely related to RAN2’s progress on the configuration methods which have not been made. To be specific, if RAN2 decides to extend the tdd-UL-DL-ConfigurationCommon IE in TS 38.331, then current XnAP specification can support the exchange of the SBFD time configuration accordingly. Otherwise, new IE or field need to be discussed to be introduced in RAN3 within TDD Info of Served Cell Information NR currently.

For the frequency configuration, the frequency configuration of TDD cell is contained in Frequency Info and Transmission Bandwidth or Carrier List IE in TDD Info of Served Cell Information NR currently. Based on the agreement that the starting RB and bandwidth of SBFD UL sub-band are indicated by a RIV-based indication per SCS configuration, the relative cell-specific SBFD frequency configuration including starting RB and bandwidth of SBFD UL sub-band may need to be discussed by RAN3 on where and how to capture them.

In RAN3#127 meeting [2], the following agreement is further made:

The SBFD time and frequency configuration is encoded as container defined by RAN2.

Therefore, form RAN3 perspective, a new IE named SBFD configuration NR which contains SBFD Time Info and SBFD Frequency Info can be introduced, and both SBFD Time Info and SBFD Frequency Info are containers which refer to parameter(s) in TS 38.331.

Observation 1: The time configuration of TDD cell is contained in TDD UL-DL Configuration Common NR IE in TDD Info of Served Cell Information NR currently.

Observation 2: The frequency configuration of TDD cell is contained in Frequency Info and Transmission Bandwidth or Carrier List IE in TDD Info of Served Cell Information NR currently.

Observation 3: For the semi-static cell-specific SBFD time and frequency location configuration design, it is closely related to RAN2’s progress on the configuration methods which have not been made.

Proposal 1: From RAN3 perspective, a new IE named SBFD configuration NR which contains SBFD Time Info and SBFD Frequency Info can be introduced in Served Cell Information NR IE. Both SBFD Time Info and SBFD Frequency Info are containers which refer to parameter(s) in TS 38.331.

Exchange of measurement resource configuration

In terms of the exchange of measurement resource configuration, there are three potential options listed below:

- Option 1: Xn Setup and/or NG-RAN node Configuration Update procedure

- Option 2: Resource Status Reporting Initiation and Resource Status Reporting procedure

- Option 3: Newly introduced procedure

For Option 1, it has been discussed in RAN3#127 meeting but no consensus has been made as following:

For exchange of SSB info over Xn, nothing to be done, already supported in RAN3 spec. For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition?

If Option 1 is supported, the measurement resource configuration can be captured in XN SETUP REQUEST/RESPONSE message and NG-RAN NODE CONFIGURATION UPDATE/ UPDATE ACKNOWLEDGE message. Note that for SSB configuration, it has been agreed that it is already contained in the Measurement Timing Configuration IE signalled as part of the Served Cell Information NR IE. For NZP-CSI-RS configuration, if RAN2 decide to introduce the set of one or more periodic NZP CSI-RS resources configuration into MeasurementTimingConfiguration, then RAN3 can support the exchange of NZP CSR-RS resources naturally. Besides, with the consideration that NZP CSI-RS resource is already specified in TS 38.331 as part of the NZP-CSI-RS-Resource IE and NZP-CSI-RS-ResourceSet IE, introducing new IE called NZP-CSI-RS Configuration, which is a container referring to NZP-CSI-RS-Resource IE and NZP-CSI-RS-ResourceSet IE in TS 38.331, is also feasible.

For Option 2, the RESOURCE STATUS REQUEST message and RESOURCE STATUS UPDATE message can be enhanced to support the exchange measurement resource configuration. With the consideration that currently RESOURCE STATUS REQUEST message and RESOURCE STATUS UPDATE message can be used to initiate the requested measurement and report the results of the requested measurements respectively, it is also technically feasible to support the exchange of measurement resource configuration by enhancing them. However, note that there is greater impact on RAN3 spec for Option 2 compared to Option 1.

For Option 3, a new procedure can be designed to exchange of measurement resource configuration. Regarding Option 3, we do not find much benefits.

Based on above analysis, we prefer Option 1, because it seems to have the least impact on the RAN3 spec.

Observation 4: There are three options for gNBs exchanging the measurement resource configuration as following:

- Option 1: Xn Setup and/or NG-RAN node Configuration Update procedure

- Option 2: Resource Status Reporting Initiation and Resource Status Reporting procedure

- Option 3: Newly introduced procedure

Observation 5: If RAN2 decide to introduce the set of one or more periodic NZP CSI-RS resources configuration into MeasurementTimingConfiguration, then RAN3 can support the exchange of NZP CSR-RS resources naturally. Besides, introducing new IE called NZP-CSI-RS Configuration, which is a container referring to NZP-CSI-RS-Resource IE and NZP-CSI-RS-ResourceSet IE in TS 38.331, is also feasible.

Proposal 2: The exchange of measurement resource configuration is performed in Xn Setup and NG-RAN node Configuration Update procedure.

Proposal 3: RAN3 is asked to wait for RAN2’s determination on the introduction of periodic NZP CSI-RS resources configuration, before discussing whether a new IE needs to be added in Served Cell Information NR IE for exchange of NZP CSI-RS over Xn.

Report of strongest DL beam information and exchange of CLI-mitigation request

In RAN3#127 meeting [2], the following agreement has been made for the report of strongest DL beam information:

Exchange of measurement result: For exchange of strongest DL beam information over Xn, SSB info is defined as SSB-Index INTEGER (0..63), NZP CSI-RS info is defined as NZP CSI-RS Resource Indicator INTEGER (1..64).

Then the remaining issue is which procedure is applied, which determining in which message should the relevant IE(s) be introduced. In general, we think that it should be possible to support that one gNB to report the strongest DL beam information and request for CLI-mitigation together with same message, that it, we support that both the strongest DL beam information and CLI-mitigation mitigation request are two optional IEs in a same message. The reason is that the strongest DL beam information from victim gNB can provide the additional information for aggressor gNB performing CLI mitigation through its implementation, especially for beam pairing case in FR-2, if victim gNB requests for CLI-mitigation.

Observation 6: The strongest DL beam information from victim gNB can provide the additional information for aggressor gNB performing CLI mitigation through its implementation, especially for beam pairing case in FR-2, if victim gNB requests for CLI-mitigation.

Proposal 4: Both the strongest DL beam information and CLI-mitigation mitigation request are two optional IEs in a same message.

In terms of the CLI-mitigation request, it has been agreed in RAN3#127 meeting that [2]:

Exchange of CLI-mitigation request: To define CLI-mitigation request as an optional IE. FFS on the details. - How to exchange the strongest DL beam information and the CLI-mitigation request over Xn, e.g. to reuse existing procedures such as NG-RAN node Configuration Update procedure or Resource Status Reporting procedure, or, to introduce new procedures?

It is clear that there are also three options for gNB reporting the strongest DL beam information and request for CLI-mitigation:

- Option 1: NG-RAN node Configuration Update procedure

- Option 2: Resource Status Reporting procedure

- Option 3: Newly introduced procedure

In fact, this issue has been discussed in email discussion during previous meeting, but no agreement has been made. From our point of view, all of the three options are technically feasible. However, there may be some difference for the beam nulling and beam pairing case. To be specific, in terms of beam nulling case, aggressor gNB may need to measure victim gNB’s reference signal based on victim gNB’s CLI-mitigation request, thus a report of strongest DL beam may not be needed in this case. While in terms of beam pairing, a report of strongest DL beam along with the CLI-mitigation request may transmitted with a same message from victim gNB to agresssor gNB. Based on above analysis, a newly introduced procedure seems to be a cleaner and more flexible solution, because it will not affect or be affected by legacy Xn Interface Management Procedure or Resource Status Reporting procedure.

In terms of the design of newly introduced procedure, a CLI Measurement Reporting procedure which is a Class 2 procedure can be introduced. CLI MEASUREMENT UPDATE message which contains the optional strongest DL beam information and optional CLI mitigation request information, initiates this procedure.

Observation 7: There may be some difference for the beam nulling and beam pairing case in terms of report of strongest DL beam and CLI-mitigation request.

Observation 8: A newly introduced procedure seems to be a cleaner and more flexible solution, because it will not affect or be affected by legacy Xn Interface Management Procedure or Resource Status Reporting procedure.

Proposal 5: RAN3 is asked to introduce a new Class 2 procedure for report of strongest DL beam and CLI-mitigation request.

Conclusions

Based on the discussions mentioned above, in this contribution we provide some discussions on random access in SBFD:

Observation 1: The time configuration of TDD cell is contained in TDD UL-DL Configuration Common NR IE in TDD Info of Served Cell Information NR currently.

Observation 2: The frequency configuration of TDD cell is contained in Frequency Info and Transmission Bandwidth or Carrier List IE in TDD Info of Served Cell Information NR currently.

Observation 3: For the semi-static cell-specific SBFD time and frequency location configuration design, it is closely related to RAN2’s progress on the configuration methods which have not been made.

Observation 4: There are three options for gNBs exchanging the measurement resource configuration as following:

- Option 1: Xn Setup and/or NG-RAN node Configuration Update procedure

- Option 2: Resource Status Reporting Initiation and Resource Status Reporting procedure

- Option 3: Newly introduced procedure

Observation 5: If RAN2 decide to introduce the set of one or more periodic NZP CSI-RS resources configuration into MeasurementTimingConfiguration, then RAN3 can support the exchange of NZP CSR-RS resources naturally. Besides, introducing new IE called NZP-CSI-RS Configuration, which is a container referring to NZP-CSI-RS-Resource IE and NZP-CSI-RS-ResourceSet IE in TS 38.331, is also feasible.

Observation 6: The strongest DL beam information from victim gNB can provide the additional information for aggressor gNB performing CLI mitigation through its implementation, especially for beam pairing case in FR-2, if victim gNB requests for CLI-mitigation.

Observation 7: There may be some difference for the beam nulling and beam pairing case in terms of report of strongest DL beam and CLI-mitigation request.

Observation 8: A newly introduced procedure seems to be a cleaner and more flexible solution, because it will not affect or be affected by legacy Xn Interface Management Procedure or Resource Status Reporting procedure.

Proposal 1: From RAN3 perspective, a new IE named SBFD configuration NR which contains SBFD Time Info and SBFD Frequency Info can be introduced in Served Cell Information NR IE. Both SBFD Time Info and SBFD Frequency Info are containers which refer to parameter(s) in TS 38.331.

Proposal 2: The exchange of measurement resource configuration is performed in Xn Setup and NG-RAN node Configuration Update procedure.

Proposal 3: RAN3 is asked to wait for RAN2’s determination on the introduction of periodic NZP CSI-RS resources configuration, before discussing whether a new IE needs to be added in Served Cell Information NR IE for exchange of NZP CSI-RS over Xn.

Proposal 4: Both the strongest DL beam information and CLI-mitigation mitigation request are two optional IEs in a same message.

Proposal 5: RAN3 is asked to introduce a new Class 2 procedure for report of strongest DL beam and CLI-mitigation request.

References

- Agenda, RAN3#126, Orlando, USA.

- Agenda, RAN3#127, Athens, Greece.