3GPP TSG-RAN WG3 Meeting #127bis														R3-252173

Wuhan, China, Apr 7 - 11, 2025

Agenda item:			11.2

Source:					CMCC

Title:						Discussion on AI-based network slice

Document Type:		Other

Introduction

During RAN3#127, the following agreements and FFS have been captured for AI/ML enabled slicing use case.

Introduce Slice Measurement Initiation Result IE to indicate the predicted radio resource status per slice or predicted slice available capacity cannot be initiated.

UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback. Other granularity of UE performance are not excluded and to be continued based on cases

Whether to introduce a new IE or extending existing bitmap to request slice UE performance in the Data Collection Request message? Taking the R18 signalling design as the starting point? How to include the failed report characteristic in the Data Collection Response message?

In this paper, we provide our consideration of AI/ML based Network Slicing.

Discussion

Slice-level UE performance

In RAN3#127, RAN3 has discussed how to introduce the slice UE performance in Data Collection Request message.

The following options are captured in the offline summary:

Option 1: To introduce a new IE to request slice UE performance in the DATA COLLECTION REQUEST message.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond | - |  |
| Slice Level UE Performance Reporting | O |  | ENUMERATED (true, …) | Indicates that UE Performance measurements are collected on a per slice granularity. | YES | ignore |

Option 2: Extending existing bitmap to request slice UE performance in the DATA COLLECTION REQUEST message:

| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Predicted Slice Available Capacity Eleventh Bit = Slice Average UE Throughput DL Twelfth Bit = Slice Average UE Throughput UL Thirteenth Bit = Slice Average Packet Delay, Fourteenth Bit = Slice Average Packet Loss DL Other bits are ignored by the NG-RAN node2. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

From our perspective, as R18 introduced UE performance with 4 bit in Report Characteristics for Data Collection in Data Collection Request message, R19 proposes to follow the R18 signalling design that extending existing bitmap for slice UE performance. So, we slightly prefer option 2, as it’s a straightforward way to configure the UE performance or slice level UE performance. And also with option 2, the local node can configure the neighbour node with different UE performance and slice UE performance. For example, with option 2, the local node can request the neighbour nodes to feedback average UE Throughput DL/UL, and slice level average UE Throughput DL/UL, slice level average Packet Delay and slice level average Packet Loss DL. But with option 1, the local node can only request the same UE performance and slice UE performance. Take above into account, extending the existing bitmap with 4bit slice UE performance maybe more flexible for feedback configuration.

Observation 1: Extending existing bitmap is a straightforward way to configure the UE performance and/or slice UE performance. The local node can flexible configure the neighbour nodes with different UE performance and slice UE performance with this option.

Proposal 1: Extending existing bitmap of Report Characteristics for Data Collection IE with 4 bit for slice UE performance in Data Collection Request message.

Predicted slice available capacity

Whether predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity?

In RAN3#127 online discussion, RAN3 has discussed whether to introduce predicted composite available capacity or Cell Capacity Class Value together with predicted slice available capacity.

In legacy, Composite Available Capacity Group IE is included in RESOURCE STATUS REQUEST message to indicate the overall available resource level per cell and per SSB area in the cell in Downlink, Uplink and Supplementary Uplink. The definition of Composite Available Capacity IE and Cell Capacity Class Value IE are as below:

    - 9.2.2.52	Composite Available Capacity

The Composite Available Capacity IE indicates the overall available resource level in the cell in either Downlink or Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Capacity Class Value | O |  | 9.2.2.53 |  |
| Capacity Value | M |  | 9.2.2.54 | ‘0’ indicates no resource is available, Measured on a linear scale. |

    - 9.2.2.53	Cell Capacity Class Value

The Cell Capacity Class Value IE indicates the value that classifies the cell capacity with regards to the other cells. The Cell Capacity Class Value IE only indicates resources that are configured for traffic purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Capacity Class Value | M |  | INTEGER (1..100,...) | Value 1 indicates the minimum cell capacity, and 100 indicates the maximum cell capacity. There should be a linear relation between cell capacity and Cell Capacity Class Value. |

    - 9.2.2.54	Capacity Value

The Capacity Value IE indicates the amount of resources per cell and per SSB area that are available relative to the total NG-RAN resources. The capacity value should be measured and reported so that the minimum NG-RAN resource usage of existing services is reserved according to implementation. The Capacity Value IE can be weighted according to the ratio of cell capacity class values, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Capacity Value | M |  | INTEGER (0..100) | Value 0 indicates no available capacity, and 100 indicates maximum available capacity with respect to the whole cell. Capacity Value should be measured on a linear scale. |
| SSB Area Capacity Value List |  | 0..1 |  |  |
| >SSB Area Capacity Value Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>SSB Index | M |  | INTEGER (0..63) |  |
| >>SSB Area Capacity Value | M |  | INTEGER (0..100) | Value 0 indicates no available capacity, and 100 indicates maximum available capacity . SSB Area Capacity Value should be measured on a linear scale. |

In legacy, the RESOURCE STATUS UPDATE message includes the Slice Available Capacity IE if the cell is requested to be reported supports more than one slice, and the Slice To Report List IE is included for the cell. The Slice Available Capacity IE indicates the amount of resources per network slice that are available per cell relative to the total NG-RAN resources per cell.

Then, from the description of the Composite Available Capacity IE, Cell Capacity Class Value IE and Slice Available Capacity IE, the Slice Available Capacity IE is the available resource per slice, which may change over time. The Composite Available Capacity IE including the Cell Capacity Class Value IE and Capacity Value IE. The Cell Capacity Class Value IE is the cell capacity with regards to other cells, which seems to be a relative static value with limits prediction usage. The Capacity Value IE indicates the available resource per cell or per SSB, which is not the slice level information, and we propose not to include this information for AI/ML enabled slicing. So, it’s proposed that only introduce the predicted slice available capacity. For the Composite Available Capacity or the Cell Capacity Class Value, the measured value is enough and can be obtained via legacy procedure.

Observation 2: The Cell Capacity Class Value IE is the cell capacity with regards to other cells, which seems to be a relative static value with limits prediction usage.

Observation 3: The Composite Available Capacity IE including the Cell Capacity Class Value IE and Capacity Value IE. Although the Capacity Value IE indicates the available resource, it’s the available resource per cell or per SSB, not the slice level information.

Proposal 2: RAN3 agrees that only introduce the predicted slice available capacity, the Composite Available Capacity or Cell Capacity Class Value can be obtained via legacy procedure.

Finer granularity UE performance feedback

UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback. Other granularity of UE performance are not excluded and to be continued based on cases

In RAN3#127, whether finer granularity of UE performance is needed for slice use case is left as FFS.

From the previous discussion, except the slice UE performance, two options are discussed:

Opt1: per S-NSSAI per PDU Session

Opt2: per-QoS flow groups/per DRB level

From our perspective, the PDU session level UE performance is not defined in SA5 and only two meetings are left for R19 WI, to avoid confusing how to calculate the PDU session level UE performance, it’s proposed not to introduce this granularity in R19. Then, for per-QoS flow groups/per DRB level, the remapping issue between DRB and QoS flows in source and target node need to be considered. So, as for the UE performance feedback of AI/ML enabled slicing, only introduce slice UE performance seems more simple.

Proposal 3: Only introduce the slice UE performance for AI/ML enabled slicing in R19.

Conclusion

In this contribution, we propose the following proposals:

Observation 1: Extending existing bitmap is a straightforward way to configure the UE performance and/or slice UE performance. The local node can flexible configure the neighbour nodes with different UE performance and slice UE performance with this option.

Proposal 1: Extending existing bitmap of Report Characteristics for Data Collection IE with 4 bit for slice UE performance in Data Collection Request message.

Observation 2: The Cell Capacity Class Value IE is the cell capacity with regards to other cells, which seems to be a relative static value with limits prediction usage.

Observation 3: The Composite Available Capacity IE including the Cell Capacity Class Value IE and Capacity Value IE. Although the Capacity Value IE indicates the available resource, it’s the available resource per cell or per SSB, not the slice level information.

Proposal 2: RAN3 agrees that only introduce the predicted slice available capacity, the Composite Available Capacity or Cell Capacity Class Value can be obtained via legacy procedure.

Proposal 3: Only introduce the slice UE performance for AI/ML enabled slicing in R19.

- References

[1] RP-242385, “New WID: Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, ZTE, NEC

- Annex: TP for 38.423

    - 9.1.3.26	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Predicted Slice Available Capacity Eleventh Bit = Slice Average UE Throughput DL Twelfth Bit = Slice Average UE Throughput UL Thirteenth Bit = Slice Average Packet Delay, Fourteenth Bit = Slice Average Packet Loss DL Other bits are ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Slice To Report List for Data Collection |  | 0..1 |  | S-NSSAI list to which the request applies. | YES | ignore |
| >>>Slice To Report Item for Data Collection |  | 1 .. < maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Trajectory Collection Configuration | O |  | 9.2.3.185 |  | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |