3GPP TSG-RAN WG3 Meeting #127bis	             R3-252183

Wuhan, CN, Apr 7th- Apr 11th, 2025

Agenda Item:	16.2

Source:	CMCC

Title:	(TP to TS 38.300) Discussion on RAN Architecture for Ambient IoT

Document for:	Other

# 1	Introduction

In RAN3#127 meeting, the discussion on RAN architecture aspects for Ambient IoT has made following agreements and remaining FFSs.

Architecture aspects: RAN3 agrees to support both the direct and indirect options for the A-IoT RAN node to communicate with AIoTF in Topology 1. WA: Take Option1 (Including AIoTF information in NGAP). AIoTF obtain the AIoT RAN information via OAM configuration. The AIoT RAN information includes supported AIoT area, served reader ID list. AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN. How to encode the requested service area information is FFS. AIoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio. AIOTF may be aware of the location of reader via OAM configuration. FFS: The details of the location of reader (e.g., room number, GNSS info) are FFS, and it is FFS on signaling support. FFS: To support selection of RAN node by AIOTF, define a new A-IoT Area, or use Tracking Area dedicated for A-IoT? Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area. Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers. WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list. FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list. About Interface management procedures: FFS: In indirect path communication, existing NG interface management procedures are used between the gNB and the AMF. FFS: In direct path communication, existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used between the gNB and the AIOTF. FFS: For interface management procedures, if reuse existing ones, how to handle the not applicable mandatory IEs, and how to handle A-IoT only support case.

In this contribution, we will provide our views on RAN architecture aspects for Ambient IoT, interface management and the A-IoT reader selection mechanism.

# 2	Discussion

### 2.1	A-IoT Architecture

During last meeting, RAN3 conducted thorough discussions on how to realize XXAP on which certain AIOTF information is included. Majority of companies support including AIOTF information in NGAP, so we made a corresponding WA. From our view, there is no essential difference between two options, and the same handling of the AIOTF containers by the gNB and AIOTF are foreseen in both options. In the indirect deployment, the AIOTF related information IEs in both options should be transparent to the AMF. It is proposed to support including AIOTF information in NGAP and turn this WA to agreement.

Observation 1: There is no essential difference between including AIOTF information in NGAP and carrying a new protocol layer over NGAP, and the same handling of the AIOTF containers by the gNB and AIOTF are foreseen in both options.

Proposal 1: Support including AIOTF information in NGAP and turn this WA to agreement.

### 2.2	Interface Management

Regarding the interface management, we need to consider both direct path deployment and indirect path deployment. In direct path deployment, gNB connects with AIOTF directly via NG interface. While in indirect path deployment, gNB connects with AIOTF indirectly via AMF, and the NG interface is between gNB and AMF. From our view, the existing NG interface management procedures can be used and enhanced to support the interface management for A-IoT services in both deployments.

Proposal 2: The existing NG interface management procedures can be used and enhanced to support the interface management for A-IoT services in both deployments.

In indirect path deployment, existing NG interface management procedures are reused and enhanced between the gNB and the AMF. In direct path deployment, excluding the AMF related NG interface management procedures, the NG interface management procedure including NG Setup, RAN Configuration Update, NG Reset and Error Indication procedure can be reused and enhanced between the gNB and the AIOTF.

Proposal 3: In indirect path deployment, existing NG interface management procedures are reused and enhanced between the gNB and the AMF.

Proposal 4: In direct path deployment, the NG interface management procedure including NG Setup, RAN Configuration Update, NG Reset and Error Indication procedure can be reused and enhanced between the gNB and the AIOTF.

Regarding the NGAP enhancement, as some mandatory IEs using in NR may not be suitable for A-IoT services. To mitigate backward compatibility risks induced by large-scale modifications to the presence of mandatory IEs, an optional A-IoT indicator IE should be introduced in the existing NG interface management procedure. The presence of IE signifies the NG-RAN node provides the A-IoT only support service and inapplicable mandatory IEs using in NR should be ignored.

Proposal 5: An optional A-IoT indicator IE should be introduced in the existing NG interface management procedure.

Proposal 6: The presence of A-IoT indicator IE signifies the NG-RAN node provides the A-IoT only support service and inapplicable mandatory IEs using in NR should be ignored.

### 2.3 A-IoT Reader Selection

In last meeting, RAN3 held extensive discussions regarding the A-IoT reader selection mechanism. Meanwhile, considering the different information carried in Inventory Request message, several cases for reader selection are summarized as follows:

Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area. Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers. WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list. FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

The WA assumes that the gNB only receives the reader list in Inventory Request, and it selects the readers indicated by the reader list. From our view, we think the gNB does not have rights to select the readers indicated by the reader list, and it only follow the guidance from A-IoT CN. The reader list provided by the A-IoT CN is built upon the mapping between the real geographic area and the reader location maintained by the A-IoT CN. The principle for A-IoT CN providing readers is to ensure that readers can perfectly cover the entire service area. If the gNB selects readers from the list, there is no guarantee mechanism to ensure coverage of the entire area.

Observation 2: The principle for A-IoT CN providing readers is to ensure that readers can perfectly cover the entire inventory area. If the gNB selects readers from the list, there is no guarantee mechanism to ensure coverage of the entire area.

Proposal 7: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

Regarding another case where the gNB receives both the area and the reader list in Inventory Request, we think it cannot be happened from the perspective of A-IoT CN. The A-IoT CN does not provide both information at the same time, since it will make the gNB confused if the area and the reader list is conflicting.

Proposal 8: The A-IoT CN does not provide both the area and the reader list in Inventory Request at the same time.

Considered the location of reader, we already agreed the AIOTF obtains the AIoT RAN information via OAM configuration, and FFS on signalling support. From our view, the reader location is relative stable and the OAM configuration is enough to provide the location of the reader to the AIOTF. We do not see the necessity to use NG signalling to transfer reader location between A-IoTF and gNB. The details of the location of reader can be GNSS information or refer to the existing TRP Position and TRP High Accuracy Access Position defined in TS 38.455 shown below.

Proposal 9: RAN3 does not support to use NG signalling to transfer reader location between A-IoTF and gNB.

Proposal 10: The details of the location of reader can be GNSS information or refer to the existing TRP Position and TRP High Accuracy Access Position defined in TS 38.455.

Another left over issue in last meeting is how to encode the Area information within service area information. The Area information would be uniformly designed for the network to enable the reader to perform A-IoT service effectively, which can be compared to the concept of tracking area. To distinguish the concept from the legacy tracking area, it is suggested to introduce the concept of A-IoT area which is identified by an A-IoT Area Identity. To facilitate management, each reader will only serve one A-IoT area. The AIOTF would be preconfigured with the supported A-IoT area of a gNB and the mapping between A-IoT area and geographical area information by the OAM. Once receiving the external geographical area from the AF, the AIOTF can turn the geographical information into requested service area which is composed of A-IoT areas. Then, the AIOTF can select suitable gNBs to perform A-IoT service operation based on the supported A-IoT area within a gNB.

Proposal 11: It is proposed to introduce the concept of A-IoT area which is identified by an A-IoT Area Identity.

# 3	Conclusion

Based on the discussions mentioned above, in this contribution we provide following observations and proposals on RAN architecture aspects for Ambient IoT:

Observation 1: There is no essential difference between including AIOTF information in NGAP and carrying a new protocol layer over NGAP, and the same handling of the AIOTF containers by the gNB and AIOTF are foreseen in both options.

Observation 2: The principle for A-IoT CN providing readers is to ensure that readers can perfectly cover the entire inventory area. If the gNB selects readers from the list, there is no guarantee mechanism to ensure coverage of the entire area.

Proposal 1: Support including AIOTF information in NGAP and turn this WA to agreement.

Proposal 2: The existing NG interface management procedures can be used and enhanced to support the interface management for A-IoT services in both deployments.

Proposal 3: In indirect path deployment, existing NG interface management procedures are reused and enhanced between the gNB and the AMF.

Proposal 4: In direct path deployment, the NG interface management procedure including NG Setup, RAN Configuration Update, NG Reset and Error Indication procedure can be reused and enhanced between the gNB and the AIOTF.

Proposal 5: An optional A-IoT indicator IE should be introduced in the existing NG interface management procedure.

Proposal 6: The presence of A-IoT indicator IE signifies the NG-RAN node provides the A-IoT only support service and inapplicable mandatory IEs using in NR should be ignored.

Proposal 7: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

Proposal 8: The A-IoT CN does not provide both the area and the reader list in Inventory Request at the same time.

Proposal 9: RAN3 does not support to use NG signalling to transfer reader location between A-IoTF and gNB.

Proposal 10: The details of the location of reader can be GNSS information or refer to the existing TRP Position and TRP High Accuracy Access Position defined in TS 38.455.

Proposal 11: It is proposed to introduce the concept of A-IoT area which is identified by an A-IoT Area Identity.

# 4	Reference

[1] 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things;

[2] 3GPP TR 38.769 V19.0.0, Study on solutions for Ambient IoT (Internet of Things) in NR;

[3] 3GPP TS 23.xyz-020, Architecture support for Ambient power-enabled Internet of Things;

# 5	Text Proposal to TS 38.300 BL CR

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

## 16.xx	Support of Ambient IoT

### 16.xx.x1	Introduction

### 16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or more readers.

-	As specified in <reference to SA2 TS to be added> the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct communication to AIOTF) or the AMF (in case of indirect communication to AIOTF).

NOTE 1:	It is not expected a deployment will deploy both direct communication and indirect communication.

-	In this version of the specification A-IoT specific data is transported between the gNB and the A-IoT CN node takes place via the NG-C interface.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Editor’s Note 3:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request includes the Inventory Request Transfer IE concerning one or multiple A-IoT devices, the Correlation Identifier and AIOTF Identifier. The Inventory Request Transfer IE includes A-IoT Device Identification information, the requested service area and, if available, some assistance information provided by the A-IoT CN, i.e., approximate number of AIoT devices and approximate D2R message size.

Editor’s Note 4:	Specification of further parameterisation of the Inventory Request message (e.g., assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Reports may be sent to the A-IoT CN node.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices,

### 16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Editor’s Note 5:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.xx.x4-1: Command procedure

1.	Prior to the Command procedure, the Inventory procedure is performed for the concerned devices.

Editor’s Note 6:	Further details on the relation and correlation between the Inventory procedure and the Command procedure needs further discussion may need input from SA2/SA3/RAN2.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB, each Command Request message only applies to a single device. In case the command operation applies to multiple devices, the A-IoT CN node should initiate multiple Command procedures over NG-C for each device.

The Command Request message includes the Command Request Transfer IE concerning one device, the Device NGAP ID association and AIOTF Identifier. The Command Request Transfer IE includes some assistance information provided by the A-IoT CN, i.e., approximate D2R message size.

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The gNB replies to the Command Request with the Command Response message.

NOTE 4:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>