3GPP TSG-RAN WG3 Meeting #127bis	R3-252193

Wuhan, CN, 7th April– 11st April 2025

Agenda Item:       10.2

Source:	ZTE Corporation

Title: (TP for SON BLCR 38.300 38.40138.413 and 38.473) LTM Ping-Pong Avoidance

Document for:	Discussion and Approval

1	Introduction

RAN3 has discussed LTM ping-pong avoidance in subsequent LTM procedures. The progress from the latest meeting is provided below:

CU can adjust the candidate Cell list for LTM based on UHI.

DU should be able to resolve LTM ping-pong based on information over F1.

CU sends UHI to target DU (always or only when ping pong is detected)? CU sends an indication (new IE in existing message, new message, …) to DU(s) that LTM ping-pong has been detected?

Include HO type to UHI entries?

CU filters out L3 HOs from the UHI sent to DU?

Other additional information in UHI?

In this contribution, we discuss the the network solution for LTM ping pong avoidance, and provide the corresponding TPs in the Annex.

2	Discussion

According to current TS38.300, see below:

TS38.300 The source NG-RAN node collects and stores the UE History Information for as long as the UE stays in one of its cells. The resulting information is then used in subsequent handover preparations by means of the Handover Preparation procedures over the NG and XN interfaces, which provide the target NG-RAN node with a list of previously visited cells and associated (per-cell) information elements. The Handover Preparation procedures also trigger the target NG-RAN node to start collection and storage of UE history Information and thus to propagate the collected information.

Observation 1 : In legacy L3 HO, the UHI is always sent to the target node for two purposes:

- To enable real-time optimization of future handovers at the target node , i.e., the target cell selection.

- To trigger the target node to begin collecting UHI and propagate it to the next node.

Since the target cell is always aware of the UHI in legacy handovers, following the same principle, it is essential to provide UHI to the target DU during each LTM cell switch. This ensures that the target DU has access to relevant historical mobility data, enabling it to make more better real-time decisions for future LTM cell switch, not only to prevent ping pong but also to optimize target cell selection.

Furthermore, as local AI is expected to play a significant role in 5GA and 6G, UHI will be crucial for improving AI/ML based predictions in selecting the optimal target cell. Restricting the provision of UHI to the target DU would limit its ability to leverage AI-driven optimizations.

Proposal 1: Following the legacy handover (HO) principle, the gNB-CU should always send the UHI to the target DU to facilitate real-time optimization of future LTM cell switch at the target DU. This ensures not only the prevention of ping-pong but also the selection of the most suitable target cell.

Based on the above inter-gNB-DU LTM procedure, we propose including UHI in Step 20. When the CU sends the CU-DU cell switch notification to the target DU, it may carry the UHI to enable the target DU to perform real-time optimization for subsequent LTM cell switch, after a successful LTM cell switch.

Proposal2 : the gNB-CU sends the UHI to the target gNB-DU via the CU-DU CELL SWITCH NOTIFICATION.

Consider a UE moving through the following sequence of cells (Cell 1 is the initial accessing cell):

| Step | Source cell | Target Cell | Handover type |
| --- | --- | --- | --- |
| 1 | cell 1 | cell 2 | L3 HO |
| 2 | cell 2 | cell 3 | L3 HO |
| 3 | cell 3 | cell 4 | LTM |

Original UHI  is (Cell1, cell2, cell3, cell4) without filtering, can be interpreted as following cell handover sequence:

(Cell 1 → Cell 2)

(Cell 2 → Cell 3)

(Cell 3 → Cell 4)

Now, if L3 HO events are filtered out and only stayed cell caused LTM cell switch in are retained, the filtered UHI (Cell1, cell4) would be interpreted as following:

(Cell 1 → Cell 4)

Clearly, the filtered UHI creates a gap, leading to incorrect pairing of the source and target cells during the handover process.

Proposal 3: Filtering out L3 HOs from the UHI creates gaps in the UHI, leading to misinterpretation of source-target cell pairs. It is proposed to include a handover type (indicating the type of cell handover in) in UHI entries while keeping the UHI is not filtered.

Based on above discussion, we provide stage 2  TP for TS38.300,TS38.401, and stage3 TP for TS38.413 and TS38.473 In annex.

Proposal 4: RAN3 to agree  the Stage 2 TP for TS38.300 and TS38.401, and the Stage 3 TP for TS38.413 and TS38.473 in the annex.

3	Conclusion

Observation 1 : In legacy L3 HO, the UHI is always sent to the target node for two purposes:

Observation 1 : In legacy L3 HO, the UHI is always sent to the target node for two purposes:

- To enable real-time optimization of future handovers at the target node , i.e., the target cell selection.

- To trigger the target node to begin collecting UHI and propagate it to the next node.

Proposal 1: Following the legacy handover (HO) principle, the gNB-CU should always send the UHI to the target DU to facilitate real-time optimization of future LTM cell switch at the target DU. This ensures not only the prevention of ping-pong but also the selection of the most suitable target cell.

Proposal2 : the gNB-CU sends the UHI to the target gNB-DU via the CU-DU CELL SWITCH NOTIFICATION.

Proposal 3: Filtering out L3 HOs from the UHI creates gaps in the UHI, leading to misinterpretation of source-target cell pairs. It is proposed to include a handover type (indicating the type of cell handover in) in UHI entries while keeping the UHI is not filtered.

Proposal 4: RAN3 to agree  the Stage 2 TP for TS38.300 and TS38.401, and the Stage 3 TP for TS38.413 and TS38.473 in the annex.

Annex A: TP to TS38.401

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell.

The gNB-CU may signal UE history information to the target DU for the future LTM cell switch preparations, when notifying the target DU about the current initiation of the cell switch command to the UE.

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>

Annex B: TP to TS38.300

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

15.5.4	UE History Information from the UE

The source NG-RAN node collects and stores the UE History Information for as long as the UE stays in one of its cells.

The UE may report the UE history information when connecting to a cell of the NG-RAN node, consisting of PCell and PSCell mobility history information, as specified in TS 38.331 [12].

When information needs to be discarded because the list is full, such information will be discarded in order of its position in the list, starting with the oldest cell record. If the list is full, and the UE history information from the UE is available, the UE history information from the UE should also be discarded.

The resulting information is then used in subsequent handover preparations by means of the Handover Preparation procedures over the NG and XN interfaces, which provide the target NG-RAN node with a list of previously visited cells and associated (per-cell) information elements. The Handover Preparation procedures also trigger the target NG-RAN node to start collection and storage of UE history Information and thus to propagate the collected information.

In case of CHO, the target NG-RAN node updates the time UE stayed in cell of the latest PCell entry (i.e. the source cell) when the UE successfully accesses a candidate cell of the target NG-RAN node. The updated value of the time UE stayed in the source cell is equal to the value received from the source NG-RAN node during the Handover Preparation plus the time from receiving the Handover Request message from the source NG-RAN node to receiving the RRC Reconfiguration Complete message from the UE. When the target NG-RAN node receives the SCG UHI from the source NG-RAN node via the Handover Request message, the target NG-RAN node also updates the time UE stayed in cell of the latest PSCell entry (i.e. the source PSCell) as specified in TS 37.340 [21].

In case of LTM cell switch, The gNB-CU collects and stores the UE History Information for as long as the UE stays in one of its cells. The resulting information is then used for future LTM cell switch preparations by means of the CU-DU CELL SWITCH NOTIFICATION procedures over F1 interface,  which provide to the target gNB-DU.

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>

Annex C: TP to TS38.473

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

8.3.10	CU-DU Cell Switch Notification

8.3.10.1	General

The purpose of the CU-DU Cell Switch Notification procedure is to enable the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-CU to the gNB-DU. The procedure uses UE-associated signalling.

8.3.10.2	Successful Operation

Figure 8.3.10.2-1: CU-DU Cell Switch Notification procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU CELL SWITCH NOTIFICATION message.

Upon reception of the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

If the LTM Cell Switch Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

If the TA Information List IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the Tag ID Pointer IE is also included, the gNB-DU shall, if supported, consider it to determine the TAG corresponding to the received TA value.

If the UE History Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, store the collected information and use it for future LTM cell switch preparation.

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGES >>>>>>>>>>>>>>>>>>>>

9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| UE History Information | O |  | OCTET STRING | UE History Information, as same definition in TS 38.413 [3]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

Annex D: TP to TS38.413

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Cell Type | M |  | 9.3.1.98 |  | - |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | - |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. | - |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.3.1.235 | The PSCell related information. | - |  |
| HO Type | O |  | ENUMERATED ( LTM, …) | The handover type for UE accessing the indicated stayed cell. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PSCell information records that can be reported in the IE. Value is 8. |

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>