3GPP TSG-RAN WG3 Meeting #R3-127-bis                                                                R3-252210

Wuhan, China, Apr 07-11, 2025

Agenda Item:	17.3

Source: 	Rakuten

Title:  	Signalling to support OD-SIB1

Document for:	Discussion and decision

# Introduction

The Rel-19 WI on enhancements of network energy savings for NR (RP-234065), with the objective to study on-demand SIB1 for idle/inactive UEs as highlighted below:

Study procedures and signalling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] Triggering method by uplink wake-up-signal using an existing signal/channel. Wake-up-signal configuration provisioning to UE Note: No modification of SSB will be discussed under this objective Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. Checkpoint for normative work in RAN#105

# NES activation at the NES cell

The NES cell is a cell that may transmit On Demand SIB1 (OD-SIB1) transmission in response to UL WUS from a UE. It need not transmit periodic SIB1 (default transmission) all the time.

When the NES Cell gNB-DU decides to transition to the NES mode, it sends a NES Activation Request to the NES Cell gNB-DU over F1AP. This request carries the NES Cell ID and the corresponding UL WUS configuration.

On receiving the request, the NES Cell gNB-CU sends a UL WUS Config Transmission Request to Cell A (as proposed in R3-247426), and it forwards the response received from Cell A to the NES Cell gNB-DU. If the NES Cell gNB-CU has received a positive acknowledgement from Cell A, then NES Cell gNB-DU can proceed with transitioning to the NES mode. Similarly, if the NES Cell gNB-CU has received a failure response from Cell A, then NES Cell gNB-DU is informed of the same by NES Cell gNB-CU. And in that case, the NES Cell gNB-DU does not transition to the NES mode. This is applicable when there is only one Cell A for the NES Cell.

NES Cell gNB-DU decides to transition to the NES mode, and it informs the NES Cell gNB-CU along with NES Cell ID and the corresponding UL WUS Configuration.

When Cell A sends a positive acknowledgement to support the NES mode of the NES cell, then the call flow will be:

When Cell A sends a failure message to support the NES mode of the NES cell, then the call flow will be:

# Support for multiple Cell As for a NES Cell and multiple NES Cells for Cell A

It has been agreed in the previous meetings of RAN1/2, that a single NES Cells can have multiple Cell As and a single Cell A can have multiple NES cells. With this agreement in place, there is a need for NES cell gNB-CU to know the list of Cell As corresponding to the NES cell. And similarly, there is a need for Cell A gNB-CU to have the list of NES cells, their corresponding UL WUS Configurations and their current NES mode status.

We propose that the OAM configures to the NES gNB-CU a list of Cell As corresponding to the NES Cell. Furthermore, NES Cell gNB-CU maintains this list along with information about the current support for the NES mode of the NES cell.

We also propose that the OAM configures to the Cell A gNB-CU a list of NES cells for which is acts as Cell A. Furthermore, Cell A gNB-CU maintains this list along with information about the corresponding UL WUS configurations and the current NES mode status.

Observation 1: A single NES Cell can have multiple Cell As associated with it and a single Cell A can have multiple NES cells associated with it.

A NES Cell gNB-CU needs to be configured with a list of all Cell As associated with the NES Cell and the Cell A gNB-CU needs to be configured with a list of all NES cells associated with it.

The behavior of the NES mode activation of the NES cell remains the same as discussed in the above section, if all the Cell As of the NES cell respond similarly to the request of transmitting UL WUS Configuration of the NES Cell.

We propose that when the NES Cell decides to transition to the NES mode, the NES Cell gNB-CU sends the UL WUS Config Transmission Request to all the Cell As that are corresponding to the NES Cell. Then the NES Cell can transition to the NES mode when at least one of the Cell As corresponding to it has responded with a positive acknowledgement. We further propose that it is the responsibility of NES Cell gNB-CU to monitor if there is support from at least one Cell A for the NES mode of the NES Cell

We present the call flow of the outcome when at least one Cell A responds to the UL WUS Config Transmission Request with a positive acknowledgement and at least one Cell A responds to the UL WUS Config Transmission Request with a failure message.

We propose that when the NES Cell decides to transition to the NES mode, the NES Cell gNB-CU sends the UL WUS Config Transmission Request to all the Cell As that correspond to the NES Cell. The NES Cell can transition to the NES mode if at least one of the Cell As corresponding to it has responded with a positive acknowledgement.

We propose that it is the responsibility of the NES Cell gNB-CU to monitor if there is support from at least one Cell A for the NES mode of the NES Cell

We present the call flow of the outcome when at least one Cell A sends a UL WUS Config Transmission Cancel procedure to the NES Cell gNB-CU with the intent to stop broadcasting UL WUS Configuration of the NES cell. We propose that when NES Cell gNB-CU receives a UL WUS Config Transmission Cancel, it checks whether any Cell A currently supports the NES mode of the NES cell. If all Cell As of the NES Cell have either sent a failure response for request to transmit the UL WUS configuration, or sent the UL WUS Config Transmission Cancel message, then the NES Cell gNB-CU can deduce that there is no support for NES Cell to continue to operate in the NES mode. In this case, it sends a NES mode deactivation message to the NES Cell gNB-DU.

We propose that when all the Cell As corresponding to a NES cell either send a UL WUS Config Transmission failure response to the request for UL WUS Config transmission, or decide to stop broadcasting UL WUS Configuration of the NES Cell and send UL WUS Config Transmission Cancel message to the NES Cell gNB-CU, then the NES gNB-CU sends a Deactivate NES Mode message over F1AP to the NES Cell gNB-DU

Therefore, when even one Cell A continues to support the NES mode of the NES Cell by transmitting UL WUS configuration of the NES cell to its UEs, then the NES Cell can continue to operate in the NES mode. It is the responsibility of the NES Cell gNB-CU to keep track of the support from all the Cell As corresponding to the NES Cell.

# Change in NES Cell UL WUS Configuration

If there is any change in the UL WUS configuration at the NES Cell, then NES Cell gNB-DU informs the NES Cell gNB-CU of the update over F1 interface. The updated UL WUS configuration is then sent to Cell A gNB-CU over the Xn interface using UL WUS Configuration Update Notification which is a class 2 procedure. The NES Cell gNB-CU sends the UL WUS Configuration Update procedure to all the Cell As from which it has previously received a positive acknowledgment to support the NES mode of the NES cell, and not received any cancelation of transmission of UL WUS configuration.

If there is any change in the UL WUS configuration at the NES Cell, then NES Cell gNB-DU informs the NES Cell gNB-CU of the update over F1 interface. The updated UL WUS configuration is then sent to Cell A gNB-CU over the Xn interface.

# Conclusion

We have the following observations and proposals:

Proposal 1	NES Cell gNB-DU decides to transition to the NES mode, and it informs the NES Cell gNB-CU along with NES Cell ID and the corresponding UL WUS Configuration.

Observation 1: A single NES Cell can have multiple Cell As associated with it and a single Cell A can have multiple NES cells associated with it.

Proposal 2	A NES Cell gNB-CU needs to be configured with a list of all Cell As associated with the NES Cell and the Cell A gNB-CU needs to be configured with a list of all NES cells associated with it.

Proposal 3	We propose that when the NES Cell decides to transition to the NES mode, the NES Cell gNB-CU sends the UL WUS Config Transmission Request to all the Cell As that correspond to the NES Cell. The NES Cell can transition to the NES mode if at least one of the Cell As corresponding to it has responded with a positive acknowledgement.

Proposal 4	We propose that it is the responsibility of the NES Cell gNB-CU to monitor if there is support from at least one Cell A for the NES mode of the NES Cell

Proposal 5	We propose that when all the Cell As corresponding to a NES cell either send a UL WUS Config Transmission failure response to the request for UL WUS Config transmission, or decide to stop broadcasting UL WUS Configuration of the NES Cell and send UL WUS Config Transmission Cancel message to the NES Cell gNB-CU, then the NES gNB-CU sends a Deactivate NES Mode message over F1AP to the NES Cell gNB-DU

Proposal 6	If there is any change in the UL WUS configuration at the NES Cell, then NES Cell gNB-DU informs the NES Cell gNB-CU of the update over F1 interface. The updated UL WUS configuration is then sent to Cell A gNB-CU over the Xn interface.

# Reference

Chairman’s Notes RAN3#125

Chairman’s Notes RAN3#125-bis