3GPP TSG-RAN WG3 #127-bis					 				                                  R3-252241

Wuhan, China, 7-11 April 2025

Title: 	(TP for BL CR to 38.300, 38.413, 38.410) Leftover issues on AIoT

Source: 	ZTE Corporation, China Telecom

Agenda item:	16.2

Document for:	Discussion and Decision

- Introduction

In the RAN3 #127 meeting, we have achieved the following progress.

RAN3 agrees to support both the direct and indirect options for the A-IoT RAN node to communicate with AIoTF in Topology 1.

WA: Take Option1 (Including AIoTF information in NGAP).

WA: The AIoT RAN node may only provide the AIoT radio.

AIoTF obtain the AIoT RAN information via OAM configuration. The AIoT RAN information includes supported AIoT area, served reader ID list.

The location of reader is FFS.

FFS on whether signaling is needed, further discussion on the detail information and necessity.

AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN.

The behavior on the AIoT RAN on how to select the proper readers needs to be further discussed?

Architecture aspects:

- AIoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

- AIOTF may be aware of the location of reader via OAM configuration.

- FFS: The details of the location of reader (e.g., room number, GNSS info) are FFS, and it is FFS on signaling support.

- FFS: To support selection of RAN node by AIOTF, define a new A-IoT Area, or use Tracking Area dedicated for A-IoT?

- Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area.

- Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers.

- WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

- FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

About Inventory Procedures:

- WA: Introduce a NGAP Class 1 Inventory Request procedure.

- Introduce INVENTORY REQUEST message, which includes Inventory Request Transfer IE.

- Introduce INVENTORY RESPONSE message, which includes Inventory Response Transfer IE.

- Introduce INVENTORY FAILURE messages, which includes Inventory Failure Transfer IE.

- Introduce a NGAP Class 2 Inventory Report procedure.

- Introduce INVENTORY REPORT message, which includes Inventory Report Transfer IE.

- The above Transfer IEs are transparent to AMF in case of indirect communication.

- FFS: For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

About Command Procedure:

- WA: Introduce a NGAP Class 1 Command Request procedure

- Introduce COMMAND REQUEST message, which includes Command Request Transfer IE.

- Introduce COMMAND RESPONSE message, which includes Command Response Transfer IE.

- Introduce COMMAND FAILURE messages, which includes Command Failure Transfer IE.

- The above Transfer IEs are transparent to AMF in case of indirect communication.

- FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

- FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure?

- FFS: Command request procedure is a per device procedure or to be used for multiple devices?

- FFS: The need for a Command Report procedure?

- FFS: Device association between gNB and AIOTF is used in Command procedure.

About Interface management procedures:

- FFS: In indirect path communication, existing NG interface management procedures are used between the gNB and the AMF.

- FFS: In direct path communication, existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used between the gNB and the AIOTF.

- FFS: For interface management procedures, if reuse existing ones, how to handle the not applicable mandatory IEs, and how to handle A-IoT only support case.

The A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report.

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF in Command Report?

AIoT RAN node sends the Inventory Report aggregated per Reader.

“Global gNB ID + Reader index” is used to uniquely identify the Reader globally.

FFS whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report.

In the RAN2 #129 meeting, the following progresses are achieved.

- Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

Agreements on paging ID

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

- Agreements - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

Agreements

- For Rel-19, only 3-step CBRA is supported for A-IoT

- We will specify both CBRA and CFRA.

- Re-use the subsequent paging message to trigger re-access.  There is no need to differentiate msg1 resource for initial access vs re-access.

Agreements

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback

- RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

Agreements - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - From device perspective, it is only required to use one AS ID. - CFRA is not supported for group ID - RAN2 assumes, AS ID is needed for CFRA at least for inventory + command procedure - For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: - Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. - Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 - Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2

- Agreements on segmentation

- To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

- R2D segmentation is not supported for R19 A-IoT.

Agreements

From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”.

Agreements on MAC PDU format design - Aim to design simple MAC PDU format design - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages - At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). - FFS whether for D2R we need message type field, any length and need for padding - Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

This paper further discusses the AIoT function based on the above progress.

- Discussion

  - Architecture aspects

Architecture aspects:

- AIoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

- AIOTF may be aware of the location of reader via OAM configuration.

- FFS: The details of the location of reader (e.g., room number, GNSS info) are FFS, and it is FFS on signaling support.

- FFS: To support selection of RAN node by AIOTF, define a new A-IoT Area, or use Tracking Area dedicated for A-IoT?

- Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area.

- Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers.

- WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

- FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

According to above cases, if the AIoT CN does not provide AIoT area in the Inventory request message, the AIoT area can be seen as the gNB’s serving area. If the AIoT CN provides area, the AIoT areas shall be full or part of the gNB’s serving area. On the contrary, if AIoT CN does not provide AIoT area, the gNB will use its serving area as AIoT area by default.

Proposal 1: If not providing area from AIoT CN, the gNB will use its serving area as AIoT area.

For instance, 4 readers (reader 1, 2, 3, 4) are deployed in the gNB serving area (grey color), 3 readers (reader 1, 2, 3) are deployed in the area provided by AIoT CN (green color), and 2 readers (reader 1 and reader 2) are suggested by AIoT CN.

- Case 1: Neither providing the area nor providing reader list

It is evident and agreed that the gNB can select reader(s) from 4 readers within the gNB serving area.

- Case 2: Providing the area but not providing reader list

It is evident and agreed that the gNB can select reader(s) from 3 readers within the area provided by AIoT CN.

- Case 3: Not providing the area but providing reader list (4 readers in the area, but 2 readers suggested)

The gNB will use its serving area as AIoT area, i.e., 4 readers in the area. However, it is strange if AIoT CN suggests 2 from 4 readers.

- Case 4: Providing both the area and reader list (3 readers in the area, but 2 readers suggested)

It is strange why AIoT CN indicates 2 from 3 readers other than all of 3 readers, perhaps due to the latest association between reader and devices in the last Inventory report.

For above four cases, it is clear for both Case 1 and Case 2. However, it is worth discussing how the gNB selects readers within the reader list in Case 3 and Case 4.

In our view, the intention of providing suggested reader(s) from AIoT CN is to help gNB to select suitable reader performing Inventory procedure with devices. Accurately, the AIoT CN mainly focuses on the Inventory result but is not care of which reader to provide it.

Observation 1: AIoT CN focuses on the Inventory result other than which reader to provide it.

From gNB side, upon receiving Inventory request message, it shall select suitable reader within either the area indicated by AIoT CN or gNB serving area by default. If receiving reader list provided by AIoT CN, the gNB can use it as suggested readers, it means that the gNB can based on the radio resources, select other reader out of reader list but within the area.

Proposal 2: Upon receiving reader list in Inventory Request message, no matter whether receiving the area in this message, the gNB uses the reader list as reference, i.e., it can select reader either in or out of the reader list within the area.

  - Inventory Procedures

About Inventory Procedures:

- WA: Introduce a NGAP Class 1 Inventory Request procedure.

- Introduce INVENTORY REQUEST message, which includes Inventory Request Transfer IE.

- Introduce INVENTORY RESPONSE message, which includes Inventory Response Transfer IE.

- Introduce INVENTORY FAILURE messages, which includes Inventory Failure Transfer IE.

- Introduce a NGAP Class 2 Inventory Report procedure.

- Introduce INVENTORY REPORT message, which includes Inventory Report Transfer IE.

- The above Transfer IEs are transparent to AMF in case of indirect communication.

- FFS: For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

In case of indirect communication deployment, one gNB can connect with one or more AIOTF via AMF, in this case, AIOT identifier shall be included in the Inventory related messages.

In case of direct communication deployment, one gNB connects with each AIOTF directly, in this case, AIOT identifier can be not included in the Inventory related messages.

Proposal 3: For Inventory related messages, Correlation Identifier and AIOTF Identifier shall be included in case of indirect communication.

  - Command Procedure

About Command Procedure:

- WA: Introduce a NGAP Class 1 Command Request procedure

- Introduce COMMAND REQUEST message, which includes Command Request Transfer IE.

- Introduce COMMAND RESPONSE message, which includes Command Response Transfer IE.

- Introduce COMMAND FAILURE messages, which includes Command Failure Transfer IE.

- The above Transfer IEs are transparent to AMF in case of indirect communication.

- FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

- FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure?

- FFS: Command request procedure is a per device procedure or to be used for multiple devices?

- FFS: The need for a Command Report procedure?

- FFS: Device association between gNB and AIOTF is used in Command procedure.

In case of indirect communication deployment, one gNB can connect with one or more AIOTF via AMF, in this case, AIOT identifier shall be included in the Command related messages.

Proposal 4: For Command related messages, Correlation Identifier and AIOTF Identifier shall be included in case of indirect communication.

In our view, the Correlation Identifier IE is use to identify a certain AIoT service procedure, so that, different AIoT service procedure shall use different Correlation Identifier value.

Proposal 5: For command related messages, the Correlation Identifier is the different one as in the related Inventory procedure.

At AIoT radio, RAN2 is discussing whether the reader can initiate one command procedure with multiple devices. However, at NG interface, the reader can receive Command request message including multiple devices then it can split them into one or more command request messages at AIoT radio.

Proposal 6: Command request procedure can be used for one or multiple devices.

This issue is raised in case of command only procedure. In this case, the latency may be long between Command request message and Command response message.

However, since command only procedure is excluded in this release, for the Inventory and command procedure, RAN2 has agreed to use AS ID to perform RACH-less procedure for the command operation, so that, command report procedure is not needed.

Proposal 7: Command Report procedure is not needed in the Inventory and command procedure.

  - Interface management procedures

About Interface management procedures:

  - RAN3 agrees to support both the direct and indirect options for the A-IoT RAN node to communicate with AIoTF in Topology 1.

  - WA: Take Option1 (Including AIoTF information in NGAP).

  - AIoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

- FFS: In indirect path communication, existing NG interface management procedures are used between the gNB and the AMF.

- FFS: In direct path communication, existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used between the gNB and the AIOTF.

- FFS: For interface management procedures, if reuse existing ones, how to handle the not applicable mandatory IEs, and how to handle A-IoT only support case.

We have agreed that A-IoT RAN node shall support both the direction connection and indirection connection with AIoTF. For the direction connection deployment, it is between RAN node and AIoTF, current NG connection between RAN node and AMF shall be extended to between RAN node and AIoTF.

Moreover, we also agreed that the gNB may only provide the AIoT radio. Based on this agreement, some mandatory IEs (e.g., Default Paging DRX, AMF Name) within current NG interface shall be ignored, and many optional IEs shall be skipped.

For our view, it is not essential whether to introduce new AIoT dedicated NG interface or to extend current NG interface. But, a new AIoT dedicated NG interface seems easy to be extended for the future proof.

Proposal 8: Introduce AIoT related NGAP protocol.

- Conclusion

Based on the above analysis, we provide the following observation and proposals.\

Observation 1: AIoT CN focuses on the Inventory result other than which reader to provide it.

Proposal 2: Upon receiving reader list in Inventory Request message, no matter whether receiving the area in this message, the gNB uses the reader list as reference, i.e., it can select reader either in or out of the reader list within the area.

Proposal 3: For Inventory related messages, Correlation Identifier and AIOTF Identifier shall be included in case of indirect communication.

Proposal 4: For Command related messages, Correlation Identifier and AIOTF Identifier shall be included in case of indirect communication.

Proposal 5: For command related messages, the Correlation Identifier is the different one as in the related Inventory procedure.

Proposal 6: Command request procedure can be used for one or multiple devices.

Proposal 7: Command Report procedure is not needed in the Inventory and command procedure.

Proposal 8: Introduce AIoT related NGAP protocol.

- TP to TS38.300

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

16.xx	Support of Ambient IoT

16.xx.x1	Introduction

16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or more readers.

-	A certain A-IoT device can be served by one or more readers.

-	As specified in <reference to SA2 TS to be added> the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct communication to AIOTF) or the AMF (in case of indirect communication to AIOTF).

NOTE 1:	It is not expected a deployment will deploy both direct communication and indirect communication.

-	In this version of the specification A-IoT specific data is transported between the gNB and the A-IoT CN node takes place via the NG-C interface.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

Editor’s Note 2:	WA: Including AIoTF information in NGAP.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Editor’s Note 3:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request may include information concerning one or multiple A-IoT devices.

Editor’s Note 4:	Specification of further parameterisation of the Inventory Request message (e.g., assistance information from 5GC, Inventory area, and reader list) needs further work.

NOTE x:	If the A-IoT CN node does not provide the Inventory area information in the Inventory Request message, the gNB uses its serving area as Inventory area.

NOTE x:	If the A-IoT CN node provides the reader list in the Inventory Request message, the gNB takes it into account for reader selection.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Reports may be sent to the A-IoT CN node.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices,

16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Editor’s Note 5:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.xx.x4-1: Command procedure

1.	Prior to the Command procedure, the Inventory procedure is performed for the concerned devices.

NOTE x: Command only procedure is excluded in this release.

Editor’s Note 6:	Further details on the relation and correlation between the Inventory procedure and the Command procedure needs further discussion may need input from SA2/SA3/RAN2.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB, including the same or different command operations for one or more AIoT devices.

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The gNB replies to the Command Request with the Command Response message.

NOTE 4:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

- TP to TS38.413

-----------------Start of the Change-----------------

8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| <Skip unchanged part> |  |  |  |
| MT Communication Handling | MT COMMUNICATION HANDLING REQUEST | MT COMMUNICATION HANDLING RESPONSE | MT COMMUNICATION HANDLING FAILURE |
| Broadcast Session Transport | BROADCAST SESSION TRANSPORT REQUEST | BROADCAST SESSION TRANSPORT RESPONSE | BROADCAST SESSION TRANSPORT FAILURE |
| Inventory Request | INVENTORY REQUEST | INVENTORY RESPONSE | INVENTORY FAILURE |
| Command Request | COMMAND REQUEST | COMMAND RESPONSE | COMMAND FAILURE |
| AIOTF Configuration Update | AIOTF CONFIGURATION UPDATE | AIOTF CONFIGURATION UPDATE ACKNOWLEDGE | AIOTF CONFIGURATION UPDATE FAILURE |
| AIoT-RAN Configuration Update | AIoT-RAN CONFIGURATION UPDATE | AIoT-RAN CONFIGURATION UPDATE ACKNOWLEDGE | AIoT-RAN CONFIGURATION UPDATE FAILURE |
| AIoT-NG Reset | AIoT-NG RESET | AIoT-NG RESET ACKNOWLEDGE |  |
| AIoT-NG Setup | AIoT-NG SETUP REQUEST | AIoT-NG SETUP RESPONSE | AIoT-NG SETUP FAILURE |

-----------------Start of the Next  Change-----------------

9.3.x	AIOTF Related IEs

9.3.x.1	Inventory Request Transfer

This IE provides the inventory request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation identifier | M | FFS |  |  |
| Device identifier | O |  | OCTET STRING (FFS) |  |
| Command Indicator | O |  | ENUMERATED (true, …) | This IE indicates Inventory and command procedure |
| Inventory area Information | O | FFS |  |  |
| Read information Iist | O | 1..8 (FFS) |  |  |
| > Serving gNB ID | O |  | Global gNB ID 9.3.1.6 |  |
| > Reader index | M |  |  |  |

9.3.x.2	Inventory Response Transfer

This IE provides the inventory response related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation identifier | M | FFS |  |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

9.3.x.3	Inventory Failure Transfer

This IE provides the inventory failure related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation identifier | M | FFS |  |  |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

9.3.x.4	Inventory Report Transfer

This IE provides the inventory report related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation identifier | M | FFS |  |  |
| Inventoried device list | M | FFS |  |  |
| >Device identifier | M |  | OCTET STRING (FFS) |  |
| Reader Information | M |  |  |  |
| > Serving gNB ID | O |  | Global gNB ID 9.3.1.6 |  |
| > Reader index | M |  |  |  |

9.3.x.5	Command Request Transfer

This IE provides the command request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation identifier | M | FFS |  |  |
| Command request List | M |  |  |  |
| >Device identifier | O |  | OCTET STRING (FFS) |  |
| >Command request |  |  | OCTET STRING (FFS) |  |

9.3.x.6	Command Response Transfer

This IE provides the command response related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation identifier | M | FFS |  |  |
| Command result List | O |  |  |  |
| >Device identifier | M |  | OCTET STRING (FFS) |  |
| >Command result Indicator | M |  | ENUMERATED (success, failure) |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

9.3.x.7	Command Failure Transfer

This IE provides the command failure related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation identifier | M | FFS |  |  |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

-----------------End of the Changes-----------------

- TP to TS38.410

<<<<<<<<<<<<<<<<<<<< First of Change >>>>>>>>>>>>>>>>>>>>

6.xx1	AIoT procedures

The following procedures are used for AIoT services:

-	AIOTF Configuration Update;

-	AIoT-RAN Configuration Update;

-	AIoT-NG Reset;

-	AIoT-NG Setup;

-	Inventory Request;

-	Inventory Report;

-	Command Request.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>