3GPP TSG RAN3 meeting #127bis   	         	                          R3-252246

7th – 11th Apr 2025  Wuhan, China

Agenda item: 	    10.2

Source: 	China Unicom

Title: 	Discussion on MRO enhancements

Document for:	Discussion

1	Introduction

At RAN#102, the new Work Item on Data Collection for SON_MDT in NR standalone and MR-DC phase 4 was agreed [1] which introduced the following objectives in the core part:

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure If needed, co-operate with RAN1, SA2, SA5, CT4.

This contribution provides our views on the scenarios of MRO enhancements which can be supported in Rel-19.

2	Discussion

2.1	LTM

MRO for LTM MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM? CU can adjust the candidate Cell list for LTM based on UHI. DU should be able to resolve LTM ping-pong based on information over F1. Network based solution is used for LTM candidate cell list. Network based or UE based solution for TA acquisition type? Will TA acquisition type always be available in the CU or is there a need for the DU to store and retrieve (partial) UE Context?

At the RAN3#127 meeting, we have been made above agreements and discussed RACH-less optimization and ping-pong avoidance.

For outdated TA case, UE may do RACH-based LTM after RACH-less LTM fails which should be distinguished from the normal failure case. Network can do optimization towards the TA value configured to the UE.

Proposal 1: RACH-less LTM failure result from outdated TA should be considered as MRO case.

For ping-pong issue in intra-NR LTM, since CU determines the LTM configuration and candidate cell list then sends to target candidate gNB-DU(s), CU should be responsible for avoiding LTM cell switch ping-pong with all the UHI information when choosing the candidate LTM cells. When inappropriate cell switch triggering made by DU, DU should be responsible for the issue. CU can do the initial analyse then sends the UHI to DUs only when inappropriate cell switch triggering made by DU.

Proposal 2: CU only sends the UHI to DUs when inappropriate cell switch triggering made by DU.

As for the solution of TA acquisition type, since UE behaviour may be adjusted during the access phase, UE -based solution is preferred with accurate information. An LS is needed to RAN2 to confirm the scenario and solution.

Proposal 3: UE-based solution is preferred for TA acquisition type, and LS to RAN2 is needed.

2.2	CHO with candidate SCGs

In last RAN3#127 meeting, we have the following agreements.

MRO for CHO with Candidate SCG(s):

In case of too late CHO execution, reusing Handover Report message from the last serving MN to the relevant candidate MN.

FFS on whether Handover Report message is needed for case 7a.

FFS on the additional information included in handover report.

New message is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization.

When to send the message? Immediately or not?

Check whether respective MN is only a candidate?

In addition to SCGFailureInformation, information needed in the new message?

CPC failure type

Since RAN3 agreed to reuse the Handover Report for too late CHO, the corresponding new Handover Report Type should be added as “Too Late Handover” .

Proposal 4: RAN3 to introduce a new Handover Report Type  as“Too Late Handover” .

Regarding the new message is used from the last serving MN to the candidate MN(s), Source PSCell ID and Failed PSCell ID should be added in the new message in addition to SCGFailureInformation.

Proposal 5: Source PSCell ID and Failed PSCell ID should be added in the new message in addition to SCGFailureInformation

2.3	Subsequent CPAC

The forwarding mechanism in BLCR for TS37.340 need to be refined.

A conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, or a SCG release, based on pre-configured subsequent CPAC configuration of candidate PSCell(s). UE would keep the subsequent CPAC configuration after a PSCell change, and if the execution condition of other one candidate PSCell is satisfied, UE may apply the stored configuration and synchronises to the candidate PSCell.

In that case, subsequent CPAC failure may occur at the first configured SN modification, as well as subsequent PSCell changes scenario. When subsequent CPAC fails, SCG FAILURE INFORMATION REPORT can be further enhanced to record the corresponding configurations for root cause analysis. Meanwhile, RAN3 needs to discuss how to trigger the forwarding of failure report to the configured node with the potential impacts on UE recording the mobility history information.

For subsequent CPAC, network cannot log the UHI since no more SN adding procedure during the mobility phase based on the existing SCG UHI mechanism. It is more straightforward for MN to adding all the visited SN while informing the source SN about the outcome of mobility events for SN initiated S-CPAC.

Proposal 6: SCG UHI mechanism can be enhanced for subsequent CPAC.

Proposal 7: MN to record the SCG UHI  for subsequent CPAC and send it back to source SN.

3	Conclusion

Proposal 1: RACH-less LTM failure result from outdated TA should be considered as MRO case.

Proposal 2: CU only sends the UHI to DUs when inappropriate cell switch triggering made by DU.

Proposal 3: UE-based solution is preferred for TA acquisition type, and LS to RAN2 is needed.

Proposal 4: RAN3 to introduce a new Handover Report Type  as“Too Late Handover” .

Proposal 5: Source PSCell ID and Failed PSCell ID should be added in the new message in addition to SCGFailureInformation

Proposal 6: SCG UHI mechanism can be enhanced for subsequent CPAC.

Proposal 7: MN to record the SCG UHI  for subsequent CPAC and send it back to source SN.

4   References

- RP-234038: New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4; TSG RAN Meeting #102, Edinburgh, Scotland, Dec 11-15, 2 023