3GPP TSG-RAN WG3 Meeting #127bis		R3-252261

Wuhan, China, 7th – 11th April 2025

Source:	vivo

Title:	Response to R3-251582 and R3-251881

Agenda Item:	13.2

Document for:	Discussion and Decision

- Introduction

In [1], SA3 provided response LS on the security handling for inter-CU LTM in non-DC cases. While in [2] and [3], this issue has been discussed and the corresponding solution has been provided.

In this response paper, we first provide our view on the key distribution approach for subsequent inter-CU LTM based on the SA3 LS, and then we discuss other remaining issues for inter-CU LTM.

- Discussion

  - Key distribution approach for subsequent inter-CU LTM

According to the LS from SA3 [1], NCC can be transmitted to the UE in plaintext via MAC CE message. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

S3-251124 LS from SA3

1. Overall Description:

SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases.

Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure.

In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future.

The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

Regarding key distribution between the source gNB and target gNB, there are two possible approaches:

- Approach 1: The source gNB transmits security keys to all candidate gNBs through the handover preparation procedure or the LTM configuration update procedure.

- Approach 2: Source gNB transmits security key only to target gNB through the Cell Switch Notification procedure.

For Approach 1, key distribution follows a process similar to that of CHO during the initial handover preparation phase; specifically, the source gNB generates security keys for all candidate cells and transmits them to each candidate cell during this phase. For subsequent inter-CU LTM operations, after completing the path switch procedure, the new source gNB must generate security keys for all candidate cells based on the unused new NH. It then distributes these newly generated security keys to all candidate gNBs again via the LTM configuration update procedure for the next LTM cell switch. The detailed process for Approach 1 is illustrated in Figure 1 and the key distribution procedure is highlighted in red.

Figure 1. Key distribution Approach 1 for inter-CU LTM

For approach 2, the source gNB generates a security key only when it determines the target cell and then transmits this security key to the target cell via the cell switch notification procedure. During each time of inter-CU LTM cell switch, the source gNB only needs to generate and transmit the security key specific to the target cell. The detailed process for Approach 2 is illustrated in Figure 2 and the key distribution procedure is highlighted in red.

Figure 2. Key distribution Approach 1 for inter-CU LTM

From the analysis above, it is evident that Approach 2 has lower computational complexity and signaling overhead compared with Approach 1, as it only requires generating and transmitting a single security key specifically for the target cell.

Observation 1: Security key distribution via the cell switch notification procedure has lower computational complexity and signaling overhead compared with security key distribution via the handover preparation procedure and the LTM configuration update procedure.

Proposal 1: Source gNB transmits the security key to the target gNB via the Cell Switch Notification message at the LTM cell switch execution phase.

  - Shared CFRA resources in the LTM cell switch command

In RAN2#129, the following agreement was made:

- Same as Rel-18 intra-CU LTM, CFRA resource configuration could be included in LTM cell switch command MAC CE for Rel-19 inter-CU LTM.

To support the CFRA resource configuration included in the LTM cell switch command MAC CE for inter-CU LTM, two approaches can be considered:

Approach 1: Allocated by Target DU

- The CFRA resources are allocated by T-DU (Target Distributed Unit).

- Before sending an LTM Cell Switch Command MAC CE, S-DU (Source Distributed Unit) retrieves dedicated CFRA resources per UE from T-DU.

Approach 2: Allocated by Source DU

- During the handover preparation phase, S-DU obtains a shared CFRA resource pool from T-DU for multiple UEs within a single DU unit;

- S-DU then allocates specific CFRA resources to UEs requiring an LTM Cell Switch.

For approach 1 (CFRA resource allocated by Target DU), specific CFRA resources need to be reserved for each UE. This could lead to significant resource wastage when subsequent LTMs are supported since CFRAs would have to remain reserved over extended periods of time. Additionally, if CFRA resources are allocated by Target DU, there would be no practical difference between including them in an LTM Cell Switch Command MAC CE versus including them in preconfigured candidate RRC configurations. This goes against our original motivation behind including CFRA resources within an LTM Cell Switch Command MAC CE.

For approach 2 (CFRA resource allocated by source DU), CFRA resources are shared among UEs within a single DU, which is similar to the early-RACH resources sharing mechanism supported in Rel-18 LTM. The S-DU is responsible for allocating specific CFRA resources for each UE to perform RACH-based LTM cell switch while simultaneously avoiding the CFRA resource conflicts. As a result, the resource wastage can be largely reduced. Therefore, we prefer using Approach 2.

Proposal 2: For inter-CU LTM, CFRA resources for RACH-based LTM could be shared among multiple UEs within a single gNB-DU and assigned by S-DU in the LTM cell switch command MAC CE.

If CFRA resources for RACH-based LTM are shared among multiple UEs, the source DU needs to inform the target DU about the CFRA resource information (e.g., preamble index and RA-RNTI) indicated in the LTM cell switch command via the cell switch notification message. This ensures that the target cell knows which UE is performing RACH-based LTM and which CFRA resources are being used.

Proposal 3: If Proposal 2 is agreed, the preamble index and RA-RNTI should be indicated in the Cell Switch Notification message if CFRA resources are indicated in the LTM cell switch command.

  - Coexistence of inter-CU LTM with CHO

The coexistence of inter-CU LTM with CHO was supported based on the agreement reached in RAN2#127 meeting.

RAN2#127 Agreement

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

However, since CHO is triggered by the UE and RRCReconfiguraiton is initiated from the network, there may be cases where the RRCReconfiguration from the source gNB conflicts with an ongoing conditional handover. This can result in the UE failing to receive the RRCReconfiguration message. If the RRCReconfiguraiton message includes information related to LTM configuration update, the target gNB of CHO must be aware of the updated LTM configuration in order to reconfigure the UE accordingly. Otherwise, a misalignment may occur between the UE and the network, potentially leading to the failure of subsequent LTM cell switch. To illustrate this issue, we refer to the example presented in the following figure.

Figure 1. Conflict of CHO and RRCReconfiguration for inter-CU LTM

To address the issue, interaction between the source gNB and the target gNB of CHO is essential.

Proposal 4: RAN3 is requested to discuss the cases of configuration misalignment between UE and the network when LTM coexists with CHO (i.e., UE fails to receive the LTM configuration update reconfiguration message due to CHO execution).

- Conclusion

In this paper, we discuss the inter-CU LTM remaining open issues and we have the following observations and proposals:

Observation 1: Security key distribution via the cell switch notification procedure has lower computational complexity and signaling overhead compared with security key distribution via the handover preparation procedure and the LTM configuration update procedure.

Proposal 1: Source gNB transmits the security key to the target gNB via the Cell Switch Notification message at the LTM cell switch execution phase.

Proposal 2: For inter-CU LTM, CFRA resources for RACH-based LTM could be shared among multiple UEs within a single gNB-DU and assigned by S-DU in the LTM cell switch command MAC CE.

Proposal 3: If Proposal 2 is agreed, the preamble index and RA-RNTI should be indicated in the Cell Switch Notification message if CFRA resources are indicated in the LTM cell switch command.

Proposal 4: RAN3 is requested to discuss the cases of configuration misalignment between UE and the network when LTM coexists with CHO (i.e., UE fails to receive the LTM configuration update reconfiguration message due to CHO execution).

- References

- S3-251124, Reply LS on security handling for inter-CU LTM in non-DC cases, CATT

- R3-251881, (TP for 38.423, 38.300, 38.420, 38.470) Inter-CU LTM	, ZTE Corporation

- R3-251582, Discussion on Inter-CU LTM security, Nokia