3GPP TSG-RAN WG3 Meeting #127-bis	R3-252441

Wuhan, China, 7-11 April, 2025

Agenda item:	11.4

Source: 	Samsung, ZTE Corporation, Nokia

Title: 	(TP to BL CR for 37.483) Data Collection procedures for UE performance

Document for:	Text Proposal

# 1. Introduction

This TP implements the agreement achieved in RAN3#127bis about introducing new E1 procedures for data collection of UE performance from CU-UP to CU-CP:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

# 2. Text Proposals to BLCR of 37.483

## 8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| gNB-CU-UP E1 Setup |  | GNB-CU-UP E1 SETUP REQUEST |  | GNB-CU-UP E1 SETUP RESPONSE |  | GNB-CU-UP E1 SETUP FAILURE |  |
| gNB-CU-CP E1 Setup |  | GNB-CU-CP E1 SETUP REQUEST |  | GNB-CU-CP E1 SETUP RESPONSE |  | GNB-CU-CP E1 SETUP FAILURE |  |
| gNB-CU-UP Configuration Update |  | GNB-CU-UP CONFIGURATION UPDATE |  | GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-UP CONFIGURATION UPDATE FAILURE |  |
| gNB-CU-CP Configuration Update |  | GNB-CU-CP CONFIGURATION UPDATE |  | GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-CP CONFIGURATION UPDATE FAILURE |  |
| E1 Release |  | E1 RELEASE REQUEST |  | E1 RELEASE RESPONSE |  |  |  |
| Bearer Context Setup |  | BEARER CONTEXT SETUP REQUEST |  | BEARER CONTEXT SETUP RESPONSE |  | BEARER CONTEXT SETUP FAILURE |  |
| Bearer Context Modification (gNB-CU-CP initiated) |  | BEARER CONTEXT MODIFICATION REQUEST |  | BEARER CONTEXT MODIFICATION RESPONSE |  | BEARER CONTEXT MODIFICATION FAILURE |  |
| Bearer Context Modification Required (gNB-CU-UP initiated) |  | BEARER CONTEXT MODIFICATION REQUIRED |  | BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| Bearer Context Release (gNB-CU-CP initiated) |  | BEARER CONTEXT RELEASE COMMAND |  | BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| IAB UP TNL Address Update |  | IAB UP TNL ADDRESS UPDATE |  | IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE |  | IAB UP TNL ADDRESS UPDATE FAILURE |  |
| BC Bearer Context Setup |  | BC BEARER CONTEXT SETUP REQUEST |  | BC BEARER CONTEXT SETUP RESPONSE |  | BC BEARER CONTEXT SETUP FAILURE |  |
| BC Bearer Context Modification (gNB-CU-CP initiated) |  | BC BEARER CONTEXT MODIFICATION REQUEST |  | BC BEARER CONTEXT MODIFICATION RESPONSE |  | BC BEARER CONTEXT MODIFICATION FAILURE |  |
| BC Bearer Context Modification Required (gNB-CU-UP initiated) |  | BC BEARER CONTEXT MODIFICATION REQUIRED |  | BC BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| BC Bearer Context Release (gNB-CU-CP initiated) |  | BC BEARER CONTEXT RELEASE COMMAND |  | BC BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| MC Bearer Context Setup |  | MC BEARER CONTEXT SETUP REQUEST |  | MC BEARER CONTEXT SETUP RESPONSE |  | MC BEARER CONTEXT SETUP FAILURE |  |
| MC Bearer Context Modification (gNB-CU-CP initiated) |  | MC BEARER CONTEXT MODIFICATION REQUEST |  | MC BEARER CONTEXT MODIFICATION RESPONSE |  | MC BEARER CONTEXT MODIFICATION FAILURE |  |
| MC Bearer Context Modification Required (gNB-CU-UP initiated) |  | MC BEARER CONTEXT MODIFICATION REQUIRED |  | MC BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| MC Bearer Context Release (gNB-CU-CP initiated) |  | MC BEARER CONTEXT RELEASE COMMAND |  | MC BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| Data Collection Reporting Initiation |  | DATA COLLECTION REQUEST |  | DATA COLLECTION RESPONSE |  | DATA COLLECTION FAILURE |  |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| Bearer Context Release Request (gNB-CU-UP initiated) | BEARER CONTEXT RELEASE REQUEST |
| Bearer Context Inactivity Notification | BEARER CONTEXT INACTIVITY NOTIFICATION |
| DL Data Notification | DL DATA NOTIFICATION |
| UL Data Notification | UL DATA NOTIFICATION |
| Data Usage Report | DATA USAGE REPORT |
| gNB-CU-UP Counter Check | GNB-CU-UP COUNTER CHECK REQUEST |
| gNB-CU-UP Status Indication | GNB-CU-UP STATUS INDICATION |
| MR-DC Data Usage Report | MR-DC DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Early Forwarding SN Transfer | EARLY FORWARDING SN TRANSFER |
| GNB-CU-CP Measurement Results Information | GNB-CU-CP MEASUREMENT RESULTS INFORMATION |
| IAB PSK Notification | IAB PSK NOTIFICATION |
| BC Bearer Context Release (gNB-CU-UP initiated) | BC BEARER CONTEXT RELEASE REQUEST |
| MC Bearer Context Release (gNB-CU-UP initiated) | MC BEARER CONTEXT RELEASE REQUEST |
| MC Bearer Notification | MC BEARER NOTIFICATION |
| Data Collection Reporting | DATA COLLECTION UPDATE |

### 8.2.AA	Data Collection Reporting Initiation

#### 8.2.AA.1	General

Editor’s Note: FFS other information that can be requested using this procedure.

This procedure is used by a gNB-CU-CP to request from a gNB-CU-UP the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.2.AA.2	Successful Operation

Figure 8.4.AA.2-1: Data Collection Reporting Initiation, successful operation

The gNB-CU-CP initiates the procedure by sending the DATA COLLECTION REQUEST message to gNB-CU-UP to start information reporting and stop information reporting. Upon receipt, the gNB-CU-UP:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to “start”; or

-	shall stop all information reporting and terminate the reporting in case the Registration Request for Data Collection IE is set to “stop”.

If the gNB-CU-UP is capable to provide all of the requested information, it shall initiate the information reporting as requested by the gNB-CU-CP and respond with the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST is present, this indicates the periodicity for the reporting of periodic information. The gNB-CU-UP shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

Interaction with other procedures

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST indicates the type of objects gNB-CU-UP shall perform measurements on. The gNB-CU-UP shall include in the DATA COLLECTION UPDATE message:

-    FFS

Editor’s Note: The procedure text can be further refined.

#### 8.2.AA.3	Unsuccessful Operation

Figure 8.4.AA.3-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information reporting cannot be initiated, gNB-CU-UP shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.2.AA.4	Abnormal Conditions

For the same Measurement ID, if the initiating gNB-CU-CP does not receive either the DATA COLLECTION RESPONSE message or the DATA COLLECTION FAILURE message, the gNB-CU-CP may reinitiate the Data Collection Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message.

If the gNB-CU-UP receives an DATA COLLECTION REQUEST message which includes the Registration Request IE set to “stop” and if the gNB-CU-UP Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the gNB-CU-UP shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

If the Report Characteristics for Data Collection IE bitmap is set to “0” (all bits are set to “0”) in the DATA COLLECTION REQUEST message then gNB-CU-UP shall initiate an DATA COLLECTION FAILURE message with an appropriate cause value.

If the gNB-CU-UP receives a DATA COLLECTION REQUEST message which includes the Registration Request IE set to “start” and the gNB-CU-CP Measurement ID IE corresponding to an existing on-going Data Collection reporting, then gNB-CU-UP shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

### 8.2.BB	Data Collection Reporting

#### 8.2.BB.1	General

This procedure is initiated by an gNB-CU-UP to report information accepted by the gNB-CU-CP following a successful Data Collection Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.2.BB.2	Successful Operation

Figure 8.2.BB.2-1: Data Collection Reporting, successful operation

The gNB-CU-UP shall report the accepted information in DATA COLLECTION UPDATE message. The accepted information is the information that was successfully initiated during the preceding Data Collection Reporting Initiation procedure.

#### 8.2.BB.3	Unsuccessful Operation

Not applicable.

#### 8.2.BB.4	Abnormal Conditions

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.CC	DATA COLLECTION REQUEST

This message is sent by gNB-CU-CP to gNB-CU-UP to initiate the requested information reporting according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-CU-UP is requested to report. First Bit = Average UE Throughput UL, Second bit = Average UE Throughput DL, Third Bit = Average Packet Delay, Other bits are ignored by the gNB-CU-UP. FFS on the coding | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

#### 9.2.1.DD	DATA COLLECTION RESPONSE

This message is sent by gNB-CU-UP to gNB-CU-CP to indicate that the requested information, for all or part of the measurement objects included in the reporting is successfully initiated.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Cause | M |  | 9.3.1.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.EE	DATA COLLECTION FAILURE

This message is sent by the gNB CU-UP to gNB-CU-CP to indicate that for all of the requested objects the reporting cannot be initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.FF	DATA COLLECTION UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the requested information.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| FFS other information |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

#### -- Elementary Procedure definitions

--

-- **************************************************************

E1AP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM E1AP-CommonDataTypes

Reset,

ResetAcknowledge,

ErrorIndication,

GNB-CU-UP-E1SetupRequest,

GNB-CU-UP-E1SetupResponse,

GNB-CU-UP-E1SetupFailure,

GNB-CU-CP-E1SetupRequest,

GNB-CU-CP-E1SetupResponse,

GNB-CU-CP-E1SetupFailure,

GNB-CU-UP-ConfigurationUpdate,

GNB-CU-UP-ConfigurationUpdateAcknowledge,

GNB-CU-UP-ConfigurationUpdateFailure,

GNB-CU-CP-ConfigurationUpdate,

GNB-CU-CP-ConfigurationUpdateAcknowledge,

GNB-CU-CP-ConfigurationUpdateFailure,

BCBearerContextSetupRequest,

BCBearerContextSetupResponse,

BCBearerContextSetupFailure,

BCBearerContextModificationRequest,

BCBearerContextModificationResponse,

BCBearerContextModificationFailure,

BCBearerContextModificationRequired,

BCBearerContextModificationConfirm,

BCBearerContextReleaseCommand,

BCBearerContextReleaseComplete,

BCBearerContextReleaseRequest,

BearerContextSetupRequest,

BearerContextSetupResponse,

BearerContextSetupFailure,

BearerContextModificationRequest,

BearerContextModificationResponse,

BearerContextModificationFailure,

BearerContextModificationRequired,

BearerContextModificationConfirm,

BearerContextReleaseCommand,

BearerContextReleaseComplete,

BearerContextReleaseRequest,

BearerContextInactivityNotification,

DLDataNotification,

ULDataNotification,

DataUsageReport,

E1ReleaseRequest,

E1ReleaseResponse,

GNB-CU-UP-CounterCheckRequest,

GNB-CU-UP-StatusIndication,

MCBearerContextSetupRequest,

MCBearerContextSetupResponse,

MCBearerContextSetupFailure,

MCBearerContextModificationRequest,

MCBearerContextModificationResponse,

MCBearerContextModificationFailure,

MCBearerContextModificationRequired,

MCBearerContextModificationConfirm,

MCBearerNotification,

MCBearerContextReleaseCommand,

MCBearerContextReleaseComplete,

MCBearerContextReleaseRequest,

MRDC-DataUsageReport,

DeactivateTrace,

TraceStart,

PrivateMessage,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

IAB-UPTNLAddressUpdate,

IAB-UPTNLAddressUpdateAcknowledge,

IAB-UPTNLAddressUpdateFailure,

CellTrafficTrace,

EarlyForwardingSNTransfer,

GNB-CU-CPMeasurementResultsInformation,

IABPSKNotification,

DataCollectionRequest,

DataCollectionResponse,

DataCollectionFailure,

DataCollectionUpdate

FROM E1AP-PDU-Contents

id-reset,

id-errorIndication,

id-gNB-CU-UP-E1Setup,

id-gNB-CU-CP-E1Setup,

id-gNB-CU-UP-ConfigurationUpdate,

id-gNB-CU-CP-ConfigurationUpdate,

id-e1Release,

id-bearerContextSetup,

id-bearerContextModification,

id-bearerContextModificationRequired,

id-bearerContextRelease,

id-bearerContextReleaseRequest,

id-bearerContextInactivityNotification,

id-dLDataNotification,

id-uLDataNotification,

id-dataUsageReport,

id-gNB-CU-UP-CounterCheck,

id-gNB-CU-UP-StatusIndication,

id-mRDC-DataUsageReport,

id-DeactivateTrace,

id-TraceStart,

id-privateMessage,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-iAB-UPTNLAddressUpdate,

id-CellTrafficTrace,

id-earlyForwardingSNTransfer,

id-gNB-CU-CPMeasurementResultsInformation,

id-iABPSKNotification,

id-BCBearerContextSetup,

id-BCBearerContextModification,

id-BCBearerContextModificationRequired,

id-BCBearerContextRelease,

id-BCBearerContextReleaseRequest,

id-MCBearerContextSetup,

id-MCBearerContextModification,

id-MCBearerContextModificationRequired,

id-MCBearerNotification,

id-MCBearerContextRelease,

id-MCBearerContextReleaseRequest,

id-dataCollectionReportingInitiation,

id-dataCollectionReporting

FROM E1AP-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

E1AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

E1AP-PDU ::= CHOICE {

initiatingMessage		InitiatingMessage,

successfulOutcome		SuccessfulOutcome,

unsuccessfulOutcome		UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

E1AP-ELEMENTARY-PROCEDURES E1AP-ELEMENTARY-PROCEDURE ::= {

E1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

E1AP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

E1AP-ELEMENTARY-PROCEDURES-CLASS-1 E1AP-ELEMENTARY-PROCEDURE ::= {

reset										|

gNB-CU-UP-E1Setup							|

gNB-CU-CP-E1Setup							|

gNB-CU-UP-ConfigurationUpdate				|

gNB-CU-CP-ConfigurationUpdate				|

e1Release									|

bearerContextSetup							|

bearerContextModification					|

bearerContextModificationRequired			|

bearerContextRelease						|

resourceStatusReportingInitiation			|

iAB-UPTNLAddressUpdate						|

bCBearerContextSetup						|

bCBearerContextModification					|

bCBearerContextModificationRequired			|

bCBearerContextRelease						|

mCBearerContextSetup						|

mCBearerContextModification					|

mCBearerContextModificationRequired			|

mCBearerContextRelease						|

dataCollectionReportingInitiation,

...

}

E1AP-ELEMENTARY-PROCEDURES-CLASS-2 E1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

bearerContextReleaseRequest				|

bearerContextInactivityNotification		|

dLDataNotification						|

uLDataNotification						|

dataUsageReport							|

gNB-CU-UP-CounterCheck					|

gNB-CU-UP-StatusIndication				|

mRDC-DataUsageReport					|

deactivateTrace							|

traceStart								|

privateMessage							|

cellTrafficTrace						|

resourceStatusReporting					|

earlyForwardingSNTransfer				|

gNB-CU-CPMeasurementResultsInformation  |

iABPSKNotification						|

bCBearerContextReleaseRequest			|

mCBearerContextReleaseRequest			|

mCBearerNotification					|

dataCollectionReporting,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

reset E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Reset

SUCCESSFUL OUTCOME		ResetAcknowledge

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

<uncahnged text omitted>

mCBearerNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MCBearerNotification

PROCEDURE CODE			id-MCBearerNotification

CRITICALITY				reject

}

mCBearerContextRelease E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MCBearerContextReleaseCommand

SUCCESSFUL OUTCOME		MCBearerContextReleaseComplete

PROCEDURE CODE			id-MCBearerContextRelease

CRITICALITY				reject

}

mCBearerContextReleaseRequest E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MCBearerContextReleaseRequest

PROCEDURE CODE			id-MCBearerContextReleaseRequest

CRITICALITY				reject

}

dataCollectionReportingInitiation	E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				DataCollectionRequest

SUCCESSFUL OUTCOME				DataCollectionResponse

UNSUCCESSFUL OUTCOME			DataCollectionFailure

PROCEDURE CODE					id-dataCollectionReportingInitiation

CRITICALITY						reject

}

dataCollectionReporting	E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DataCollectionUpdate

PROCEDURE CODE			id-dataCollectionReporting

CRITICALITY				ignore

}

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for E1AP

--

-- **************************************************************

E1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

Cause,

CriticalityDiagnostics,

GNB-CU-CP-MBS-E1AP-ID,

GNB-CU-UP-MBS-E1AP-ID,

GNB-CU-CP-UE-E1AP-ID,

GNB-CU-UP-UE-E1AP-ID,

UE-associatedLogicalE1-ConnectionItem,

GNB-CU-UP-ID,

GNB-CU-UP-Name,

Extended-GNB-CU-UP-Name,

GNB-CU-CP-Name,

Extended-GNB-CU-CP-Name,

CNSupport,

PLMN-Identity,

Slice-Support-List,

NR-CGI-Support-List,

QoS-Parameters-Support-List,

SecurityInformation,

BitRate,

BearerContextStatusChange,

DRB-To-Setup-List-EUTRAN,

DRB-Setup-List-EUTRAN,

DRB-Failed-List-EUTRAN,

DRB-To-Modify-List-EUTRAN,

DRB-Measurement-Results-Information-List,

DRB-Modified-List-EUTRAN,

DRB-Failed-To-Modify-List-EUTRAN,

DRB-To-Remove-List-EUTRAN,

DRB-Required-To-Remove-List-EUTRAN,

DRB-Required-To-Modify-List-EUTRAN,

DRB-Confirm-Modified-List-EUTRAN,

DRB-To-Setup-Mod-List-EUTRAN,

DRB-Setup-Mod-List-EUTRAN,

DRB-Failed-Mod-List-EUTRAN,

PDU-Session-Resource-To-Setup-List,

PDU-Session-Resource-Setup-List,

PDU-Session-Resource-Failed-List,

PDU-Session-Resource-To-Modify-List,

PDU-Session-Resource-Modified-List,

PDU-Session-Resource-Failed-To-Modify-List,

PDU-Session-Resource-To-Remove-List,

PDU-Session-Resource-Required-To-Modify-List,

PDU-Session-Resource-Confirm-Modified-List,

PDU-Session-Resource-To-Setup-Mod-List,

PDU-Session-Resource-Setup-Mod-List,

PDU-Session-Resource-Failed-Mod-List,

PDU-Session-To-Notify-List,

DRB-Status-Item,

DRB-Activity-Item,

Data-Usage-Report-List,

TimeToWait,

ActivityNotificationLevel,

ActivityInformation,

New-UL-TNL-Information-Required,

GNB-CU-CP-TNLA-Setup-Item,

GNB-CU-CP-TNLA-Failed-To-Setup-Item,

GNB-CU-CP-TNLA-To-Add-Item,

GNB-CU-CP-TNLA-To-Remove-Item,

GNB-CU-CP-TNLA-To-Update-Item,

GNB-CU-UP-TNLA-To-Remove-Item,

TransactionID,

Inactivity-Timer,

DRBs-Subject-To-Counter-Check-List-EUTRAN,

DRBs-Subject-To-Counter-Check-List-NG-RAN,

PPI,

GNB-CU-UP-Capacity,

GNB-CU-UP-OverloadInformation,

DataDiscardRequired,

PDU-Session-Resource-Data-Usage-List,

RANUEID,

GNB-DU-ID,

TraceID,

TraceActivation,

SubscriberProfileIDforRFP,

AdditionalRRMPriorityIndex,

RetainabilityMeasurementsInfo,

Transport-Layer-Address-Info,

HW-CapacityIndicator,

RegistrationRequest,

ReportCharacteristics,

ReportingPeriodicity,

TNL-AvailableCapacityIndicator,

DLUPTNLAddressToUpdateItem,

ULUPTNLAddressToUpdateItem,

NPNContextInfo,

NPNSupportInfo,

MDTPLMNList,

PrivacyIndicator,

URIaddress,

DRBs-Subject-To-Early-Forwarding-List,

CHOInitiation,

ExtendedSliceSupportList,

TransportLayerAddress,

AdditionalHandoverInfo,

Extended-NR-CGI-Support-List,

DirectForwardingPathAvailability,

IAB-Donor-CU-UPPSKInfo-Item,

ECGI-Support-List,

MDTPollutedMeasurementIndicator,

UESliceMaximumBitRateList,

SCGActivationStatus,

GlobalMBSSessionID,

BCBearerContextToSetup,

BCBearerContextToSetupResponse,

BCBearerContextToModify,

BCBearerContextToModifyResponse,

BCBearerContextToModifyRequired,

BCBearerContextToModifyConfirm,

MCBearerContextToSetup,

MCBearerContextToSetupResponse,

MCBearerContextToModify,

MCBearerContextToModifyResponse,

MCBearerContextToModifyRequired,

MCBearerContextToModifyConfirm,

MBSMulticastF1UContextDescriptor,

MBS-ServiceArea,

GNB-CU-UP-MBS-Support-Info,

SDTContinueROHC,

MDTPLMNModificationList,

InactivityInformationRequest,

UEInactivityInformation,

MBSSessionResourceNotification,

MT-SDT-Information,

MT-SDT-Information-Request,

SDT-data-size-threshold,

SDT-data-size-threshold-Crossed,

RegistrationRequestForDataCollection,

ReportCharacteristicsForDataCollection,

ReportingPeriodicityForDataCollection

FROM E1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-SingleContainer{},

E1AP-PRIVATE-IES,

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers

id-AssociatedSessionID,

id-Cause,

id-CriticalityDiagnostics,

id-gNB-CU-CP-UE-E1AP-ID,

id-gNB-CU-UP-UE-E1AP-ID,

id-ResetType,

id-UE-associatedLogicalE1-ConnectionItem,

id-UE-associatedLogicalE1-ConnectionListResAck,

id-gNB-CU-UP-ID,

id-gNB-CU-UP-Name,

id-Extended-GNB-CU-UP-Name,

id-gNB-CU-CP-Name,

id-Extended-GNB-CU-CP-Name,

id-CNSupport,

id-SupportedPLMNs,

id-NPNSupportInfo,

id-NPNContextInfo,

id-SecurityInformation,

id-UEDLAggregateMaximumBitRate,

id-BearerContextStatusChange,

id-System-BearerContextSetupRequest,

id-System-BearerContextSetupResponse,

id-System-BearerContextModificationRequest,

id-System-BearerContextModificationResponse,

id-System-BearerContextModificationConfirm,

id-System-BearerContextModificationRequired,

id-DRB-Status-List,

id-Data-Usage-Report-List,

id-TimeToWait,

id-ActivityNotificationLevel,

id-ActivityInformation,

id-New-UL-TNL-Information-Required,

id-GNB-CU-CP-TNLA-Setup-List,

id-GNB-CU-CP-TNLA-Failed-To-Setup-List,

id-GNB-CU-CP-TNLA-To-Add-List,

id-GNB-CU-CP-TNLA-To-Remove-List,

id-GNB-CU-CP-TNLA-To-Update-List,

id-GNB-CU-UP-TNLA-To-Remove-List,

id-DRB-To-Setup-List-EUTRAN,

id-DRB-To-Modify-List-EUTRAN,

id-DRB-To-Remove-List-EUTRAN,

id-DRB-Required-To-Modify-List-EUTRAN,

id-DRB-Required-To-Remove-List-EUTRAN,

id-DRB-Setup-List-EUTRAN,

id-DRB-Failed-List-EUTRAN,

id-DRB-Measurement-Results-Information-List,

id-DRB-Modified-List-EUTRAN,

id-DRB-Failed-To-Modify-List-EUTRAN,

id-DRB-Confirm-Modified-List-EUTRAN,

id-DRB-To-Setup-Mod-List-EUTRAN,

id-DRB-Setup-Mod-List-EUTRAN,

id-DRB-Failed-Mod-List-EUTRAN,

id-PDU-Session-Resource-To-Setup-List,

id-PDU-Session-Resource-To-Modify-List,

id-PDU-Session-Resource-To-Remove-List,

id-PDU-Session-Resource-Required-To-Modify-List,

id-PDU-Session-Resource-Setup-List,

id-PDU-Session-Resource-Failed-List,

id-PDU-Session-Resource-Modified-List,

id-PDU-Session-Resource-Failed-To-Modify-List,

id-PDU-Session-Resource-Confirm-Modified-List,

id-PDU-Session-Resource-Setup-Mod-List,

id-PDU-Session-Resource-Failed-Mod-List,

id-PDU-Session-Resource-To-Setup-Mod-List,

id-PDU-Session-To-Notify-List,

id-TransactionID,

id-Serving-PLMN,

id-UE-Inactivity-Timer,

id-System-GNB-CU-UP-CounterCheckRequest,

id-DRBs-Subject-To-Counter-Check-List-EUTRAN,

id-DRBs-Subject-To-Counter-Check-List-NG-RAN,

id-PPI,

id-gNB-CU-UP-Capacity,

id-GNB-CU-UP-OverloadInformation,

id-UEDLMaximumIntegrityProtectedDataRate,

id-DataDiscardRequired,

id-PDU-Session-Resource-Data-Usage-List,

id-RANUEID,

id-GNB-DU-ID,

id-TraceID,

id-TraceActivation,

id-SubscriberProfileIDforRFP,

id-AdditionalRRMPriorityIndex,

id-RetainabilityMeasurementsInfo,

id-Transport-Layer-Address-Info,

id-gNB-CU-CP-Measurement-ID,

id-gNB-CU-UP-Measurement-ID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-ReportingPeriodicity,

id-TNL-AvailableCapacityIndicator,

id-HW-CapacityIndicator,

id-DLUPTNLAddressToUpdateList,

id-ULUPTNLAddressToUpdateList,

id-ManagementBasedMDTPLMNList,

id-TraceCollectionEntityIPAddress,

id-PrivacyIndicator,

id-URIaddress,

id-DRBs-Subject-To-Early-Forwarding-List,

id-CHOInitiation,

id-ExtendedSliceSupportList,

id-AdditionalHandoverInfo,

id-Extended-NR-CGI-Support-List,

id-DirectForwardingPathAvailability,	id-IAB-Donor-CU-UPPSKInfo,

id-ECGI-Support-List,

id-MDTPollutedMeasurementIndicator,

id-UESliceMaximumBitRateList,

id-SCGActivationStatus,

id-GNB-CU-CP-MBS-E1AP-ID,

id-GNB-CU-UP-MBS-E1AP-ID,

id-GlobalMBSSessionID,

id-BCBearerContextToSetup,

id-BCBearerContextToSetupResponse,

id-BCBearerContextToModify,

id-BCBearerContextToModifyResponse,

id-BCBearerContextToModifyRequired,

id-BCBearerContextToModifyConfirm,

id-MCBearerContextToSetup,

id-MCBearerContextToSetupResponse,

id-MCBearerContextToModify,

id-MCBearerContextToModifyResponse,

id-MCBearerContextToModifyRequired,

id-MCBearerContextToModifyConfirm,

id-MBSMulticastF1UContextDescriptor,

id-gNB-CU-UP-MBS-Support-Info,

id-SDTContinueROHC,

id-ManagementBasedMDTPLMNModificationList,

id-MBS-ServiceArea,

id-InactivityInformationRequest,

id-UEInactivityInformation,

id-MBSSessionResourceNotification,

id-MT-SDT-Information,

id-MT-SDT-Information-Request,

id-SDT-data-size-threshold,

id-SDT-data-size-threshold-Crossed,

id-RegistrationRequestForDataCollection,

id-ReportCharacteristicsForDataCollection,

id-ReportingPeriodicityForDataCollection

maxnoofErrors,

maxnoofSPLMNs,

maxnoofDRBs,

maxnoofTNLAssociations,

maxnoofIndividualE1ConnectionsToReset,

maxnoofTNLAddresses,

maxnoofPSKs

FROM E1AP-Constants;

-- **************************************************************

--

<unchanged text omitted>

-- **************************************************************

--

#### -- DATA COLLECTION REQUEST

--

-- **************************************************************

DataCollectionRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionRequest-IEs}},

...

}

DataCollectionRequest-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID	PRESENCE mandatory}|

{ ID id-gNB-CU-CP-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID										PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID					CRITICALITY ignore	TYPE Measurement-ID										PRESENCE conditional}|

-- This IE shall be present if the Registration Request for Data Collection IE is set to the value "stop".

{ ID id-RegistrationRequestForDataCollection		CRITICALITY reject	TYPE RegistrationRequestForDataCollection		PRESENCE mandatory}|

{ ID id-ReportCharacteristicsForDataCollection		CRITICALITY reject	TYPE ReportCharacteristicsForDataCollection	PRESENCE conditional}|

-- This IE shall be present if the Registration Request for Data Collection IE is set to the value "start".

{ ID id-ReportingPeriodicityForDataCollection		CRITICALITY ignore	TYPE ReportingPeriodicityForDataCollection		PRESENCE optional}|

...

}

-- **************************************************************

--

#### -- DATA COLLECTION RESPONSE

--

-- **************************************************************

DataCollectionResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionResponse-IEs}},

...

}

DataCollectionResponse-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID	PRESENCE mandatory}|

{ ID id-gNB-CU-CP-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID										PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID										PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional},

...

}

-- **************************************************************

--

#### -- DATA COLLECTION FAILURE

--

-- **************************************************************

DataCollectionFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionFailure-IEs}},

...

}

DataCollectionFailure-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID	PRESENCE mandatory}|

{ ID id-gNB-CU-CP-Measurement-ID				CRITICALITY reject	TYPE Measurement-ID									PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID				CRITICALITY reject	TYPE Measurement-ID									PRESENCE mandatory}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- DATA COLLECTION UPDATE

--

-- **************************************************************

DataCollectionUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{DataCollectionUpdate-IEs}},

...

}

DataCollectionUpdate-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID	PRESENCE mandatory}|

{ ID id-gNB-CU-CP-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID												PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID					CRITICALITY reject	TYPE Measurement-ID												PRESENCE mandatory}|

...

}

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

id-BCBearerContextNGU-TNLInfoatNGRAN-Request,

id-QoSMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-QoSMonitoringDisabled,

id-PDCP-StatusReportIndication,

id-RedundantCommonNetworkInstance,

id-redundant-nG-UL-UP-TNL-Information,

id-redundant-nG-DL-UP-TNL-Information,

id-RedundantQosFlowIndicator,

id-TSCTrafficCharacteristics,

id-ExtendedPacketDelayBudget,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-AdditionalPDCPduplicationInformation,

id-RedundantPDUSessionInformation,

id-RedundantPDUSessionInformation-used,

id-QoS-Mapping-Information,

id-MDTConfiguration,

id-TraceCollectionEntityURI,

id-EHC-Parameters,

id-DAPSRequestInfo,

id-EarlyForwardingCOUNTReq,

id-EarlyForwardingCOUNTInfo,

id-AlternativeQoSParaSetList,

id-MCG-OfferedGBRQoSFlowInfo,

id-Number-of-tunnels,

id-DataForwardingtoE-UTRANInformationList,

id-DataForwardingtoNG-RANQoSFlowInformationList,

id-MaxCIDEHCDL,

id-ignoreMappingRuleIndication,

id-EarlyDataForwardingIndicator,

id-QoSFlowsDRBRemapping,

id-SecurityIndicationModify,

id-DataForwardingSourceIPAddress,

id-M4ReportAmount,

id-M6ReportAmount,

id-M7ReportAmount,

id-PDUSession-PairID,

id-SurvivalTime,

id-UDC-Parameters,

id-SecurityIndication,

id-SecurityResult,

id-SDTindicatorSetup,

id-SDTindicatorMod,

id-DiscardTimerExtended,

id-MCForwardingResourceRequest,

id-MCForwardingResourceIndication,

id-MCForwardingResourceResponse,

id-MCForwardingResourceRelease,

id-MCForwardingResourceReleaseIndication,

id-PDCP-COUNT-Reset,

id-MBSSessionAssociatedInfoNonSupportToSupport,

id-VersionID,

id-MBSAreaSessionID,

id-Secondary-PDU-Session-Data-Forwarding-Information,

id-MBSSessionResourceNotification,

id-MCBearerContextInactivityTimer,

id-MCBearerContextStatusChange,

id-SpecialTriggeringPurpose,

id-F1UTunnelNotEstablished,

id-PDUSetQoSParameters,

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-ECNMarkingorCongestionInformationReportingStatus,

id-PDUSetbasedHandlingIndicator,

id-IndirectPathIndication,

id-F1U-TNL-InfoToAdd-List,

id-F1U-TNL-InfoAdded-List,

id-F1U-TNL-InfoToAddOrModify-List,

id-F1U-TNL-InfoAddedOrModified-List,

id-F1U-TNL-InfoToRelease-List,

id-BroadcastF1U-ContextReferenceE1,

id-PSIbasedDiscardTimer,

id-UserPlaneErrorIndicator,

id-MaximumDataBurstVolume,

id-PDCPSNGapReport,

id-UserPlaneFailureIndication,

maxnoofMBSAreaSessionIDs,

maxnoofSharedNG-UTerminations,

maxnoofMRBs,

maxnoofMBSSessionIDs,

maxnoofQoSParaSets,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofSPLMNs,

maxnoofMDTPLMNs,

maxnoofExtSliceItems,

maxnoofDataForwardingTunneltoE-UTRAN,

maxnoofExtNRCGI,

maxnoofECGI,

maxnoofSMBRValues,

maxnoofCellsforMBS,

maxnoofTAIforMBS,

maxnoofMBSServiceAreaInformation,

maxnoofDUs

FROM E1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM E1AP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-SingleContainer{},

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers;

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

#### -- R

RANUEID ::= OCTET STRING (SIZE (8))

RAT-Type	::=	ENUMERATED	{

e-UTRA,

nR,

...

}

RedundantQoSFlowIndicator::= ENUMERATED {true,false}

RedundantPDUSessionInformation ::= SEQUENCE {

rSN					RSN,

iE-Extensions		ProtocolExtensionContainer { {RedundantPDUSessionInformation-ExtIEs} }	OPTIONAL,

...

}

RedundantPDUSessionInformation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-PDUSession-PairID	CRITICALITY ignore	EXTENSION PDUSession-PairID	PRESENCE optional	},

...

}

RSN ::= ENUMERATED {v1, v2, ...}

RetainabilityMeasurementsInfo	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Removed-Item

RegistrationRequest	::=	ENUMERATED	{

start,

stop,

...

}

RegistrationRequestForDataCollection ::= ENUMERATED {start, stop, ... }

ReportCharacteristics	::=	BIT STRING (SIZE(36))

ReportCharacteristicsForDataCollection ::= BIT STRING(SIZE(32))

ReportingPeriodicity	::=	ENUMERATED	{

ms500, ms1000, ms2000, ms5000, ms10000, ms20000, ms30000, ms40000, ms50000, ms60000, ms70000, ms80000, ms90000, ms100000, ms110000, ms120000,

...

}

ReportingPeriodicityForDataCollection ::= ENUMERATED {

half-thousand-ms,

one-thousand-ms,

two-thousand-ms,

five-thousand-ms,

ten-thousand-ms,

...

}

RequestedAction4AvailNGUTermination ::= ENUMERATED {

apply-available-configuration,

apply-requested-configuration,

...,

apply-available-configuration-if-same-as-requested

}

RLC-Mode	::=	ENUMERATED	{

rlc-tm,

rlc-am,

rlc-um-bidirectional,

rlc-um-unidirectional-ul,

rlc-um-unidirectional-dl,

...

}

ROHC-Parameters	::= CHOICE {

rOHC					ROHC,

uPlinkOnlyROHC			UplinkOnlyROHC,

choice-Extension		ProtocolIE-SingleContainer { { ROHC-Parameters-ExtIEs} }

}

ROHC-Parameters-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

ROHC	::= SEQUENCE {

maxCID							INTEGER (0..16383, ...),

rOHC-Profiles					INTEGER (0..511, ...),

continueROHC					ENUMERATED {true, ...}								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { ROHC-ExtIEs } } 		OPTIONAL

}

ROHC-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

#### -- U

UDC-Parameters	::= SEQUENCE {

bufferSize					BufferSize,

dictionary					Dictionary											OPTIONAL,

continueUDC					ENUMERATED {true, ...}								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UDC-Parameters-ExtIEs } } 		OPTIONAL

}

UDC-Parameters-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-VersionID CRITICALITY ignore EXTENSION INTEGER (0..15) PRESENCE optional},

...

}

UE-Activity	::=	ENUMERATED	{

active,

not-active,

...

}

UE-associatedLogicalE1-ConnectionItem ::= SEQUENCE {

gNB-CU-CP-UE-E1AP-ID		GNB-CU-CP-UE-E1AP-ID	 OPTIONAL,

gNB-CU-UP-UE-E1AP-ID		GNB-CU-UP-UE-E1AP-ID	 OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UE-associatedLogicalE1-ConnectionItemExtIEs} }	OPTIONAL,

...

}

UE-associatedLogicalE1-ConnectionItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UESliceMaximumBitRateList ::= SEQUENCE (SIZE(1.. maxnoofSMBRValues)) OF UESliceMaximumBitRateItem

UESliceMaximumBitRateItem ::= SEQUENCE {

sNSSAI		SNSSAI,

uESliceMaximumBitRateDL		BitRate,

iE-Extensions		ProtocolExtensionContainer { { UESliceMaximumBitRateItem-ExtIEs} } OPTIONAL,

...

}

UESliceMaximumBitRateItem-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-reset														ProcedureCode ::= 0

id-errorIndication												ProcedureCode ::= 1

id-privateMessage												ProcedureCode ::= 2

id-gNB-CU-UP-E1Setup											ProcedureCode ::= 3

id-gNB-CU-CP-E1Setup											ProcedureCode ::= 4

id-gNB-CU-UP-ConfigurationUpdate								ProcedureCode ::= 5

id-gNB-CU-CP-ConfigurationUpdate								ProcedureCode ::= 6

id-e1Release													ProcedureCode ::= 7

id-bearerContextSetup											ProcedureCode ::= 8

id-bearerContextModification									ProcedureCode ::= 9

id-bearerContextModificationRequired							ProcedureCode ::= 10

id-bearerContextRelease											ProcedureCode ::= 11

id-bearerContextReleaseRequest									ProcedureCode ::= 12

id-bearerContextInactivityNotification							ProcedureCode ::= 13

id-dLDataNotification											ProcedureCode ::= 14

id-dataUsageReport												ProcedureCode ::= 15

id-gNB-CU-UP-CounterCheck										ProcedureCode ::= 16

id-gNB-CU-UP-StatusIndication									ProcedureCode ::= 17

id-uLDataNotification											ProcedureCode ::= 18

id-mRDC-DataUsageReport											ProcedureCode ::= 19

id-TraceStart													ProcedureCode ::= 20

id-DeactivateTrace												ProcedureCode ::= 21

id-resourceStatusReportingInitiation							ProcedureCode ::= 22

id-resourceStatusReporting										ProcedureCode ::= 23

id-iAB-UPTNLAddressUpdate										ProcedureCode ::= 24

id-CellTrafficTrace												ProcedureCode ::= 25

id-earlyForwardingSNTransfer									ProcedureCode ::= 26

id-gNB-CU-CPMeasurementResultsInformation						ProcedureCode ::= 27

id-iABPSKNotification											ProcedureCode ::= 28

id-BCBearerContextSetup											ProcedureCode ::= 29

id-BCBearerContextModification									ProcedureCode ::= 30

id-BCBearerContextModificationRequired							ProcedureCode ::= 31

id-BCBearerContextRelease										ProcedureCode ::= 32

id-BCBearerContextReleaseRequest								ProcedureCode ::= 33

id-MCBearerContextSetup											ProcedureCode ::= 34

id-MCBearerContextModification									ProcedureCode ::= 35

id-MCBearerContextModificationRequired							ProcedureCode ::= 36

id-MCBearerContextRelease										ProcedureCode ::= 37

id-MCBearerContextReleaseRequest								ProcedureCode ::= 38

id-MCBearerNotification											ProcedureCode ::= 39

id-dataCollectionReportingInitiation							ProcedureCode ::= X1

id-dataCollectionReporting										ProcedureCode ::= X2

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofErrors								INTEGER ::= 256

maxnoofSPLMNs								INTEGER ::= 12

maxnoofSliceItems							INTEGER ::= 1024

maxnoofIndividualE1ConnectionsToReset		INTEGER ::= 65536

maxnoofEUTRANQOSParameters					INTEGER ::= 256

maxnoofNGRANQOSParameters					INTEGER ::= 256

maxnoofDRBs									INTEGER	::= 32

maxnoofNRCGI								INTEGER	::= 512

maxnoofPDUSessionResource					INTEGER	::= 256

maxnoofQoSFlows								INTEGER	::= 64

maxnoofUPParameters							INTEGER	::= 8

maxnoofCellGroups							INTEGER	::= 4

maxnooftimeperiods							INTEGER	::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofTLAs									INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofTNLAddresses							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofDataForwardingTunneltoE-UTRAN		INTEGER ::= 256

maxnoofExtNRCGI 							INTEGER	::= 16384

maxnoofPSKs									INTEGER	::= 256

maxnoofECGI 								INTEGER	::= 512

maxnoofSMBRValues							INTEGER	::= 8

maxnoofMBSAreaSessionIDs					INTEGER ::= 256

maxnoofSharedNG-UTerminations				INTEGER ::= 8

maxnoofMRBs									INTEGER ::= 32

maxnoofMBSSessionIDs						INTEGER ::= 512

maxnoofCellsforMBS							INTEGER ::= 512

maxnoofTAIforMBS							INTEGER ::= 512

maxnoofMBSServiceAreaInformation			INTEGER ::= 256

maxnoofDUs									INTEGER ::= 512

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-gNB-CU-CP-UE-E1AP-ID 										ProtocolIE-ID ::= 2

id-gNB-CU-UP-UE-E1AP-ID											ProtocolIE-ID ::= 3

id-ResetType													ProtocolIE-ID ::= 4

id-UE-associatedLogicalE1-ConnectionItem						ProtocolIE-ID ::= 5

id-UE-associatedLogicalE1-ConnectionListResAck					ProtocolIE-ID ::= 6

id-gNB-CU-UP-ID													ProtocolIE-ID ::= 7

id-gNB-CU-UP-Name												ProtocolIE-ID ::= 8

id-gNB-CU-CP-Name												ProtocolIE-ID ::= 9

id-CNSupport													ProtocolIE-ID ::= 10

id-SupportedPLMNs												ProtocolIE-ID ::= 11

id-TimeToWait													ProtocolIE-ID ::= 12

id-SecurityInformation											ProtocolIE-ID ::= 13

id-UEDLAggregateMaximumBitRate									ProtocolIE-ID ::= 14

id-System-BearerContextSetupRequest								ProtocolIE-ID ::= 15

id-System-BearerContextSetupResponse							ProtocolIE-ID ::= 16

id-BearerContextStatusChange									ProtocolIE-ID ::= 17

id-System-BearerContextModificationRequest						ProtocolIE-ID ::= 18

id-System-BearerContextModificationResponse						ProtocolIE-ID ::= 19

id-System-BearerContextModificationConfirm						ProtocolIE-ID ::= 20

id-System-BearerContextModificationRequired						ProtocolIE-ID ::= 21

id-DRB-Status-List												ProtocolIE-ID ::= 22

id-ActivityNotificationLevel									ProtocolIE-ID ::= 23

id-ActivityInformation											ProtocolIE-ID ::= 24

id-Data-Usage-Report-List										ProtocolIE-ID ::= 25

id-New-UL-TNL-Information-Required								ProtocolIE-ID ::= 26

id-GNB-CU-CP-TNLA-To-Add-List									ProtocolIE-ID ::= 27

id-GNB-CU-CP-TNLA-To-Remove-List								ProtocolIE-ID ::= 28

id-GNB-CU-CP-TNLA-To-Update-List								ProtocolIE-ID ::= 29

id-GNB-CU-CP-TNLA-Setup-List									ProtocolIE-ID ::= 30

id-GNB-CU-CP-TNLA-Failed-To-Setup-List							ProtocolIE-ID ::= 31

id-DRB-To-Setup-List-EUTRAN										ProtocolIE-ID ::= 32

id-DRB-To-Modify-List-EUTRAN									ProtocolIE-ID ::= 33

id-DRB-To-Remove-List-EUTRAN									ProtocolIE-ID ::= 34

id-DRB-Required-To-Modify-List-EUTRAN							ProtocolIE-ID ::= 35

id-DRB-Required-To-Remove-List-EUTRAN							ProtocolIE-ID ::= 36

id-DRB-Setup-List-EUTRAN										ProtocolIE-ID ::= 37

id-DRB-Failed-List-EUTRAN										ProtocolIE-ID ::= 38

id-DRB-Modified-List-EUTRAN										ProtocolIE-ID ::= 39

id-DRB-Failed-To-Modify-List-EUTRAN								ProtocolIE-ID ::= 40

id-DRB-Confirm-Modified-List-EUTRAN								ProtocolIE-ID ::= 41

id-PDU-Session-Resource-To-Setup-List							ProtocolIE-ID ::= 42

id-PDU-Session-Resource-To-Modify-List							ProtocolIE-ID ::= 43

id-PDU-Session-Resource-To-Remove-List							ProtocolIE-ID ::= 44

id-PDU-Session-Resource-Required-To-Modify-List					ProtocolIE-ID ::= 45

id-PDU-Session-Resource-Setup-List								ProtocolIE-ID ::= 46

id-PDU-Session-Resource-Failed-List								ProtocolIE-ID ::= 47

id-PDU-Session-Resource-Modified-List							ProtocolIE-ID ::= 48

id-PDU-Session-Resource-Failed-To-Modify-List					ProtocolIE-ID ::= 49

id-PDU-Session-Resource-Confirm-Modified-List					ProtocolIE-ID ::= 50

id-DRB-To-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 51

id-DRB-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 52

id-DRB-Failed-Mod-List-EUTRAN									ProtocolIE-ID ::= 53

id-PDU-Session-Resource-Setup-Mod-List							ProtocolIE-ID ::= 54

id-PDU-Session-Resource-Failed-Mod-List							ProtocolIE-ID ::= 55

id-PDU-Session-Resource-To-Setup-Mod-List						ProtocolIE-ID ::= 56

id-TransactionID												ProtocolIE-ID ::= 57

id-Serving-PLMN													ProtocolIE-ID ::= 58

id-UE-Inactivity-Timer											ProtocolIE-ID ::= 59

id-System-GNB-CU-UP-CounterCheckRequest							ProtocolIE-ID ::= 60

id-DRBs-Subject-To-Counter-Check-List-EUTRAN					ProtocolIE-ID ::= 61

id-DRBs-Subject-To-Counter-Check-List-NG-RAN					ProtocolIE-ID ::= 62

id-PPI															ProtocolIE-ID ::= 63

id-gNB-CU-UP-Capacity											ProtocolIE-ID ::= 64

id-GNB-CU-UP-OverloadInformation								ProtocolIE-ID ::= 65

id-UEDLMaximumIntegrityProtectedDataRate						ProtocolIE-ID ::= 66

id-PDU-Session-To-Notify-List									ProtocolIE-ID ::= 67

id-PDU-Session-Resource-Data-Usage-List							ProtocolIE-ID ::= 68

id-SNSSAI														ProtocolIE-ID ::= 69

id-DataDiscardRequired											ProtocolIE-ID ::= 70

id-OldQoSFlowMap-ULendmarkerexpected							ProtocolIE-ID ::= 71

id-DRB-QoS														ProtocolIE-ID ::= 72

id-GNB-CU-UP-TNLA-To-Remove-List								ProtocolIE-ID ::= 73

id-endpoint-IP-Address-and-Port									ProtocolIE-ID ::= 74

id-TNLAssociationTransportLayerAddressgNBCUUP					ProtocolIE-ID ::= 75

id-RANUEID														ProtocolIE-ID ::= 76

id-GNB-DU-ID													ProtocolIE-ID ::= 77

id-CommonNetworkInstance										ProtocolIE-ID ::= 78

id-NetworkInstance												ProtocolIE-ID ::= 79

id-QoSFlowMappingIndication										ProtocolIE-ID ::= 80

id-TraceActivation												ProtocolIE-ID ::= 81

id-TraceID														ProtocolIE-ID ::= 82

id-SubscriberProfileIDforRFP									ProtocolIE-ID ::= 83

id-AdditionalRRMPriorityIndex									ProtocolIE-ID ::= 84

id-RetainabilityMeasurementsInfo								ProtocolIE-ID ::= 85

id-Transport-Layer-Address-Info									ProtocolIE-ID ::= 86

id-QoSMonitoringRequest											ProtocolIE-ID ::= 87

id-PDCP-StatusReportIndication									ProtocolIE-ID ::= 88

id-gNB-CU-CP-Measurement-ID										ProtocolIE-ID ::= 89

id-gNB-CU-UP-Measurement-ID										ProtocolIE-ID ::= 90

id-RegistrationRequest											ProtocolIE-ID ::= 91

id-ReportCharacteristics										ProtocolIE-ID ::= 92

id-ReportingPeriodicity											ProtocolIE-ID ::= 93

id-TNL-AvailableCapacityIndicator								ProtocolIE-ID ::= 94

id-HW-CapacityIndicator											ProtocolIE-ID ::= 95

id-RedundantCommonNetworkInstance								ProtocolIE-ID ::= 96

id-redundant-nG-UL-UP-TNL-Information							ProtocolIE-ID ::= 97

id-redundant-nG-DL-UP-TNL-Information							ProtocolIE-ID ::= 98

id-RedundantQosFlowIndicator									ProtocolIE-ID ::= 99

id-TSCTrafficCharacteristics									ProtocolIE-ID ::= 100

id-CNPacketDelayBudgetDownlink									ProtocolIE-ID ::= 101

id-CNPacketDelayBudgetUplink									ProtocolIE-ID ::= 102

id-ExtendedPacketDelayBudget									ProtocolIE-ID ::= 103

id-AdditionalPDCPduplicationInformation							ProtocolIE-ID ::= 104

id-RedundantPDUSessionInformation								ProtocolIE-ID ::= 105

id-RedundantPDUSessionInformation-used							ProtocolIE-ID ::= 106

id-QoS-Mapping-Information										ProtocolIE-ID ::= 107

id-DLUPTNLAddressToUpdateList									ProtocolIE-ID ::= 108

id-ULUPTNLAddressToUpdateList									ProtocolIE-ID ::= 109

id-NPNSupportInfo												ProtocolIE-ID ::= 110

id-NPNContextInfo												ProtocolIE-ID ::= 111

id-MDTConfiguration												ProtocolIE-ID ::= 112

id-ManagementBasedMDTPLMNList									ProtocolIE-ID ::= 113

id-TraceCollectionEntityIPAddress								ProtocolIE-ID ::= 114

id-PrivacyIndicator												ProtocolIE-ID ::= 115

id-TraceCollectionEntityURI										ProtocolIE-ID ::= 116

id-URIaddress													ProtocolIE-ID ::= 117

id-EHC-Parameters												ProtocolIE-ID ::= 118

id-DRBs-Subject-To-Early-Forwarding-List						ProtocolIE-ID ::= 119

id-DAPSRequestInfo												ProtocolIE-ID ::= 120

id-CHOInitiation												ProtocolIE-ID ::= 121

id-EarlyForwardingCOUNTReq										ProtocolIE-ID ::= 122

id-EarlyForwardingCOUNTInfo										ProtocolIE-ID ::= 123

id-AlternativeQoSParaSetList									ProtocolIE-ID ::= 124

id-ExtendedSliceSupportList										ProtocolIE-ID ::= 125

id-MCG-OfferedGBRQoSFlowInfo									ProtocolIE-ID ::= 126

id-Number-of-tunnels											ProtocolIE-ID ::= 127

id-DRB-Measurement-Results-Information-List						ProtocolIE-ID ::= 128

id-Extended-GNB-CU-CP-Name										ProtocolIE-ID ::= 129

id-Extended-GNB-CU-UP-Name										ProtocolIE-ID ::= 130

id-DataForwardingtoE-UTRANInformationList						ProtocolIE-ID ::= 131

id-QosMonitoringReportingFrequency								ProtocolIE-ID ::= 132

id-QoSMonitoringDisabled										ProtocolIE-ID ::= 133

id-AdditionalHandoverInfo										ProtocolIE-ID ::= 134

id-Extended-NR-CGI-Support-List									ProtocolIE-ID ::= 135

id-DataForwardingtoNG-RANQoSFlowInformationList					ProtocolIE-ID ::= 136

id-MaxCIDEHCDL													ProtocolIE-ID ::= 137

id-ignoreMappingRuleIndication									ProtocolIE-ID ::= 138

id-DirectForwardingPathAvailability								ProtocolIE-ID ::= 139

id-EarlyDataForwardingIndicator									ProtocolIE-ID ::= 140

id-QoSFlowsDRBRemapping											ProtocolIE-ID ::= 141

id-DataForwardingSourceIPAddress								ProtocolIE-ID ::= 142

id-SecurityIndicationModify										ProtocolIE-ID ::= 143

id-IAB-Donor-CU-UPPSKInfo										ProtocolIE-ID ::= 144

id-ECGI-Support-List											ProtocolIE-ID ::= 145

id-MDTPollutedMeasurementIndicator								ProtocolIE-ID ::= 146

id-M4ReportAmount 												ProtocolIE-ID ::= 147

id-M6ReportAmount 												ProtocolIE-ID ::= 148

id-M7ReportAmount 												ProtocolIE-ID ::= 149

id-UESliceMaximumBitRateList									ProtocolIE-ID ::= 150

id-PDUSession-PairID											ProtocolIE-ID ::= 151

id-SurvivalTime													ProtocolIE-ID ::= 152

id-UDC-Parameters												ProtocolIE-ID ::= 153

id-SCGActivationStatus											ProtocolIE-ID ::= 154

id-GNB-CU-CP-MBS-E1AP-ID										ProtocolIE-ID ::= 155

id-GNB-CU-UP-MBS-E1AP-ID										ProtocolIE-ID ::= 156

id-GlobalMBSSessionID											ProtocolIE-ID ::= 157

id-BCBearerContextToSetup										ProtocolIE-ID ::= 158

id-BCBearerContextToSetupResponse								ProtocolIE-ID ::= 159

id-BCBearerContextToModify										ProtocolIE-ID ::= 160

id-BCBearerContextToModifyResponse								ProtocolIE-ID ::= 161

id-BCBearerContextToModifyRequired								ProtocolIE-ID ::= 162

id-BCBearerContextToModifyConfirm								ProtocolIE-ID ::= 163

id-MCBearerContextToSetup										ProtocolIE-ID ::= 164

id-MCBearerContextToSetupResponse								ProtocolIE-ID ::= 165

id-MCBearerContextToModify										ProtocolIE-ID ::= 166

id-MCBearerContextToModifyResponse								ProtocolIE-ID ::= 167

id-MCBearerContextToModifyRequired								ProtocolIE-ID ::= 168

id-MCBearerContextToModifyConfirm								ProtocolIE-ID ::= 169

id-MBSMulticastF1UContextDescriptor								ProtocolIE-ID ::= 170

id-gNB-CU-UP-MBS-Support-Info									ProtocolIE-ID ::= 171

id-SecurityIndication											ProtocolIE-ID ::= 172

id-SecurityResult												ProtocolIE-ID ::= 173

id-SDTContinueROHC												ProtocolIE-ID ::= 174

id-SDTindicatorSetup											ProtocolIE-ID ::= 175

id-SDTindicatorMod												ProtocolIE-ID ::= 176

id-DiscardTimerExtended											ProtocolIE-ID ::= 177

id-ManagementBasedMDTPLMNModificationList						ProtocolIE-ID ::= 178

id-MCForwardingResourceRequest									ProtocolIE-ID ::= 179

id-MCForwardingResourceIndication								ProtocolIE-ID ::= 180

id-MCForwardingResourceResponse									ProtocolIE-ID ::= 181

id-MCForwardingResourceRelease									ProtocolIE-ID ::= 182

id-MCForwardingResourceReleaseIndication						ProtocolIE-ID ::= 183

id-PDCP-COUNT-Reset												ProtocolIE-ID ::= 184

id-MBSSessionAssociatedInfoNonSupportToSupport					ProtocolIE-ID ::= 185

id-VersionID													ProtocolIE-ID ::= 186

id-InactivityInformationRequest									ProtocolIE-ID ::= 187

id-UEInactivityInformation										ProtocolIE-ID ::= 188

id-MBSAreaSessionID												ProtocolIE-ID ::= 189

id-Secondary-PDU-Session-Data-Forwarding-Information			ProtocolIE-ID ::= 190

id-MBSSessionResourceNotification								ProtocolIE-ID ::= 191

id-MCBearerContextInactivityTimer								ProtocolIE-ID ::= 192

id-MCBearerContextStatusChange									ProtocolIE-ID ::= 193

id-MT-SDT-Information 											ProtocolIE-ID ::= 194

id-MT-SDT-Information-Request									ProtocolIE-ID ::= 195

id-SDT-data-size-threshold										ProtocolIE-ID ::= 196

id-SDT-data-size-threshold-Crossed								ProtocolIE-ID ::= 197

id-SpecialTriggeringPurpose										ProtocolIE-ID ::= 198

id-AssociatedSessionID											ProtocolIE-ID ::= 199

id-MBS-ServiceArea												ProtocolIE-ID ::= 200

id-PDUSetQoSParameters											ProtocolIE-ID ::= 201

id-N6JitterInformation											ProtocolIE-ID ::= 202

id-ECNMarkingorCongestionInformationReportingRequest			ProtocolIE-ID ::= 203

id-ECNMarkingorCongestionInformationReportingStatus				ProtocolIE-ID ::= 204

id-PDUSetbasedHandlingIndicator 								ProtocolIE-ID ::= 205

id-IndirectPathIndication										ProtocolIE-ID ::= 206

id-F1UTunnelNotEstablished										ProtocolIE-ID ::= 207

id-F1U-TNL-InfoToAdd-List										ProtocolIE-ID ::= 208

id-F1U-TNL-InfoAdded-List										ProtocolIE-ID ::= 209

id-F1U-TNL-InfoToAddOrModify-List								ProtocolIE-ID ::= 210

id-F1U-TNL-InfoAddedOrModified-List								ProtocolIE-ID ::= 211

id-F1U-TNL-InfoToRelease-List									ProtocolIE-ID ::= 212

id-BroadcastF1U-ContextReferenceE1								ProtocolIE-ID ::= 213

id-PSIbasedDiscardTimer											ProtocolIE-ID ::= 214

id-UserPlaneErrorIndicator										ProtocolIE-ID ::= 215

id-MaximumDataBurstVolume 										ProtocolIE-ID ::= 216

id-BCBearerContextNGU-TNLInfoatNGRAN-Request					ProtocolIE-ID ::= 217

id-PDCPSNGapReport												ProtocolIE-ID ::= 218

id-UserPlaneFailureIndication									ProtocolIE-ID ::= 219

id-RegistrationRequestForDataCollection							ProtocolIE-ID ::= XX1

id-ReportCharacteristicsForDataCollection						ProtocolIE-ID ::= XX2

id-ReportingPeriodicityForDataCollection						ProtocolIE-ID ::= XX3

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>>>