3GPP TSG-RAN WG3 #127bis	R3-252498

# Wuhan, China, 7th- 11th April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1436 | rev | 6 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Network Energy Saving Enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, CMCC, Samsung, Qualcomm, Nokia, ZTE, Lenovo, Deutsche Telekom, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-04-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specification should support Rel 19 Network Energy Saving Enhancement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce one new class 1 procedure <Name FFS> for providing UL WUS Configuration Information from NES Cell gNB to Cell A gNB. Introduce one new class 2 procedure <Name FFS> from Cell A gNB to NES Cell gNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for Rel-19 Network Energy Saving Enhancement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x,x, 8.x..x.1, 8.x.x.2, 8.x.x.3, 8.x.x.4, 8.x.y, 8.x.y.1, 8.x.y.2, 8.x.y.3, 8.x.y.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR 1531 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 6: Include the agreed TP R3-252467 Rev5: Update with “one user” in the track change. Rev4: BL CR submission to R3#127bis. Rebase on the new specification Rev3: Include the agreed TP R3- R3-250912 Rev2: BL CR submission to R3#127 |  |  |  |  |  |  |  |  |

Editor’s Note: The procedure, message and the IE names are FFS

Editors Note: How to address the role of the NG-RAN node1 and NG-RAN node2 to be further discussed.

### 8.x.x	UL WUS Configuration Provision (FFS)

#### 8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2 “and request NG-RAN node2 to transmit UL WUS configuration information (FFS)”.

“The procedure is also used to enable an NG-RAN node1 to request NG-RAN node2 to stop transmission of UL WUS configuration.  (FFS)”

The procedure uses non UE-associated signaling.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

If the “start with UL WUS Configuration” (details on this codepoint/choice of “start with UL WUS Configuration” are FFS) is included in the UL WUS CONFIGURATION PROVISION REQUEST (Naming FFS) message, the Cell A gNB (Naming FFS) shall, if supported, broadcast UL WUS Configuration (Naming FFS) in SIBx (SIB naming FFS) and reply with the UL WUS CONFIGURATION PROVISION RESPONSE (Naming FFS) message. The Cell A gNB (Naming FFS) stores the UL WUS configuration information (Naming FFS).

If the “stop” is included in the UL WUS CONFIGURATION PROVISION REQUEST (Naming FFS) message, the Cell A gNB (Naming FFS) shall, if supported, stop broadcasting UL WUS Configuration (Naming FFS) in SIBx (SIB naming FFS). Cell A gNB (Naming FFS) removes the 0received UL WUS Configuration.

“Cell A stores the UL WUS configuration information after it has been requested to be discontinued (FFS)”

“Cell A removes the UL WUS configuration information after it has been requested to be stopped (FFS)”

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

If the Cell A gNB (Naming FFS) cannot broadcast UL WUS Configuration in SIBx (Naming FFS), or a failure occurs during UL WUS Configuration Provision (Naming FFS), the Cell A gNB (Naming FFS) shall send UL WUS CONFIGURATION PROVISION FAILURE (Naming FFS) message. The message shall contain the Cause IE with an appropriate value.

#### 8.x.x.4	Abnormal Conditions

Void.

### 8.x.y	UL WUS Configuration Transmission Status Update (FFS)

#### 8.x.y.1	General

The procedure uses non UE-associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Transmission Status Update, successful operation

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

*************** Next change ***************

### 9.x.x.x	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request or stop provisioning of an UL WUS configuration in one or more cells.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >> UL WUS Configuration | M |  | OCTET STRING | Includes the xyz message as defined in subclause a.b.c of TS 38.331 [10]. |  |  |
| >Stop |  |  |  |  |  |  |