3GPP TSG-RAN WG3 Meeting #127	R3-250200

Athens, Greece, 17-21 Feb, 2025

Title:	(TPs to TS 38.410 TS 38.413 BL CRs) A-IoT Command procedures

Agenda Item:	16.2

Source:	Huawei, China Telecom, China Unicom

Document for:	other

# 1	Introduction

In RAN3 part of RAN TR 38.769 [1], the candidate procedures for A-IoT Command for Topology 1 were captured. In R19 RAN WID [2], RAN3 scope is concluded:

R19 RAN WID: - RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

Based on the current study item progress and R19 RAN WID, this contribution futher discuss the signaling and procedures over CN-RAN interface to support A-IoT Command, and provide the corresponding TPs.

# 2	Discussion

## 2.1	Command procedure after inventory procedure

Based on RAN TR 38.769, the Command operation is supported by command procedure after Inventory procedure over NG interface:

Figure 6.5.2.2-1: Message flow for A-IoT Command in Topology 1

It is also noticed that in the TR, it is said that “Command can be sent by the A-IoT CN for a single device”.

Based on these, to support the command operation of a device, we need to introduce a Class 1 Command Request procedure, including COMMAND REQUEST, COMMAND RESPONSE and COMMAND FAILURE messages.

- Proposal 1: To support the command operation of a device, introduce a Class 1 Command Request procedure, with COMMAND REQUEST, COMMAND RESPONSE and COMMAND FAILURE messages.

## 2.2	Device association management

In case of multiple devices inventory (group of devices or all devices), if there is command transmission afterwards, we should allow parallel transactions between the gNB and AIOTF for the command transmissions for different devices.

Proposal 2: Parallel transactions between the gNB and AIOTF for the command transmissions for different devices should be supported.

To enable parallel transactions, we either introduce transaction ID per device, or as legacy UE association, we introduce device association, and maybe we can choose the latter one as it follows the tranditional handling.

- Proposal 3: To supprot parallel transactions for the command transmissions of devices, Device association between the gNB and the AIOTF should be used.

More specifically, to establish the device association, as shown in Figure 1 below, the AIOTF can include a Command indication IE in the Inventory Request Transfer IE in INVENTORY REQUEST message to indicates there is subsequent command. Upon receiving the Command indication IE, the gNB allocates RAN Device ID and includes it in INVENTORY REPORT message together with the NAS PDU (Device Identifier) received from the device. Then, AIOTF allocates CN Device ID and includes the RAN/CN device ID pair in the COMMAND REQUEST message for this device.

Figure 1: Device association in Command operation

- Proposal 4: AIOTF indicates whether there is subsequent command in the INVENTORY REQUEST message, via an optional Command Indication IE.

- Proposal 5: The gNB allocates RAN Device ID and includes it in INVENTORY REPORT message together with the NAS PDU (Device Identifier) received from the device.

- Proposal 6: The AIOTF allocates CN Device ID, and includes the RAN/CN Device ID pair in the COMMAND REQUEST message.

In case of indirect communication, in our understanding there is no need for the AMF to be aware of the device association, the command indication and the RAN/CN device ID pair should be included in the Transfer IEs which are transparent to the AMF. But we should allow parallel transactions for the commands to different devices between the gNB and the AMF, therefore, an additional transaction ID should be included outside of the AIOTF Transfer IEs.

- Proposal 7: The Command Indication and the RAN/CN device ID should be included in the Transfer IEs which are transparent to the AMF.

- Proposal 8: In indirect communication, introduce Transaction ID outside of the Transfer IEs in all NGAP command messages, to allow parallel transactions between gNB and AMF for the commands of different devices after the same inventory operation.

## 2.3	Other Information in Command Request Transfer IE

As captured in current RAN TR 38.769, the gNB can differentiate between command and inventory. In RAN2#128 meeting, there are the following agreements about service type/command type:

- RAN2#128 agreements on service type in the Paging Message Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID. - RAN2#128 agreements on visibility - The reader needs to know whether a response is expected and expected D2R message size. FFS how the reader gets this information. Wait for SA2 LS response on expected D2R message size to resolve the FFS. FFS if command type is explicit, if it can be inferred from expected D2R message size if available. - Ask SA2 if they can provide information that RAN2 agreed on some information that are useful for the reader and ask whether they can be provided (e.g. service type). Ask SA2 if there are requirements from SA2 perspective (e.g. on latency of completion time of procedure)

And the up-to-date SA2 TR 13.700-13 also captured the following related conclution:

- SA2 TR 13.700-13: - 2. The AIoTF performs the following functionality. e. The AIOTF may provide the following assistance information to RAN/UE Reader: - AIoT service type (e.g. Inventory, Command) - approximate number of AIoT devices based on AF request - approximate D2R message size based on AF request The AF provides one or some of the following information: - Information about AIoT services - Service type indicating e.g., inventory or command (e.g. read or write) - Information to be used for AIoT reader selection - At least one of the UE reader ID or the target area information - Optionally, Information about the target AIoT device(s) - At least one of the AIoT device ID(s) or the filtering information which can be used to associate with multiple AIoT devices. - Optionally, Information to be used for resource allocation that is detailed in TR 38.769 [8] - Approximate number of AIoT devices - Approximate D2R message size

Based on SA2 TR conclution, when AF provides command type (e.g. read or write) to AIOTF, the AIOTF can indicate the information to the gNB in COMMAND REQUEST message. Then, the gNB can differentiate command type and take it into account for scheduling.

- Proposal 9: Introduce Command Type (i.e., read/write/disable) in Command Request Transfer IE in the COMMAND REQUEST message.

- In RAN2#128 meeting, there is the following agreements:

- RAN2#128 agreements on visibility: - The reader needs to know whether a response is expected and expected D2R message size. FFS how the reader gets this information. Wait for SA2 LS response on expected D2R message size to resolve the FFS. FFS if command type is explicit, if it can be inferred from expected D2R message size if available. - Ask SA2 if they can provide information that RAN2 agreed on some information that are useful for the reader and ask whether they can be provided (e.g. service type). Ask SA2 if there are requirements from SA2 perspective (e.g. on latency of completion time of procedure)

And current RAN TR 38.769 captured two options on how a reader knows D2R message size, including 1) from the CN and 2) from the device. As an alternative approach, AIOTF can indicate the expected data size of the following D2R transmission to the gNB when sending INVENTORY REQUEST message, then the gNB can schedule the corresponding D2R message according to the CN’s indication, e.g., indicating TBS together with transmitting A-IoT paging messages.

RAN2 has sent LS to SA2 in R2-2409412, to ask whether CN can provide D2R message size to the reader. As described in SA2 LS reply S2-2413035 and conclusions captured in the up-to-date SA2 TR 23.700-13, there is no final conclusion on whether AIOTF can provide D2R message size if AF does not provide it.

SA2 LS reply S2-2413035: Q1: Can the CN provide, to the reader, an estimate of the expected size of the following D2R message(s) in response to the service request? SA2 answer: If provided by AF, AIOTF provides the reader with the approximate D2R message size based on AF request. Whether the CN can determine the approximate D2R message size if not provided by AF is under discussion in SA2.

SA2 TR 13.700-13: The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work: Editor's note: Final conclusions are assumed to be taken in coordination with RAN WGs. 1. A new core network function is introduced to support Ambient IoT (e.g. AIoTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality. e. The AIOTF may provide the following assistance information to RAN/UE Reader: - AIoT service type (e.g. Inventory, Command); - approximate number of AIoT devices based on AF request; - approximate D2R message size based on AF request.

SA2 LS S2-2501241: Q1: Is it feasible for CN to provide the following information to the reader? - the service type of A-IoT (e.g. inventory, command) - whether the service is targeted for a single or multiple devices; - the approximate number of target devices (if available). SA2 answer: CN can provide the information listed above to the reader if available. Q2: Can there be a case where a D2R response is not expected for a R2D message for command service type? If so, can the CN indicate to the reader whether a message is expected in D2R direction in response to an R2D message for command service type (e.g., either explicitly or implicitly by indicating the response message size if available)? SA2 answer: No, a D2R response is always expected for a R2D message for command service type (e.g., the service data response for 'Read' service operation, and the operation result indication response for 'Write' or 'Disable' service operation).

- Proposal 10: Introduce “expected D2R message size” in Command Request Transfer IE in the COMMAND REQUEST message.

# 3	Conclusion

In this contribution, we further discussed the Command procedure over CN interface, and got the following proposals:

- Proposal 1: To support the command operation of a device, introduce a Class 1 Command Request procedure, with COMMAND REQUEST, COMMAND RESPONSE and COMMAND FAILURE messages.

Proposal 2: Parallel transactions between the gNB and AIOTF for the command transmissions for different devices should be supported.

- Proposal 3: To supprot parallel transactions for the command transmissions of devices, Device association between the gNB and the AIOTF should be used.

- Proposal 4: AIOTF indicates whether there is subsequent command in the INVENTORY REQUEST message, via an optional Command Indication IE.

- Proposal 5: The gNB allocates RAN Device ID and includes it in INVENTORY REPORT message together with the NAS PDU (Device Identifier) received from the device.

- Proposal 6: The AIOTF allocates CN Device ID, and includes the RAN/CN Device ID pair in the COMMAND REQUEST message.

- Proposal 7: The Command Indication and the RAN/CN device ID should be included in the Transfer IEs which are transparent to the AMF.

- Proposal 8: In indirect communication, introduce Transaction ID outside of the Transfer IEs in all NGAP command messages, to allow parallel transactions between gNB and AMF for the commands of different devices after the same inventory operation.

- Proposal 9: Introduce Command Type (i.e., read/write/disable) in Command Request Transfer IE in the COMMAND REQUEST message.

- Proposal 10: Introduce “expected D2R message size” in Command Request Transfer IE in the COMMAND REQUEST message.

- The corresponding TPs to TS 38.410 and TS38.413 BL CRs are provided in section 5 and section 6.

# 4. Reference

[1] 3GPP TR 38.769 V2.0.1, Study on solutions for ambient IoT (Internet of Things), 2024-12

- [2] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Dec. 2024

- [3] 3GPP TR 23.700-13 V1.3.0, Study on Architecture support of Ambient power-enabled Internet of Things (Release 19), 2025-01

# 5. TPs for TS 38.410 BL CR

---------------Start of the First Change---------------

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AIOTF: as defined in TS 23.501 [8].

gNB: as defined in TS 38.300 [7].

NB-IoT: as defined in TS36.300 [11].

ng-eNB: as defined in TS 38.300 [7].

NG-RAN node: as defined in TS 38.300 [7].

UPF: as defined in TS 23.501 [8].

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

A-IoT	Ambient IoT

AIOTF	Ambient IoT Function

AMF	Access and Mobility Management Function

CIoT	Cellular IoT

CN	Core Network

DRX	Discontinuous Reception

MBS	Multicast/Broadcast Service

MT	Mobile Terminated

NB-IoT	Narrow Band Internet of Things

NG-U	NG User plane interface

PTP	Point to Point

PTM	Point to Multipoint

QMC	QoE Measurement Collection

QoE	Quality of Experience

RIM	Remote Interference Management

SMF	Session Management Function

UP	User Plane

UPF	User Plane Function

---------------Start of the Next Change---------------

## 5.y	Command function

The Command function enables the 5GC to request the NG-RAN node to transfer the command towards an A-IoT device, and enables the NG-RAN node to report the outcome for the Command Session over the NG interface.

---------------Start of the Next Change---------------

## 6.y	Command Procedure after Inventory Procedure

The following procedures are used by the 5GC to request the NG-RAN node to transfer the command towards an A-IoT device, and forward the command response from the A-IoT device, after Inventory procedure:

-	Command Request procedure.

# 5. TPs for TS 38.413 BL CR

---------------Start of the First Change---------------

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

ACL	Access Control List

A-IoT	Ambient IoT

AIOTF	Ambient IoT	 Function

AMF	Access and Mobility Management Function

AUN3	Authenticable Non-3GPP

CAG	Closed Access Group

CGI	Cell Global Identifier

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DC	Dual Connectivity

DL	Downlink

EPC	Evolved Packet Core

FN-RG	Fixed Network Residential Gateway

GUAMI	Globally Unique AMF Identifier

HFC	Hybrid Fiber-Coax

IAB	Integrated Access and Backhaul

IOT	Internet of Things

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

MBS	Multicast/Broadcast Service

MT	Mobile Terminated

N3IWF	Non 3GPP InterWorking Function

NB-IoT	Narrow Band Internet of Things

NGAP	NG Application Protocol

---------------Start of the Next Change---------------

# 8	NGAP Procedures

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Setup | BROADCAST SESSION SETUP REQUEST | BROADCAST SESSION SETUP RESPONSE | BROADCAST SESSION SETUP FAILURE |
| Broadcast Session Modification | BROADCAST SESSION MODIFICATION REQUEST | BROADCAST SESSION MODIFICATION RESPONSE | BROADCAST SESSION MODIFICATION FAILURE |
| Broadcast Session Release | BROADCAST SESSION RELEASE REQUEST | BROADCAST SESSION RELEASE RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |
| Multicast Session Activation | MULTICAST SESSION ACTIVATION REQUEST | MULTICAST SESSION ACTIVATION RESPONSE | MULTICAST SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAST SESSION DEACTIVATION REQUEST | MULTICAST SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |
| MT Communication Handling | MT COMMUNICATION HANDLING REQUEST | MT COMMUNICATION HANDLING RESPONSE | MT COMMUNICATION HANDLING FAILURE |
| Broadcast Session Transport | BROADCAST SESSION TRANSPORT REQUEST | BROADCAST SESSION TRANSPORT RESPONSE | BROADCAST SESSION TRANSPORT FAILURE |
| Command Request | COMMAND REQUEST | COMMAND RESPONSE | COMMAND FAILURE |

---------------Start of the Next Change---------------

### 8.xx.3	Command Request

#### 8.xx.3.1	General

The purpose of the Command Request procedure is to request the NG-RAN node to transfer the command towards an A-IoT device. The procedure uses device associated signalling.

#### 8.xx.3.2	Successful Operation

Figure 8.xx.3.2-1: Command Request initiated by AIOTF directly, successful operation.

Figure 8.xx.3.2-2: Command Request initiated by AIOTF via AMF, successful operation.

The AIOTF initiates the procedure by sending a COMMAND REQUEST message to the NG-RAN node. If the NG-RAN node accepts the request, the NG-RAN node responds with the COMMAND RESPONSE message.

In case of Command Request initiated by AIOTF via AMF, the AMF shall include the AIOTF Identifier IE in the COMMAND REQUEST message, and the NG-RAN node, shall include the same value of the AIOTF Identifier IE in the COMMAND RESPONSE message.

#### 8.xx.3.3	Unsuccessful Operation

Figure 8.xx.3.3-1: Command Request, unsuccessful operation.

If the NG-RAN node is not able to transmit the received command towards the A-IoT device, it shall send the COMMAND FAILURE message with an appropriate cause value.

In case of Command Request initiated by AIOTF via AMF, the NG-RAN node shall, include same value of the AIOTF Identifier IE as received from the COMMAND REQUEST message in the COMMAND REPORT message.

#### 8.xx.3.4		Abnormal Conditions

Void.

---------------Start of the Next Change---------------

### 9.2.x		A-IoT Messages

#### 9.2.x.y5	COMMAND REQUEST

This message is used for carrying command over the NG interface.

Direction: AMF  NG-RAN node, or AIOTF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.xx0 |  | YES | reject |
| Correlation Identifier | M |  | 9.3.1.xx1 |  | YES | reject |
| AIOTF Identifier | O |  | 9.3.3.ee |  | YES | reject |
| Command Request Transfer | M |  | OCTET STRING | Containing the Command Request Transfer IE specified in subclause 9.3.6.5. | YES | reject |

#### 9.2.x.y6	COMMAND RESPONSE

This message is used for carrying the command response over the NG interface.

Direction: NG-RAN node  AMF, or NG-RAN node  AIOTF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.xx0 |  | YES | reject |
| Correlation Identifier | M |  | 9.3.1.xx1 |  | YES | reject |
| AIOTF Identifier | O |  | 9.3.3.ee |  | YES | reject |
| Command Response Transfer | M |  | OCTET STRING | Containing the Command Response Transfer IE specified in subclause 9.3.6.5. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.y7	COMMAND FAILURE

This message is used for carrying command failure over the NG interface.

Direction: AMF  NG-RAN node, or AIOTF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.xx0 |  | YES | reject |
| Correlation Identifier | M |  | 9.3.1.xx1 |  | YES | reject |
| AIOTF Identifier | O |  | 9.3.3.ee |  | YES | reject |
| Command Failure Transfer | M |  | OCTET STRING | Containing the Command Failure Transfer IE specified in subclause 9.3.6.5. | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

---------------Start of the Next Change---------------

#### 9.3.1.xx0	Transaction ID

This IE uniquely identifies a transaction.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transaction ID | M |  | FFS |  |

#### 9.3.1.xx1	Correlation Identifier

This IE uniquely identifies an A-IoT service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Correlation Identifier | M |  | OCTET STRING (SIZE(FFS)) | Encoded as defined in TS 23.003 [23]. |

---------------Start of the Next Change---------------

#### 9.3.3.aa	CN Device ID

This IE uniquely identifies the Device association within the AIOTF, as described in TS 38.401 [2].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN Device UE ID | M |  | INTEGER (0..240 -1) |  |

#### 9.3.3.bb	RAN Device ID

This IE uniquely identifies the Device association over the NG interface within the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAN Device ID | M |  | INTEGER (0..232 -1) |  |

#### 9.3.3.ee	AIOTF Identifier

This IE uniquely identifies the AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AIOTF Identifier | M |  | FFS |  |

---------------Start of the Next Change---------------

### 9.3.x	AIOTF Related IEs

#### 9.3.x.1	Inventory Request Transfer (grey part from Inventory TP)

This IE is transparent to the AMF, if via AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Device Identification for Paging | M |  | 9.3.3.xx |  |
| Inventory Target information | O |  | 9.3.1.xx2 |  |
| Inventory Assistance Information | O |  | 9.3.1.xx3 |  |
| Command Indication | O |  | ENUMERATED (true, …) |  |

#### 9.3.x.5	Command Request Transfer

This IE is transparent to the AMF, if via AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAN Device ID | M |  | 9.3.3.bb |  |
| CN Device ID | M |  | 9.3.3.aa |  |
| A-IoT NAS PDU | M |  | OCTET STRING |  |
| Command Type | O |  | ENUMERATED (read, write, disable …) |  |
| Estimate of Expected D2R Message Size | O |  | INTEGER (0..1000, …) | Unit: bit. |

#### 9.3.x.6	Command Response Transfer

This IE is transparent to the AMF, if via AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAN Device ID | M |  | 9.3.3.bb |  |
| CN Device ID | M |  | 9.3.3.aa |  |
| A-IoT NAS PDU | O |  | OCTET STRING |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

#### 9.3.x.7	Command Failure Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAN Device ID | M |  | 9.3.3.bb |  |
| CN Device ID | M |  | 9.3.3.aa |  |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |