3GPP TSG-RAN WG3 #127	R3-20xxxx

Online, 1-11 June 2020

Agenda Item:	9.2

Source:	Vodafone (moderator)

Title:	Summary of Offline Discussion on UE Context Modification Failure for LTM as in R3-250082

Document for:	Approval

# Introduction

This document aims to clarify open issues related to R3-250082. As per chairlady report:

“Check other issues raised in 0082 (the first change) and 0420 (whether a LS to RAN2 is needed)”?

# Discussion [if needed]

## Issue 1 [if needed]

Please provide your comments on R3-250082. Please concentrate on the first change:

[Gg] I inserted this table for easier discussion.

| LTM Scenario | A: Acquire candidate cell config (including CSI Report Config) | B: Acquire candidate cell config (not including CSI Report Config) | C: Acquire serving cell config (including CSI Report Config) | D: Not specified (Failure case) |
| --- | --- | --- | --- | --- |
| SpCell ID | Y | Y | N | N |
| LTM indicator | Y | Y | Y | Y |
| LTM CSI Resource Config | Y | N | Y | N |

| Company | Comment |
| --- | --- |
| Google | Per current text, the gNB-DU shall report failure for the scenarios C and D as above; however, scenario C is a legit scenario to acquire serving cell configuration, especially for the CSI report configuration. For the two changes proposed in R3-250082: - The first one is to correct for scenario C so that the gNB-DU won’t send the UE Context Modification Failure message to the gNB-CU when the gNB-CU tries to acquire CSI Report Config. The second one is to specify scenario D and clarify that the gNB-DU sends the UE Context Modification Failure message in such a case. The two changes are necessary because the gNB-DU would incorrectly report scenario C if we don’t agree on the first change. - As for the second change, if we agreed on the first change (which is definitely needed), then the gNB-DU would not report scenario D if we don’t capture it. |

## Issue 2 [if needed]

Do you agree to send an LS R3-250420. If you agree on, please provide your suggestion on how to update

| Company | Comment |
| --- | --- |
| Google | We are neutral whether to send the LS to state the situation. If we agree to send the LS, it should be clarified that for the inter-DU scenario the resource from rach-ConfigCommon is not available at the source gNB-DU. |

# Conclusion, Recommendations [if needed]

If needed

# References