3GPP TSG-RAN WG3 Meeting #127	R3-25xxxx

Athens, GR, 17-21 Feb, 2025

Agenda Item:	16.2

Source:	Huawei (moderator)

Title:	Summary of SBFD offline discussion

Document for:	Discussion and decision

# Introduction

This is a summary of discussions for the following CB, based on the submitted contributions [1]~[12].

CB: # SBFD

- Check the open issues above

- Capture agreements and provide TPs if agreeable

(moderator - HW)

Summary of offline disc R3-250803

# For the Chairman’s Notes

Issue 1: Exchange of SBFD configuration

Issue 2: Exchange of measurement configuration

Issue 3: Exchange of measurement result

Issue 4: Exchange of CLI-mitigation request

# Discussion

## Exchange of SBFD configuration

Based on the submitted contributions, the following proposals are on the table for discussion and approval:

For Xn, it seems that now RAN3 is waiting for the progress working groups, e.g. RAN2, so the thing in RAN3 now is whether to send an LS asking RAN1 and RAN2 to work on the detailed parameters and IE design or, do nothing and just wait.

Q1.1: For exchange of SBFD configuration over Xn, shall we send an LS asking RAN1 and RAN2 to work on the detailed parameters and IE design or, do nothing and just wait.

Companies are invited to express their view on Q1.1 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Send LS to RAN2, cc RAN2 | Some progress are needed, after 3 meeting. |
| ZTE | LS to RAN1， CC RAN2 | I believe the detailed signaling design is finished by RAN1, and RAN1 will send the higher layer parameter table including new added signaling to RAN2. |
| CATT | No strong view | RAN2 anyway in cc in R3-245006. RAN2 knows they should do something. |
| Ericsson | Send LS to RAN2, Cc RAN1 | RAN3 should ask RAN2 to define the SBFD configuration in RRC in a way that it can be used transparently in RAN3 specifications, just like the tdd-UL-DL-ConfigurationCommon IE contained in the ServingCellConfigCommon IE defined in TS 38.331, which is used in RAN3 specifications |

For F1, the remaining open point is how to exchange SBFD time and frequency configuration over F1, here there are two sub-points:

- the gNB-DU provide the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE over F1

- gNB-CU can provide the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in gNB-CU Configuration Update message.

Q1.2: For exchange of SBFD configuration over F1, RAN3 to discuss and agree:

- gNB-DU provide the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and

- gNB-CU can provide the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in gNB-CU Configuration Update message.

Companies are invited to express their view on Q1.2 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Agree with both proposals above in Q1.2 |  |
| ZTE | Agree |  |
| CATT | Agree | Specifically, in Neighbour Cell Information List for second bullet. |
| Ericsson | Agree |  |

## Exchange of measurement configuration

According to RAN1 LS, It seems there are different understandings on this issue, basically there are two different understanding, as follows:

- SSB and/or periodic NZP CSI-RS were already defined in RAN2, i.e. SSB already exchanged in MeasTiming field of inter-node RRC message Measurement Timing Configuration and, NZP-CSI-RS-ResourceSet in Measurement Timing Configuration but not included it in the inter-node message; with this understanding, anyway they are already defined in RAN2, so no further work needed in RAN3 but just put it in Served Cell Information NR IE with reference to RAN2;

- Since NZP CSI-RS is not included in inter-node message, RAN3 needs to discuss how to exchange NZP CSI-RS, i.e. to ask RAN2 to include it in the inter-node message.

So RAN3 needs to decide:

For Xn

- For exchange of SSB info, nothing to be done in RAN3

- For exchange of NZP CSI-RS, to put it in Served Cell Information NR IE with reference to RAN2 or, ask RAN2 to include in inter-node message?

Q2.1: For exchange of measurement configuration over Xn, there are two basic sub-points:

- Point 1: For exchange of SSB info, nothing to be done in RAN3

- Point 2: For exchange of NZP CSI-RS, to put a new IE in Served Cell Information NR IE with reference to RAN2 definition or, ask RAN2 to include in inter-node message?

Companies are invited to express their view on Q2.1 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Point 1: yes, nothing to be done in RAN3; Point 2: either way works, but prefer to adopt the former one, i.e. “to put a new IE in Served Cell Information NR IE” | This way is a pure RAN3 work, should be quicker, the detailed IE design could refer to 0470 as an example. |
| ZTE | Point1: SSB resource is exchanged via a container in Xn interface, so do not see spec impact in RAN3 Point2: slightly prefer the later one. The measurement resource of SSB and CSI-RS has been exchanged via MTC container. So prefer a unified design to exchange the NZP CSI-RS | RAN3 anyway need to send the LS to RAN1/2 to design SBFD configuration. So, do not think the former one would be quicker for RAN3. |
| CATT | Point 1: ok Point 2: put it in Served Cell Information NR IE with reference to RAN2 | RAN2 is busy We can do this by ourselves. |
| Ericsson | Point 1: ok Point 2, put a new IE in Served Cell Information NR IE with reference to RAN2 definition |  |

Then there are also some other proposals trying to guide the measurement behavior, e.g. periodical measurement, start or stop measurement, etc., while some companies think there is no such need, such behavior could be left to gNB implementations. RAN3 needs to discuss and agree:

- Whether to introduce more info in the measurement configuration info, e.g. periodical measurement, start or stop measurement, etc.

It was also proposed in some contributions that there might be a need to have an explicit procedure asking the peer node to perform CLI measurement, so that the peer node could response and update with measurement results, then RAN3 needs to discuss and decide, while some companies prefer to reuse existing procedures, e.g. NG-RAN node Configuration Update procedure:

- Whether to introduce an explicit procedure asking the peer node to perform CLI measurement

Q2.2: Two further questions need to be discussed:

- Whether to introduce more info in the measurement configuration info, e.g. periodical measurement, start or stop measurement, etc., or just leave to gNB implementation

- Whether to introduce a dedicated procedure asking the peer node to perform CLI measurement

Companies are invited to express their view on Q2.2 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Not needed for both | When gNB receives the measurement configuration, it is up to gNB implementation whether to perform measurement. |
| ZTE | We are open to this question. | We think such mechanism can provide a better CLI control and signaling optimization. |
| Charter | Yes, we should send an LS to RAN1, informing that RAN3 can provide more detailed information, which the receiver of the information, can decide how much to use. | We believe this would provide more information that would allow better CLI mitigation, and feedback to the aggressors on how whatever mitigation they engage in is actually improving the situation. Charter has contribution R3-250194 to this meeting addressing this very issue. |
| CATT | Yes for periodical measurement or event triggered measurement | It is up to aggressor gNB decides whether to perform interference mitigation. The aggressor gNB should control how often receiving strongest DL beam from victim to avoid signalling overhead or miss some strongest DL beam information, rather than victim. |
| Ericsson | No need to introduce more info in the measurement configuration info, e.g. periodical measurement. Note that the WID clearly states that “No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3”, therefore some of the proposed additions are explicitly forbidden by the WID. No need for a dedicated procedure asking the peer node to perform CLI measurement. This was discussed in RAN1 and RAN1 has not identified such requirement. |  |

For F1, the open points should be similar, thus it would be better to first conclude on Xn, then we could just refer to the conclusions reached for Xn.

## Exchange of measurement result

This issue seems to be more controversial. According to RAN1 agreement in the LS, it clearly says:

- Information exchange of strongest DL beam information

While in some contributions, it was proposed that in addition to strongest DL beam information (SSB index or NZP CSI-RS Resource Indicator), the concrete measurement result associated with the strongest DL beam should also be included, some proposals even suggested to include measurement results of other beams. Thus, RAN3 should first needs to discuss and agree:

- To exchange strongest DL beam information, i.e. SSB index or NZP CSI-RS Resource Indicator

Companies also gave the concrete format of the strongest DL beam information, i.e. to use an NZP-CSI-RS-ResourceId with INTEGER (1..64) and, the strongest beam of SSB as a SSB-Index with INTEGER (0..63).

In addition, RAN3 also needs to discuss and agree

- whether to exchange the concrete measurement result associated with the strongest DL beam; if yes, whether measurement results of other beams should also be exchanged

Thus, we should first conclude on the following two issues as described above.

Q3: For exchange of measurement result over Xn, RAN3 to discuss and agree

- Point 1: To exchange strongest DL beam information, i.e. SSB index with SSB-Index INTEGER (0..63) or NZP CSI-RS Resource Indicator INTEGER (1..64)

- Point 2: Whether to exchange the concrete measurement result associated with the strongest DL beam; if yes, whether measurement results of other beams should also be exchanged

- Companies are invited to express their view on Q3 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Agree with point 1; No need for point 2 | According to RAN1, just the index/ID of strongest beam is needed. |
| ZTE | Agree with P1; Fine with P2 | Technically speaking, concrete measurement result can provide better CLI control. Supposing that RSRP the second strongest beam is close to the first strongest beam, then reporting only one strongest beam can not eliminate the CLI. |
| Charter | Agree with P1; Agree with P2, i.e., provide results of the measured interference power. And send not only the highest interfering but a list of all beams and cells which have exceeded what the victim perceives as excessive CLI power. | This would definitely allow the receiver of the information to make a more effective decision on what measures to take to mitigate the interference. We are not mandating that the receiver take an action, but we want to definitely allow the best information to be available if the receiver decides to proactively engage in CLI mitigation. Charter has a contribution to this meeting R3-250194, which address this very issue. |
| CATT | Agree P1. Not agree P2 | We may revisit RAN1 agreement when they are not correct. But in this case, this is a feasible and clear solution agreed by RAN1. No need to further discuss. |
| Ericsson | Agree to P1 Do not agree to P2. Note that RAN1 has explicitly describe how the procedures for CLI mitigation request and Strongest DL beam reporting should work in R3-245006. RAN3 should follow the agreements from RAN1 |  |

Similarly, for F1, we could first refer to the conclusions reached for Xn.

## Exchange of CLI-mitigation request

Here the issue is very straight forward, there are mainly two sub-issues, 1) what this request looks like; 2) how to transfer this request.

For the first issue, RAN3 needs to discuss and agree if the CLI-mitigation request is just simple indication, i.e a per cell optional IE, such as an ENUMERATED value “CLI-mitigation requested”, or are there any addition info needed.

Q4: RAN3 needs to discuss and agree:

- CLI-mitigation request is just simple indication, i.e. a per cell optional IE, such as an ENUMERATED value “CLI-mitigation requested”, or are there any addition info needed.

Companies are invited to express their view on Q4 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Agree to introduce a per cell optional IE, such as an ENUMERATED value “CLI-mitigation requested”; Others are not needed |  |
| ZTE | CLI-Mitigation may also include the MTC or the on indication of CLI-RS included in served cell info. | RAN1 design two different CLI solutions. For beam nulling, victim beam sending the CLI-mitigation request will also need to configure the CLI-RS. |
| Charter | Charter has contribution R3-250194 that explains our view, which consists of sending a message indicating request for mitigation to the the particular gNB and specific cells/beams of that gNB (identified based on the RS signal detected, and the actual measured interference power caused by that RS) | Our current contribution addresses only the beam nulling situation, but we intend to bring to next meeting a contribution addressing the beam pairing scenario (we currently see it as a variation of the testing request that we currently have on our contribution R3-250194). |
| CATT | Yes | What the following behaviors after receiving CLI mitigation is based on implementation. The CLI mitigation message and measurement configuration can be sent in any order. |
| Ericsson | Agree that the CLI Mitigation request is a per cell optional IE, such as an ENUMERATED value “CLI-mitigation requested” |  |

## How to exchange measurement result and CLI-mitigation request over Xn

There is a common issue for exchange of measurement result and exchange of CLI-mitigation request, i.e., how to exchange measurement result and CLI-mitigation request. Basically, there are two options proposed, according to the submitted contributions, one is to reuse existing procedures, the other is to introduce new procedure. While for the former, there are further two sub-options, one is to reuse interface management procedures, the other is to reuse Resource Status Reporting procedure. Here the rapporteur assumes that whatever the option would be adopted, the measurement result, i.e. the strongest DL beam information and CLI-mitigation request could be included as two optional IEs in the same message.

Q5.1: RAN3 to discuss and agree to exchange the strongest DL beam information and the CLI-mitigation request as two optional IEs in the same message over Xn.

Companies are invited to express their view on Q5.1 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Agree | Anyway, both are optional IEs, and gNB should allow to provide measurement results together with CLI mitigation request. |
| ZTE | Fine, no strong view |  |
| Charter | We refer to our proposal in contribution R3-250194 that relies on the use of the existing RESOURCE STATUS REQUEST message and related messages RESOURCE STATUS RESPONSE, RESOURCE STATUS UPDATE, and RESOURCE STATUS FAILURE, with added IEs, and added values to existing IEs. | As previously mentioned, we advocate for sending not only the strongest cell/beam indication, but all cell/beam that are causing excessive CLI interference from the point of view of the victim. |
| CATT | Too early to have such conclusion. | Should be discussed after we have conclusion on Q5.2 because CLI mitigation cannot be period but strongest DL beam can be sent period. May be different procedures are used for these two IEs. |
| Ericsson | Agree, the two pieces of information can be exchanged in the Xn Setup and NG-RAN Node configuration update procedures |  |

Q5.2: RAN3 to discuss which options to support the exchange of strongest DL beam information and CLI-mitigation request over Xn:

- Option 1: To reuse interface management procedures: NG-RAN node Configuration Update procedure

- Option 2: To reuse Resource Status Reporting procedure

- Option 3: To introduce new procedures

Companies are invited to express their view on Q5.2 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Each option could work, but option 3 is preferred | Option 3, i.e. to introduce a new message seems to be a cleaner way. |
| ZTE | Option1 is not preferred, prefer option3. | Strongest DL beam and CLi-mitigation request is event triggered signaling and may happens frequently. Configuration update is to exchange the semi-static information. |
| Charter | Option 2 | Reuses the existing RESOURCE STATUS REQUEST, RESOURCE STATUS RESPONSE, RESOURCE STATUS UPDATE, RESOURCE STATUS FAILURE, with identified added IE, and added values to existing IE. The proposal in R3-250194 allows the mitigation request to be sent, the aggressor to decide if it will accept the request or not, and if the aggressor decides to engage in mitigating the interference, allows the victim to provide feedback, if additional mitigation measures needs to be continued to lower the interference level to acceptable levels from the point of view of the victim. |
| CATT | Option 2 | It is up to aggressor gNB decides whether to perform interference mitigation. The aggressor gNB should control how often receiving strongest DL beam from victim to avoid signalling overhead or miss some strongest DL beam information, rather than victim. |
| Ericsson | Option 1. Option 2 does not work for a number of reasons. One of them being that it is the aggressor that triggers a request for CLI mitigation or Strongest DL b eam reporting, while it should be the victim in control of whether to signal these IEs when interference is detected. Also, as mentioned already, the WID is clearly stating that the CLI mitigation procedures should not have a “stop” mechanism, while option 2 implies this will be in place. |  |

Similarly, RAN3 needs to discuss the corresponding F1 impacts, and the solution should be straight forward, i.e. just to adopt the conclusions reached for Xn.

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-250163 Discussion on CLI handling support (NEC)

- R3-250193 Signaling design to enable NR SBFD operation (Qualcomm Incorporated)

- R3-250194 CLI Mitigation (Charter Communications, Inc)

- R3-250329 Discussion on Evolution of Duplex Operation (CATT)

- R3-250382 Discussion on issues of CLI handling in SBFD (LG Electronics Inc.)

- R3-250431 Discussion on Information exchange for SBFD (ZTE corporation)

- R3-250432 (TP for BLCR 38.300&401&420&423&473) SBFD (ZTE corporation)

- R3-250469 Discussion on RAN3 impacts to support enhancements for CLI handling for SBFD (Ericsson)

- R3-250537 Further discussion on network support for SBFD operation and CLI management (Nokia)

- R3-250622 Discussion on information exchanges among gNBs for CLI handling (Samsung)

- R3-250651 Further discussions on SBFD related information exchange (Huawei, China Telecom,China Unicom)

- R3-250662 Discussion on supporting CLI handling in SBFD (CMCC)