3GPP TSG-RAN WG3 Meeting #127	R3-25xxxx

Athens, GR, 17-21 Feb, 2025

Agenda Item:	16.2

Source:	Huawei (moderator)

Title:	Summary of SBFD offline discussion

Document for:	Discussion and decision

# Introduction

This is a summary of discussions for the following CB, based on the submitted contributions [1]~[12].

CB: # SBFD

- Check the open issues above

- Capture agreements and provide TPs if agreeable

(moderator - HW)

Summary of offline disc R3-250803

# For the Chairman’s Notes

Issue 1: Exchange of SBFD configuration

Issue 2: Exchange of measurement configuration

Issue 3: Exchange of measurement result

Issue 4: Exchange of CLI-mitigation request

# Discussion

## Exchange of SBFD configuration

Based on the submitted contributions, the following proposals are on the table for discussion and approval:

For Xn, it seems that now RAN3 is waiting for the progress working groups, e.g. RAN2, so the thing in RAN3 now is whether to send an LS asking RAN1 and RAN2 to work on the detailed parameters and IE design or, do nothing and just wait.

Q1.1: For exchange of SBFD configuration over Xn, shall we send an LS asking RAN1 and RAN2 to work on the detailed parameters and IE design or, do nothing and just wait.

Companies are invited to express their view on Q1.1 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Send LS to RAN2, cc RAN2 | Some progress are needed, after 3 meeting. |

For F1, the remaining open point is how to exchange SBFD time and frequency configuration over F1, here there are two sub-points:

- the gNB-DU provide the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE over F1

- gNB-CU can provide the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in gNB-CU Configuration Update message.

Q1.2: For exchange of SBFD configuration over F1, RAN3 to discuss and agree:

- gNB-DU provide the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and

- gNB-CU can provide the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in gNB-CU Configuration Update message.

Companies are invited to express their view on Q1.2 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Agree with both proposals above in Q1.2 |  |

## Exchange of measurement configuration

According to RAN1 LS, It seems there are different understandings on this issue, basically there are two different understanding, as follows:

- SSB and/or periodic NZP CSI-RS were already defined in RAN2, i.e. SSB already exchanged in MeasTiming field of inter-node RRC message Measurement Timing Configuration and, NZP-CSI-RS-ResourceSet in Measurement Timing Configuration but not included it in the inter-node message; with this understanding, anyway they are already defined in RAN2, so no further work needed in RAN3 but just put it in Served Cell Information NR IE with reference to RAN2;

- Since NZP CSI-RS is not included in inter-node message, RAN3 needs to discuss how to exchange NZP CSI-RS, i.e. to ask RAN2 to include it in the inter-node message.

So RAN3 needs to decide:

For Xn

- For exchange of SSB info, nothing to be done in RAN3

- For exchange of NZP CSI-RS, to put it in Served Cell Information NR IE with reference to RAN2 or, ask RAN2 to include in inter-node message?

Q2.1: For exchange of measurement configuration over Xn, there are two basic sub-points:

- Point 1: For exchange of SSB info, nothing to be done in RAN3

- Point 2: For exchange of NZP CSI-RS, to put a new IE in Served Cell Information NR IE with reference to RAN2 definition or, ask RAN2 to include in inter-node message?

Companies are invited to express their view on Q2.1 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Point 1: yes, nothing to be done in RAN3; Point 2: either way works, but prefer to adopt the former one, i.e. “to put a new IE in Served Cell Information NR IE” | This way is a pure RAN3 work, should be quicker, the detailed IE design could refer to 0470 as an example. |

Then there are also some other proposals trying to guide the measurement behavior, e.g. periodical measurement, start or stop measurement, etc., while some companies think there is no such need, such behavior could be left to gNB implementations. RAN3 needs to discuss and agree:

- Whether to introduce more info in the measurement configuration info, e.g. periodical measurement, start or stop measurement, etc.

It was also proposed in some contributions that there might be a need to have an explicit procedure asking the peer node to perform CLI measurement, so that the peer node could response and update with measurement results, then RAN3 needs to discuss and decide, while some companies prefer to reuse existing procedures, e.g. NG-RAN node Configuration Update procedure:

- Whether to introduce an explicit procedure asking the peer node to perform CLI measurement

Q2.2: Two further questions need to be discussed:

- Whether to introduce more info in the measurement configuration info, e.g. periodical measurement, start or stop measurement, etc., or just leave to gNB implementation

- Whether to introduce a dedicated procedure asking the peer node to perform CLI measurement

Companies are invited to express their view on Q2.2 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Not needed for both | When gNB receives the measurement configuration, it is up to gNB implementation whether to perform measurement. |

For F1, the open points should be similar, thus it would be better to first conclude on Xn, then we could just refer to the conclusions reached for Xn.

## Exchange of measurement result

This issue seems to be more controversial. According to RAN1 agreement in the LS, it clearly says:

- Information exchange of strongest DL beam information

While in some contributions, it was proposed that in addition to strongest DL beam information (SSB index or NZP CSI-RS Resource Indicator), the concrete measurement result associated with the strongest DL beam should also be included, some proposals even suggested to include measurement results of other beams. Thus, RAN3 should first needs to discuss and agree:

- To exchange strongest DL beam information, i.e. SSB index or NZP CSI-RS Resource Indicator

Companies also gave the concrete format of the strongest DL beam information, i.e. to use an NZP-CSI-RS-ResourceId with INTEGER (1..64) and, the strongest beam of SSB as a SSB-Index with INTEGER (0..63).

In addition, RAN3 also needs to discuss and agree

- whether to exchange the concrete measurement result associated with the strongest DL beam; if yes, whether measurement results of other beams should also be exchanged

Thus, we should first conclude on the following two issues as described above.

Q3: For exchange of measurement result over Xn, RAN3 to discuss and agree

- Point 1: To exchange strongest DL beam information, i.e. SSB index with SSB-Index INTEGER (0..63) or NZP CSI-RS Resource Indicator INTEGER (1..64)

- Point 2: Whether to exchange the concrete measurement result associated with the strongest DL beam; if yes, whether measurement results of other beams should also be exchanged

- Companies are invited to express their view on Q3 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Agree with point 1; No need for point 2 | According to RAN1, just the index/ID of strongest beam is needed. |

Similarly, for F1, we could first refer to the conclusions reached for Xn.

## Exchange of CLI-mitigation request

Here the issue is very straight forward, there are mainly two sub-issues, 1) what this request looks like; 2) how to transfer this request.

For the first issue, RAN3 needs to discuss and agree if the CLI-mitigation request is just simple indication, i.e a per cell optional IE, such as an ENUMERATED value “CLI-mitigation requested”, or are there any addition info needed.

Q4: RAN3 needs to discuss and agree:

- CLI-mitigation request is just simple indication, i.e. a per cell optional IE, such as an ENUMERATED value “CLI-mitigation requested”, or are there any addition info needed.

Companies are invited to express their view on Q4 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Agree to introduce a per cell optional IE, such as an ENUMERATED value “CLI-mitigation requested”; Others are not needed |  |

## How to exchange measurement result and CLI-mitigation request over Xn

There is a common issue for exchange of measurement result and exchange of CLI-mitigation request, i.e., how to exchange measurement result and CLI-mitigation request. Basically, there are two options proposed, according to the submitted contributions, one is to reuse existing procedures, the other is to introduce new procedure. While for the former, there are further two sub-options, one is to reuse interface management procedures, the other is to reuse Resource Status Reporting procedure. Here the rapporteur assumes that whatever the option would be adopted, the measurement result, i.e. the strongest DL beam information and CLI-mitigation request could be included as two optional IEs in the same message.

Q5.1: RAN3 to discuss and agree to exchange the strongest DL beam information and the CLI-mitigation request as two optional IEs in the same message over Xn.

Companies are invited to express their view on Q5.1 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Agree | Anyway, both are optional IEs, and gNB should allow to provide measurement results together with CLI mitigation request. |

Q5.2: RAN3 to discuss which options to support the exchange of strongest DL beam information and CLI-mitigation request over Xn:

- Option 1: To reuse interface management procedures: NG-RAN node Configuration Update procedure

- Option 2: To reuse Resource Status Reporting procedure

- Option 3: To introduce new procedures

Companies are invited to express their view on Q5.2 above:

| Company | View or preferred option | Comment |
| --- | --- | --- |
| Huawei | Each option could work, but option 3 is preferred | Option 3, i.e. to introduce a new message seems to be a cleaner way. |

Similarly, RAN3 needs to discuss the corresponding F1 impacts, and the solution should be straight forward, i.e. just to adopt the conclusions reached for Xn.

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-250163 Discussion on CLI handling support (NEC)

- R3-250193 Signaling design to enable NR SBFD operation (Qualcomm Incorporated)

- R3-250194 CLI Mitigation (Charter Communications, Inc)

- R3-250329 Discussion on Evolution of Duplex Operation (CATT)

- R3-250382 Discussion on issues of CLI handling in SBFD (LG Electronics Inc.)

- R3-250431 Discussion on Information exchange for SBFD (ZTE corporation)

- R3-250432 (TP for BLCR 38.300&401&420&423&473) SBFD (ZTE corporation)

- R3-250469 Discussion on RAN3 impacts to support enhancements for CLI handling for SBFD (Ericsson)

- R3-250537 Further discussion on network support for SBFD operation and CLI management (Nokia)

- R3-250622 Discussion on information exchanges among gNBs for CLI handling (Samsung)

- R3-250651 Further discussions on SBFD related information exchange (Huawei, China Telecom,China Unicom)

- R3-250662 Discussion on supporting CLI handling in SBFD (CMCC)