3GPP TSG-RAN WG3#127 	R3-250060

Athens, Greece, 17-21 February 2025

Agenda item:	13.3 Support for Conditional LTM

Source:	Nokia

Title:	Discussion on Conditional LTM in disaggregated gNB architecture

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-242356 NR mobility enhancements Phase 4), where the objectives relevant to conditional LTM (CLTM) are described as:

• Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] o Specify UE evaluated conditions for triggering LTM o Aim to support conditional LTM including subsequent LTM o Limit specifying the conditional LTM to the scenario where the UE is in non-DC o Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint • Specify RRM requirements related to the above objectives as necessary [RAN4]

The primary motivation for CLTM is to introduce robustness to the LTM procedure where the UE may not be able to receive the cell-switch command at right radio condition which may lead to radio link failure. Compared to CHO, the conditional LTM is expected to bring the benefits of robustness along with the interruption time reduction benefits of LTM. In this contribution, we discuss base requirements from RAN3 perspective to support Intra-CU Conditional LTM in disaggregated gNB architecture, considering the agreements so far in RAN2 provided in R3-247111 and R2-2411134.

# 2	Discussion

The following phases are considered when analyzing support for Intra-CU Conditional LTM:

Phase # 1: Conditional cell switch condition configuration

Phase # 2: Conditional cell switch condition evaluation

Phase # 3: Early synchronization

Phase # 4: Conditional cell switch execution

Phase #1 (configuration) and Phase #3 (early sync) will incur F1AP specification changes. However, given that once configured, the evaluation of conditional LTM execution conditions and triggering of LTM cell switch is to be carried out the UE, we do not foresee F1AP specification impact for Phase #2 (evaluation) and #4 (execution).

## 2.1	Conditional cell switch condition configuration

Conditional LTM follows Rel-18 LTM principle which reserves cell and RACH resources when configuring LTM. Likewise, if Conditional LTM configured at a cell is to be based on L1 measurements, the gNB-CU needs to request gNB-DU to generate the L1 execution conditions, which are reported back to the gNB-CU. Further, this process is required for each candidate LTM cell where Conditional LTM is to be configured. After the L1 execution conditions have been signaled to the gNB-CU, it can generate the RRC message containing the L1 execution conditions of each cell and signal it to the UE.

A straightforward approach is to reuse existing Rel-18 LTM configuration messages to optionally also request a Source/Candidate gNB-DU to generate the L1 execution conditions and report them back to the gNB-CU.

Proposal 1: For L1 measurement based Conditional LTM, F1AP UE CONTEXT SETUP and UE CONTEXT MODIFICATION REQUEST messages are used for the gNB-CU to requests a gNB-DU to generate L1 execution conditions. The gNB-DU signals the L1 execution conditions via the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages.

Proposal 2: Whether the L1 executions conditions should be embedded with the CSI report config or not is spending RAN2 progress.

Similarly, if Conditional LTM configured at a cell is based on L3 measurements we do not foresee F1AP impact, given that the condition is determined by the gNB-CU. Hence, the existing Rel-18 LTM framework is sufficient and there is not additional interaction needed between the gNB-CU and Source/Candidate gNB-DU(s).

## 2.2	Early Synchronization

L3 based Conditional LTM considerations

Conditional LTM is meant to support early synchronization. This results in two possible cases, one for L1 measurements based, and another for L3 measurements case.

In our view, for L1 measurement based conditional LTM, the existing Rel-18 LTM framework is sufficient for triggering early sync related procedures and we foresee no F1AP specification impact for this case.

However, for the L3 measurement based conditional LTM, there is need for supporting a mechanism for the gNB-CU to request a Source gNB-DU to trigger only early sync related procedures but not the LTM cell switch command.

Proposal 3: For L3 measurement based Conditional LTM, the gNB-CU requests the Source gNB-DU to trigger early sync related procedures independently (i.e. without having the gNB-DU trigger also a LTM cell switch) via F1AP.

A possible alternative to support L3 measurement based early sync for Conditional LTM is to add an additional codepoint to the CR for L3 based LTM support in Rel-18 at the last RAN3#126 meeting (R3-247868).

This would trigger two actions at the gNB-DU for UL sync, (a) send a PDDCH command to the UE triggering TA Acquisition toward a given candidate cell, and (b) send the received TA value received from gNB-CU to the UE via MAC CE command. Likewise, the additional codepoint avoids the gNB-DU from executing LTM cell switch, since for of Conditional LTM, the UE would be in charge of the cell switch execution when the evaluation conditions are met. The updates to the aforementioned CR (R3-247868) would be limited to the following.

#### 9.2.2.x	CU-DU MOBILITY INITIATION REQUEST

This message is sent by the gNB-CU to inform the gNB-DU to trigger the cell switch command and/or early TA acquisition to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Mobility Triggering Information |  |  | 0..1 |  | YES | ignore |
| >Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >Triggering Indication | M |  | ENUMERATED (cell switch, early sync and cell switch, …, early sync) | Indicates whether the CU-DU Mobility Initiation procedure is requested for cell switch, or early synchronisation and cell switch. | - |  |

Beam indication considerations for Conditional LTM

Upon LTM cell switch change, the Rel-18 LTM framework signals the activated TCI States to the Candidate LTM cell in order to be prepared for the incoming UE in the appropriate resources. This is signalled via the DU-CU/DU-CU CELL SWITCH NOTIFICATION messages.

For Conditional LTM a similar handling is required. However, given that the cell switch execution is determined by the UE, the Candidate LTM cell is unaware of the TCI States that the UE will utilize. Hence, F1AP signalling is needed so that when DL synchronization takes place, the Source-DU provides the TCI State IDs to the Candidate-DU LTM cells (via the gNB-CU) indicating these have been activated for Conditional LTM.

Proposal 4: For a UE configured with Conditional LTM, the Source gNB-DU signals over F1AP to the Candidate gNB-DU (via the gNB-CU) about the activated TCI States.

Proposal 5: The F1AP CU-DU CELL NOTIFICATION and CU-DU CELL SWITCH NOTIFICATION messages are reused to indicate the activated TCI States to the Candidate gNB-DU for Conditional LTM.

#### 9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >Conditional LTM | O |  | ENUMERATED(true, …) | Indicates whether the message relates to a Conditional LTM configuration. | YES | ignore |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

# 5	Conclusions

Proposal 1: For L1 measurement based Conditional LTM, F1AP UE CONTEXT SETUP and UE CONTEXT MODIFICATION REQUEST messages are used for the gNB-CU to requests a gNB-DU to generate L1 execution conditions. The gNB-DU signals the L1 execution conditions via the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages.

Proposal 2: Whether the L1 executions conditions should be embedded with the CSI report config or not is spending RAN2 progress.

Proposal 3: For L3 measurement based Conditional LTM, the gNB-CU requests the Source gNB-DU to trigger early sync related procedures independently (i.e. without having the gNB-DU trigger also a LTM cell switch) via F1AP.

Proposal 4: For a UE configured with Conditional LTM, the Source gNB-DU signals over F1AP to the Candidate gNB-DU (via the gNB-CU) about the activated TCI States.

Proposal 5: The F1AP CU-DU CELL NOTIFICATION and CU-DU CELL SWITCH NOTIFICATION messages are reused to indicate the activated TCI States to the Candidate gNB-DU for Conditional LTM.

# References

- 3GPP TS 38.401

- RP-242356 NR mobility enhancements Phase 4, Intel (moderator)

- R3-247111 LS on RAN2 agreements for inter-CU LTM - (RAN2#127 and RAN2#127bis)

- R2-2411134 RAN2 agreements on Inter-CU LTM and Conditional LTM - (RAN2#128)

- R3-247868 F1 support for L3 measurements-based LTM (CR TS 38.473), Ericsson, Vodafone, CATT, ZTE, NTT DoCoMo, Nokia, Samsung, LG Electronics,