3GPP TSG-RAN WG3 Meeting #127	R3-250084

Athens, Greece, 17 – 21 February, 2025

Agenda item:	10.2

Source:	Nokia

Title:	(TP for SON BL CR for TS 38.473, TP for SON BL CR for TS 37.340, TP for SON BL CR for TS 38.300) MRO Enhancements for LTM and CHO with Candidate SCG(s)

Document for:	Discussion

# 1	Introduction

This is a contribution related to MRO for LTM and CHO with candidate SCG(s) in Rel-19.

# 2	Discussion

## 2.1 MRO for LTM

### 2.1.1 Agreements achieved in RAN3 and RAN2

The agreements achieved in RAN3 #126 [1]:

For TP to BL CR to TS 38.473:

- In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI.

For TP to BL CR to TS 38.401:

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the failure DU in case of too late LTM.

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the source DU in case of too early LTM, or LTM to wrong cell.

Whether network based solution is needed? What’s the UE context to be stored in RAN side?

UHI and ping-pong:

- Which node is responsible for avoiding LTM cell switch ping-pong (CU or DU)?

- CU sends UHI to DUs?

The agreements achieved in RAN3 #125bis [2]:

- For TP to BL CR to TS 38.300:

  - Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions.

  - For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM.

  - Change“successful handover" definition, as proposed in R3-245230.

- For TP to BL CR to TS 38.401:

  - Capture wrong LTM prepared cell selection.

- Add failure type to F1AP ACCESS AND MOBILITY INDICATION message.

- CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell.

The agreements achieved in RAN2 #128 [3]:

- No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues.

- Reuse the existing ra-InformationCommon for the RA-based LTM failure.

- If RAN3 does not address this meaning that we need a RAN2 solution, add a list indicating which LTM candidates the UE had at RLF.

The agreements achieved in RAN2 #127bis [4]:

- The UE shall log cell IDs such as reestablishment cell ID, source, failed, reconnect cell ID, following the same principle as RLF, HOF and successful recovery, incl. the time between UE executing the LTM command and the failure.

- If RA-based LTM failure happens the UE logs and reports RACH info in the RLF report. Additional information is TBD.

- Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined).

- Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation.

- Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell.

- UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

### 2.1.2 Near Failure Monitoring for MCG LTM

Main benefits of the LTM shall be seen in avoiding the complete reset of L3, avoiding RACH procedure which shall result into reduction of the interruption time. The interruption time is significantly impacted with RACH procedure. In case the TA of the candidate target LTM cell is not included into Cell Switch Command (MAC CE) the UE will initiate the legacy RACH procedure to access the target candidate cell if there is no TA value obtained by UE based on measurements. Despite the procedure can be monitored by means of T304 timer, but the quality in terms of having it in a RACH-less manner is not considered. However, it is essential to get information whether the cell change occurred RACH-less which is key for LTM. It resulted into proposal to define new SHR triggers specific to LTM near failure cases related to RACH-based LTM by some of companies. However, during the RAN2 # 127 meeting it was agreed not to use this as a new trigger event for SHR for LTM. As we think this conclusion in RAN2 was made quickly and not considering all the aspects we propose to discuss it again in RAN3.

Proposal 1-1: RAN3 discuss new SHR trigger specific to LTM near failure cases related to RACH-based LTM.

On the other hand, in case the RACH-less cell change is triggered, i.e. MAC-CE msg contains TA, the first UL transmission is done in RLC AM. So, it may require several RLC retransmissions to get the transmission through.

Proposal 1-2: RAN3 agrees to define a threshold related to number of RLC retransmissions of the first UL transmission in RLC AM for RACH less LTM as new SHR trigger.

The problem may not only be related to degrading radio conditions in the target cell. The UE may move into area where TA included in Cell Switch Command is not valid for a while and then moving back to area where the TA is valid. Unsuccessful RLC transmissions may have their basis in HARQ DTX which may point out there was no synchronization in UL between UE and network.  It may be thus desirable to identify the “Almost wrong TA” case when the increased interruption time was caused by invalid TA for some limited time. Reporting the SHR with the new cause “Almost wrong TA” could help operator to properly optimize the network.

Proposal 1-3: RAN3 agrees to define a threshold related to number of RLC retransmissions of the first UL transmission in RLC AM for RACH less LTM for which there was no UL synchronization between UE and network as new SHR trigger.

### 2.1.3 Connection Failure Monitoring for MCG LTM

Based on the agreements the gNB-CU is responsible for optimizing LTM candidate cells. Two options are possible to optimize the LTM candidate cell list:

- UE based solution

- Network based solution

LTM compared to CHO supports subsequent LTM and therefore the UE stores LTM configuration also after successful LTM execution. UE can therefore report LTM candidate cell list also in scenario when RLF occurred shortly after successful LTM execution.  On the other hand, as Rel-18 only supports intra-CU LTM and therefore the gNB-CU can have the LTM candidate cells in its UE context even if there is inter-DU mobility within the gNB-CU.

Observation 1-1: To optimize LTM candidate cell list both UE and network-based solutions may be used for Rel-18 LTM cell switch.

As the info on LTM candidate cells is available in the network (gNB-CU) it is proposed not to ask UE to monitor this information.

In the context of RAN3 # 126 meeting also the network-based solutions for TA acquisition in RLF Report was discussed. Majority of companies agreed with network-based solutions as source gNB can store the Early UL sync configuration and UE-based TA configuration and can determine whether the UE had determined the TA via UE-based TA measurement or gNB indicated TA value in Cell Switch Command by “retrieving” the UE context upon receiving the RLF Report. Some companies shared a view that it may be difficult for network to remember such information as according to their view the configuration of method for TA acquisition may dynamically change. However, the configuration is done based on operator’s setting therefore it shall not change on short term basis. Even the UE has both TA received in Cell Switch Command and obtained via UE-based TA measurement the UE shall consider the TA from Cell Switch Command as message sent from network has always higher priority.  As the info on TA acquisition is available in the network it is proposed not to ask UE to monitor this information.

Proposal 1-4: RAN3 agrees on network-based solution for optimizing the LTM candidate cell list and the info on TA acquisition.

In RAN3 # 126 meeting the following was agreed:

For TP to BL CR to TS 38.473:

- In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI.

The reasoning why the C-RTNI is added shall be part of stage 3. It is therefore proposed to update the Semantics description for the C-RNTI in TS 38.473 as proposed in the attached TP.

Proposal 1-5: RAN3 agrees the attached TP for TS 38.473 to update the Semantics description for the C-RNTI.

There may be a plurality of problems causing the RLF. Besides the wrong cell selection at cell switch and wrong cell switch timing the problem may be related to wrong or missing TA in the Cell switch command.  In general, it may be related to an issue in Early TA Acquisition procedure which oversees source DU. Despite the TA was included in Cell Switch Command it may become outdated because the UE significantly moved between the time of the Early TA Acquisition and the time of the LTM execution. The UE then fails to attempt the target LTM cell and had to switch RACH based access via LTM recovery/re-establishment procedure when new TA value is provided from network in RAR message. We think that reporting of the information about the difference between the TA reported in the Cell switch command for which the RA less LTM failed, and TA received consequently in RAR for recovery/re-establishment cell may help to optimize the wrong cell switch timing.

- Proposal 1-6: RAN3 to discuss the following information to be included in the RLF report:

- TA difference between the TA provided by network in Cell Switch Command for which the RA less LTM failed, and TA received in RAR for recovery/re-establishment cell.

- In case of invalid TA, the UE may re-connect or re-establish into the same target LTM cell to which original access failed. Even such failure case is related to MRO it neither fits to too late, not too early nor to LTM to wrong cell. It may therefore be desirable to introduce new failure cause “Invalid TA” which CU communicates to source DU.

- Proposal 1-7: RAN3 agrees on new failure cause “Invalid TA” to be communicated from CU to the source DU.

There may be TA obtained by UE based on measurements. However, if the network includes a TA in the Cell switch command the UE must use it. It may happen that the TA obtained based on UE measurements is very close to TA received consequently in RAR for recovery/re-establishment in case of RA less LTM failure for TA reported in the Cell switch command. The UE based on some similar observation of other LTM cell switch procedures may prove the trend of better precision of UE measurement-based TA. Then it may be desirable the UE signals to network the preference of using the UE measurement-based TA.

Proposal 1-8: RAN3 agrees on signalling from UE to network the preference of using the UE measurement-based TA.

## 2.2 MRO for CHO with Candidate SCG(s)

### 2.2.1 Agreements achieved in RAN3

The agreements achieved in RAN3 #126 [1]:

Case 1a), 2a), 3a), 9a): Too late handover

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 7a), 8a): Too early HO or wrong cell Handover to wrong cell

Case 7b), 8b): Too Early CPC/CPA Execution or CPC/CPA Execution to wrong PSCell

The failure types defined for CHO in TS38.300 and for CPAC defined in TS37.340 are used as baseline. Clarification or amendment could be made on top of that if needed.

In case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization.

Which message is used from the last serving MN to the candidate MN(s):

- Handover Report

- via new message

SCGFailureInfomation handling for case 1b)/2b)/3b)/9b) and case 7b)/8b):

The serving MN which receives SCGFailureInformation from the UE performs the initial analysis.

The serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed (e.g for case 3b).

The agreements achieved in RAN3 #125bis [2]:

RLF report is enhanced to including

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

  - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

  - The fulfilled events before the RLF is encountered?

The agreements achieved in RAN3 #128 [3]:

RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements.

Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

The agreements achieved in RAN2 #127bis [4]:

UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS.

Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

### 2.2.3 Near Failure Monitoring for CHO with Candidate SCG(s)

In case of CHO with candidate SCG(s), it may happen that if one of the execution conditions is not configured properly, the entire execution will be delayed as the configuration cannot be executed partially. Even the CHO with candidate SCG(s) is at the end successfully completed, the delayed execution may have negative impact on end user when one of the carriers (MCG or SCG) is temporarily unavailable due to running T310/312 on the carrier. Even if there is a general agreement between the companies on the need to monitor the time between fulfilment the associated conditions, the details of such monitoring and optimization steps to reduce this time have not been discussed yet. Some companies come with proposal how to possibly correlate SHR and SPR in near failure case however the basic cases like for connection failure scenario have not been agreed yet. We did a proposal for near failure cases in RAN3 # 125bis meeting in R3-245277 and repeated in RAN3 # 126 meeting in R3-247321 but due to lack of time it was not discussed in any of these meetings.

Despite the near failure monitoring for CHO with Candidate SCG(s) was agreed for Rel.19 the discussion on that has not started at all in RAN3. There are only four RAN3 meetings and considering there is still a lot of open topics which have not been yet concluded it is questionable there is enough time for this topic in Rel.19.

Proposal 2-1: RAN3 shall decide whether the near failure monitoring for CHO with Candidate SCGs is in Rel. 19 or if intention is rather to move it to Rel.20.

### 2.2.4 Connection Failure Monitoring for CHO with Candidate SCG(s)

During the RAN3 # 126 the following was agreed:

Case 1a), 2a), 3a), 9a): Too late handover

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 7a), 8a): Too early HO or wrong cell Handover to wrong cell

Case 7b), 8b): Too Early CPC/CPA Execution or CPC/CPA Execution to wrong PSCell

Consequently, it was agreed to modify the detection mechanism for MRO for too late handover failure case in the TP for TS 38.300 (R3-247905) via adding the part the UE may be configured with CHO with candidate SCG(s). The corresponding definition of too late CPC in TS 37.340 which reads as follows was not modified at all:

Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

However, for the case 2b CPC execution conditions are satisfied but due to waiting for fulfilment of the CHO execution conditions the SCG failure occurs. It does not fit to the current definition of the too late CPC failure case in TS 37.340. The too late CPC failure cases shall be modified as follows:

Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution condition is satisfied; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

Proposal 2-2: RAN3 agrees the attached TP for TS 37.340 to update the definition of the too late CPC failure case.

During the RAN3 # 126 also the following was agreed:

In case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization.

Which message is used from the last serving MN to the candidate MN(s):

- Handover Report

- via new message

In our understanding this agreement applies for too late CHO execution to case 3a and 9a only. In these cases, the CHO execution conditions are fulfilled and due to waiting for CPC/CPA execution conditions fulfilment the RLF is declared on PCell. After UE reconnects or re-establish to another PCell of possible another MN and sends the RLF report to this MN which is then forwarded to the MN where failure occurred via RLF indication, this MN based on enhanced RLF report containing first fulfilled execution condition, also PSCell(s) measurements may do initial analysis and identify the candidate MN(s) for final optimization related to CPC/CPA part. For the case 1a when neither CHO nor CPC/CPA execution conditions were fulfilled and RLF is declared on PCell the optimization shall focus on fixing the CHO related part issue. Similar for case 2a when CPC/CPA execution conditions are fulfilled as first and consequently there is RLF on PCell.

Observation 2-1: The agreement that in case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization applies for case 3a and 9a.

However, there may be a need to forward the RLF report also in case not relevant to too late CHO execution. The case 7a when CHO execution conditions fulfilled first then with some delay the CPA/CPC execution conditions are also fulfilled consequently the CHO execution is unsuccessful while CPA/CPC execution is successful, which may be caused by delayed fulfilment of CPA/CPC execution conditions which may require optimization on CPA/CPC related part. The RLF report received by source (serving) MN may also need to be forwarded to candidate target MN.

Observation 2-2: The agreement that the last serving MN may need to send message to the candidate MN(s) which may need optimization applies also for case 7a.

The part in TS 38.300 which was added based on the above agreement in RAN3 # 126 meeting shall therefore be updated accordingly.

Proposal 2-3: RAN3 agrees the attached TP for TS 38.300 to update the scenario when the last serving MN may need to send message to the candidate MN(s) which may need optimization.

In RAN3 # 126 also the following was agreed:

SCGFailureInfomation handling for case 1b)/2b)/3b)/9b) and case 7b)/8b):

The serving MN which receives SCGFailureInformation from the UE performs the initial analysis.

The serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed (e.g for case 3b).

In our understanding this agreement applies to case 3b and 9b when CHO execution conditions are fulfilled and during waiting for CPC/CPA execution conditions fulfilment RLF is declared on SCG. UE sends the SCGFailureInformation to the serving MN (source MN), while optimization on CPC/CPA related part is needed which requires forwarding the SCGFailureInformation to respective candidate MN(s). For case 1b when no one of the execution conditions is fulfilled the serving MN based on SCGFailureInformation may decide some optimization is needed also for CPC/CPA part and may forward the SCGFailureInformation to the candidate MN(s). For case 2b when CPC/CPAC execution conditions are fulfilled but due to waiting for the CHO execution conditions fulfilment there is RLF on SCG when issue may be in the CHO related part which is configured by source (serving) MN there may be no need to forward the SCGFailureInformation to candidate MN(s). Concerning the case 7b when CPC/CPA execution conditions were fulfilled first then with some delay the CHO execution conditions were also fulfilled but CPC/CPA execution is unsuccessful while CHO execution is successful, which may be caused by delayed fulfilment of CHO execution conditions which may require optimization in CHO related part. The SCGFailureInformation received by target (serving) MN may need to be forwarded to source MN. Considering the same case 7b but with CHO execution conditions fulfilled first then with some delay the CPA/CPC execution conditions but CPC/CPA execution is unsuccessful while CHO execution is successful, which may point to CPA/CPC issue configured by target (serving) MN and thus non need to forward the SCGFailureInformation. Regarding the case 8b when CPC/CPA execution conditions were fulfilled first then with some delay the CHO execution conditions were fulfilled which was followed consequently with successful CHO and CPC/CPA execution, but shortly there is SCG failure which may be caused by delayed fulfilment of CHO execution conditions which may require optimization in CHO related part. The SCGFailureInformation received by target (serving) MN may need to be forwarded to source MN.  Considering the same case 8b but with CHO execution conditions fulfilled first then with some delay the CPA/CPC execution conditions and consequent successful CHO and CPC/CPA execution, but shortly there is SCG failure which may point of CPA/CPC issue configured by target (serving) MN and thus non need to forward the SCGFailureInformation.

Observation 2-3: The agreement that the serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed applies for case 1b, 3b, 7b, 8b, 9b.

To forward the SCGFailureInformation for UE configured with CHO with Candidate SCG(s) also to NG-RAN node different from SN relevant stage 2 text shall be added into TS 38.300 as proposed in Annex 3.

Proposal 2-4: RAN3 agrees the attached TP for TS 38.300 to add stage 2 text about the SCGFailureInformation forwarding from MN to NG-RAN node different from SN.

# 3	Conclusions

We have the following observations and proposals to enhance MRO for LTM:

Proposal 1-1: RAN3 discuss new SHR trigger specific to LTM near failure cases related to RACH-based LTM.

Proposal 1-2: RAN3 agrees to define a threshold related to number of RLC retransmissions of the first UL transmission in RLC AM for RACH less LTM as new SHR trigger.

Proposal 1-3: RAN3 agrees to define a threshold related to number of RLC retransmissions of the first UL transmission in RLC AM for RACH less LTM for which there was no UL synchronization between UE and network as new SHR trigger.

Observation 1-1: To optimize LTM candidate cell list both UE and network-based solutions may be used for Rel-18 LTM cell switch.

Proposal 1-4: RAN3 agrees on network-based solution for optimizing the LTM candidate cell list and the info on TA acquisition.

Proposal 1-5: RAN3 agrees the attached TP for TS 38.473 to update the Semantics description for the C-RNTI.

- Proposal 1-6: RAN3 to discuss the following information to be included in the RLF report:

- TA difference between the TA provided by network in Cell Switch Command for which the RA less LTM failed and TA received in RAR for recovery/re-establishment cell.

- Proposal 1-7: RAN3 agrees on new failure cause “Invalid TA” to be communicated from CU to the source DU.

Proposal 1-8: RAN3 agrees on signalling from UE to network the preference of using the UE measurement-based TA.

To support MRO for CHO with candidate SCG(s), we propose what follows:

Proposal 2-1: RAN3 shall decide whether the near failure monitoring for CHO with Candidate SCGs is in Rel. 19 or intention is rather to move it Rel.20.

Proposal 2-2: RAN3 agrees the attached TP for TS 37.340 to update the definition of the too late CPC failure case.

Observation 2-1: The agreement that in case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization applies for case 3a and 9a.

Observation 2-2: The agreement that the last serving MN may need to send message to the candidate MN(s) which may need optimization applies also for case 7a.

Proposal 2-3: RAN3 agrees the attached TP for TS 38.300 to update the scenario when the last serving MN may need to send message to the candidate MN(s) which may need optimization.

Observation 2-3: The agreement that the serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed applies for case 1b, 3b, 7b, 8b, 9b.

Proposal 2-4: RAN3 agrees the attached TP for TS 38.300 to add stage 2 text about the SCGFailureInformation forwarding from MN to NG-RAN node different from SN.

# References

- Report of 3GPP TSG RAN3 meeting #126

- Report of 3GPP TSG RAN3 meeting #125bis

- Report of 3GPP TSG RAN2 meeting #128

- Report of 3GPP TSG RAN2 meeting #127bis

# Annex 1: Text proposal to TS 38.473 (MRO for LTM)

First change, ommited text not changed

# 9	Elements for F1AP Communication

[snip]

## 9.2	Message Functional Definition and Content

[snip]

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU ® gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU added in case the gNB-DU responsible for the LTM failure is not the gNB-DU serving the UE at the time of LTM failure. | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

Remaining text not changed

# Annex 2: Text proposal to TS 37.340 (MRO for CHO with Candidate SCG(s))

First change, ommited text not changed

### 10.18.3	Conditional PSCell addition or change failure

One of the functions of self-optimization for CPAC is to detect CPAC failures that occur due to Too late CPC execution or Too early CPC/CPA execution, or CPC/CPA execution to wrong PSCell. These problems are defined as follows:

-	Too Late CPC Execution: UE receives CPC configuration, while a SCG failure occurs before CPC execution; a suitable PSCell different from source PSCell is found based on the measurements reported from the UE.

-	Too Early CPC/CPA Execution: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; in case of CPC, the source PSCell is still the suitable PSCell based on the measurements reported from the UE; in case of CPA, no suitable PSCell is found based on the measurements reported from the UE.

-	CPC/CPA Execution to wrong PSCell: CPC/CPA execution is not successful or an SCG failure occurs shortly after a successful CPC/CPA execution; a suitable PSCell different from the source PSCell or the target PSCell is found based on the measurements reported from the UE. There are two sub-cases for wrong candidate PSCell list selection:

-	if the suitable PSCell is one of the candidate target PSCells provided by the node initiating the CPC or by the MN initiating the CPA, but not one of the candidate PSCells selected by the candidate or target SN, it is wrong target PSCell selection at the candidate or target SN;

-	else, it is wrong candidate PSCell list selection at the node initiating the CPC or at the MN initiating the CPA.

In the definition above, the "successful CPC/CPA execution" refers to the UE state, namely the successful completion of the RA procedure.

Remaining text not changed

# Annex 3: Text proposal to TS 38.300 (MRO for CHO with Candidate SCG(s))

First change, ommited text not changed

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO or CHO with candidate SCG(s) is configured but the CHO execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell is the cell that served the UE at the last handover initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO execution until connection failure.

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

In case UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may inform with the failure event the candidate NG-RAN node(s) controlling the candidate PSCell where the CPA/CPC configuration caused the failure.

Editor’s note: FFS on the definition of failure event.

For MRO analysis, in case of RLFs or handover failures, the source gNB may take into account the information regarding the LBT failures occurred in UL and DL for a specific UE. For UL, the information may be collected and provided by the UE to the network in the RLF report. For DL, the information may be collected at the source and target gNBs. The target gNB may provide the information to the source gNB via the Access and Mobility Indication procedure over Xn, if requested by the source gNB during the handover preparation.

Retrieval of information needed for problem analysis

In order to retrieve relevant information collected at the network side as part of the UE context, the UE provides C-RNTI used in the last serving cell. If the cause for the failure is identified as a "Too Early HO" or a "HO to Wrong Cell", the NG-RAN node controlling the last serving cell shall, include in the HANDOVER REPORT message the C-RNTI used in the source cell of the last completed handover before the failure. If the NG RAN node controlling that source cell provided the Mobility Information, it is also included in the HANDOVER REPORT message. If used, the Mobility Information is prepared at the source NG RAN node of a handover and may refer to or identify any handover-related data at this NG RAN node. In the Handover Preparation procedure, the source gNB can request the target gNB to provide information on DL LBT failures at the target gNB during handover execution.

Handling multiple reports from a single failure event

In case the RRC re-establishment fails and the RRC connection setup succeeds, MRO evaluation of intra-RAT mobility connection failures may be triggered twice for the same failure event. In this case, only one failure event should be counted.

Next change, ommited text not changed

#### 15.5.2.6	PSCell addition/change failure

For analysis of PSCell addition/change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN as defined in TS 37.340 [21].

For UE configured with CHO with candidate SCG(s), the MN receiving SCGFailureInformation from the UE performs an initial analysis. If the SCG failure is caused by other NG-RAN node different from SN, the MN transfers the SCGFailureInformation together with the analysis results to the NG-RAN node which should perform the optimization.

Remaining text not changed