3GPP TSG-RAN WG3 Meeting #127	R3-250090

Athens, Greece, 17-21 February 2025

Agenda Item:		13.2

Source:		Ofinno

Title:	Issues on Data Forwarding for Inter-CU LTM

Document for:		Discussion and Decision

# Introduction

After RAN3 #126 meeting, the following progress has been achieved on the agenda item 13.2, i.e., support for Inter-CU LTM:

RAN3#126:

RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution.

The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB.

Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation?

There is no consensus on the data forwarding issues.

This contribution discusses the data forwarding issue above and issues on MR-DC LTM.

# Discussion

## 2.1	Data Forwarding for Inter-CU/gNB LTM

In last RAN3 meeting, (normal or late) data forwarding for inter-CU LTM was discussed [3]. The following two options exist:

Option 1:

(Late or normal) data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message. [3]

Option 2:

(Late or normal) data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation [1].

From Rel-16/17, HANDOVER SUCCESS message was introduced for CHO, in which (Late or normal) data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message. For CHO, it’s clear that it totally depends on UE when to trigger/perform handover after source gNB configures potential target cells to UE. Thus, source gNB does not know when to start (Late or normal) data forwarding before receiving HANDOVER SUCCESS message from target gNB.

However, for inter-CU/gNB LTM, source gNB decides on when to trigger handover, i.e., sending LTM Cell Switch Command to UE. Thus, source gNB knows the timing of performing the handover and so that data forwarding can be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE. It seems that waiting for receiving HANDOVER SUCCESS message from target gNB is not necessary.

On the other hand, in Rel-18 LTM with gNB-CU-UP change scenario, source gNB-CU-CP/UP may initiate data forwarding after gNB-DU sends LTM Cell Switch Command to the UE.

FIG. 1 Proposed changes for BLCR to TS 38.300 (Figure 9.2.3.5.X-1 in [4])

Based on the analysis above, the following proposals are suggested to RAN3:

Proposal 1: To adopt option 2, i.e., (Late or normal) data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE.

Proposal 2: To adopt the proposed changes in FIG. 1 for BLCR to TS 38.300 (Figure 9.2.3.5.X-1 in [4]).

## 2.2	Data Forwarding for MR-DC LTM

In Rel-17/18, we introduced several new IEs (e.g., CHO MR-DC Indicator, CHO MR-DC Early Data Forwarding Indicator, CPC Data Forwarding indicator) in XN-U ADDRESS INDICATION message as shown in the table below. The motivation was to trigger early data forwarding for CHO MR-DC or CPC scenarios.

#### 9.1.1.11	XN-U ADDRESS INDICATION

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| … |  |  |  |  |  |  |
| CHO MR-DC Indicator | O |  | ENUMERATED (true, ..., coordination-only) | Indicating that the XN-U ADDRESS INDICATION message is for Conditional Handover, as specified in TS 37.340 [8]. | YES | reject |
| CHO MR-DC Early Data Forwarding Indicator | O |  | ENUMERATED (stop, ...) |  | YES | ignore |
| CPC Data Forwarding indicator | O |  | ENUMERATED (triggered, early data transmission stop, ..., coordination-only) | Indicating that the XN-U ADDRESS INDICATION message is for a Conditional PSCell Change. | YES | reject |
| … |  |  |  |  |  |  |
| … |  |  |  |  |  |  |
| … |  |  |  |  |  |  |

In this release, early data forwarding is also supported for Inter-CU/gNB LTM as shown step 9a [4].

FIG. 2 Part of signalling procedure for inter-gNB LTM (Figure 9.2.3.5.X-1 in [4])

Similar to Rel-17/18 CHO MR-DC or CPC scenarios, there are two scenarios for Inter-CU LTM, which are within the scopes of WID.

The first scenario is about inter-MN change while the source secondary node is finally released as shown in FIG. 3.

FIG. 3 Part of signalling procedure for inter-CU LTM (MR-DC case).

The second scenario is about inter-SN change while the master node is not changed as shown in FIG. 4.

FIG. 4 Part of signalling procedure for inter-SN LTM (PSCell change case).

In order to support early data forwarding for the scenarios showed in FIG. 3 and FIG. 4, XN-U ADDRESS INDICATION message should be enhanced. One possible solution is to introduce two indicators, for example, one LTM MR-DC data forwarding indicator and one LTM PSCell change data forwarding indicator. The other solution is to change the stage 3 descriptions to cover the LTM MR-DC and LTM PSCell change scenarios.

Based on the analysis above, the following proposal is suggested to RAN3:

Proposal 3: To enhance XN-U ADDRESS INDICATION message to cover the LTM MR-DC and LTM PSCell change scenarios.

- Option 1: Introduce two indicators: LTM MR-DC data forwarding indicator and LTM PSCell change data forwarding indicator

- Option 2: to change the stage 3 descriptions, e.g., semantics and text

# Proposals

Proposal 1: To adopt option 2, i.e., (Late or normal) data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE.

Proposal 2: To adopt the proposed changes in FIG. 1 for BLCR to TS 38.300 (Figure 9.2.3.5.X-1 in [4]).

Proposal 3: To enhance XN-U ADDRESS INDICATION message to cover the LTM MR-DC and LTM PSCell change scenarios.

- Option 1: Introduce two indicators: LTM MR-DC data forwarding indicator and LTM PSCell change data forwarding indicator

- Option 2: to change the stage 3 descriptions, e.g., semantics and text

# Reference

[1] R3-250001, RAN3#127 Meeting Agenda

[2] R3-250002, RAN3#126 Meeting Report

[3] R3-247824, Summary of offline discussion on Mobility Enh

[4] R3-250060, (BL CR to 38.300) Support for Inter-CU LTM