3GPP TSG-RAN3 Meeting #127	R3-250099

Athens, Greece, Feb 17th – 21nd, 2025

Agenda item:	10.3.1

Source:	Qualcomm Incorporated

Title:	SON MDT for NTN

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss potential SON/MDT enhancements for NTN based on agreements and open issues in the previous meetings.

# Discussion

  - MRO enhancements for NTN

### Time-based CHO

#### Distinguishing RLF before or after T1-Threshold

RAN3 discussed and agreed the following failure scenario of Too Late CHO Execution (case 1) when UE is configured with a time-based CHO trigger condition:

Case 1 (Too Late CHO Execution): RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell.

In case of Time-based CHO, to distinguish whether RLF occurred before or after T1-Threshold, down-select among below three alternatives:

Alt 1: RLF report includes the Absolute time (measured UTC time) when RLF occurred

Alt 2: RLF Report includes a flag to indicate whether the RLF happens before or after T1-Threshold

Alt 3: based on existing timers in RLF report

Existing timers can be reused to identify whether RLF happens before or after T1 with 100ms accuracy, FFS if the accuracy is a problem to be solved.

We divide Case 1 into 2 subcases:

Case 1a: RLF before time-based CHO window has started

UE encounters RLF before window for time-based CHO has even started (i.e., UE has not even attempted time-based CHO execution yet)

Time CHO config is sent + timeSinceCHO-Reconfig < T1-Threshold

If the network is able to determine that a lot of RLFs (e.g., 90%) are seen before the T1-Threshold, the network can move the T1-Threshold to the left (i.e., decrease its value) so that the UEs can start time-based CHO execution earlier and thereby reducing the possibility of these RLFs.

Case 1b: RLF after time-based CHO window has started (but before CHO execution)

UE encounters RLF after window for time-based CHO has started but CHO has not been executed

(i.e., UE has not yet executed CHO as it has till t1-Threshold + Duration to execute the CHO)

Time CHO config is sent + timeSinceCHO-Reconfig > T1-Threshold

If the network is able to determine that a lot of RLFs (e.g., 90%) are seen after the T1-Threshold but before the Duration and the CHO is not yet executed, it is perhaps because the Duration is set too large. With this knowledge, the network can perhaps reduce the Duration so that UEs execute CHO more quickly. Or this could be simply because of a certain UE implementation which executes time-based CHO “late” into the time-based CHO window. In this case, the network should not optimize Duration unnecessarily because this is happening because of the UE implementation and not because of improperly configured CHO parameters.

There were few proposals in the previous meetings to report the absolute timestamp (e.g., timestamp when RLF occurred or timestamp when CHO occurred). But we think there is no need for UE to report the absolute timestamps as existing relative timer (timeSinceCHO-Reconfig) in RLF Report and time points information at the gNB is sufficient to distinguish between case 1a and case 1b.

The definition of timeSinceCHO-Reconfig is also follows:

| timeSinceCHO-Reconfig | In case of handover failure, this field is used to indicate the time elapsed between the initiation of the last handover execution towards the target cell and the reception of the latest conditional reconfiguration. In case of radio link failure, this field is used to indicate the time elapsed between the radio link failure and the reception of the latest conditional reconfiguration while connected to the source PCell. TimeSinceCHO-Reconfig-r17:: = INTEGER (0..1023) Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer. |
| --- | --- |

t1-Threshold: The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900).

It is to be noted the gNB knows the time CHO config is sent to the UE and the CHO configuration parameters (T1-Threshold and Duration).

Therefore, the network can determine from the timeSinceCHO-Reconfig sent in RLF Report whether the RLF encountered by UE is due to case 1a (occurs before T1-Threshold) or case 1b (occurs after the T1-Threshold) by adding the time it sent CHO configuration to the UE with timeSinceCHO-Reconfig received in RLF Report and comparing it with T1-Threshold.

gNB knows the time CHO configuration is sent to the UE and the CHO configuration parameters (T1-Threshold and Duration).

gNB can store the time it sent the CHO configuration to the UE and the CHO configuration parameters (T1-Threshold and Duration) and retrieve it via the C-RNTI and timeConnFailure received in RLF Report

timeSinceCHO-Reconfig is reported in RLF Report with a granularity of 100 ms and T1-Threshold is an absolute UTC time. In the worst case, the RLFs that occur within +/- 100 ms of T1-Threshold can’t be distinguished via using timeSinceCHO-Reconfig and such a small error can be ignored.

gNB can distinguish between case 1a and case 1b (i.e., whether an RLF occurred before T1-Threshold or after T1-Threshold) via the following formula (with a small error of +/-100 ms):

- Time it sent CHO configuration to the UE + timeSinceCHO-Reconfig < or > T1-Threshold.

Also reporting the absolute timestamp adds to a lot of overhead and not all UEs might have absolute time sync (e.g., with the UTC time). We therefore propose the following:

Reporting the absolute time stamp (e.g., UTC timestamp when RLF occurs) adds to a lot of overhead

To distinguish between case 1a and case 1b, there is no need for UE to report any absolute timestamp information in RLF Report

#### Whether to distinguish RLF before or after Duration

RAN3 also agreed on the following case last meeting:

Case 2 (Too Early CHO Execution/CHO Execution to Wrong Cell): HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

FFS on whether it matters if RLF happens within the duration or after the duration period.

Case 2a: RLF “shortly” after time-based CHO execution but before T1-Threshold + Duration

Case 2b: RLF “shortly” after time-based CHO execution but after T1-Threshold + Duration

We don’t think there is a need to distinguish whether RLF occurs before or after T1-Threshold + Duration. gNB can categorize this as Too Early HO/HO to wrong cell as long the RLF occurs within the T_UEcntxt_store timer. Therefore, the following is proposed:

In case of RLF shortly after a successful time-based CHO execution, there is no need to distinguish whether RLF occurs before or after T1-Threshold + Duration. gNB can categorize this as Too Early HO/HO to wrong cell as long the RLF occurs within the T_UEcntxt_store timer

### Location-based CHO

The following location-based CHO trigger conditions are supported for NTN:

CondEvent D1:

Distance between UE and a reference location referenceLocation1 becomes larger than configured threshold distanceThreshFromReference1 and

between UE and a reference location referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2

CondEvent D2:

Distance between UE and a moving reference location determined based on movingReferenceLocation and its corresponding satellite ephemeris and epoch time broadcast in SIB19 for the serving cell becomes larger than configured threshold distanceThreshFromReference1 and

distance between UE and a moving reference location determined based on referenceLocation2 of conditional reconfiguration candidate becomes shorter than configured threshold distanceThreshFromReference2

In the previous meetings, there were proposals that Location-based CHO parameters in CondEvent D1/D2 (e.g., distanceThreshFromReference1, distanceThreshFromReference2) might not have been configured properly, therefore resulting in a Too early/Too late/wrong cell CHO.

Further, RAN3 agreed the following:

In case of Location-based CHO, there is no need to report the measured distance from the UE for earth fixed case. FFS on the earth moving case.

WA: There is no need to report any other info in the RLF report for the location-based CHO

And there were proposals to report the relative distance between UE’s exact location and the reference location/moving reference location of source/target/candidate cell when the connection failure happens (or when CHO execution happens).

We think that the existing location information reported in RLF Report is sufficient and there is no need to report any relative distance(s) to optimize location-based CHO.

Even for earth moving case, we think the gNB is aware of the “moving” reference location based on the timeConnFailure reported in RLF-Report based on the stored CHO configuration information. For example, if the RLF Report indicates timeConnFailure = 2 hours (i.e., RLF occurred before 2 hours), the gNB can determine the moving reference location 2 hours ago and can know the UE’s location 2 hours ago via the location information reported in RLF Report and the satellite ephemeris location.

Even for earth moving case, gNB is aware of the “moving” reference location based on the timeConnFailure reported in RLF-Report based on the stored CHO configuration information and can know the exact UE location where RLF happened via the location information reported in RLF Report and the satellite ephemeris location.

Convert the WA into agreement: There is no need for UE to report any other info (e.g., distance information) in RLF Report, even for the earth moving case.

  - Logged MDT enhancements for NTN

### Location-based event trigger for logged MDT

NTN supports location-based measurements in RRC_IDLE and RRC_INACTIVE. Based on this, the UE will measure in RRC_IDLE and RRC_INACTIVE if it has moved a certain distance larger than a threshold. Current event triggers in logged MDT are only for Out-of-coverage or when signal strength is below a threshold. We propose to introduce a location-based event trigger for logged MDT.

Introduce a new “location-based” event trigger to collect logged MDT e.g., only when the UE has moved a certain distance w.r.t. a reference location.

# Conclusion

2.1.1	Time-based CHO

Observation 1.	gNB knows the time CHO configuration is sent to the UE and the CHO configuration parameters (T1-Threshold and Duration).

Proposal 1:	gNB can store the time it sent the CHO configuration to the UE and the CHO configuration parameters (T1-Threshold and Duration) and retrieve it via the C-RNTI and timeConnFailure received in RLF Report

Observation 2.	timeSinceCHO-Reconfig is reported in RLF Report with a granularity of 100 ms and T1-Threshold is an absolute UTC time. In the worst case, the RLFs that occur within +/- 100 ms of T1-Threshold can’t be distinguished via using timeSinceCHO-Reconfig and such a small error can be ignored.

Proposal 2:	gNB can distinguish between case 1a and case 1b (i.e., whether an RLF occurred before T1-Threshold or after T1-Threshold) via the following formula (with a small error of +/-100 ms):

Time it sent CHO configuration to the UE + timeSinceCHO-Reconfig < or > T1-Threshold.

Observation 3.	Reporting the absolute time stamp (e.g., UTC timestamp when RLF occurs) adds to a lot of overhead

Proposal 3:	To distinguish between case 1a and case 1b, there is no need for UE to report any absolute timestamp information in RLF Report

Proposal 4:	In case of RLF shortly after a successful time-based CHO execution, there is no need to distinguish whether RLF occurs before or after T1-Threshold + Duration. gNB can categorize this as Too Early HO/HO to wrong cell as long the RLF occurs within the T_UEcntxt_store timer

2.1.2	Location-based CHO

Observation 4.	Even for earth moving case, gNB is aware of the “moving” reference location based on the timeConnFailure reported in RLF-Report based on the stored CHO configuration information and can know the exact UE location where RLF happened via the location information reported in RLF Report and the satellite ephemeris location.

Proposal 5:	Convert the WA into agreement: There is no need for UE to report any other info (e.g., distance information) in RLF Report, even for the earth moving case.

2.2.1	Location-based event trigger for logged MDT

Proposal 6:	Introduce a new “location-based” event trigger to collect logged MDT e.g., only when the UE has moved a certain distance w.r.t. a reference location.