3GPP TSG-RAN WG3 Meeting #127	R3-250201

Athens, Greece, 17-21 Feb, 2025

Title:	(TP for TS 38.413) A-IoT Device Locating

Agenda Item:	16.3

Source:	Huawei, China Telecom, Lenovo, China Unicom

Document for:	other

# 1	Introduction

In RAN3#126 meeting, some related agreements about A-IoT Device Locating were achieved in the following:

RAN3#126 agreements on A-IoT Device Locating; For topology 1, Reader ID is included in the Inventory report and Command response message in addition of AIoT RAN node ID. For topology 1, only one reader ID included in Inventory report and Command response message. If AIoT RAN node would receive multiple responses for the same AIoT device it can send multiple inventory report messages. For topology 1, no need to include an AIoT Device location in the Inventory report and Command response message. Whether to have awareness of reader location in AIoT CN needs further discussion. For topology 2, the AIOT CN may get UE reader location via ULI in NGAP messages or via legacy LCS mechanism.

In RAN WID [1], RAN3 scope is concluded:

R19 RAN WID: - RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In this paper, we futher discuss the support of Ambient IoT device locating, and provide the corresponding TP.

# 2	Discussion

## 2.1	A-IoT Device Locating

- Based on RAN and SA2 study item conclusion, R19 RAN WID, we need to support Device location reporting at reader ID granularity via Inventory Report message. There are two ways to report Device location at reader ID granularity via Inventory Report:

- Option 1:	report a list of {A-IoT NAS PDU (device identifier) + Reader ID}

- NAS PDU 1 (device Identifier 1) + Reader ID 1

- NAS PDU 2 (device Identifier 2) + Reader ID 1

- NAS PDU 3 (device Identifier 3) + Reader ID 2

- NAS PDU 4 (device Identifier 4) + Reader ID 2

- Option 2:	report a list of {Reader ID + List of A-IoT NAS PDU (device identifier)}

- Reader ID 1

  - NAS PDU 1 (device Identifier 1)

  - NAS PDU 2 (device Identifier 2)

- Reader ID 2

  - NAS PDU 3 (device Identifier 3)

  - NAS PDU 4 (device Identifier 4)

- Option 2 is more efficient than option 1, especially if there are large number of devices acces from same reader. It is also noticed that option 2 makes the report of reader id as a mandatory feature, and in option 1, the reader id can be defined as optional for each device.

- Proposal 1: RAN3 make decision on the way to provide reader id in the Inventory Report, either report a list of {A-IoT NAS PDU (device identifier) + Reader ID}, or report a list of {Reader ID + List of A-IoT NAS PDU (device identifier)}.

## 2.2	Proximity determination

Regarding to proximity determination, the following two solutions are captured in RAN TR 38.769:

RAN TR 38.769: Proximity determination is feasible with either of the two following solutions. Potential specification impact or not will not be determined in this study item. Solution 1: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader. Solution 2: If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader based on measurements at the reader side. Proximity determination based on device-side measurements is not considered.

And proximity determination via Solution 1 was adopted in RAN WID:

R19 RAN WID: General Scope - Proximity determination via Solution 1 in TR 38.769 only.

Based on the Solution 1, after the reader successfully receives the A-IoT NAS PDU (Device Identifier) from the device, the gNB is able to report Reader ID together with the NAS PDU (Device Identifier) received from the device via the Inventory Report message. Therefore, the solution 1 for proximity determination can be covered by the reader ID granularity device locating.

- Proposal 2: Proximity determination solution 1 can be covered by the reader ID granularity device locating.

# 3	Conclusion

In this contribution, we further discussed the AIoT Command procedure over CN-RAN interface for Topology 1, and got the following proposals:

- Proposal 1: RAN3 make decision on the way to provide reader id in the Inventory Report, either report a list of {A-IoT NAS PDU (device identifier) + Reader ID}, or report a list of {Reader ID + List of A-IoT NAS PDU (device identifier)}.

- Proposal 2: Proximity determination solution 1 can be covered by the reader ID granularity device locating.

- The corresponding TP is provided in section 5.

# 4. Reference

- [1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Dec. 2024

- [2] 3GPP TR 38.769 V2.0.1, Study on solutions for ambient IoT (Internet of Things), 2024-12

# 5. TPs for TS 38.413 (grey part overlapped with Inventory TP)

#### 9.3.x.4	Inventory Report Transfer --- Option 1

This IE is transparent to the AMF, if via AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Device ID to Report List |  | 1 |  |  |
| >Device ID to Report Item |  | 1..<maxnoofDeviceIDstoReport> |  |  |
| >>A-IoT NAS PDU | M |  | OCTET STRING |  |
| >>Reader Identify | O |  | 9.3.3.zz |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDeviceIDstoReport | Maximum no. of Device IDs to report. Value is FFS. |

#### 9.3.x.4	Inventory Report Transfer --- Option 2

This IE is transparent to the AMF, if via AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Per Reader Report List |  | 1 |  |  |
| >Per Reader Report Item |  | 1..<maxnoofReaderstoReport> |  |  |
| >>Reader ID | M |  | 9.3.3.zz |  |
| >>Device ID to Report List |  | 1 |  |  |
| >>>Device ID to Report Item |  | 1..<maxnoofDeviceIDstoReport> |  |  |
| >>>>A-IoT NAS PDU | M |  | OCTET STRING |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDeviceIDstoReport | Maximum no. of Device IDs to report. Value is FFS. |
| maxnoofReaderstoReport | Maximum no. of Readers to report. Value is FFS. |