3GPP TSG-RAN WG3 Meeting #127	R3-250209

Athens, Greece, 17 – 21 February 2025

Agenda Item:	12.3

Source: 				LG Electronics Inc.

Title:		 Further discussion on access control in 5G Femto

Document for:		Discussion and Decision

1.	Introduction

In last RAN3 meeting, it was agreed to capture the access control for NR Femto into the BL CR to TS 38.300 [1]. In this contribution, we will discuss further clarification on access control in a cell shared by PLMN and PNI-NPN, and provide our view on it.

2.	Discussion

In last meeting, RAN3 captured the TP for access control in NR Femto in [1].

In R3-247829 [1], … 4.x.4 Access Control - Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription. NOTE: The NR Femto node may use the CAG mechanism for PNI NPN (see clause 16.7.4) as follows: - The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG. - The NR Femto may activate a cell shared by both PLMN and CAG, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

In LTE, the hybrid cell is defined as the cell which is accessible as a CSG cell by UEs which are members of the CSG and as a normal cell by all other UEs. The hybrid cell in LTE broadcasts a CSG indicator set to false and a specific CSG identity. As highlighted one above, the cell shared by PLMN and PNI-NPN is introduced for the NR Femto, which is similar with the hybrid cell in 4G Femto. That is, the cell broadcasting the npn-IdentityInfoList-r16 IE and the cellReservedForOtherUse IE set to false is used to support both the CAG supporting UE and CAG non-supporting UE.

Observation 1: The NR Femto cell shared by PLMN and PNI-NPN is introduced for the NR Femto, which is similar with the hybrid cell in 4G Femto.

We think that some clarification on how to support the access control in the NR Femto cell shared by PLMN and PNI-NPN is needed. When the UE accesses to the shared cell, the NR Femto cannot differentiate a CAG supporting UE with a CAG non-supporting UE because it does not have knowledge on UE CAG capability. Therefore, during the UE’s initial access, the NR Femto always provides the supported CAG ID(s) of the shared cell to the AMF irrespective of the UE CAG capability. The AMF performs the access control for the CAG supporting UE as described in TS 23.501. However, we think that since the AMF does not know whether the CAG non-supporting UE accesses to AMF via the cell shared by PLMN and PNI-NPN, the AMF rejects the Registration Request from the CAG non-supporting UE based on UE subscription and the supported CAG ID(s) of the cell as follows:

From TS 24.501, … ##### 5.5.1.2.8 Abnormal cases on the network side The following abnormal cases can be identified: … j) Based on operator policy, if the initial registration request from a UE not supporting CAG is rejected due to CAG restrictions, the network shall reject the initial registration with a 5GMM cause value other than the 5GMM cause #76 (Not authorized for this CAG or authorized for CAG cells only). …

Observation 2: According to the current specification, the AMF rejects the Registration Request from the CAG non-supporting UE which accesses to the cell shared by PLMN and PNI-NPN.

Proposal 1: To control the UE initial access via the NR Femto, the AMF needs to know whether the UE’s serving cell is shared by the PLMN and PNI-NPN.

If the access of the CAG non-supporting UE via the shared cell in NR Femto is always rejected by the AMF, there is no need to introduce the cell shared by the PLMN and PNI-NPN in NR Femto. In order to avoid this problem, therefore, the NR Femto needs to provide to the AMF whether the UE’s serving cell is shared by the PLMN and PNI-NPN. That is, a new indication needs to be defined into the INITIAL UE MESSAGE message.

Proposal 2: The NR Femto provides to the AMF whether the UE’s serving cell is shared by the PLMN and PNI-NPN in the INITIAL UE MESSAGE message.

As described above, since the AMF behaviour needs to be updated to support the access of the CAG non-supporting UE via the shared cell in NR Femto, it is better to send a LS to SA2 to provide the RAN3 agreements on the access control for NR Femto.

Proposal 3: It is proposed to send a LS to SA2 to provide the RAN3 agreements on the access control for NR Femto in Appendix.

In EPS, CSG is used for access control of Femto cell (HeNB) and there is a hybrid cell, which is similar with an NR Femto cell shared by PLMN and PNI-NPN. When a UE is accessing via hybrid cell, based on subscription of an UE, an MME indicates whether the UE is a CSG member to RAN and S-GW/P-GW/PCRF for differentiated treatment and charging between CSG and non-CSG members. See details in the following.

From TS 23.401, … #### 5.3.2.1 E-UTRAN Initial Attach … 12. If an ESM container was not included in the Attach Request, steps 12, 13,14,15,16 are skipped. If the attach type is not set to "Emergency" or "RLOS", and the ESM container was included in the Attach Request, and the UE has indicated support for Attach without PDN Connectivity, and the network supports Attach without PDN Connectivity, and the PDN Connection Restriction is set in the subscriber data, then the new MME shall not establish PDN connection, and steps 12, 13, 14, 15 and 16 are skipped. … If the subscription context does not indicate that the APN is for a PDN connection to an SCEF, the new MME selects a Serving GW as described in clause 4.3.8.2 on Serving GW selection function and allocates an EPS Bearer Identity for the Default Bearer associated with the UE. Then it sends a Create Session Request (IMSI, MSISDN, MME TEID for control plane, PDN GW address, PDN Address, APN, RAT type, LTE-M RAT type reporting to PGW flag, Default EPS Bearer QoS, PDN Type, APN-AMBR, EPS Bearer Identity, Protocol Configuration Options, Handover Indication, ME Identity (IMEISV), User Location Information (ECGI and TAI), UE Time Zone, User CSG Information, MS Info Change Reporting support indication, Selection Mode, Charging Characteristics, Trace Reference, Trace Type, Trigger Id, OMC Identity, Maximum APN Restriction, Dual Address Bearer Flag, the Protocol Type over S5/S8, Serving Network, APN Rate Control Status) message to the selected Serving GW. If Control Plane CIoT EPS Optimisation applies, then the MME shall also indicate S11-U tunnelling of NAS user data and send its own S11-U IP address and MME DL TEID for DL data forwarding by the SGW. User CSG Information includes CSG ID, access mode and CSG membership indication. … 13. The Serving GW creates a new entry in its EPS Bearer table and sends a Create Session Request (IMSI, MSISDN, APN, Serving GW Address for the user plane, Serving GW TEID of the user plane, Serving GW TEID of the control plane, RAT type, Default EPS Bearer QoS, PDN Type, PDN Address, subscribed APN-AMBR, EPS Bearer Identity, Protocol Configuration Options, Handover Indication, ME Identity, User Location Information (ECGI), UE Time Zone, User CSG Information, MS Info Change Reporting support indication, PDN Charging Pause Support indication, Selection Mode, Charging Characteristics, Trace Reference, Trace Type, Trigger Id, OMC Identity, Maximum APN Restriction, Dual Address Bearer Flag, Serving Network, APN Rate Control Status) message to the PDN GW indicated by the PDN GW address received in the previous step. After this step, the Serving GW buffers any downlink packets it may receive from the PDN GW without sending a Downlink Data Notification message to the MME until it receives the Modify Bearer Request message in step 23 below. The MSISDN is included if received from the MME. … 14. If dynamic PCC is deployed and the Handover Indication is not present, the PDN GW performs an IP-CAN Session Establishment procedure as defined in TS 23.203 [6], and thereby obtains the default PCC rules for the UE. If the UE is accessing over WB-E-UTRA, this may lead to the establishment of a number of dedicated bearers following the procedures defined in clause 5.4.1 in association with the establishment of the default bearer, which is described in Annex F. … If the CSG information reporting triggers are received from the PCRF, the PDN GW should set the CSG Information Reporting Action IE accordingly. … 17. If an APN Restriction is received, then the MME shall store this value for the Bearer Context and the MME shall check this received value with the stored value for the Maximum APN Restriction to ensure there are no conflicts between values. If the Bearer Context is accepted, the MME shall determine a (new) value for the Maximum APN Restriction. If there is no previously stored value for Maximum APN Restriction, then the Maximum APN Restriction shall be set to the value of the received APN Restriction. MME shall not deactivate bearer(s) with emergency ARP, if present, to maintain valid APN restriction combination. … If the UE initiates the Attach procedure at a hybrid cell, the MME shall check whether the CSG ID is contained in the CSG subscription and is not expired. The MME shall send an indication whether the UE is a CSG member to the RAN along with the S1-MME control message. Based on this information, the RAN may perform differentiated treatment for CSG and non-CSG members. …

Observation 3: Similar with an NR Femto cell shared by PLMN and PNI-NPN, a hybrid cell in LTE is used to allow access both for a membership UE and a non-membership UE.

Observation 4: For hybrid CSG cell support, an MME indicates whether a UE is a CSG member to a RAN and S-GW/P-GW/PCRF for differentiated treatment and charging between CSG and non-CSG members.

However, it is unclear whether the feature similar with the CSG hybrid cell in LTE is introduced in NR Femto. That is, it is required for RAN3 to discuss whether the NR Femto performs differentiated handling between a CAG supporting UE and a CAG non-supporting UE. We believe that differentiated handling is required between a CAG supporting UE and a CAG non-supporting UE, if a shared cell is used in NR Femto. Otherwise, it seems that there is no difference between a normal PLMN cell and a shared cell. If RAN3 decides to support differentiated handling between a CAG supporting UE and a CAG non-supporting UE in the shared cell, the NR Femto needs to know whether a CAG non-supporting UE is accessing via a shared cell, e.g. based on the AMF provided information.

Proposal 4: An NR Femto cell shared by PLMN and PNI-NPN should be able to support differentiated handling between a CAG supporting UE and a CAG non-supporting UE.

Proposal 5: It is proposed to discuss how the NR Femto becomes aware of whether a CAG non-supporting UE is accessing via a shared cell.

We also think that the connected mode mobility considering a cell shared by PLMN and PNI-NPN should be supported in NR Femto. For example, the source RAN should be able to hand over the CAG non-supporting UE served by the PLMN cell towards the shared cell in target NR Femto. To this end, the source RAN also needs to know whether the target cell is a cell shared by PLMN and PNI-NPN in NR Femto.

Proposal 6: It is proposed to discuss how to support the connected mode mobility considering a cell shared by PLMN and PNI-NPN.

In order to capture the above observation and proposals, the following proposal is also suggested to RAN3:

Proposal 7: It is proposed to agree the corresponding TPs in [2].

3.	Conclusion

In this contribution, we focused on further clarification on access control in a cell shared by PLMN and PNI-NPN, and then provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: To control the UE initial access via the NR Femto, the AMF needs to know whether the UE’s serving cell is shared by the PLMN and PNI-NPN.

Proposal 2: The NR Femto provides to the AMF whether the UE’s serving cell is shared by the PLMN and PNI-NPN in the INITIAL UE MESSAGE message.

Proposal 3: It is proposed to send a LS to SA2 to provide the RAN3 agreements on the access control for NR Femto in Appendix.

Proposal 4: An NR Femto cell shared by PLMN and PNI-NPN should be able to support differentiated handling between a CAG supporting UE and a CAG non-supporting UE.

Proposal 5: It is proposed to discuss how the NR Femto becomes aware of whether a CAG non-supporting UE is accessing via a shared cell.

Proposal 6: It is proposed to discuss how to support the connected mode mobility considering a cell shared by PLMN and PNI-NPN.

Proposal 7: It is proposed to agree the corresponding TPs in [2].

4.	Reference

- R3-247829, “Access Control with NR Femto – Stage 2 TP,” Ericsson, Huawei, Nokia, China Telecom, Lenovo, Samsung, NTT DOCOMO, ZTE, Verizon, BT, November, 2024.

- R3-250210, “(TP to TS 38.300 and 38.413) TP for access control in NR Femto,” LG Electronics, February, 2025.

5.	Appendix: Draft LS to SA2 for access control in NR Femto

This appendix provides the draft LS to SA2 based on the proposal of this contribution.

----------------Start of the First Change---------------

3GPP TSG-RAN WG3 #127 R3-25xxxx Athens, Greece, 17 – 21, February, 2025 Title: [Draft] LS on handling of location information in multi-path operation Response to: - Release: Rel-19 Work Item: NR_WAB_5GFemto-Core, 5G_Femto Source: RAN3 To: SA2 Cc: - Contact Person: Name: Seokjung Kim E-mail Address: seokjung.kim@lge.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: none 1. Overall description RAN3 agreed to capture the access control for NR Femto into RAN3 BL CR to TS 38.300 (R3-247936). NOTE: The NR Femto node may use the CAG mechanism for PNI NPN (see clause 16.7.4) as follows: - The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG. - The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.. RAN3 has discussed that in order to control the UE initial access via the NR Femto, the AMF needs to know whether the UE’s serving cell is shared by the PLMN and PNI-NPN. Therefore, it is agreed to introduce a new indication to inform the cell shared by PLMN and PNI-NPN in INITIAL UE MESSAGE message. RAN3 also believes that the AMF behaviour needs to be updated to support the access of the CAG non-supporting UE via the shared cell in NR Femto. RAN3 is also discussing how to support differentiated handling between CAG supporting UE and CAG non-supporting UE, which may require AMF support. RAN3 will continue to discuss how to support differentiated handling in NR Femto and inform SA2 if AMF support is required. 2. Actions: To SA2: ACTION: RAN3 kindly asks SA2 to take above information into consideration and update the corresponding specifications. 3. Date of Next RAN3 Meetings: 3GPP TSG RAN3#127bis 7 – 11 April 2024 Wuhan, CN 3GPP TSG RAN3#128 19 – 23 May Malta, MT

-----------------End of the First Change---------------