3GPP TSG-RAN WG3 #127	          R3-250215

Athens, Greece, 17th – 21st Feb 2025

Agenda item:	9.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the UE Location Information for NB-IoT NTN

Document for:	Discussion and Decision

# 1	Introduction

RAN3 received a SA2 LS ([1]) on UE Location Information for NB-IoT NTN. This contribution discusses the RAN3 impact and propose a reply LS.

# 2	Discussion

The SA2 LS ([1]) states:

1. Overall Description: SA2 thanks RAN2 and RAN3 for the reply LS on UE Location Information for NB-IoT NTN. SA2 approved the attached CR which allows the MME, if requested from eNB, to send the coarse location information received from the UE to NB-IoT eNB. SA2 nevertheless observed that the CR approved by RAN3 (TS 36.413 CR1957) did not include the “Coarse UE Location Requested” IE in UL NAS Transport, Resume Request and the “Coarse UE Location IE” in Resume Response. Even though Coarse Location Information can be requested once in the Initial UE message also in these cases (and provided to the eNB in a downlink S1-AP message), this prevents the Coarse Location Information of the UE to be requested by eNB and provided from MME to eNB at a later stage during a RRC connection when user plane bearers are established. The relevant procedures in the SA2 specification are “Establishment of S1-U bearer during Data Transport in Control Plane CIoT EPS Optimisation” (see clause 5.3.4B.4 of TS 23.401), “Connection Resume procedure” (see clause 5.3.5A) and “MT-EDT procedure for User Plane CIoT EPS Optimisation” (see clause 5.3.5B). SA2 would like to understand if RAN2 and RAN3 has performed any analysis on whether Coarse Location Information is needed by the eNB in any of the above procedures. In case RAN2 and RAN3 conclude that Coarse Location Information is needed in these cases, SA2 would like to highlight that AS security is activated during these procedures which would also allow the UE to provide the Coarse Location Information to the eNB via the RRC protocol. 2. Actions: To RAN2, RAN3: ACTION: SA2 asks RAN2 and RAN3 to take the above information into account and indicate whether Coarse Location Information is needed by the eNB in the procedures defined in clause 5.3.4B.4, 5.3.5A and 5.3.5B.

In last meeting, RAN3 agreed

- eNB sends MME the INITIAL UE MESSAGE message including the request for UE coarse location.

- If available, MME sends the UE coarse location in the INITIAL CONTEXT SETUP REQUEST, DOWNLINK NAS TRANSPORT, or CONNECTION ESTABLISHMENT INDICATION messages may contain the coarse UE location.

For UP-CIOT:

Before we discuss the detail, we’d better check the history. The issue was originally discussed by RAN2 about the coarse UE location reporting for NB-IoT NTN in both Rel-17 and Rel-18. Based on the input from [2], it was recognized that the majority of NB-IoT NTN devices will support the CP CIoT optimization solution, in which AS security cannot be assumed to be in place.

Excerpt from R2-2310192 [2]: Observation 3: In practice, we have to recognize that majority of NB-IoT NTN devices will focus on CP CIoT Optimizations with Data over NAS, thus AS security cannot be assumed to be in place.

Therefore, RAN2 focused on the coarse UE location reporting requirements for NB-IoT CP solution without AS security activation and asked SA2/CT1 and RAN3 to design the UE location reporting via NAS layer in Rel-18.

Excerpt from LS R2-2311326 [3]: No consensus could be reached in RAN2 as to whether an AS-based solution of reporting UE location, even coarse-grained, for NB-IoT UEs using control plane optimization could be supported, due to privacy and security concerns raised by some companies. RAN2 would thus like to ask SA2 and CT1 if UE location reporting via NAS layer can be considered in Rel-18, to satisfy this operator requirement.

Accordingly, the NAS-based coarse UE location reporting has been specified in Rel-18 after extensive discussions in CT1, SA2, RAN3 and RAN2, with multiple LS [1][3][4][5][6][7] exchange between the WGs. According to the current specification, the coarse UE location Information request and report can be sent in NAS signalling. The location information can be delivered not only to the MME but also can be sent back from the MME to the eNB, if requested.

Observation 1: For NB-IoT NTN, it is recognized that the majority of NB-IoT NTN devices support the CP CIoT solution, thus AS security cannot be assumed to be in place.

Observation 2: For NB-IoT NTN, according to the current specification, the coarse UE location information request and report can be sent in NAS signalling even the AS security is not activated. This information can be delivered not only to the MME but also from the MME back to the eNB, if requested.

For the cases raised by SA2 LS [1], it is mainly for the UE supporting User Plane CIoT EPS optimization, in which the AS security can be activated between the UE and the network. However, as indicated in observation 1, it seems the number of NB-IoT NTN device only supporting UP solution is quite limited. Therefore, first of all, it needs to be confirmed whether the requirements to support the coarse UE location reporting for UP solution is really needed.

Observation 3: The requirement for supporting coarse UE location reporting in cases where the UE only supports UP CIoT solution is unclear.

Back to the SA2 LS, SA2 question is whether support the UE coarse location retrieval from UE during the Connection Resume procedure. In the SA2 LS, the attached S2-2412643 CR states:

In the case of satellite NB-IoT, the UE and the MME may support reporting of Coarse Location Information (with the format defined in TS 24.301 [46]) from the UE to MME in the NAS Security Mode Command procedure. As described for the Attach procedure (clause 5.3.2.1), TAU procedures (clauses 5.3.3.1 and 5.3.3.2) and Service Request procedure (clause 5.3.4.1), in the case of NB-IoT, the MME may, if supported by the UE and if the MME has obtained user consent based on proprietary mechanisms depending on local regulations, request the UE, in a NAS Security Mode Command message, to report its Coarse Location Information. If requested, the UE provides its Coarse Location Information in the NAS Security Mode Complete message.

The above TS23.401 text does not describe the MME can request the UE coarse location in Connection Resume procedure (TS 23.401 Clause 5.3.5A). So unless TS23.401 text is updated, we assume the coarse location reporting is not supported during the resume procedure. Thus no enhancement is needed for S1AP UE Context Resume procedure, unless it is required by RAN2/SA2.

Observation 4: SA2 spec only describes the MME can request the coarse location from UE during Attach, TAU and Service Request procedure, but not for Connection Resume procedure.

Proposal 1: For User Plane CIoT EPS optimisation, RAN3 does not see the scenario requiring the eNB to request the UE coarse location when UE initiates connection resumes procedure, unless the scenario is confirmed by RAN2/SA2.

For CP-CIOT:

SA2 question is whether the eNB can request the UE coarse location via UL NAS TRANSPORT message. But the first S1AP message from the eNB is the INITIAL UE MESSAGE. It is unclear why cannot reuse the existing method (i.e. INITIAL UE MESSAGE).

Observation 5: For CP-CIOT, the existing Initial UE Message procedure can be used to request the UE coarse location from the MME.

In a summary, it seems current mechanism is enough. There is no need to introduce additional S1AP enhancements.

Proposal 2: For Control Plane CIoT EPS optimisation, the first S1AP message from the eNB is the INITIAL UE MESSAGE, so the eNB can request the UE coarse location via the Initial UE Message procedure. There is no need to enhance UL NAS TRANSPORT message.

The draft reply LS can be found in [8]

# 3	Conclusion

In this contribution, we discussed the SA2 LS. Our proposals are:

Observation 1: For NB-IoT NTN, it is recognized that the majority of NB-IoT NTN devices support the CP CIoT solution, thus AS security cannot be assumed to be in place.

Observation 2: For NB-IoT NTN, according to the current specification, the coarse UE location information request and report can be sent in NAS signalling even the AS security is not activated. This information can be delivered not only to the MME but also from the MME back to the eNB, if requested.

Observation 3: The requirement for supporting coarse UE location reporting in cases where the UE only supports UP CIoT solution is unclear.

Observation 4: SA2 spec only describes the MME can request the coarse location from UE during Attach, TAU and Service Request procedure, but not for Connection Resume procedure. Observation 5: For CP-CIOT, it is unclear why the eNB cannot reuse the existing method (i.e. INITIAL UE MESSAGE).

Proposal 1: For User Plane CIoT EPS optimisation, RAN3 does not see the scenario requiring the eNB to request the UE coarse location when UE initiates connection resumes procedure, unless the scenario is confirmed by RAN2/SA2.

Observation 5: For CP-CIOT, the existing Initial UE Message procedure can be used to request the UE coarse location from the MME.

Proposal 2: For Control Plane CIoT EPS optimisation, the first S1AP message from the eNB is the INITIAL UE MESSAGE, so the eNB can request the UE coarse location via the Initial UE Message procedure. There is no need to enhance UL NAS TRANSPORT message.

The draft reply LS can be found in [8]

# References

- R3-250017/S2-2412665 	Reply LS on UE Location Information for NB-IoT NTN	SA2(Qualcomm)

- R2-2310192 NB-IoT NTN Coarse UE location reporting, Inmarsat, Viasat, Sateliot, Novamint, ESA, Thales

- R2-2311326 - LS on UE Location Information for NB-IoT

- C1-239363 LS on UE Location Information for NB-IoT NTN

- S2-2403851 Reply LS on UE Location Information for NB-IoT NTN

- R2-2403770 LS on UE Location Information for NB-IoT NTN (R2-2402143/ S2-2403851)

- R3-245819 Reply LS on UE Location Information for NB-IoT NTN

- R3-250216 [draft] Reply LS on UE Location Information for NB-IoT NTN