3GPP TSG-RAN WG3 Meeting #127															R3-250217

Athens, Greece, 17th – 21st Feb 2025

Agenda item:	9.2

Source:	Nokia, Nokia Shanghai Bell, Samsung, CATT, Thales, Huawei, Qualcomm, Vivo

Title:	Discussion on inter-gNB RACH-less HO in NTN

Document for:	Discussion and Decision

# 1	Introduction

RAN3 received a LS ([1]) from RAN2 regarding inter-gNB RACH-less HO in NTN.  This contribution discusses the related issue and propose a reply LS.

# 2	Discussion

RAN2 LS states:

1. Overall Description: At RAN2#128 it has been discussed whether RACH-less handover (described in 9.2.3.6 of TS 38.300) can be also supported in inter-gNB scenario in NTN. RAN2 has assumed that in NTN RACH-less handover can be supported for intra/inter-satellite handover with same gateway/gNB and intra/inter-satellite handover with gateway/gNB switch. However, RAN2 would like to confirm with RAN3 that the assumption made at RAN2#128 meeting for inter-gNB scenario in NTN is feasible from RAN3 point of view. 2. Actions: To TSG RAN WG3. ACTION: RAN2 respectfully asks for RAN3 feedback on any potential issue if RACH-less handover in inter-gNB NTN scenario is supported.

Before we discuss RACH-less HO for NTN, it is worthy to note that RAN3 actually discussed the inter-gNB RACH-less HO for mobile IAB in Rel-18. During Mobile IAB-DU migration procedure, The UE(s) connected to the mobile IAB-node are handed over from the cell(s) of the source logical mobile IAB-DU that have F1 set up with the source F1-terminating IAB-donor-CU to the cell(s) of the target logical mobile IAB-DU that have F1 set up with the target F1-terminating IAB-donor-CU.

RAN3 sent a LS ([2]) to RAN2 asking whether it is feasible to use RACH-less handover. Here is the RAN3 LS:

1. Overall Description: RAN3 would like to thank RAN2 for their LS on UE RACH-less handover for mobile IAB (R3-233713/R2-2306817). RAN3 has discussed RACH-less handover for UEs served by a mobile IAB during DU migration. RAN3 identified the following issues: (1) During DU migration, UE handovers may not only occur from the source logical DU’s cell but also from other cells to the co-located target logical DU’s cell. RAN3 assumes that RACH-less handover can only be applied to those UEs that are handed over from the source logical DU’s cell. The co-located target logical DU therefore needs to be able to derive from the information it receives during UE handover preparation, whether the UE is presently connected to the source logical DU. (2) When the target logical DU configures the UE’s beam to be used in the target cell for RACH-less handover based on network-implementation-specific knowledge (i.e., without measurement report), it needs to identify the beam configuration this UE presently uses in the source logical DU’s cell. For this purpose, the target logical DU needs to know a UE ID the logical source DU uses for this UE in order to identify the UE in the source logical DU during the UE handover preparation procedure. (3) When the target logical DU configures the UE’s beam to be used in the target cell for RACH-less handover based on legacy measurements, it needs to be able to obtain the beam information the UE reported to the source logical DU’s CU in the measurement report. RAN3 asks RAN2 to take the above feedback into account, and to consider whether the existing RRC container (i.e., HandoverPreparationInformation) contains sufficient information to address the above issues, and to provide feedback, if any. 2. Actions: To RAN2 group. ACTION: RAN3 asks RAN2 to take the above feedback into account, and to consider whether the existing RRC container (i.e., HandoverPreparationInformation) contains sufficient information to address the above issues, and to provide feedback, if any.

RAN2 discussed and agreed no issue to support RACH-less HO in mobile IAB Du migration. Here is the RAN2 reply LS ([3]):

1. Overall Description: RAN2 would like to thank RAN3 for the Reply LS on UE RACH-less handover for mobile IAB. In the Reply LS, RAN3 raised 3 issues and asked RAN2, whether the existing RRC container (i.e., HandoverPreparationInformation) contained sufficient information to address these issues. RAN2 discussed the matter and reached the following conclusion: Issue 1 in RAN3’s Reply LS: - During DU migration, UE handovers may not only occur from the source logical DU’s cell but also from other cells to the co-located target logical DU’s cell. RAN3 assumes that RACH-less handover can only be applied to those UEs that are handed over from the source logical DU’s cell. The co-located target logical DU therefore needs to be able to derive from the information it receives during UE handover preparation, whether the UE is presently connected to the source logical DU. RAN2’s response on issue 1: The target logical DU is able to derive from the PCI and the NCGI contained in the HandoverPreparationInformation IE whether the UE is presently connected to the source logical DU. Issue 2 in RAN3’s Reply LS: - When the target logical DU configures the UE’s beam to be used in the target cell for RACH-less handover based on network-implementation-specific knowledge (i.e., without measurement report), it needs to identify the beam configuration this UE presently uses in the source logical DU’s cell. For this purpose, the target logical DU needs to know a UE ID the logical source DU uses for this UE in order to identify the UE in the source logical DU during the UE handover preparation procedure. RAN2’s response on issue 2: The target logical DU is able to identify the UE based on the C-RNTI this UE uses in the source logical DU and which is contained in the HandoverPreparationInformation IE. Issue 3 in RAN3’s Reply LS: (3) When the target logical DU configures the UE’s beam to be used in the target cell for RACH-less handover based on legacy measurements, it needs to be able to obtain the beam information the UE reported to the source logical DU’s CU in the measurement report. RAN2’s response on issue 3: The target logical DU is able to obtain the beam information the UE reported to the source logical DU’s CU in the measurement report from the MeasResultList2NR IE contained in the HandoverPreparationInformation IE. 2. Actions: To RAN3: RAN2 respectfully asks RAN3 to take the above into account.

The main RAN3 related aspect for RACH-less HO is the target gNB need to configure the correct beam for the UE. RAN2 agreed that the target gNB can configure the correct beam based on the beam information the UE reported to source gNB in the measurement report from the MeasResultList2NR IE contained in the HandoverPreparationInformation IE.

The conclusion made for mobile IAB is also applicable to intra/inter-satellite handover with gateway/gNB switch in NR NTN. We do not see any blocking issues to allow RACH-less handover also in inter-gNB scenario. It should be allowed if NW implementation can handle it.

Then as described in TS 38.300, currently the RACH-less can be configured for a UE during the intra-gNB NTN procedure. For NTN inter-gNB handover case, RAN2 is discussing whether the RACH-less can be also applicable. The detailed discussions are excerpted below.

R2-2410438 Various Corrections and Open Points for Rel-18 NTN Nokia, Nokia Shanghai Bell discussion Rel-18 NR_NTN_enh-Core Proposal 1: RAN2 assumes the requirements and parameters for FR2-1 are generally applicable to FR2-NTN, unless specified otherwise. Proposal 2: Consider a general statement in RAN2 Technical Specifications or a pointer to TS 38.101 to reflect which requirements apply to FR2-NTN. Proposal 3: If Proposal 2 is not agreeable, RAN2 agrees a single CR fixing the FR2-NTN requirements for all affected fields/IEs. Proposal 4: Support RACH-less HO also in inter-gNB scenarios for NTN. Proposal 5: RAN2 reopens the discussion on whether the UE can measure an NTN cell while camping or being connected to a TN cell. RAN2 reflects RAN4 requirements in the appropriate specifications. Proposal 6: Align with RAN4 regarding the requirements for SIB19 acquisition in TN cell. [AT128][302][R18 NR NTN] LS to RAN3 on RACH-less HO (Nokia) Scope: Draft an LS to RAN3 on RACH-less HO in NTN Intended outcome: Draft LS Deadline for draft LS (in R2-2410967): Thursday 2024-11-22 08:00 R2-2410967 Draft LS on RACH-less HO in NTN (Nokia) LSout To:RAN3, Cc:RAN1 NR_NTN_enh-Core Remove “Based on the analysis provided in several TDocs (e.g. R2-2410527 or R2-24010438)” Change “conclusion” to “assumption” Remove Draft Revised in R2-2411191 R2-2411191 LS on RACH-less HO in NTN (Nokia) LSout To:RAN3, Cc:RAN1 NR_NTN_enh-Core Approved R2-2410642 Correction on RACH-less HO in NR-NTN vivo, Samsung, Nokia, Nokia Shanghai Bell, THALES, Huawei, HiSilicon, CATT CR Rel-18 38.300 18.3.0 0904 1 F NR_NTN_enh-Core R2-2408013 Postponed

When referring to the TS 38.331, the rach-LessHO IE is included in the ReconfigurationWithSync, which includes the timing advance value encoded as zero or source, and the beam indication. This is included in the inter-node RRC message – HandoverCommand.

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                     OPTIONAL,

<Skip the irrelevant >

[[

rach-LessHO-r18                 RACH-LessHO-r18                                                 OPTIONAL,   -- Need N

sl-IndirectPathMaintain-r18     ENUMERATED{true}                                                OPTIONAL     -- Cond MP

]]

}

RACH-LessHO-r18 ::=                 SEQUENCE {

targetNTA-r18                       ENUMERATED {zero, source}                                   OPTIONAL,   -- Need N

beamIndication-r18                  CHOICE {

tci-StateID-r18                     TCI-StateId,

ssb-Index-r18                       SSB-Index

}                                                                                               OPTIONAL,   -- Need N

...

}

Observation: For RACH-less handover, the target gNB can decide the timing advance value encoded as zero or source, and the beam indication, and send to the UE via the inter-node RRC message (i.e. the HandoverCommand).

The LS asks whether there is any potential issue if RACH-less handover in inter-gNB NTN scenario from RAN3 perspective.

First, the target NTA timing advance value, either zero, or the same as the source can be decided by the target gNB based on its own discretion. One example is that for the intra-satellite and intra-gNB case with cell change, the gNB can configure the target NTA as source. While for other cases, e.g., for inter-satellite case, the gNB can configure to be zero. Then the UE can autonomously pre-compensate the TTA for the RTT between the UE and the reference point, as indicated in TS 38.300.

Second, the target gNB can decide the beam identifier included in the RACH-LessHO by the UE at the target cell based on the measurement results encoded as MeasResultList2NR contained in the HandoverPreparationInformation inter-node RRC message. This includes measurement results pre SSB or CSI-RS of the target cell.

Last, even for configured grant case, the target gNB can configure the associated beam information towards the UE, then it is up to the UE to use the configured grant based on the SSB measurement results.

Hence based on the above analysis, there is no issue for the RACH-less handover in the inter-gNB NTN scenario.

So we propose to reply RAN2 that there is no issue to support RACH-less handover also in inter-gNB scenario.

Proposal: RAN3 send a reply LS to RAN2, that there is no issue to support RACH-less handover also in inter-gNB scenario.

The draft reply LS can be found in ([4])

# 3	Conclusion

In this contribution, we discussed inter-gNB RACH-less HO in NTN. Our proposals are:

Proposal: RAN3 send a reply LS to RAN2, that there is no issue to support RACH-less handover also in inter-gNB scenario.

The draft reply LS can be found in ([4])

# References

- R3-250010/R2-2411191 LS on inter-gNB RACH-less HO in NTN

- R3-238048	Reply LS on UE RACH-less handover for mobile IAB

- R3-241518/R2-2401937	Reply LS on UE RACH-less handover for mobile IAB

- R3-250218	[DRAFT] Reply LS on inter-gNB RACH-less HO in NTN