3GPP TSG-RAN WG3 Meeting #R3-127		R3-250227     Athens, Greece, Feb 17-21, 2025

Title: 	Initial considerations on Inter-CU LTM

Source: 	Rakuten

Agenda item:	13.2

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution, we discuss the resource management issues associated with configuration and maintenance of multiple candidate cells, the impacts on gNB-DU and the importance of gNB-DU initiated reconfiguration of LTM target cell configuration.

# 2. Discussion

## 2.1 Assistance information for target cell preparation

LTM could be associated with multiple flavours like Inter Cell Beam Management (ICBM), ICBM with dynamic switching, LTM Serving cell switch, LTM serving cell change with dynamic switching etc. With the addition of conditional LTM (CLTM), the candidate/target gNB-DU needs to be aware of the variant that that gNB-CU is requesting.

Each of these variants need minor variation in resource reservation at the candidate/target cell.

For example, LTM serving cell switch could be considered as the primary form where the UE undergoes serving cell change by adopting the target cell configuration.

In Inter Cell Beam Management (ICBM), CSI measurement configuration and TCI states for PDCCH and PDSCH are modified to include target cell (intra DU or inter DU) aspects.

Configurations are extended to include resources of multiple cells.

- The reporting configuration for the UE is maintained as common across all cells associated with ICBM, the target cells may reserve resources at the same location in PUSCH/PUCCH for reporting.

- Modification of beam management related configuration to enable efficient beam switching operation without impacting cell specific beam refinement operation within same cell is needed.

- Paging and System Information Broadcast are performed from the serving cell.

Dynamic switching between two intra gNB-DU or inter gNB-DU cells is when ping-pong is expected between them, and the involved candidate cells have to be prepared for that without causing any additional damage to the user-plane interruption time.

In CLTM, the candidate cell has to provide CLTM execution condition for subsequent LTM.

Hence, candidate/target gNB-DU needs to be made aware of the exact variant of LTM for which the candidate cell configuration is being sought.

Proposal 1: gNB-CU indicates to the target gNB-DU, the LTM variant that is being requested, during the candidate cell preparation.

Since gNB-CU is responsible for preparation of LTM target cells, we think two aspects are very important here.

- Awareness at gNB-CU regarding the technical feasibility of the desired target cell configuration at the target gNB-DU.

- Resource situation at the target gNB-DU to accede to the request made by gNB-CU.

For a), the gNB-CU may need to be aware of the UE’s current configuration at serving gNB-DU. On top of this, the gNB-CU needs to be able to decide if the same or a different configuration is feasible at the target gNB-DU. Some of this may need serving gNB-DU specific configuration information at the gNB-CU.

For b), with each candidate gNB-DU, the gNB-CU needs awareness of resource availability of each variant before making a request, failing which there could be a lot of rejections.

Observation 1: Awareness of source configuration at gNB-DU is required at gNB-CU to determine technical feasibility of the desired LTM target cell configuration at candidate/target gNB-DU and awareness of resource situation at each target gNB-DU is required at gNB-CU before initiating candidate/target cell preparation.

Proposal 2: RAN3 discusses and agrees a method to enable selection of the appropriate variant of LTM during candidate cell preparation.

## 2.2 	Security impacts of inter-CU LTM on intra-CU LTM

The scenario described in [2] is gNB-CU-UP change inside the same gNB. In this scenario, the PDCP anchor of the source CU-UP and the target CU-UP are different. Hence this intra gNB use case may also require security key refresh. In this section, we describe the inter gNB-DU LTM scenario and discuss the security aspects associated with it.

The most recent Reply LS from SA3 (S3-244316) to RAN2 for Rel 19 inter-CU LTM with security key change is given below:

# 1 Overall description SA WG3 thanks RAN WG2 for the LS on the security handling for inter-CU LTM in non-DC cases. SA WG3 has been considering the security mechanisms proposed for the inter-CU LTM feature. For this feature, SA WG3 want to preserve the principle that when there is a PDCP anchor change, then it is necessary to perform a key change (i.e. for solutions or handovers that do not require a PDCP anchor change, there is no need to provide rekeying). The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure. SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like, - Security threat of the unprotected MAC CE to deliver the NCC value. - Sending NCC value per candidate cell via RRC message during LTM preparation phase. SA WG3 will keep RAN WG2 and RAN WG3 informed as we make further progress on this topic.

From the above, whenever there is a PDCP anchor change, security key refresh is required.

In Rel 19, RAN2 #125-bis made the above agreement which is to support inter-CU LTM with security key change. This is because the source and target PDCP entitties located in the gNB-CU-UP are different in an inter-CU LTM scenario. However, there is an impact of this on intra-CU LTM also.

We illustrate the scenario described in [2] using the below figure which depicts a gNB with the following elements,

  - A gNB having a gNB-CU-CP and two gNB-CU-UPs serving a subset of gNB-DUs.

  - gNB-CU-UP1 serves gNB-DUs, DU1..DU5.

  - gNB-CU-UP2 serves gNB-DUs, DU6..DU10.

It should be noted here that when a UE undergoes LTM cell switch from a cell belonging to DU5 to a cell belonging to DU6, it also undergoes intra gNB inter-gNB-CU-UP relocation. When a UE undergoes intra gNB gNB-CU-UP relocation, it changes the PDCP anchor point and hence there is a need to refresh the security keys. This implies that new security keys generated have to be delivered to the UE. This also implies that the gNB-CU-UP has to be included while preparing such an LTM candidate cell.

This scenario was excluded in Rel 18 as no security change was supported for LTM. This has to be considered in Rel 19 along with inter-CU LTM with security key change.

Figure 1: Inter gNB-DU LTM HO with intra gNB gNB-CU-UP relocation

Observation 2: Security key refresh is required when a UE undergoes inter gNB-DU mobility that results in an intra gNB gNB-CU-UP relocation.

Proposal 3: 	RAN3 asks RAN2 to consider Security Key refresh for inter gNB-DU LTM, when gNB-CU-UP relocation occurs inside the same gNB.

# 3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Observation 1: Awareness of gNB-DU specific information at gNB-CU to determine technical feasibility of the desired LTM target cell configuration at target gNB-DU and awareness of resource situation at each target gNB-DU is required at gNB-CU before initiating target cell preparation.

Observation 2: Security key refresh is required when a UE undergoes inter gNB-DU mobility that results in an intra gNB gNB-CU-UP relocation.

Proposal 1: gNB-CU indicates to the target gNB-DU, the LTM variant that is being requested, during the candidate cell preparation.

Proposal 2: RAN3 discusses and agrees a method to enable selection of the appropriate variant of LTM during candidate cell preparation.

Proposal 3: RAN3 asks RAN2 to consider Security Key refresh for inter gNB-DU LTM, when gNB-CU-UP relocation occurs inside the same gNB.

# 4. Reference

RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”

RP-241106 : Rel-18 LTM correction for LTM with gNB-CU-UP change