3GPP TSG-RAN WG3 Meeting #127	R3-250235

Athens, Greece, 17 - 21 February 2025

Agenda Item:	12.3

Source:	Huawei

Title:	Discussion on functional split and remaining issues for NR Femto

Document for:	Discussion

1	Introduction

In last meeting RAN3 achieved the following agreements for the NR Femto architecture and functional split [1].

An NR Femto may serve more than one cell.

Capture the following in section 4.x.3.1 of the BLCR for TS38.300

NG-U is defined as specified in clause 4.3.1.1 regardless of whether it is concentrated in the NR Femto GW.

In case of user plane transport concentration at the Femto GW, the Femto GW takes the role described in Option 3 (routing at the IP). FFS whether NAT support.

TS 38.300 captures reference to the specification section describing NG control plane stack for NR Femto without NR Femto GW.

In cases of NR Femto connecting to a NR Femto GW, the NR Femto shall only connect to a single NR Femto GW at any point in time.

The NR Femto GW supports NG-Flex configuration and can simultaneously connect to multiple AMFs.

This contribution further discusses functional split, node ID, and the mobility issues for NR Femto.

2	Discussion

2.1 Functional split

We observe that in TS 36.300 the HeNB GW is responsible for some determination and verification, such as the HeNB identity verification, CSG mode determination and verification, and CSG ID verification. Genrally the verification of gNB ID in NG SETUP REQUEST, PWS RESTART INDICATION and PWS FAILURE INDICATION is implemented by the AMF. If the NR Femto GW is deployed, the Femto node’s NG interface is terminated to the Femto GW, and it is straightforward the NR Femto GW verifies the Femto node ID. That needs to be checked with SA2 and SA3 for confirmation.

- RAN3 assumes NR Femto GW verifies the Femto node ID in NG SETUP REQUEST, PWS RESTART INDICATION and PWS FAILURE INDICATION, and check with SA2 and SA3 for confirmation.

----------Excerpt from TS 36.300----------

4.6.2	Functional Split

......

The HeNB GW hosts the following functions:

--------unrelated parts are skipped---------

-	In case of S1 INITIAL UE MESSAGE message, S1 PATH SWITCH REQUEST and S1 HANDOVER REQUEST ACKNOWLEDGE message, verifying, as defined in TS 33.320 [53], for a closed HeNB, that the indicated cell access mode and CSG ID are valid for that HeNB.

-	Terminating non-UE associated S1 application part procedures towards the HeNB and towards the MME. In case of S1 SETUP REQUEST message, verifying, as defined in TS 33.320 [53], that the identity used by the HeNB is valid and determining whether the access mode of the HeNB is closed or not. In case of S1 PWS RESTART INDICATION message and PWS FAILURE INDICATION message, verifying, as defined in TS 33.320 [53], that the indicated cell identity is valid and replacing the HeNB ID by the HeNB GW ID before sending the PWS RESTART INDICATION message (respectively the PWS FAILURE INDICATION message) to the MME.

--------unrelated parts are skipped---------

In addition to functions specified in clause 4.1, the MME hosts the following functions:

--------unrelated parts are skipped---------

-	Supervising the E-UTRAN action after the change in the membership status of a UE.

-	In case of a HeNB directly connected:

-	verifying as defined in TS 33.320 [53], that the identity used by the HeNB is valid when receiving the S1 SETUP REQUEST message and determining whether the access mode of the HeNB is closed or not;

-	verifying as defined in TS 33.320 [53], for a closed HeNB, that the indicated cell access mode and CSG ID are valid when receiving the S1 INITIAL UE MESSAGE message, the S1 PATH SWITCH REQUEST and the S1 HANDOVER REQUEST ACKNOWLEDGE message;

-	and verifying, as defined in TS 33.320 [53], that the indicated HeNB identity is valid when receiving the S1 PWS RESTART INDICATION message and the S1 PWS FAILURE INDICATION message.

The access mode “open”, “hybrid” and “closed” were defined for 4G HeNB based on the CSG mechanism, and as agreed in the last RAN3 #126 meeting, using 5G CAG functionality can equally support all the three modes as 4G without any stage 3 impact. In 4G HeNB, the “access mode” is defined and indicated in S1 interface, and the HeNB GW or MME (depends on which one the HeNB directly connects) needs to determine and verify the access mode for the HeNB. Regarding the highlighted sentence in the TS 36.300 description as excerpted above for 4G HeNB, MME and/or the HeNB GW should determine/verify the access mode of the HeNB is closed or not. Based on our understanding, such determination and verification were introduced from security point of view, which should be out of RAN3’s scope. For access control of NR femto, RAN3 agreed to reuse the 5G CAG functionality, and we may further coordinate with SA2 and SA3 about whether the NR femto GW/AMF needs to determine/verify that the NR femto serves NPN-only cell(s) or cell(s) shared by both PLMN and CAG.

- RAN3 to further coordinate with SA2 and SA3 about whether the NR femto GW/AMF needs to determine/verify that the NR femto serves NPN-only cell(s) or cell(s) shared by both PLMN and CAG.

We provide the draft LS to SA2 and SA3 in the Annex 2. In addition, regarding the functional split, we also provide the TP for 38.300 in [2] for capturing the functional supported by the NR femto GW.

2.2 gNB ID for NR femto node

In the last meeting we agreed that “an NR Femto may have more than one cell”, as a normal gNB, which is different from the LTE HeNB. In TS 36.300, it is captured “one HeNB serves only one cell.” And we also observed that in TS 36.413, the HeNB ID is represented as the ECGI of the cell served by the HeNB.

---Start of excerpt from TS 36.413---

#### 9.2.1.37	Global eNB ID

This information element is used to globally identify an eNB (see TS 36.401 [2]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.3.8 |  |
| CHOICE eNB ID | M |  |  |  |
| >Macro eNB ID |  |  |  |  |
| >>Macro eNB ID | M |  | BIT STRING (SIZE(20)) | Equal to the 20 leftmost bits of the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of each cell served by the eNB. |
| >Home eNB ID |  |  |  |  |
| >>Home eNB ID | M |  | BIT STRING (SIZE(28)) | Equal to the Cell Identity IE contained in the E-UTRAN CGI IE (see subclause 9.2.1.38) of the cell served by the eNB. |
| >Short Macro eNB ID |  |  |  |  |
| >> Short Macro eNB ID | M |  | BIT STRING (SIZE(18)) | Equal to the 18 leftmost bits of the Cell Identity IE (see subclause 9.2.1.38) of each cell served by the eNB. |
| >Long Macro eNB ID |  |  |  |  |
| >> Long Macro eNB ID | M |  | BIT STRING (SIZE(21)) | Equal to the 21 leftmost bits of the Cell Identity IE (see subclause 9.2.1.38) of each cell served by the eNB. |

---End of excerpt from TS 36.413---

---Start of excerpt from TS 38.413---

9.3.1.6	Global gNB ID

This IE is used to globally identify a gNB (see TS 38.300 [8]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.3.5 |  |
| CHOICE gNB ID | M |  |  |  |
| >gNB ID |  |  |  |  |
| >>gNB ID | M |  | BIT STRING (SIZE(22..32)) | Equal to the leftmost bits of the NR Cell Identity IE contained in the NR CGI IE of each cell served by the gNB. |

---End of excerpt from TS 38.413---

- One HeNB serves only one cell, and the HeNB ID is the ECGI of the cell served by the HeNB.

Now, the NR Femto node may serve more than one cells. It is unreasonable to still use the NCGI of a served cell as the Femto ID. And for stage 3, we need to discuss how to identify the NR femto node. A straightforward solution is just to reuse the global gNB ID for NR Femto node. Alternatively, dedicated NR Femto node ID may be considered.

It is proposed:

- RAN3 to reuse the Global gNB ID to identify the NR femto node. Otherwise, dedicated NR femto node ID needs to be introduced.

2.3 NG mobility impact

For the NG-based UE handover in traditional NR architecture, the target AMF needs to find the correct target gNB for sending the HANDOVER REQUEST, which is based on the target ID included in the HANDOVER REQUIRED message. When the target gNB is an NR femto node and connects to the NR Femto GW, the target ID should be the ID of the target femto node, then how does the target AMF find the correct target NR Femto GW?

In the HeNB architecture, it is noted in the 36.300 that “if routing ambiguities are to be avoided, a TAI used in a HeNB GW should not be reused in another HeNB GW”, which means only one HeNB GW can be deployed in a TA. During NG handover, if the target eNB is a HeNB, the MME sends the HANDOVER REQUEST message to the HeNB GW in the target HeNB’s TA (included as the Selected TAI IE in the Target ID IE).

Techinically such mechanism can be resued in NR Femto, but that depends on whether we still want to restrict the implementation that only one Femto GW can be deployed in a TA. In our view, such restriction is too strong, and considering the coverage of gNB may be smaller than eNB, more Femto nodes may be deployed in a TA causing heavy workload to the Femto GW. Therefore, RAN3 need to discuss how to find the correct Femto GW during NG handover, i.e., how the AMF knows the association of the Femto GW and its corresponding Femto node.

- RAN3 to discuss how the AMF knows the association of the Femto GW and the Femto node.

3	Conclusion

This paper focus on the functional split, Femto ID, and NG mobility in the NR femto. We also provide the TP for TS 38.300 to capture the functional split descriptions for NR Femto, and provide the LS to SA3.

Issue of functional split

- RAN3 assumes NR Femto GW verifies the Femto node ID in NG SETUP REQUEST, PWS RESTART INDICATION and PWS FAILURE INDICATION, and check with SA3 for confirmation.

- RAN3 to further coordinate with SA2 and SA3 about whether the NR femto GW/AMF needs to determine/verify that the NR femto serves NPN-only cell(s) or cell(s) shared by both PLMN and CAG.

Issue of Femto ID

- One HeNB serves only one cell, and the HeNB ID is the ECGI of the cell served by the HeNB.

- RAN3 to reuse the Global gNB ID to identify the NR femto node. Otherwise, dedicated NR femto node ID needs to be introduced.

Issue of NG mobility impact

- RAN3 to discuss how the AMF knows the association of the Femto GW and the Femto node.

4	Reference

- RAN3 #126 chairman notes.

- R3-250236, (TP for BL CR for TS38.300) Functional split for NR Femto, Huawei, RAN3 #127.

Annex 1——LS on femto ID and mode verifications

3GPP TSG-RAN3 Meeting #127	TDoc <TDoc#>

Athens, Greece, 17 – 21 February, 2025

Title:	[DRAFT] LS on femto ID and mode verifications

Response to:

Release:	Rel-19

Work Item:		NR_WAB_5GFemto

Source:	Huawei [will be RAN3]

To:	SA2, SA3

Cc:

Contact person:	Yuanping Zhu

zhuyuanping@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1	Overall description

RAN3 discussed the functional split for the NR Femto architecture considering the architecture with and without the NR femto GW. Similar as 4G HeNB, RAN3 assumes the NR Femto GW also verifies the NR Femto node ID in the NG SETUP REQUEST, PWS RESTART INDICATION and PWS FAILURE INDICATION messages.

In addition, regarding access control for NR femto, RAN3 agreed that reusing 5G CAG functionality without any stage 3 impact. In this context, RAN3 would like to further coordination with SA2 and SA3 about whether the NR femto GW/AMF needs to determine/verify that the NR femto serves NPN-only cell(s) or cell(s) shared by both PLMN and CAG.

2	Actions

To SA2, SA3:

ACTION: 	RAN3 kindly asks SA2 and SA3 to provide feedback on:

- Whether NR Femto GW verifies the Femto node ID in NG SETUP REQUEST, PWS RESTART INDICATION and PWS FAILURE INDICATION.

- Whether the NR femto GW/AMF needs to determine/verify that the NR femto serves NPN-only cell(s) or cell(s) can shared by both PLMN and CAG.

3	Dates of next RAN3 meetings

RAN3#127bis           	07 – 11, April 2025	China

RAN3#128	19 – 23, May 2025		Malta