3GPP TSG-RAN3 Meeting #127	R3-250237

Athens, Greece, Feb 17-21, 2025

Agenda Item:	9.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on Coarse UE Location Information for NB-IoT UEs

Document for:	Discussion/Decision

# Background

In RAN#125bis it was agreed to request from MME by RAN and receive Coarse UE location for NB-IoT UEs only once. Subsequently in RAN3#126 CRs to TS 36.413 was agreed in R3-247816 and CR to TS 36.300 was agreed in R3-247815. RAN3 agreed to add coarse UE location in The INITIAL CONTEXT SETUP REQUEST, DOWNLINK NAS TRANSPORT, and CONNECTION ESTABLISHMENT INDICATION messages. LS was sent to SA2 informing them on the RAN3 agreement and CRs. SA2 has sent LS response in R3-245819 mentioning that RAN3 has not considered few scenarios for UL NAS Transport and Resume Request.

R3-250017 SA2 thanks RAN2 and RAN3 for the reply LS on UE Location Information for NB-IoT NTN. SA2 approved the attached CR which allows the MME, if requested from eNB, to send the coarse location information received from the UE to NB-IoT eNB. SA2 nevertheless observed that the CR approved by RAN3 (TS 36.413 CR1957) did not include the “Coarse UE Location Requested” IE in UL NAS Transport, Resume Request and the “Coarse UE Location IE” in Resume Response. Even though Coarse Location Information can be requested once in the Initial UE message also in these cases (and provided to the eNB in a downlink S1-AP message), this prevents the Coarse Location Information of the UE to be requested by eNB and provided from MME to eNB at a later stage during a RRC connection when user plane bearers are established. The relevant procedures in the SA2 specification are “Establishment of S1-U bearer during Data Transport in Control Plane CIoT EPS Optimisation” (see clause 5.3.4B.4 of TS 23.401), “Connection Resume procedure” (see clause 5.3.5A) and “MT-EDT procedure for User Plane CIoT EPS Optimisation” (see clause 5.3.5B). SA2 would like to understand if RAN2 and RAN3 has performed any analysis on whether Coarse Location Information is needed by the eNB in any of the above procedures. In case RAN2 and RAN3 conclude that Coarse Location Information is needed in these cases, SA2 would like to highlight that AS security is activated during these procedures which would also allow the UE to provide the Coarse Location Information to the eNB via the RRC protocol.

In this paper we discuss our views on the SA2 LS response.

# Discussion

## Connection Resume Procedure

SA2 has mentioned in the LS response that Coarse UE location request and response needs to be provided in S1AP UE Context Resume Request and Response message. Coarse UE location may have been provided in Initial UE Context Request message. But then UE might go Idle and resume after long gap during which the coarse UE location may have changed. Hence it is beneficial to receive Coarse UE location from MME in the Connection Resume procedure.

UE may go Idle and resume after long gap during which the coarse UE location may have changed. Hence it is beneficial to receive Coarse UE location from MME in the Connection Resume procedure

During Connection Resume procedure, there is no NAS interaction between the UE and the MME. Hence MME may not have updated UE location during the Connection Resume procedure to send it to eNB in the S1AP UE Context Resume Response. Once the Connection Resume procedure is successful, MME may have to request for coarse UE location from UE and then send it to eNB in any other S1AP message. RAN requesting and receiving coarse UE location in connection Resume procedure is not beneficial to RAN.

During Connection Resume procedure, there is no NAS interaction between the UE and the MME and hence MME cannot provide updated UE location in the S1AP UE Context Resume Response to eNB.

- Connection Resume Response does not include updated Coarse UE location, as MME will request for UE location from UE, after UE goes into Connected state. Hence it is not beneficial to request and receive Coarse UE location from MME in Connection Resume procedure by RAN.

## UL NAS Transport

SA2 has also mentioned in their LS response that “Coarse UE Location Requested” needs to be added to UL NAS Transport as this would be needed when user plane bearers are established. We think in the duration between coarse UE location provided in Initial UE Context Setup procedure and when user plane bearers are established will be less and coarse UE location will not change much for NB-IoT UEs. Hence, we think it is not necessary to request coarse UE location in UL NAS Transport message. Corresponding LS response is provided in this document in Section 4.

Duration between coarse UE location provided in Initial UE Context Setup procedure and when user plane bearers are established may be less and coarse UE location will not change much for NB-IoT UEs.

- RAN requesting coarse UE location in UL NAS Transport to MME is not necessary. Corresponding LS response provided below.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	UE may go Idle and resume after long gap during which the coarse UE location may have changed. Hence it is beneficial to receive Coarse UE location from MME in the Connection Resume procedure

Observation 2.	During Connection Resume procedure, there is no NAS interaction between the UE and the MME and hence MME cannot provide updated UE location in the S1AP UE Context Resume Response to eNB.

Observation 3.	Duration between coarse UE location provided in Initial UE Context Setup procedure and when user plane bearers are established may be less and coarse UE location will not change much for NB-IoT UEs.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Connection Resume Response does not include updated Coarse UE location, as MME will request for UE location from UE, after UE goes into Connected state. Hence it is not beneficial to request and receive Coarse UE location from MME in Connection Resume procedure by RAN.

Proposal 2:	RAN requesting coarse UE location in UL NAS Transport to MME is not necessary. Corresponding LS response provided below.

# LS Response

# Title:	Reply LS on UE Location Information for NB-IoT NTN

# Response to:	Reply LS on UE Location Information for NB-IoT NTN (R3-250017)

# Release:	Release 18

# Work Item:	IoT_NTN_enh-Core

Source:	RAN3

To:	SA2, RAN2

Cc:	CT1

Contact Person:

Name:	Geetha Rajendran

Tel. Number:

E-mail Address:	geethapr@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN3 thanks SA2 for their reply LS on UE Location Information for NB-IoT NTN and sharing the corresponding SA2 CR.

RAN3 analysed the scenarios mentioned in the SA2 LS to add “Coarse UE Location Requested” in UL NAS Transport message and in Connection Resume Request. RAN3 thinks that Coarse UE Location provided in the above procedures will not be different than the coarse UE location provided in Initial UE Context setup procedure. Hence RAN3 thinks it is not necessary to add coarse UE location in UL NAS Transport and Connection Resume procedures.

2. Actions:

To RAN2, RAN3:

ACTION: 	RAN3 asks SA2 to kindly take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

RAN3#127bis		07-11 April 2025	Wuhan, China

RAN3#128		19-23 May 2025	Malta, Malta