3GPP TSG-RAN3 Meeting #127	R3-250243

Athens, Greece, Feb 17-21, 2025

Agenda Item:	11.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on remaining issues for NG-RAN AI/ML for Network Slicing

Document for:	Discussion/Decision

# Background

In RAN3#126 most of the stage 3 details for predicted slice-based resource status and predicted slice available capacity was agreed. The only open issues are UE performance reporting for slice and failure reporting.

RAN3#126 First Bit (Predicted Radio Resource Status) can be reused for requesting predicted slice radio resource status with list of Slice in the DATA COLLECTION REQUEST message. Introduce the new bit for Predicted slice available capacity in the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message. Introduce a Predicted Slice Available Capacity IE in the DATA COLLECTION UPDATE message to report the requested predicted slice available capacity to the requesting node. Update the Semantics Description of Predicted Radio Resource Status IE in the DATA COLLECTION UPDATE message to indicate that this IE also includes the Slice Radio Resource Status List IE. Source node can request per UE per S-NSSAI granularity for UE performance feedback. There is no consensus if UE performance can be measured in per UE per S-NSSAI and reported from target node to source node. Continue to work on the stage2/3 details…

In this paper we discuss our views on the remaining open issues on failure reporting and UE performance reporting at slice level.

# Discussion

## Slice based UE Performance

In the last meeting we agreed that “Source node can request per UE per S-NSSAI granularity for UE performance feedback”. Though there is still no consensus on UE performance reporting granularity. Different levels of requesting and reporting UE performance was discussed as shown in the options below.

- Option 1 – DRB level UE Performance metrics

- Option 2 – QoS level UE Performance metrics

- Option 3 – PDU Session level UE Performance metrics

- Option 4 – Slice level UE Performance metrics

Slice is mapped to a PDU Session with multiple QoS flows. In a disaggregated gNB, gNB-CU-UP is aware of the PDU sessions associated to a Slice and a DRB, whereas gNB-DU is unaware of the PDU sessions associated to Slice and DRB. Hence collecting UE performance metrics from gNB-DU at PDU session level is not possible. gNB-DU can report UE performance at DRB level and gNB-CU-CP needs to aggregate the UE performance for a PDU session. Since DRB level is the lowest granularity of reporting possible, it is better to report at UE performance at DRB level and the receiving node can aggregate the DRB level performance to PDU or QoS or Slice level. DRB level UE performance will be beneficial for UE performance feedback reporting for NR-DC case as well.

Since there is no consensus on the aggregation of DRB level UE performance at PDU or QoS or Slice level, it is easier to report UE performance at lowest possible granularity DRB level and the receiving node can aggregate DRB level UE performance into PDU level, Slice level etc. DRB level reporting is also beneficial for AI/ML based NR-DC use case.

As a compromised approach the reporting node can have flexibility to report either DRB level UE performance or slice level UE performance. If DRB level UE performance is reported the receiving node can aggregate, it to slice level UE performance. Sample IE structure is shown below –

9.2.3.x2	UE performance

This IE indicates the UE performance at per slice or per DRB level.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Slice UE performance List |  | 0..1 |  |  |
| > Slice UE performance Item |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 |  |
| >S-NSSAI UE performance List |  | 1 |  |  |
| >>S-NSSAI UE performance Item |  | 1..<maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>UE Performance | O |  | 9.2.3.179 |  |
| DRB UE performance List |  | 0..1 |  |  |
| > DRB UE performance Item |  | 1..< maxnoofDRBs > |  |  |
| >>DRB ID | M |  | 9.2.3.33 |  |
| >>Slice ID | O |  | 9.2.3.21 |  |
| >>Cell ID | O |  | 9.2.2.7 |  |
| >>UE Performance | O |  | 9.2.3.179 |  |

To provide flexibility to the reporting node to report UE performance at either slice level or DRB level as a compromised approach. DRB level UE performance reporting may be needed for AI/ML based NR-DC use case.

## Failed Slice Reporting

It was agreed to introduce Predicted Slice Resource Status and Available Capacity reporting in Data Collection procedure. If there is a failure in reporting the Predicted Slice metrics, the failed Slices and the causes can be reported using the existing Data Collection Response message. The failed Sliced to the reported can be added as a sub-list within the List of cells or as a separate IE. UE performance failure reporting can be discussed, once UE performance reporting granularity is agreed.

To add failed Slice Report Characteristics and Cause in the Cell Measurement Initiation List IE or as a Separate Slice Measurement Initiation List IE in the Data Collection Response procedure. UE performance failure reporting can be discussed, once UE performance reporting granularity is agreed

# Summary

Based on the discussion above, we have following observations:

Observation 1.	Since there is no consensus on the aggregation of DRB level UE performance at PDU or QoS or Slice level, it is easier to report UE performance at lowest possible granularity DRB level and the receiving node can aggregate DRB level UE performance into PDU level, Slice level etc. DRB level reporting is also beneficial for AI/ML based NR-DC use case.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	To provide flexibility to the reporting node to report UE performance at either slice level or DRB level as a compromised approach. DRB level UE performance reporting may be needed for AI/ML based NR-DC use case.

Proposal 2:	To add failed Slice Report Characteristics and Cause in the Cell Measurement Initiation List IE or as a Separate Slice Measurement Initiation List IE in the Data Collection Response procedure. UE performance failure reporting can be discussed, once UE performance reporting granularity is agreed