3GPP TSG-RAN WG3 Meeting #127	R3-250248

Athens, GR, 17-21 Feb, 2025

Title:	(TP for BLCR for SON for 38.413, 36.423) Rel-18 SON and MDT leftovers

Agenda Item:	10.4

Source:	Huawei

Document for:	other

# 1. Introduction

This paper discusses the Rel-18 SON and MDT leftovers.

# 2. Discussion

## 2.1 UHI and MHI enhancement for SCG deactivation/activation

The potential anhancement of MHI/UHI was put on hold in RAN3 while waiting for RAN2 progress. This was

The hierarchical structure of the UHI is a list of 16 PCells where each PCell can include a list of 16 PSCells. This IE is defined as below in NGAP:

#### 9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. |

As can be seen above, the IE already contains the stay time. It is also worth noting that the stay time is capped at 4096 seconds. Hence, the most straightforward solution would be to add an optional IE representing the ratio of time the SCG is in activated state, i.e. as indicated below:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| ... |  |  |  |  |  |  |
| Relative time in SCG activated state | O |  | INTEGER(0..100) |  | YES | ignore |

Agree to include the relative time in SCG activated state as an optional IE in Last Visited PSCell Information.

We think a relative value gives enough information for the receiving node. Please note that this information is sent in handover prearation. In case we try to get a more accurate value (absolute time) it may also make sense to update this value after the handover is executed.

We do not see the need for additional information. If RAN3 anyway prefer to discuss this, we would think that it is better to include more statistical information regarding the outcome (e.g. average time in activated state) rather than including information about how the source node set the conditions for activations. We use UHI in the HO preparation message to observe the SCG usage status for SCG resource allocation rather than oberve how easily or hard the previous gNBs take actions to deactive/activate SCG. If we for example add a throughput threshold used, it does not bring so much information to a node that prefers to use another threshold or that prefers to use other criteria. The SCG activation/deactivation is determined by the gNB based on the UE preference of battery saving, throughput, SN load factors and so on. Specifying all possible criteria also seems like an impossible task.

## 2.2	MRO for MR-DC SCG failure

In the work so far, we have assumed that a node does not have to implement the RRC of another RAT, i.e., we need to duplicate important information into separate IEs ofver the network interfaces. With this agreement in mind, the tricky part is to determine what information is important enough to be duplicated.

If the assumption is that all information cannot be decoded, it would have made more sense to use dedicated IEs. Therefore, we think it is also important to differentiate the need to duplicate information. First of all, the previous agreement is to only duplicate information needed for basic functionality. This may however be difficult to agree on. But we think another more important differentiator is wheter the receiving node is an eNB vs an NG-RAN node. It is more reasonable to duplicate for an eNB since this is a legacy node where it may be more complicated to support the decoding of the RRC of another RAT.

Only duplicate information needed for basic functionality and especially in cases where the receiver is an eNB.

To facilitate the UE with EN-DC, the LTM MN is capable of selecting a target PSCell for PSCell addition purpose based on SSB RRM measurement. SSB RRM measurement is sufficient for the mobility decision made by the MN for PSCell addition or PSCell change or SCG failure handling procedure.

For MRO analysis, the measurement result of SCG frequencies configured by the MN for SSB RRM measurement may not be as complete as the one configured by the SN.

When the last serving SN receives the SCGfailureInformation and analyses that it is not its fault, the SN could indicate the suitable PSCell in the SCG failure transfer to help the MN identify why the sutiable cell is not selected as candidate cell, which may be the wrong cell selection of initiating node or candidate SN. If it is the candidate SN’s wrong selection, the MN could forward to the candidate SN which the suitable PSCell belongs to.

Therefore, we propose:

Include the suitable PSCell ID in SCG failure transfer message and agree the TP as included in the annex.

The RAN2#127bis meeting has agreed to add several additional parameters in SCGFailureInformationNR message for EN-DC case as below:

Agreements

- Add reporting of the following parameters for SCG failure report in EN-DC scenario:

⁻	For failedPSCellId and previousPSCellId: frequency and the PCI of the PSCell;

⁻	For timeSCGFailure: value range 0-1023;

⁻	For failureType: Reuse the legacy field.

-	perRA-InfoListA

After reception of SCGFailureInformationNR message, the MN forwards the SCG failure information to SN for MRO analysis in the SCG FAILURE INFORMATION REPORT, as defined in TS 38.423.

Some company propose to add additional information from MN to SN, e.g., failure type and perRA-InfoList. Based on the reasoning above, we do not think this is crucial. The receives is an NG-RAN node.

There is no need to add the duplicating info from the MN to SN, e.g., failure type and perRA-InfoList,.

# 3	Conclusion

This document proposes:

Proposal 1:	Agree to include the relative time in SCG activated state as an optional IE in Last Visited PSCell Information.

Proposal 2:	Only duplicate information needed for basic functionality and especially in cases where the receiver is an eNB.

Proposal 3:	Include the suitable PSCell ID in SCG failure transfer message and agree the TP as included in the annex.

Proposal 4:	There is no need to add the duplicating info from the MN to SN, e.g., failure type and perRA-InfoList,.

# Annex	TP for UHI for 38.413

#### 9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. | - |  |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. | - |  |
| Relative time in SCG activated state | O |  | INTEGER(0..100) |  | YES | ignore |

[snip]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

[snip]

id-UserPlaneFailureIndication,

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-RelativeTimeInSCGActivatedState,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

maxnoofAoIMinusOne,

[snip]

LastVisitedPSCellInformation ::= SEQUENCE {

pSCellID			NGRAN-CGI	 			OPTIONAL,

timeStay			INTEGER (0..40950),

iE-Extensions		ProtocolExtensionContainer { {LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-RelativeTimeInSCGActivatedState	CRITICALITY ignore	EXTENSION RelativeTimeInSCGActivatedState		PRESENCE optional

...

}

[snip]

RelativeAMFCapacity ::= INTEGER (0..255)

RelativeTimeInSCGActivatedState ::= INTEGER (0..100)

ReportArea ::= ENUMERATED {

cell,

...

}

# Annex	TP for MR-DC SCG failure for 36.423

### 8.7.y	SCG Failure Transfer

#### 8.7.y.1	General

The purpose of the SCG Failure Transfer procedure is to indicate to the MeNB that the root cause of the SCG failure may have occurred in the other nodes.

The procedure uses UE-associated signalling.

#### 8.7.y.2	Successful Operation

Figure 8.7.y.2-1: SCG Failure Transfer, successful operation

en-gNB initiates the procedure by sending the SCG FAILURE TRANSFER message to MeNB.

If received, MeNB uses the information according to TS 36.300 [15].

#### 8.7.y.3	Unsuccessful Operation

Not applicable.

#### 8.7.y.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.4.y	SCG FAILURE TRANSFER

This message is sent by the en-gNB to the MeNB to indicate that the root cause of the SCG failure may have occurred in the other nodes.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE XnAP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| Suitable PSCell ID | O |  | NR CGI 9.2.111 |  | YES | ignore |

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

[snip]

id-IABAuthorized,

id-SourcePSCellCGI,

id-FailedPSCellCGI,

id-SCG-FailureReportContainer,

id-TimeSCG-Failure,

id-SuitablePSCellID,

maxCellineNB,

maxnoofBearers,

maxnoofPDCP-SN,

maxFailedMeasObjects,

[snip]

-- **************************************************************

--

#### -- SCG FAILURE TRANSFER

--

-- **************************************************************

ScgFailureTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ScgFailureTransfer-IEs}},

...

}

ScgFailureTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID					CRITICALITY ignore		TYPE UE-X2AP-ID								PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID					CRITICALITY ignore		TYPE SgNB-UE-X2AP-ID						PRESENCE mandatory}|

{ ID id-SuitablePSCellID					CRITICALITY ignore		TYPE NRCGI									PRESENCE mandatory},

...

}

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

[snip]

id-IABAuthorized															ProtocolIE-ID ::= 449

id-SourcePSCellCGI															ProtocolIE-ID ::= xx1

id-FailedPSCellCGI															ProtocolIE-ID ::= xx2

id-SCG-FailureReportContainer												ProtocolIE-ID ::= xx3

id-TimeSCG-Failure															ProtocolIE-ID ::= xx4

id-SuitablePSCellID															ProtocolIE-ID ::= xx5