3GPP TSG-RAN WG3 Meeting #127	R3-250257

Athens, GR, 17th – 21st February, 2025

Title: 	Support of multi-hop relay

Source: 	Huawei

Agenda item:	22.2

Document Type:	Discussion and Decision

# 1. Introduction

RAN2 has had two meetings (RAN2#127 and RAN2#128) to discuss the multi-hop relay, which covers the main part of the work item objectives [1]. In this paper, we discuss the control plane procedures for multi-hop relay.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

- Discussion

Figure 1 shows the multi-hop relay scenario that remote UE accesses to the RAN node via multi-hop relay UEs. The relay UEs between the remote UE and the last relay UE can be called the intermediate relay UE as well.

Figure 1. Multi-hop relay scenario

There is still two candidate approaches on the table for the support of the multi-hop relay.

- Approach 1: The network needs to directly control each of the intermediate relay UEs via Uu RRC.

- Approach 2: Only the last relay UE requires control by the network via Uu RRC.

Based on the discussions in [POST127][402][Relay] [2] and in last meeting [3], more companies preferred approach 1 since the gNB can control all the UEs along the path and Rel-17 U2N relay mechanisms can be reused as much as possible. In the following, we analyse how to support the multi-hop relay in CU-DU split structure for approach 1.

## 2.1 Protocol stack

In RAN2#127 meeting, RAN2 had agreed to reuse the SRAP layer in multi-hop relay, i.e., reuse the Rel-17/Rel-18 structure, for the protocol stack.

- The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18) - The multi-hop UP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

Based on approach 1, the remote UE can connect to the RAN node through more than one relay UE. At the RAN node side, the protocol stack can be the same as legacy, i.e., SRAP placed on the DU node. Therefore, the protocol stack for multi-hop relay can be a simple extension of the Figure 6.1.6-1 and Figure 6.1.6-2 in TS 38.401 [4] by adding additional relay UE hops between the remote UE and the U2N relay UE.

### **TS 38.401 Clause 6.1.6** ### 6.1.6 Protocol stacks of L2 UE-to-Network Relay The protocol stacks for the user plane and control plane of L2 U2N Relay architecture are described in Figure 6.1.6-1 and Figure 6.1.6-2, respectively. The Uu SRAP is terminated between U2N relay UE and gNB-DU. Figure 6.1.6-1: User plane protocol stack for L2 UE-to-Network Relay Figure 6.1.6-2: Control plane protocol stack for L2 UE-to-Network Relay

Observation 1: If RAN2 select approach 1, the protocol stack for multi-hop relay in CU-DU split can be a simple extension of the Figure 6.1.6-1 and Figure 6.1.6-2 in TS 38.401 by adding additional relay UE hops between the remote UE and the U2N relay UE.

## 2.2 RRC Connection Establishment

Figure 2 shows the remote UE’s initial access procedure in multi-hop relay scenario.

Figure 2. Remote UE initial access procedure for multi-hop relay

- The remote UE perform discovery procedure to find proper multi-hop relaying path, and establish PC5 connection using NR ProSe procedure. The details can be left to RAN2.

- The remote UE sends an RRCSetupRequest message to the first  UE via PC5 Relay RLC Channel (if following legacy, it is the specified PC5 relay RLC channel for SRB0).

- The first relay UE withholds the received RRC message and sends the SidelinkUEInformationNR message to the gNB-DU. Whether and how to support the RRC_IDLE/RRC_INACTIVE relay UE can be left to RAN2 discussion. The same as Rel-17/Rel-18, the first relay UE contains the remote UE’s L2 ID and the local ID request indication in the SUI message.

- The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the first relay by encapsulating the SidelinkUEInformationNR message to gNB-CU, and gNB-CU allocates the local ID of U2N Remote UE.

- The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message of the first relay UE to gNB-DU, to request the establishment or modification of PC5 Relay RLC channel for the transmission of remote UE’s SRB0 message if necessary.

- The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message of the first relay UE to gNB-CU.

- The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the first Relay UE to gNB-DU by encapsulating the RRCReconfiguration message, which contains the local ID allocated to the remote UE, bearer mapping for relaying of remote UE’s SRB0, and the PC5 Relay RLC channel configuration if not configured before.

- The gNB-DU sends the RRCReconfiguration message to the first relay UE to configure the local ID of the remote UE, the PC5 Relay RLC channel configuration and bearer mapping for relaying of remote UE’s SRB0.

- The first relay UE sends the RRCReconfigurationComplete message to gNB-DU.

- The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the first relay UE by encapsulating the RRCReconfigurationComplete message to gNB-CU.

- The gNB-CU also trigger the UE CONTEXT MODIFICATION procedure to configure the PC5 relay RLC channel(s)/Uu relay RLC channel if needed, bearer mapping for remote UE’s SRB0, and the remote UE local ID at the relay UE except for the first relay UE.

- Based on the received relaying configuration, the relay UE(s) can forward the RRCSetupRequest message of the remote UE to gNB-DU.

- The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the remote UE and sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N Remote UE. In addition, the INITIAL UL RRC MESSAGE TRANSFER message contains the remote UE local ID, PC5 relay RLC channel configuration for remote UE’s SRB1 message, for the gNB-CU to generate the configuration in RRCSetup message for the remote UE.

- The gNB-CU allocates a gNB-CU UE F1AP ID for the remote UE and generates a RRCSetup message towards the remote UE. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message, and includes the configurations of PC5 Relay RLC channel and bearer mapping for the transmission of remote UE’s SRB1.

- The gNB-DU sends the RRCSetup message to the remote UE via mult-hop relay UEs, including the last relay UE, first relay  UE and other intermediate relay UE(s).

- The gNB-CU configures the relay UEs with PC5 Relay RLC channel, Uu Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1.

- The remote UE sends the RRCSetupComplete message to the gNB-DU via the relay UEs.

- The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

- Upon receiving the RRCSetupComplete message of remote UE, the gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

- The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

- The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the remote UE context in the gNB-DU. Such message may request the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs, and may also encapsulate the SecurityModeCommand message.

- The gNB-DU sends the SecurityModeCommand message to the remote UE via multi-hop relay UEs.

- The gNB-DU sends the UE CONTEXT SETUP RESPONSE message of the remote UE to the gNB-CU, which contains the configuration of PC5 Relay RLC channels for the transmission of remote UE’s SRB2 and DRBs.

- The remote UE responds with the SecurityModeComplete message.

- The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

- The gNB-CU generates the RRCReconfiguration message for remote UE and encapsulates it in the DL RRC MESSAGE TRANSFER message. The RRCReconfiguration message contains the configuration of PC5 Relay RLC channels and bearer mapping for the transmission of U2N Remote UE’s SRB2 and DRBs.

- The gNB-DU sends RRCReconfiguration message to the U2N Remote UE via the relay UEs.

- The U2N Remote UE sends RRCReconfigurationComplete message to the gNB-DU via the relay UEs.

- The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

- The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

- The gNB-CU configures additional Uu Relay RLC channels between the gNB-DU and the last relay UE, and PC5 Relay RLC channels for the relay UEs for relaying of remote UE’s DRBs and SRBs. Also, such step may configure the bearer mapping between remote UE’s DRB/SRB and PC5/Uu Relay RLC channel at the U2N Relay UE.

The above UE initial access procedure for multi-hop relay is similar to that for remote UE initial access in single-hop relay. The difference is the gNB-CU and gNB-DU need to configure the relaying configuration for multi-hop relay UEs along the path, which will not impact the baseline procedure for CU-DU split structure.

Observation 2: If RAN2 select approach 1, RAN3 can adopt the procedures in Figure 2 as the baseline procedure for remote UE’s initial access in multi-hop relay.

Besides, no matter one addition hop or two/more additional hops relays on top of Rel-17 U2N relay, the procedure is unified and the above procedure can be applied.

Observation 3: If RAN2 select approach 1, there no additional complexity from specification point of view to support more than one intermediate node.

RAN3 need to wait for RAN2 to conclude on which approach to use. When RAN2 has decided, RAN3 can continue looking at the impact on RAN3 specifications.

Proposal 1: RAN3 need to wait for RAN2 to decide which approach to use.

# 3. Conclusion

Based on the above discussion, we have the following observation and proposals:

Observation 1: If RAN2 select approach 1, the protocol stack for multi-hop relay in CU-DU split can be a simple extension of the Figure 6.1.6-1 and Figure 6.1.6-2 in TS 38.401 by adding additional relay UE hops between the remote UE and the U2N relay UE.

Observation 2: If RAN2 select approach 1, RAN3 can adopt the procedures in Figure 2 as the baseline procedure for remote UE’s initial access in multi-hop relay.

Observation 3: If RAN2 select approach 1, there no additional complexity from specification point of view to support more than one intermediate node.

Proposal 1: RAN3 need to wait for RAN2 to decide which approach to use.

# 4. Reference

- RP-241609, New WID on NR sidelink multi-hop relay, TSG RAN Meeting #104.

- R2-2410006, Report of [POST127][402][Relay] Multi-hop relay control plane.

- Session Chair Notes on Relay and Positioning RAN2 #128;

- 3GPP, TS 38.401: “NG-RAN; Architecture description”.