3GPP TSG-RAN WG3 Meeting #R3-127		R3-250263 Athens, Greece, Feb 17-21, 2025

Title: 	gNB-DU initiated LTM candidate cell reconfiguration

Source: 	Rakuten, Qualcomm Inc

Agenda item:	13.2

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution, we discuss the resource management issues associated with configuration and maintenance of multiple candidate cells, the impacts on gNB-DU and the importance of gNB-DU initiated reconfiguration of LTM target cell configuration.

# 2. Discussion

## 2.1 Resource management for multiple LTM target cells

The lead time (duration between LTM candidate cell preparation and execution) in an LTM cell switch could be unpredictably long. Secondly, not all prepared candidate cell configurations end up in LTM cell switches. Radio resources are scarce and considering that a UE can be configured with multiple LTM candidate cells, resource reservation is always going to be an issue. Reservation of radio resources in all the configured LTM candidate cells for a UE leads to sub-optimal performance and overhead on the network. Especially considering that this is applicable on a large scale, for all UEs configured with LTM in the network.

Rel18 LTM requires the UE to perform UL sync to acquire timing advance from the candidate/target cell before the serving cell switch with the objective of performing a RACH-less LTM cell switch and therefore reducing HO latency and reducing user service interruption.

Based on the UE’s L1 measurements, the serving gNB-DU may ask the UE to perform UL sync with the candidate/target cell to acquire UE’s candidate/cell timing advance. Hence, UE performing UL sync with the candidate/target cell can be considered to indicate a likelihood of UE undergoing LTM cell switch to the given target cell.

To address the resource reservation issues, the following could be considered - at the time of preparing an LTM candidate cell, the gNB-CU could indicate to the candidate/target gNB-DU whether or not the UE is going to be configured to perform RACH based TA acquisition, as there could be UEs who could be configured with UE capability-based TA acquisition. Based on this, the target gNB-DU shall not reserve all radio resources immediately at the time of LTM candidate/target cell configuration preparation.

Based on the UE performing TA acquisition with a candidate cell, the candidate/target gNB-DU can anticipate an incoming LTM cell switch and initiate resource reservation at the candidate/target cell including update of a RACH resource configuration corresponding to the UE’s best beam/beam-group at that point of time.

This introduces multiple benefits

- This ensures that the resource reservation at the target gNB-DU is performed very close to the LTM serving cell switch and hence optimized, as the likelihood of an LTM cell switch is strong. This also prevents unnecessary blocking of resources for a longer duration in case of long lead times.

- This also ensures that the right RACH resources corresponding to the best beam/beam-group are configured at the candidate/target gNB-DU.

- The UE performing UL sync can be a good indicator of the likelihood of a LTM serving cell change. This enables the candidate/target gNB-DU to issue a target cell re-configuration (if required) based on the prevalent resource conditions at that time.

The resource reservation changes may result in a modified candidate/target cell configuration at the candidate/target gNB-DU and could be delivered to the UE via the gNB-CU using a gNB-DU initiated F1: UE Context Modification Required procedure.

Observation 1: gNB-DU initiated target cell re-configuration can be used to enable optimized resource reservation.

However, in Rel18, gNB-DU initiated LTM candidate cell re-configuration is not permitted by RAN3. This is a limitation considering the fact that there could be changes in resource status and availability between LTM candidate cell preparation and execution. As an example, a simple RACH preamble modification based on the best beam or beam-group change isn’t possible after the candidate cell configuration has been prepared. Once the candidate cell configuration is prepared, it is either executed or cancelled. This, in our view, is too inflexible and limiting genuine cases requiring re-configuration.

Proposal 1: serving gNB-CU indicates to the candidate/target gNB-CU about the timing acquisition mechanism configured to the UE.

Proposal 2: RAN3 reconsiders gNB-DU initiated candidate/target cell re-configuration in Release 19 and permits re-configuration of resources initiated by the gNB-DU.

# 3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Observation 1: gNB-DU initiated target cell re-configuration can be used to enable optimized resource reservation.

Proposal 1: Serving gNB-CU indicates to the candidate/target gNB-CU about the timing acquisition mechanism configured to the UE.

Proposal 2: RAN3 reconsiders gNB-DU initiated candidate/target cell re-configuration in Release 19 and permits re-configuration of resources initiated by the gNB-DU.

# 4. Reference

RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”