3GPP TSG-RAN WG3 #127	R3-250275

Athens, GR, 17th – 21st Feb 2025

Agenda Item:	11.4

Source:	Lenovo

Title: 		Discussion on UE performance metric in AIML for NR-DC

Document for:	Discussion and Approval

# Introduction

In this paper, we discuss issues related to the UE performance collection in DC scenario.

# Discussion

Mobility Optimization for NR-DC As a starting point, SN Addition and MN-initiated SN Change procedures are considered to be enhanced to trigger the collection of UE performance from SN to MN. This assumes that the involved gNBs have an existing Data Collection Reporting Initiation procedure in place configuring the collection of UE performance. Whether to use the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side or whether to reuse existing DC procedure to configure the collection of UE performance at SN side and a new procedure for the reporting of UE performance needs further discussion. Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side. Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

So far, above RAN3 agreements have been made related to the Xn interface procedure for MN to collect UE performance feedback from SN after SN addition or SN change.

Another aspect to be discussed is if the existing list of UE performance metrics can be reused for DC scenario or not. Note that after SN addition or SN change, the following types of radio bearers could exist depending on where it is anchored.

- MN terminated MCG/SCG/split bearer with QoS flow anchored at MN

- SN terminated MCG/SCG/split bearer with QoS flow anchored at SN

There are six types of radio bearers in DC scenario: MN terminated MCG bearer, MN terminated SCG bearer, MN terminated split bearer, SN terminated MCG bearer, SN terminated SCG bearer, SN terminated split bearer

In the meanwhile, the existing UE performance IE consists of average UE throughput DL, average UE throughput UL, average packet delay, and average packet loss DL as below:

#### 9.2.3.179 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average UE Throughput DL; O; Bit Rate 9.2.3.4. Average UE Throughput UL; O; Bit Rate 9.2.3.4. Average Packet Delay; O; 9.2.3.187. Average Packet Loss DL; O; Packet Loss Rate 9.2.3.11.

## 2.1	Packet Delay Measurement by SN

The average packet delay metric is defined in 38.314 as following.

#### 4.2.1.2 Packet delay ##### 4.2.1.2.1 General Packet delay includes RAN part of delay and CN part of delay. The RAN part of DL packet delay measurement comprises: - D1 (DL delay in over-the-air interface), referring to Average delay DL air-interface in TS 28.552 [2] 5.1.1.1.1. - D2 (DL delay on gNB-DU), referring to Average delay in RLC sublayer of gNB-DU in TS 28.552 [2] 5.1.3.3.3. - D3 (DL delay on F1-U), referring to Average delay on F1-U in TS 28.552 [2] 5.1.3.3.2. - D4 (DL delay in CU-UP), referring to Average delay DL in CU-UP in TS 28.552 [2] 5.1.3.3.1. The DL packet delay measurements, i.e. D1 (the DL delay in over-the-air interface ), D2 (the DL delay in gNB-DU), D3 (the DL delay on F1-U) and D4 (the DL delay in CU-UP), should be measured per DRB per UE. NOTE: The delay measurements D1, D2 and D4 are also applicable for EUTRA in case of EN-DC related DL delay measurements on the MN side. The RAN part (including UE) of UL packet delay measurement comprises: - D1 (UL PDCP packet average delay, as defined in clause 4.3.1.1). - D2.1 (average over-the-air interface packet delay, as defined in 4.2.1.2.2). - D2.2 (average RLC packet delay, as defined in 4.2.1.2.3). - D2.3 (average delay UL on F1-U, it is measured using the same metric as the average delay DL on F1-U defined in TS 28.552 [2] clause 5.1.3.3.2). - D2.4 (average PDCP re-ordering delay, as defined in 4.2.1.2.4). The UL packet delay measurements, i.e. D1(UL PDCP packet average delay), D2.1(average over-the-air interface packet delay), D2.2(average RLC packet delay), D2.3(average delay UL on F1-U) and D2.4(average PDCP re-ordering delay), should be measured per DRB per UE. The unit of D1, D2.1, D2.2, D2.3 and D2.4 is 0.1ms. NOTE: The delay measurements D1, D2.1, D2.2 and D2.4 are also applicable for EUTRA in case of EN-DC related UL delay measurements on the MN side.

From the definition, the RAN part of DL or UL packet delay consists of lower layer delay measured by DU and upper layer delay measured by CU-UP in split architecture. For DC, depending on the radio bearer type, that would mean:

| DC radio bearer type | Lower layer delay measured by | Upper layer delay measured by |
| --- | --- | --- |
| MN terminated MCG bearer | MN | MN |
| MN terminated SCG bearer | SN | MN |
| MN terminated split bearer | MN, SN | MN |
| SN terminated MCG bearer | MN | SN |
| SN terminated SCG bearer | SN | SN |
| SN terminated split bearer | MN, SN | SN |

In order to accurately reflect the delay caused by SN and be understood by MN, RAN3 discusses the following two options:

- Support SN to measure and report packet delay per DRB level for all types of bearers. In this case, according to the configuration MN can understand if the measured packet delay includes lower layer delay or upper layer delay or both.

- Support SN to measure and report average packet delay ONLY for SN terminated SCG bearer and SN terminated split bearer by SN. For which, the average packet delay considers both lower layer and upper layer delay.

To reflect the packet delay and be understood by MN correctly, RAN3 discusses the following two options:

  - Support SN to measure and report packet delay per DRB level for all types of bearers, which may consider lower layer delay or upper layer delay or both depending on the bearer type.

  - Support SN to measure and report averaged packet delay (per UE level) ONLY considering SN terminated SCG bearer and SN terminated split bearer by SN, which considers both lower layer and upper layer delay.

## 2.2	DL Packet Loss Measurement by SN

4.2.1.5 Packet Loss Rate 4.2.1.5.1 Packet Uu Loss Rate in the DL per DRB per UE The objective of this measurement is to measure packets that are lost at Uu transmission, for OAM performance observability or for QoS verification of MDT. Protocol Layer: RLC Table 4.2.1.5.1-1: Definition for Packet Uu Loss Rate in the DL per DRB per UE Definition; Uu Packet Loss Rate in the DL per DRB per UE. One packet corresponds to one RLC SDU. The measurement is done separately per DRB. Detailed Definition: M(T,drbid)=Dloss(T,drbid)*1000000N(T,drbid)+Dloss(T,drbid), where explanations can be found in the table 4.2.1.5.1-2 below.. 4.2.1.5.2 Packet Uu Loss Rate with delay threshold in the DL per DRB per UE The objective of this measurement is to measure the DL packets loss including any packets not successfully transmitted or packets successfully received but delayed more than a delay threshold at Uu transmission, for OAM performance observability or for QoS verification of MDT. Protocol Layer: RLC Table 4.2.1.5.2-1: Definition for Packet Uu Loss Rate with delay threshold in the DL per DRB per UE Definition; Uu Packet Loss Rate with delay threshold in the DL per DRB per UE: One packet corresponds to one RLC SDU. The measurement is done separately per DRB. Detailed definition: M_dt(T,drbid)=[DlossT,drbid+DexdT, drbid]*1000000N_dtT,drbid+DlossT,drbid+DexdT, drbid Where explanations can be found in the table 4.2.1.5.2-2 below..

As defined in TS 38.314, the packet loss in DL is measured per DRB per UE and at RLC protocol layer. In the DC scenario, the RLC entity may locate at MN for MCG bearer or located at SN for SCG bearer, thus the packet loss could be measured by MN or SN depending on the DC radio bearer type, as summarized in the table below.

| DC radio bearer type | Packet loss measured by |
| --- | --- |
| MN terminated MCG bearer | MN |
| MN terminated SCG bearer | SN |
| MN terminated split bearer | MN, SN |
| SN terminated MCG bearer | MN |
| SN terminated SCG bearer | SN |
| SN terminated split bearer | MN, SN |

In order to accurately reflect the DL packet loss caused by SN and be understood by MN, RAN3 discusses the following two options:

- Support SN to measure and report DL packet loss per DRB level for SCG bearers and split bearers.

- Support SN to measure and report averaged DL packet loss for SCG bearers and split bearers.

To reflect the DL packet loss and be understood by MN correctly, RAN3 discusses the following two options:

  - Support SN to measure and report DL packet loss per DRB level for SCG bearers and split bearers.

  - Support SN to measure and report averaged DL packet loss (per UE level) considering SCG bearers and split bearers.

## 2.3	Throughput Measurement by SN

In the last RAN3 meeting, RAN3 sends an LS to SA5 asking how per UE level UL/DL throughput is calculated, which is relevant to the throughput measurement performed by SN as well. Thus, we suggest to discuss the throughput measurement by SN after receiving the reply from SA5.

RAN3 waits for SA5’s reply related to per UE UL/DL throughput calculation before discussing the throughput measurement by SN.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	There are six types of radio bearers in DC scenario: MN terminated MCG bearer, MN terminated SCG bearer, MN terminated split bearer, SN terminated MCG bearer, SN terminated SCG bearer, SN terminated split bearer

Based on the discussion above, we propose:

Proposal 1	To reflect the packet delay and be understood by MN correctly, RAN3 discusses the following two options:

a.	Support SN to measure and report packet delay per DRB level for all types of bearers, which may consider lower layer delay or upper layer delay or both depending on the bearer type.

b.	Support SN to measure and report averaged packet delay (per UE level) ONLY considering SN terminated SCG bearer and SN terminated split bearer by SN, which considers both lower layer and upper layer delay.

Proposal 2	To reflect the DL packet loss and be understood by MN correctly, RAN3 discusses the following two options:

a.	Support SN to measure and report DL packet loss per DRB level for SCG bearers and split bearers.

b.	Support SN to measure and report averaged DL packet loss (per UE level) considering SCG bearers and split bearers.

Proposal 3	RAN3 waits for SA5’s reply related to per UE UL/DL throughput calculation before discussing the throughput measurement by SN.