3GPP TSG-RAN WG3 Meeting #127	R3-250284

Athens, Greece, 17 - 21 February, 2025

Agenda item:	13.2

Source: 	Lenovo

Title: 	General aspects of inter-CU SCG LTM

Document for:	Discussion and Approval

1. Introduction

In this contribution, we discuss the remaining issues of inter-CU SCG LTM related to RAN3 impact.

2. Discussion

The revised work item of NR mobility enhancements phase 4 at the RAN#105 meeting has the following objectives [1].

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

The WID includes the following scenarios on the NR-DC:

- Scenario 1: Inter-MN PCell change, where the configurations at the SN are either releases or reconfigured. RAN2 also assumes the inter-CU MCG LTM with SCG addition is supported.

- Scenario 2: Inter-SN PSCell change, where the configurations at the MN are unchanged.

In our view, the scenario 2 is simpler than the scenario 1, and most of the principles proposed by inter-CU LTM without DC could be reused. In addition, the scenario 2 avoids unnecessary tight coordination between MN and SN. In contrast, the scenario 1 is more complex. For example, if the SN is released, this information should be propagated to all other candidate MNs that prepare the LTM candidate configurations relying on using the SN, to avoid a misalignment between the UE configuration and the LTM candidate configuration.

Therefore, we discuss the scenario 2 (i.e., inter-CU SCG LTM) under inter-CU LTM work in Rel-19 first.

For the inter-CU SCG LTM, RAN2 agreed only SN-initiated inter-SN LTM is supported in Rel-19 at RAN2#127 meeting [2].

Agreements on inter-CU LTM Inter-CU SCG LTM: - Inter-CU SCG LTM preparation can be initiated by source SN. - The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. - For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. - RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. - Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

For the SN initiated inter-SN LTM, the source SN initially triggers the candidate cell preparation of inter-SN LTM procedure by sending a SN Change Required message to the MN. In existing S-CPAC functionality, a S-CPAC Request IE was introduced in the SN Change Required message to indicate the SN change is for S-CPAC. A similar approach could be used to enable inter-CU SCG LTM, where an indicator is introduced in the SN Change Required message to indicate the SN change is for inter-CU SCG LTM.

Proposal 1: An indicator is introduced in the SN Change Required message from the source SNG to the MN, to indicate that the SN change is for inter-CU SCG LTM.

On receipt of the SN Change Required message, the MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedures. In existing S-CPAC functionality, a S-CPAC Request Information IE was introduced in the SN Addition Request message to indicate the SN addition is for S-CPAC preparation. A similar approach could be used to enable inter-CU SCG LTM, where an indicator is introduced in the SN Addition Request message to indicate the SN Addition is for inter-CU SCG LTM.

Proposal 2: An indicator is introduced in the SN Addition Request message from the MN to the candidate SN, to indicate that the SN addition is for inter-CU SCG LTM.

To help the candidate SN determining the LTM candidate cells, the source SN needs to provide a list of candidate PSCells to the candidate SN via the MN. There are two options:

- Option 1: The source SN provides the measurement results of candidate PSCells to the candidate SN via the MN, and then the candidate SN determines the LTM candidate cells based on the measurement results.

- Option 2: The source SN provides a list of LTM candidate cells to the candidate SN via the MN, and the candidate SN accepts or rejects these LTM candidate cells.

The option 1 is more like the legacy SN-initiated inter-SN change, while the option 2 is introduced mainly for the inter-SN CPAC. Unlike CPAC, there is no execution condition related to the LTM candidate cell in the inter-CU SCG LTM. Therefore, the source SN doesn’t need to decide the LTM candidate cells, and the candidate SN can make the decision. The option 1 is also preferred by RAN2 [5].

Agreements on inter-CU LTM: - RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

Therefore, we propose to use the option 1 for determination of LTM candidate cells.

Proposal 3: The source SN sends the measurement results of candidate PSCells to the MN via the SN Change Required message, which will be forwarded to the candidate SN via the SN Addition Request message.

Based on the measurement results of candidate PSCells received from the source SN via the MN, the candidate SN determines the LTM candidate cells. For each LTM candidate cell, the candidate SN provides the corresponding SCG part configuration, and may also provide the L1 RS configuration (e.g., a list of SSB or CSI-RS) for L1 measurement, early UL sync configuration or TCI-state configuration. The candidate SN sends those configurations to the MN via the SN Addition Request ACK message, and the MN forwards those configurations to the source SN via the SN Modification Request message.

Proposal 4: The source SN sends the SCG part configuration, L1 RS configuration for L1 measurement, early UL sync configuration or TCI state configuration of the LTM candidate cell to the MN via the SN Addition Request ACK message, which will be forwarded to the source SN via the SN Modification Request message.

In the inter-CU LTM without DC, the source gNB generates a common LTM CSI resource configuration outside of the LTM candidate configuration and sends the common LTM CSI resource configuration to the candidate gNB. The common LTM CSI resource configuration includes the RS configurations of multiple LTM candidate cells, and it is used by the candidate gNB to prepare the LTM CSI report configuration for each candidate cell.

A similar approach could be used to enable inter-CU SCG LTM. That is, the source SN generates a common LTM CSI resource configuration and sends it to the candidate SN via MN. The candidate SN then generates the LTM CSI report configuration for each candidate cell, based on the received common LTM CSI resource configuration. This approach is also agreed by RAN2 at RAN2#127bis meeting [4].

Agreements on inter-CU LTM: - The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells. - The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN. - In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

The common LTM CSI resource configuration could be included in the SN Modification Request ACK message from the source SN to the MN, and then be included in the SN Modification Request message from the MN to the candidate SN. On receipt of the common LTM CSI resource configuration, the candidate SN provides the LTM CSI report configuration for each LTM candidate cell.

Proposal 5: The source SN provides the common LTM CSI resource configuration for inter-CU SCG LTM to the MN via the SN Modification Request ACK message, which will be forwarded to the candidate SN via the SN Modification Request message.

Early synchronization is an important feature for the LTM to reduce the latency. For the PDCCH order triggered early RACH, the candidate SN calculates the TA value of LTM candidate cell, and then transfers the TA information to the source SN via the MN. The new introduced TA Information Transfer message over the Xn for the inter-CU LTM without DC could be reused. That is, the candidate SN sends the TA information to the MN via the TA Information Transfer message, where the MN sends the received TA information to the source SN via the TA Information Transfer message.

Proposal 6: The candidate SN sends the TA information to the source SN via the MN using the TA Information Transfer message.

When the source SN decides to trigger the LTM cell switch for the UE, it should send the target cell information to the target SN (the selected candidate SN), which helps the target SN to prepare the necessary resource for the UE to access. In the inter-CU LTM without DC, the new Cell Switch Notification message over the Xn is introduced. The same message could be reused for the inter-CU SCG LTM. For example, the source SN sends the Cell Switch Notification message including the target cell ID and the selected TCI-state of the target cell to the MN, and the MN transfers the message to the target SN, to indicate the triggering of LTM to the UE.

Proposal 7: The source SN sends the Cell Switch Notification message to the target SN via the MN to indicate the triggering of LTM to the UE.

For the LTM reference configuration for the inter-CU SCG LTM, the LTM reference configuration includes the MCG part and the SCG part. At RAN2#128 meeting [5], RAN2 agreed:

Agreements on inter-CU LTM: - It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

If the MN doesn’t have the SCG part configuration, or the existing SCG part configuration is not suitable for the LTM candidate cells, the MN may request one of the candidate SNs to provide the SCG part configuration, which will be used as the SCG part configuration of the LTM reference configuration. With that, the MN provides the LTM reference configuration including the MCG part configuration and the SCG part configuration to the UE.

Proposal 8: The MN may request the candidate SN to provide the SCG part of the LTM reference configuration.

At RAN2#126 meeting [3], RAN2 agreed:

Agreements on inter-CU LTM: Clarification of DC - An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

To avoid simultaneous LTM configurations at both the MN and the SN, a simple approach is to inform the other node that the LTM is to be configured. For one example, if the MN decides to configure the inter-CU MCG LTM, the MN indicates the SN about the inter-CU MCG LTM configuration, which is used by the SN to avoid configuring the inter-CU SCG LTM. For another example, if the SN decides to configure the inter-CU SCG LTM, it will send a request using the SN Change Required message to the MN, where the MN may reject the request.

Proposal 9: The MN indicates the SN about inter-CU MCG LTM configuration, to avoid the simultaneous inter-CU MCG and inter-CU SCG LTM configurations.

Proposal 10: The MN may reject the request for inter-CU SCG LTM configuration from the SN, to avoid the simultaneous inter-CU MCG and inter-CU SCG LTM configurations.

Therefore, the illustration of signalling flow of the inter-CU SCG LTM is shown as below.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Proposal 1: An indicator is introduced in the SN Change Required message from the source SNG to the MN, to indicate that the SN change is for inter-CU SCG LTM.

Proposal 2: An indicator is introduced in the SN Addition Request message from the MN to the candidate SN, to indicate that the SN addition is for inter-CU SCG LTM.

Proposal 3: The source SN sends the measurement results of candidate PSCells to the MN via the SN Change Required message, which will be forwarded to the candidate SN via the SN Addition Request message.

Proposal 4: The source SN sends the SCG part configuration, L1 RS configuration for L1 measurement, early UL sync configuration or TCI state configuration of the LTM candidate cell to the MN via the SN Addition Request ACK message, which will be forwarded to the source SN via the SN Modification Request message.

Proposal 5: The source SN provides the common LTM CSI resource configuration for inter-CU SCG LTM to the MN via the SN Modification Request ACK message, which will be forwarded to the candidate SN via the SN Modification Request message.

Proposal 6: The candidate SN sends the TA information to the source SN via the MN using the TA Information Transfer message.

Proposal 7: The source SN sends the Cell Switch Notification message to the target SN via the MN to indicate the triggering of LTM to the UE.

Proposal 8: The MN may request the candidate SN to provide the SCG part of the LTM reference configuration.

Proposal 9: The MN indicates the SN about inter-CU MCG LTM configuration, to avoid the simultaneous inter-CU MCG and inter-CU SCG LTM configurations.

Proposal 10: The MN may reject the request for inter-CU SCG LTM configuration from the SN, to avoid the simultaneous inter-CU MCG and inter-CU SCG LTM configurations.

4. Reference

- RP-242356, Revised work item: NR mobility enhancements phase 4.

- RAN2#127 meeting minutes.

- RAN2#126 meeting minutes.

- RAN2#127bis meeting minutes.

- RAN2#128 meeting minutes.