3GPP TSG-RAN WG3 Meeting #127	R3-250286

Athens, Greece, 17-21 Feb, 2025

Agenda item:	16.2

Source: 	Lenovo

Title: 	On A-IOT Architecture and Interface

Document for:	Discussion and Approval

- Introduction

This paper discusses the architectural aspects and interface for A-IOT as agreed in the WID [1]:

- RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

- Discussion

For topology 1, both direct path and indirect path between AIOT RAN node and A-IOTF are supported. For direct patch, the AIOTF communicates with a AIOT RAN via a direct interface reference point Nx as descried in TR 23.700 – 13 [2].

Figure 1. Example Protocol Stack between AIOTF and AIoT Device for Topology 1 (Direct Path)

For indirect path, the AIOT RAN and the AIoTF communicate indirectly via an AMF. NGAP over the N2 reference point between the AIOT RAN and AMF supports Ambient IoT services including delivery of inventory/command messages.

Figure 2. Example Protocol Stack between AIOTF and AIoT Device for Topology 1 (Direct Path)

As agreed by SA2 and RAN3, NGAP is used for both direct path and indirect path and the signalling transport for NGAP at the A-IOT RAN node is SCTP/IP.

NOTE 2: NGAP is used when the AIOTF directly communicates with the AIOT RAN (i.e. over the Nx reference point) or indirect communication with AIOT RAN via an AMF (i.e. over the N2 reference point). NOTE 3: It is assumed Ambient IoT services can be deployed independently from existing deployments. NOTE 4: It is not expected a deployment will use both direct communication between AIOT RAN and an AIOTF and indirect communication between AIOT RAN and an AIOTF via an AMF. The deployment choice can be based on, for example, using a direct communication path for a local deployments. - The signalling transport for NGAP at the A-IoT RAN node is SCTP/IP.

Proposal 1: Enhance NG-AP to enable AIoT functionalities and procedures for both direct path and indirect path of topology 1.

For the ‘AIOT Reader Control’ shown in the Figure 1 and Figure 2, it is feasible to use ‘SMF Transfer’ liked IEs to carry AIOT related IEs, i.e. by defining AIOT related transfer IEs in NG-AP.

Proposal 2: Define AIOT related transfer IEs in NG-AP for carrying AIOT related IEs.

As described in TR 23.700 – 13 [2]:

1. A new core network function is introduced to support Ambient IoT (e.g. AIOTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality. a. The AIOTF registers itself in the NRF with its NF profile, this is to enable the discovery of AIOTF instances e.g. by an NEF. The NF profile at least includes the AIOTF ID/address, NF type, information used to allow the NEF to discover AIOT instances, e.g. based on the target area information in AF request. Other details of the NF profile, if needed, will be completed in normative phase. b. For topology 1, the AIOTF selects AIOT RAN nodes and optionally a list of the BS readers. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN. NOTE 1: Providing the UE Reader list to the RAN only applies to the RRC-based option. For topology 1, NGAP is terminated at an AIOT RAN node. An AIOT RAN node that supports one or more BS readers, may report its supported BS reader information e.g. BS reader IDs and their corresponding service areas. The OAM may configure the AIOTF with BS Reader information including e.g. their corresponding service area when the BS Readers are integrated in the network. NOTE 2: Whether RAN3 will specify BS reader information (e.g. BS reader ID, service area) reporting to AIOTF over NGAP depends on RAN3 decision and if so SA WG2 may include this feature in SA WG2 specifications. NOTE 3: If the request from the AIOTF to the AIOT RAN does not include a BS reader list, then AIOT RAN can use all available BS readers. NOTE 4: The BS reader’s service area needs to coordinate with RAN WG(s).

AIOT RAN can support one or more BS readers, and it may report its supported BS reader information e.g., BS reader IDs and their corresponding service area to AIOTF. Alternatively, the BS reader information can be configured by OAM.

Proposal 3: AIOT RAN optionally reports its supported BS reader ID and their corresponding service area to AIOTF.

3. Conclusion

This paper discusses the architectural aspects and interface for A-IOT with the following proposals:

Proposal 1: Enhance NG-AP to enable AIoT functionalities and procedures for both direct path and indirect path of topology 1.

Proposal 2: Define AIOT related transfer IEs in NG-AP for carrying AIOT related IEs.

Proposal 3: AIOT RAN optionally reports its supported BS reader ID and their corresponding service area to AIOTF.

Text Proposal

[1] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR [Ambient_IoT_Solutions]

[2]	3GPP TR 23.700-13 Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)