3GPP TSG-RAN WG3 Meeting #127	R3-250287

Athens, Greece, 17-21 Feb, 2025

Agenda item:	16.2

Source: 	Lenovo

Title: 	On A-IOT Inventory Procedure

Document for:	Discussion and Approval

- Introduction

This paper discusses AIOT inventory procedure as agreed in the WID [1]:

- RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

- Discussion

The basic procedure for A-IOT inventory for topology has been discussed in study phase and it was concluded in the TR 38.769, as shown in the Figure 1:

Figure 1. Message flow for A-IoT Inventory in Topology 1

To support A-IOT, two basic procedures are needed, i.e., one class 1 Inventory Request procedure and one class 2 Inventory Report procedure.

Proposal 1: Introduce a new class 1 AIOT Inventory Request procedure and a new class 2 Inventory Report procedure.

Contents of inventory request

Firstly, there may be multiple inventory procedures which are performed in parallel. In this case, it needs to distinguish the inventor procedures in the interface between AIOT RAN node and AIOT CN node. It makes sense to introduce a inventory session or task ID or correlation ID to identify different inventory procedures.

Proposal 2: Introduce an inventory session or task ID or corelation ID to identify different inventory procedures in the interface between AIOT RAN NODE and AIOT CN node.

As discussed in the study phase and following the conclusion in RAN2, the Inventory Request may include the following IDs

-	the Inventory Request includes an identifier of a single A-IoT device;

-	the Inventory Request includes a group ID that maps to multiple A-IoT devices

-	the Inventory Request includes multiple identifiers of A-IoT devices (FFS)

-	the Inventory Request does not contain any identifier, i.e., indicating all A-IoT devices that can receive the A-IoT paging message need to respond.

Proposal 3: The Inventory Request message should support the following types of identifiers:

-	Single Device Identifier: an identifier for a single A-IoT device.

-	Group Identifier: a group ID that maps to multiple A-IoT devices.

-	Multiple Device Identifiers: multiple identifiers for individual A-IoT devices (FFS).

-	No Specific ID: The Inventory Request does not contain any specific identifier, indicating that all A-IoT devices capable of receiving the A-IoT inventory message should respond.

In the study phase, RAN3 agreed to include scope of inventory request (e.g., a certain area in which the inventory is to be triggered) in the inventory request.

Similarly, SA2 concluded that for topology 1, the AIOTF selects AIOT RAN nodes and optionally a list of the BS readers. An AIOT RAN node that supports one or more BS readers, may report its supported BS reader information e.g. BS reader IDs and their corresponding service areas. The OAM may configure the AIOTF with BS Reader information including e.g. their corresponding service area when the BS Readers are integrated in the network. The service area needs further study.

Proposal 4: The Inventory Request message should include the scope of inventory request: BS reader IDs and/or service areas (FFS).

As captured in the TR 23.700-13 [2], the assistance information needs to be provided to AIOT RAN from AIOTF.

e. The AIOTF may provide the following assistance information to AIoT RAN/UE Reader: - AIoT service type (e.g. Inventory, Command); - approximate number of AIoT devices based on AF request; - approximate D2R message size based on AF request. NOTE 5: If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation. NOTE 6: The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3. NOTE 7: Further assistance information can be added during the normative phase and in cooperation with other WGs if necessary.

The assistance information at least comprises AIOT service type, with the service type, the AIOT RAN knows whether there is Command procedure followed by the Inventory.

Proposal 5: The Inventory Request message should include the AIOT service type (Inventory, Inventory and Command).

We also think the number of A-IOT Devices for which ID to be collected should also be provided in the inventory request message. According to RAN1’s agreement, Slotted ALOHA-based random-access procedure is used as baseline for inventory. In this case, the AIOT RAN node needs to know how many times of Query or QueryRep procedures should be performed towards A-IOT devices IDs. Following the Slotted ALOHA-based random access procedure in RFID, the random access procedure in 5G system could be:

-	DL triggering (Msg 0). A gNB sends DL signal (similar as Query or QueryRep in RFID) to trigger the A-IoT device(s) to perform access.

-	A-IoT message 1 transmission (Msg 1). Each A-IoT device joining this access round randomly selects one slot number j among N=fun.(Q) slots. If j=0, the A-IoT device performs UL transmission based on the scheduling information indicated by the Reader, including at least a (Random Number) RN16.

-	Reader Access Response (Msg 2). gNB sends ACK with the same RN16 to acknowledge the successful reception and scheduling information for next UL transmission.

-	A-IOT Device ID Reporting (Msg 3). Based on the Msg2, the A-IOT device sends corresponding parameters asked by paging for inventory, i.e., the A-IOT Device ID, as PC/XPC, EPC, packet CRC in RFID, this is referred to Msg3.

-	gNB trigger next round of random access procedure.

We think the number of A-IOT Devices for which ID to be collected should be provided for AIOT RAN Node deciding the number of time slots for A-IOT device ID reporting in air interface.

Proposal 6: The Inventory Request message should include the estimated number of A-IOT Devices per reader for which ID to be collected.

Considering that AIoT devices have limited energy storage and rely on energy harvesting from radio waves, light, or other suitable power sources, they cannot always be in an “always-on” state to receive messages from the AIoT RAN node. Therefore, the AIoT RAN node needs to determine the optimal inventory starting time or completion time for these devices.

As discussed in RAN1, if the antenna is shared between the RF energy harvester and the receiver/transmitter, the AIoT RAN node cannot communicate directly with the AIoT device while it is performing energy harvesting. To address this, it would be beneficial for the AIoT-aware CN node to provide assistance information in the Inventory Request message to the AIoT RAN node. This information can help the AIoT RAN node determine the inventory timing for the AIoT device.

For example, based on the antenna type (e.g., whether it is shared or separate between the RF energy harvester and receiver/transmitter) provided by the AIoT-aware CN node, the AIoT RAN node can determine the duration for which RF energy should be provided to the AIoT device for harvesting, or the waiting time required for the AIoT device to complete energy harvesting.

Proposal 7: The Inventory Request message should include assistance information, such as the A-IoT device type or category. This information will enable the A-IoT RAN node to more effectively determine the optimal inventory starting time or completion time.

Contents of inventory report

Considering the number of ambient IOT devices could be very large, it would be better to allow the AIOT RAN Considering the potentially large number of ambient IoT devices, it is more efficient to allow the A-IoT RAN node to report a list of A-IoT device IDs in a single message. For example, the A-IoT RAN node can store the A-IoT Device IDs received from the devices until all IDs have been collected. The A-IoT RAN node then reports all A-IoT Device IDs in a single message to the A-IoT CN node.

According to Proposal 5, the A-IoT RAN node already knows the estimated number of A-IoT Devices for which IDs need to be collected. This information can be used by the A-IoT RAN node to determine the optimal timing for sending the device IDs to the A-IoT CN node.

Proposal 8: The Inventory Request message should include a list of A-IOT Device IDs.

3. Conclusion

This paper discusses A-IOT inventory procedure with the following proposals:

Proposal 1: Introduce a new class 1 AIOT Inventory Request procedure and a new class 2 Inventory Report procedure.

Proposal 2: Introduce an inventory session or task ID or corelation ID to identify different inventory procedures in the interface between AIOT RAN NODE and AIOT CN node.

Proposal 3: The Inventory Request message should support the following types of identifiers:

-	Single Device Identifier: an identifier for a single A-IoT device.

-	Group Identifier: a group ID that maps to multiple A-IoT devices.

-	Multiple Device Identifiers: multiple identifiers for individual A-IoT devices (FFS).

-	No Specific ID: The Inventory Request does not contain any specific identifier, indicating that all A-IoT devices capable of receiving the A-IoT inventory message should respond.

Proposal 4: The Inventory Request message should include the scope of inventory request: BS reader IDs and/or service areas (FFS).

Proposal 5: The Inventory Request message should include the AIOT service type (Inventory, Inventory and Command).

Proposal 6: The Inventory Request message should include the estimated number of A-IOT Devices per reader for which ID to be collected.

Proposal 7: The Inventory Request message should include assistance information, such as the A-IoT device type or category. This information will enable the A-IoT RAN node to more effectively determine the optimal inventory starting time or completion time.

Proposal 8: The Inventory Request message should include a list of A-IOT Device IDs.

Reference

[1] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR [Ambient_IoT_Solutions]

[2]	3GPP TR 23.700-13 Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)