3GPP TSG-RAN WG3 Meeting #127	R3-250292

Athens, Greece, 17 – 21 February, 2025

Agenda item:	8.1

Source: 	Huawei

Title: 	Discussion on the XR UL bit rate control

Document for:	Discussion and Decision

# 1. Introduction

RAN2 has agreed to support a rate indication from the gNB to the UE on a per QoS flow level. There is an LS (R2-2411218/ R3-250012) from RAN2 requiring RAN3 to take the solution into account. In this contribution, we will analyse the solution from RAN3 perspective and give our views.

# 2. Discussion

During RAN2#128 meeting, RAN2 agreed to support per QoS flow level UL rate control. There is an LS in R2-2411218 [1] which informs the latest agreement of RAN2 and asks RAN3 to take it into consideration. We excerpt the main part of the LS as below.

1. Overall Description: RAN2 has made the following agreement regarding a solution on UL bit rate control - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. 2. Actions: To SA4, RAN3: ACTION: RAN2 kindly asks SA4 and RAN3 to take above into consideration and provide feedback on the agreement provided, if any.

From the LS, it can be seen that RAN2 decides to support a per QoS flow level rate indication from the gNB to the UE. To provide the indication, the gNB should estimate the recommended bit rate for the QoS flow. In case of CU-DU split, it should be the gNB-DU to be responsible to estimate and provide the recommended bit rate.

From RAN3 perspective, it is feasible and up to implementation for the gNB-DU to decide the recommended bit rate for a QoS flow. It is noted that F1AP is already able to transfer the QoS flow level QoS parameters to the gNB-DU. So, as one possible implementation, the gNB-DU can derive the available rate for each QoS flow based on the available resource and the QoS requirements. For example, for one DRB, assuming there are two QoS flows with their GFBR equal to 10Mbps (QoS flow 1) and 20Mbps (QoS flow 2) respectively, if the gNB-DU can provide up to 45Mbps for this DRB, then it can determine the recommended bit rate as 15Mbps for QoS flow 1 and 30Mbps for QoS flow 2.

Another potential implementation is that the gNB-DU may monitor the actual bit rate of each QoS flow by taking statistics of the received DL or UL packets. Specifically, if there is only one QoS flow mapping to the DRB, the gNB-DU can measure the data rate for the DRB, which equals to the rate of the QoS flow. If there are multiple QoS flows mapping to the DRB, the gNB-DU can distinguish the packets belong to different QoS flows via the QFI carried in the GTP-U header (for DL) or the SDAP header (for DL/UL) so that the rate of each QoS flow can be measured. It is worth noting that the CU can configure the DL/UL SDAP header to be included if N to 1 mapping will be performed for the QoS flow which supports bit rate control. With monitoring the actual rate for the QoS flows, the gNB-DU can determine the recommended rate for each QoS flow by taking the available resource, measured date rate per QoS flow and the QoS requirement of each Qos flow into account.

Both the above implementations have no specification impact. RAN3 can confirm that even for the disaggregated RAN architecture, it is feasible for the gNB-DU to provide the QoS flow level recommended bit rate to UE, regardless of how the QoS flows and the DRBs are mapped. But how will the gNB-DU determine the value for each QoS flow level recommended bit rate should be up to implementation, no specification effort is needed.

Proposal 1: RAN3 reply to RAN2 that it is feasible for the gNB-DU to provide the QoS flow level recommended bit rate to UE, and how to derive the recommended bit rate is up to implementation.

Moreover, there are two options to implement the indication as mentioned by RAN2. One is to indicate per QoS flow recommended rate via MAC CE directly. The other is to indicate which QoS flow is configured with rate control by RRC signalling and then use per DRB MAC CE to transfer the recommended bit rate.

From RAN3 perspective, we prefer to support the first option since it is more effective and accurate. With the above analysis, it can be seen that the gNB is feasible to provide the QoS flow level recommended bit rate even in case of CU-DU split. There is no need to use the DRB level indication, which may cause some uncertainty when allocating the recommended rate to multiple QoS flows.

Proposal 2: RAN3 reply to RAN2 that QoS flow level MAC CE indication is preferred.

Based on the discussion above, we prepare a draft reply LS to RAN2 in [2].

# 3. Conclusion

In this contribution, we analyzed the LS on UL rate control. The following proposal was made:

Proposal 1: RAN3 reply to RAN2 that it is feasible for the gNB-DU to provide the QoS flow level recommended bit rate to UE, and how to derive the recommended bit rate is up to implementation.

Proposal 2: RAN3 reply to RAN2 that QoS flow level MAC CE indication is preferred.

# 4. Reference

[1]. R2-2411218, TSG RAN WG2, LS on XR UL bit rate control.

[2]. R3-250293, [draft] Reply LS on XR UL bit rate control, Huawei.