3GPP TSG-RAN WG3 Meeting #127	R3-250297

Athens, GR, 17 - 21 Feb, 2025

Title:	(TP for LTM BLCR for TS38.473): Further discussion on inter-CU LTM procedure

Agenda Item:	13.2

Source:	Huawei

Document for:	Other

1	Introduction

In this contribution, we continue to discuss the basic procedures of inter-CU LTM, including the standalone case and the NR-DC case.

2	Discussion

2.1	Inter-CU LTM in standalone case

2.1.1 Semi-persistent CSI-RS measurement

RAN3 received an LS [1] from RAN1, which requests RAN3 to analyse the feasibility of supporting semi-persistent (SP) CSI-RS measurement in inter-DU LTM and inter-CU LTM. In RAN3 #126 meeting, RAN3 replied an LS [2] and confirmed that it is feasible to specify the signaling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s), and RAN3 will continue to work on the details.

Reply LS for LS on the support of semi-persistent CSI-RS resource for LTM candidate cells 1. Overall Description: RAN3 would like to thank RAN1 for their LS about supporting semi-persistent CSI-RS resource for LTM candidate cells. RAN3 confirmed it is feasible to specify the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s), and RAN3 will continue to work on the details.

The semi-persistent CSI-RS is designed to only send the CSI-RS when needed to reduce the transmission overhead over air interface. In order to support semi-persistent CSI-RS measurement in LTM, the UE needs to know the CSI-RS resource to be measured and the starting time to measure, which is also the point of time that the candidate gNB-DU starts to transmit the corresponding CSI-RS resource.

To align the measuring time and transmission time between the UE and the candidate gNB-DU, the source gNB-DU needs to coordinate with both the UE and the candidate gNB-DU which may bring some network signalling requirements for intra-CU inter-DU LTM and inter-CU LTM.

There are different options which are shown in the figure below. Intra-CU inter-DU LTM is taken as an example.

Figure 1. Procedures to support SP CSI-RS measurement in intra-CU inter-DU case

As illustrated in Figure 1, after receiving SSB measurement report from the UE, the source gNB-DU determines to start semi-persistent CSI-RS measurement for the candidate cell. Before sending the SP CSI-RS measurement indication to the UE, the source gNB-DU has three options to trigger the candidate gNB-DU to transmit the CSI-RS correspondingly.

- Option 1: The source gNB-DU determines the CSI-RS resources that to be transmitted, and sends the CSI-RS resource (set) IDs to the candidate gNB-DU via the gNB-CU to request the candidate gNB-DU to transmit the CSI-RS. In this option, the source gNB-DU needs to know the SSB and CSI-RS association of the candidate cell, to determine the suitable CSI-RS resources.

- Option 2: The source gNB-DU forwards the SSB measurement report to the candidate gNB-DU and the candidate gNB-DU determines the CSI-RS resources. The candidate gNB-DU sends that information back to the source gNB-DU.

- Option 3: The gNB-CU decides to trigger semi-persistent CSI-RS measurement and sends the selected CSI-RS resource (set) IDs to the candidate gNB-DU and the source gNB-DU based on L3 measurement results. The candidate gNB-DU starts to transmit CSI-RS following the indication of the gNB-CU, and the source gNB-DU triggers the UE measurement.

Currently, it is the source DU that generates the L1 report configuration, triggers SP CSI-RS measurement to the UE, and then determines the target cell and the target beam. With the same principle, the source DU could also determine the SP CSI-RS resource (set) and send it to the candidate DU.

Therefore, comparing Option 1 and Option 2, we think that the Option 1 is preferable. Since the L3 measurement triggered LTM is still under discussion in Rel-18 and the SP CSI-RS measurement at this moment is only L1 measurement related, Option 3 is not supported at the current stage.

The detailed procedure of Option 1 includes:

- In LTM preparation step, the candidate gNB-DU provides SP CSI-RS resource configuration to the gNB-CU (via UE Context Setup Response) and then to the source gNB-DU (via UE Context Modification Request).

- The source gNB-DU decides to activate the SP CSI-RS measurement, e.g., by the received periodic SSB L1 report. The source gNB-DU determines SP CSI-RS resource (set) and send to the candidate gNB-DU via the gNB-CU, and the candidate gNB-DU sends a response to confirm the SP CSI-RS transmission. This can be done in a new class 1 procedure.

- The source gNB-DU triggers the UE to measure the SP CSI-RS in the candidate cells. This is done by existing signalling, e.g., by SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE.

Proposal 1a: For SP CSI-RS measurement in intra-CU scenario, the source gNB-DU determines the CSI-RS resources that to be transmitted in the candidate cells, and sends it to the candidate gNB-DU via the gNB-CU.

For inter-CU scenario, the similar procedure can be applied. The source gNB-DU determines the SP CSI-RS resources, and sends a request message to the candidate gNB-DU via Xn between the source gNB-CU and the candidate gNB-CU.

Proposal 1b: For SP CSI-RS measurement in inter-CU scenario, the source gNB-DU determines the CSI-RS resources that to be transmitted in the candidate cells, and sends it to the candidate gNB-DU via Xn between the source gNB-CU and the candidate gNB-CU.

2.1.2 LTM Preparation

a) L1 measurement and report configuration

RAN3 had the following agreement in RAN3 #125bis meeting:

RAN3 #125bis Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase. Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

The SSB configuration exchanged in F1/Xn Setup does not include the SSB transmit power information, which is needed in the UE LTM configuration.

SSB related RRC configuration at the UE side: SSB related RRC configuration transferred in Xn Setup:

Therefore, in Rel-18, the SSB Information is included in the UE Context Setup Response message from the candidate DU to the CU, to provide the full required SSB related configuration.

In Rel-19, similarly, the candidate gNB-CU should send the full SSB Time/Frequency Configuration of candidate cells to the source gNB-CU in the Handover Request ACK message.

Proposal 2: The candidate gNB-CU responds the full SSB Time/Frequency Configuration (in SSB Information IE) of candidate cells to the source gNB-CU in the Handover Request ACK message.

b) Early RACH configuration

In order to support PDCCH ordered early RACH, candidate DU first needs to allocate early RACH resources in the preparation step. To avoid RACH resource conflict, the early RACH resources of one candidate cell should be different for different source gNB-DUs. This is the reason why RAN3 introduced the source gNB-DUs List in Early UL Sync Configuration in Rel-18. The gNB-DUs List ensures that the candidate gNB-DUs can allocate non-overlapping RACH resources for different source gNB-DUs.

In Rel-19 inter-CU LTM, two gNB-DUs of difference gNB-CUs may have the same gNB-DU ID. Therefore, to avoid early RACH resource conflict, the pair of (gNB ID, gNB-DU ID) needs to be transferred to the candidate gNB-CU, and then the candidate gNB-DUs during handover preparation. If preferred, the gNB-DU ID IE can be replaced by a new name, e.g., early RACH resource request ID to avoid explicitly exposing the gNB internal architecture.

Similar with L1 measurement and report configuration, the early RACH configuration can be allocated by Handover Request and Handover Request ACK procedure.

Proposal 3: The source gNB-CU sends the pair of (gNB ID, early RACH resource request ID) to the candidate gNB-CU via the Handover Request message to request early RACH configuration.

Proposal 4: The candidate gNB sends the early RACH configuration to the source gNB-CU in the Handover Request Acknowledge message.

c) Reference configuration

RAN2 agreed to support one single reference configuration in Rel-19 inter-CU LTM, as shown below:

RAN2 #127 1. Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

In RAN2 #127bis meeting, such agreement is retained. Basically, it means that the source gNB-CU can generate a common reference configuration and sends the reference configuration to all candidate gNBs in Handover Request message. Some companies in [3] proposed to support that the candidate gNB provides reference configuration in Handover Request Acknowledge to the source gNB. This could be beneficial for RRC signalling overhead reduction when this candidate gNB are configured with many candidate cells.

Proposal 5: The source gNB generates a reference configuration by itself, or obtains a reference configuration from a candidate gNB. Then the source gNB sends it to candidate gNBs in Handover Request message.

2.1.3 Early Synchronization

In Rel-19 inter-CU LTM, in PDCCH ordered early RACH procedure, in order to transfer the TA information between the candidate gNB and the source gNB, RAN3 agreed to introduce a new procedure on Xn.

RAN3 #124 Introduce a new procedure on Xn to transfer the TA information.

In RAN3 #125 meeting, there was no consensus on whether the TA information transfer message is UE associated message or non-UE associated message.

According to the RAN2 progress on PDCCH ordered early RACH, there is no intention in RAN2 to further enhance the Rel-18 mechanism. In this case, when a candidate cell receives a preamble for early TA acquisition, it does not know the UE identity because the early RACH resources are shared by multiple UEs in the source cell. Therefore, the TA information transfer procedure should be a new non-UE associated class 2 procedure also on Xn interface.

Proposal 6: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

After the candidate gNB-DU receives a preamble for early TA acquisition, it transfers the TA value to the source gNB-DU. In Rel-18, on F1 interface, messages (e.g. DU-CU/CU-DU TA Information Transfer) were introduced to transfer the TA information from candidate gNB-DU to source gNB-DU. They can be re-used between the candidate gNB-DU and the candidate gNB-CU, and between the source gNB-CU and the source gNB-DU.

Proposal 7: The F1AP messages DU-CU TA Information Transfer and CU-DU TA Information transfer are reused to transfer the TA information between the candidate gNB-DU and the candidate gNB-CU, and between the source gNB-CU and the source gNB-DU.

The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) included in F1AP TA Information Transfer messages are reused for inter-CU LTM. In addition, to let the candidate gNB-CU know where to forward the TA information, a source gNB ID is need in the corresponding DU-CU TA Information Transfer message.

Proposal 8: The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) in F1AP TA Information Transfer messages are reused for inter-CU LTM. In addition, a source gNB ID is included in the DU-CU TA Information Transfer message.

2.1.4 LTM configuration sync up among candidate gNBs

In addition, to support subsequent inter-CU LTM, some LTM configurations, e.g., early sync configuration, configuration ID, and data forwarding addresses, etc may need to sync up among candidate gNBs.

In general, there are three options to sync up those LTM configurations among candidate gNB-CUs. Option 1 is to sync up just after the initial handover preparation. Option 2 is that the source gNB sends the configuration to the target gNB once the LTM is initiated. Option 3 is when receiving the successful access notification from the target gNB. The three options are illustrated in Figure 1.

Figure 3: LTM Configuration Update for subsequent inter-CU LTM

In Rel-18 inter-DU intra-CU LTM, the following note is captured in stage 2:

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

Basically, the three options above can be mapped to the three cases of NOTE 2.

Proposal 9: To use the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

2.1.5 Candidate gNB-initiated LTM Cancel

In previous RAN3 meetings, RAN3 had the following agreement to support candidate gNB-initiated LTM cancellation:

RAN3 #125bis Introduce a new procedure for candidate gNB-initiated LTM cancellation. RAN3 #126 Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

Because of the serving gNB change during subsequent LTM, it needs to discuss and decide to which node, the candidate gNB initiate the LTM cancel message.

One option is that the candidate gNB always contacts the initial source gNB, then the initial source gNB forwards the LTM cancel message to the current serving gNB of the UE. This makes the initial source gNB behave like an anchor gNB of the UE and requires the initial source gNB should be always informed by the new serving gNB after each LTM cell switch.

Another option could be that after each LTM cell switch, the target gNB (the new serving gNB) notifies all candidate gNBs about the serving gNB change, so that the LTM Cancel message is sent from the candidate gNB to the new serving gNB directly.

The LTM Configuration Update procedure can be used for this purpose, i.e., when LTM cell switch is executed, the new serving gNB sends the LTM Configuration Update message to all other candidate gNBs to notify the serving cell change.

Since the first option may be not workable if the source gNB is not a candidate gNB in subsequent case, we propose the candidate gNB to contact the new serving gNB directly.

Proposal 10: To support candidate gNB initiated LTM cancel, the target gNB (new serving gNB) notifies all other candidate gNBs about the serving gNB change by the LTM Configuration Update message.

Proposal 11: The candidate gNB initiates the LTM Cancel message towards the current UE serving gNB to cancel the prepared LTM candidate cells in it.

2.2.5 UE association setup between the new serving gNB and candidate gNBs

The following agreements were made at previous RAN3 meetings.

RAN3 #125bis Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM, when and how to establish the UE association is FFS. RAN3 #126 To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

After each cell switch, the new serving gNB signals the new allocated source gNB UE XnAP ID(s) to other candidate gNBs in the LTM Configuration Update message. Then the candidate gNB replies with the target gNB UE XnAP ID in LTM Configuration Update Acknowledgement message.

One issue that RAN3 needs to discuss is how does the candidate gNB identify the UE Context. The candidate gNB needs to know some kinds of UE identity to associate the new allocated UE XnAP ID with the UE context. To solve this issue, in the LTM Configuration Update message, the UE XnAP ID that the candidate gNB allocated for the first source gNB should be included in the LTM Configuration Update message

Proposal 12: After each cell switch, the new serving gNB signals the new allocated source gNB UE XnAP ID(s) to other candidate gNBs in the LTM Configuration Update message. The candidate gNB replies with the target gNB UE XnAP ID in LTM Configuration Update Acknowledgement message. In addition, the UE XnAP ID allocated by the candidate gNB for the initial source gNB is included in the LTM Configuration Update message.

2.2	Inter-CU LTM in NR-DC

SN initiated inter-CU SCG LTM with MN unchanged

Two LSs from RAN2 in [4] [5] are received containing the following RAN2 agreements related to inter-CU LTM. The inter-CU SCG LTM related agreements are extracted as follow:

RAN2 #127 10. Inter-CU SCG LTM preparation can be initiated by source SN. 11. The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. 12. For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. 13. RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. 14. Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19. RAN2 #127bis 3. The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3. 4. Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline: - The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part. - FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations. - The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. - Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs. 5. For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN. 6. The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells. 7. The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN. 8. In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN. 9. Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. 10. Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM. RAN2 #128 2. For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3. 3. For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard). 8. RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message. 10. It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM. 11. RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

Since only SN-initiated inter-CU SCG LTM is supported, and the procedure is similar with SN-initiated subsequent CPAC, except that the cell switch triggering in LTM is by MAC CE, instead of by event evaluation. We suggest to take the SN-initiated subsequent CPAC in TS 37.340 section 10.20 as baseline to design the inter-CU SCG LTM procedure.

Proposal 13: RAN3 to design the SN-initiated inter-CU SCG LTM by taking into account the above RAN2 agreements and take the SN-initiated subsequent CPAC procedure in TS 37.340 section 10.20 as baseline.

MN initiated inter-CU MCG LTM with SN unchanged/released/reconfigured

RAN2 agreements related to inter-CU MCG LTM with SN case are extracted as follows:

RAN2 #127 15. SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). 16. Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied). 17. For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure. RAN2 #128 4. RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it. 5. RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

When a candidate MN receives Handover Request message from the source MN for LTM preparation, it decides whether to release the SN. If the SN is not released, the candidate MN sends SN Addition Request message to the SN, and the SN will generate the corresponding SCG configuration.

Proposal 14: In inter-CU MCG LTM with SN case, the candidate MN decides whether to release the SN. If not, the candidate MN sends SN Addition Request message to the SN to request SCG part LTM configuration.

3	Conclusion

This document proposes:

Semi-persistent CSI-RS measurement:

Proposal 1a: For SP CSI-RS measurement in intra-CU scenario, the source gNB-DU determines the CSI-RS resources that to be transmitted in the candidate cells, and sends it to the candidate gNB-DU via the gNB-CU.

Proposal 1b: For SP CSI-RS measurement in inter-CU scenario, the source gNB-DU determines the CSI-RS resources that to be transmitted in the candidate cells, and sends it to the candidate gNB-DU via Xn between the source gNB-CU and the candidate gNB-CU.

LTM Preparation:

Proposal 2: The candidate gNB-CU responds the full SSB Time/Frequency Configuration (in SSB Information IE) of candidate cells to the source gNB-CU in the Handover Request ACK message.

Proposal 3: The source gNB-CU sends the pair of (gNB ID, early RACH resource request ID) to the candidate gNB-CU via the Handover Request message to request early RACH configuration.

Proposal 4: The candidate gNB sends the early RACH configuration to the source gNB-CU in the Handover Request Acknowledge message.

Proposal 5: The source gNB generates a reference configuration by itself, or obtains a reference configuration from a candidate gNB. Then the source gNB sends it to candidate gNBs in Handover Request message.

Early Synchronization:

Proposal 6: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Proposal 7: The F1AP messages DU-CU TA Information Transfer and CU-DU TA Information transfer are reused to transfer the TA information between the candidate gNB-DU and the candidate gNB-CU, and between the source gNB-CU and the source gNB-DU.

Proposal 8: The parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) in F1AP TA Information Transfer messages are reused for inter-CU LTM. In addition, a source gNB ID is included in the DU-CU TA Information Transfer message.

LTM configuration sync up:

Proposal 9: To use the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

Candidate gNB-initiated LTM Cancel:

Proposal 10: To support candidate gNB initiated LTM cancel, the target gNB (new serving gNB) notifies all candidate gNBs about the serving gNB change by LTM Configuration Update message.

Proposal 11: The candidate gNB initiates the LTM Cancel message towards the current UE serving gNB to cancel the prepared LTM candidate cells in it.

UE association setup:

Proposal 12: After each cell switch, the new serving gNB signals the new allocated source gNB UE XnAP ID(s) to other candidate gNBs in the LTM Configuration Update message. The candidate gNB replies with the target gNB UE XnAP ID in LTM Configuration Update Acknowledgement message. In addition, the UE XnAP ID allocated by the candidate gNB for the initial source gNB is included in the LTM Configuration Update message.

Inter-CU LTM in NR-DC:

Proposal 13: RAN3 to design the SN-initiated inter-CU SCG LTM by taking into account the above RAN2 agreements and take the SN-initiated subsequent CPAC procedure in TS 37.340 section 10.20 as baseline.

Proposal 14: In inter-CU MCG LTM with SN case, the candidate MN decides whether to release the SN. If not, the candidate MN sends SN Addition Request message to the SN to request SCG part LTM configuration.

The corresponding TPs are provided in the annex and [6].

4	Reference

- R1-2409283, LS on the support of semi-persistent CSI-RS resource for LTM candidate cells, RAN1.

- R3-247911, Reply LS for LS on the support of semi-persistent CSI-RS resource for LTM candidate cells, RAN3.

- R3-247126, (TP to BL CR to 38.300 and 38.423) Support for reference configurations in the inter-CU MCG LTM, Google.

- R2-2409378, LS on RAN2 agreements for inter-CU LTM, RAN2.

- R2-2411134, RAN2 agreements on Inter-CU LTM and Conditional LTM, RAN2.

- R3-250298,	(TP for LTM BLCR for TS38.300): Update on inter-CU LTM basic procedure.

Annex – Text Proposal for LTM BLCR for TS 38.473

/*********************Start of changes***********************/

9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM Configuration ID | M |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [8]. | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs List |  | 0..1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  |  |  |
| >>>LTM gNB ID | M |  | Global gNB ID 9.3.1.305 |  | - |  |
| Path Addition Information | O |  | 9.3.1.296 | This IE contains either the Indirect Path Addition IE or the N3C Indirect Path Addition IE. | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |

/*********************Next change***********************/

#### 9.2.1.24	DU-CU TA INFORMATION TRANSFER

This message is sent by the gNB-DU to inform the gNB-CU about TA information.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DU to CU TA Information List |  | 0..1 |  |  | YES | ignore |
| >DU to CU TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Preamble Index | M |  | INTEGER (0..63) |  | - |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [16]. | - |  |
| >>Source gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>Source gNB ID | M |  | Global gNB ID 9.3.1.305 |  | - |  |

/*********************End of changes***********************/