3GPP TSG-RAN WG3 #127                                                 R3-250325

Athens, GR, 17th – 21st Feb, 2025

Agenda Item:	10.2

Source:	CATT

Title:	(TP for 37.340, 38.473, 38.300 and 38.401) MRO for R18 mobility mechanisms

Document for:	other

Introduction

In last RAN3 meeting, the following agreements are captured:

LTM:

For TP to BL CR to TS 38.473:

In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI.

For TP to BL CR to TS 38.401:

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the failure DU in case of too late LTM.

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the source DU in case of too early LTM, or LTM to wrong cell.

MRO for CHO with Candidate SCG(s)

Case 1a), 2a), 3a), 9a): Too late handover

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 7a), 8a): Too early HO or wrong cell Handover to wrong cell

Case 7b), 8b): Too Early CPC/CPA Execution or CPC/CPA Execution to wrong PSCell

The failure types defined for CHO in TS38.300 and for CPAC defined in TS37.340 are used as baseline. Clarification or amendment could be made on top of that if needed.

In case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization.

SCGFailureInfomation handling for case 1b)/2b)/3b)/9b) and case 7b)/8b):

The serving MN which receives SCGFailureInformation from the UE performs the initial analysis.

The serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed (e.g for case 3b).

Whether the above mechanism works depends on RAN2 progress on SCGFailureInfomation.

Based on above agreements, here we provide some analysis on MRO for R18 mobility mechanisms.

Discussion

  - MRO for LTM

UE-based solutions or network-based solution

In last RAN3 meeting, RAN3 disused the following issue:

Discuss RAN2 agreements on UE-based solutions for TA acquisition and indication of LTM candidate cells in RLF Report. RAN3 to discuss the following network-based solution: - Source gNB can store the Early UL sync configuration and UE-based TA configuration and can determine whether the UE had determined the TA via UE-based TA measurement or gNB indicated TA value in CSC by “retrieving” the UE context upon receiving the RLF Report? - There is no need for UE to report the LTM candidate cell list (or an indication whether a cell was LTM candidate or not) in RLF Report. Existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF.

During intra-CU LTM, CU could always keep LTM related configuration including TA configuration and LTM candidate cell list. But there may be a case that RLF Report is not fetched in time when failure occurs during LTM, and after that, UE re-connects to other CU via L3 HO who triggers the fetching of RLF Report. In this case the source CU could know the failure occurs for this UE, and then keep necessary UE context (including LTM configuration) to wait for RLF Report, the network-based solution is still feasible.

Proposal 1: It is proposed to adopt network-based solution to store TA type and LTM candidate cell list.

RACH-less and RACH-based access

First, once UE starts RACH-less access, there would be no fall back from RACH-less to RACH-based access.

UE is triggered to perform LTM via a MAC CE (LTM Cell Switch Command), not legacy handover command. And LTM is specified in subsection 9.2.3.5 in TS38.300, not the subsection 9.2.3.6.

More specifically, the following text describes UE behavior when receiving LTM Cell Switch Command captured in TS38.321.

- 5.18.35 LTM Cell Switch Command The network may instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE described in clause 6.1.3.75. The MAC entity shall: 1> if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell: 2> indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE; 2> if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers: 3> if Timing Advance Command value (hexa-decimal) is not set as FFF: 4> process the received Timing Advance Command (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing; 4> if the MAC entity is associated with SCG: 5> indicate to upper layers to skip the Random Access procedure for this LTM cell switch. 3> else if the Timing Advance measurement is configured as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the indicated LTM target: 4> process the measured Timing Advance (see clause 5.2); 4> consider the RACH-less LTM cell switch to be ongoing. 4> if the MAC entity is associated with SCG: 5> indicate to upper layers to skip the Random Access procedure for this LTM cell switch. 3> consider the SSB associated to the TCI state indicated by TCI state ID field as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell for RACH-less LTM cell switch (as in clause 5.8.2); 3> indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

We can see that UE would perform RACH less LTM cell switch when Timing Advance is configured. Then UE performs RACH-less access until RACH failure occurs, i.e. t304 expires. There is not fallback mechanism in above procedure.

Proposal 2: If valid TA is provided via LTM Cell Switch Command or measured by UE, UE would perform RACH-less LTM cell switch until T304 expires. There is no fall back mechanism after RACH-less procedure start.

UE-based TA related optimization:

For UE-based TA measurement, the description on RACH-less and RACH-based LTM is as below:

In the case where UE-based TA measurement is configured, but no valid TA value is provided in the cell switch command, the UE applies the valid TA value by itself if available. The UE performs RACH-less LTM cell switch upon receiving the cell switch command whenever a valid TA value is available. If no valid TA value is available, the UE performs RACH-based LTM cell switch.

Network could configure UE-based TA measurement. If UE cannot calculate valid TA, UE would perform RACH-based LTM cell switch which may delay handover procedure.

Observation 1: If UE is configured UE-based TA measurement, but UE cannot calculate valid TA, UE would perform RACH-based LTM cell switch.

From UE perspective, how to calculate TA value is UE implementation for which is out of the SON scope.

From network perspective, OAM configures UE-based TA measurement and then calculate RACH-less access related KPI data from all of DU to decide whether UE-based TA measurement should be configured. It is also up to network implementation.

Proposal 3: For the case of UE-based TA measurement, calculating TA value is UE implementation which is out of the SON scope. Also, the optimization on whether to configure UE-based TA can be achieved by network implementation which has no specification impact.

Network-based TA related optimization:

For the timing of triggering early TA acquisition procedure, if triggering too early, the TA may be outdated when starting LTM cell switch. On the contrary, if triggering too late, the early TA acquisition procedure may be not finished. So, it may be needed to discuss how to trigger early TA acquisition procedure at the correct time.

Early TA acquisition procedure by PDCCH order may periodic triggering. Shorter periodicity may better meet the requirement of TA validity with the cost of impact UE and network performance while longer periodicity may not provide valid TA when triggering LTM cell switch. So, Network may decrease the periodicity step by step until RACH-less handover successful rate is satisfied. As for RACH-less handover successful rate, RACH-less handover failure can be achieved by RLF Report which has been agreed to include access type according to RAN2 agreement:

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

Proposal 4: The optimization on avoiding network based TA outdates can be performed based on network implementation e.g., shorten TA obtain period.

Information in ACCESS AND MOBILITY INDICATION message

In last RAN3 meeting, the agreement and FFS on ACCESS AND MOBILITY INDICATION message is as below:

For TP to BL CR to TS 38.473:

In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional.

FFS on stage2 or stage3 to define the condition to include this optional C-RNTI.

For the case that RLF occurs shortly after successful handover, UE context should be released in source gNB when receiving UE CONTEXT RELEASE message. In inter-DU LTM case, DU UE F1AP ID as part of UE context may be also released in source DU, therefore, only C-RNTI could be used. When receiving ACCESS AND MOBILITY INDICATION message for "Too Early HO" or "HO to Wrong Cell" failure type, source CU could forward Source C-RNTI and RLF included in ACCESS AND MOBILITY INDICATION message to source DU if needed.

Observation 2: For the case that RLF occurs shortly after successful handover and "Too Early HO" or "HO to Wrong Cell" failure type is detected, source CU should forward Source C-RNTI and RLF report included in ACCESS AND MOBILITY INDICATION message to source DU.

According to above understanding, following description may be included in stage2 text:

If the cause for RLF which occurs shortly after successful handover is identified as a "Too Early HO" or a "HO to Wrong Cell", the CU shall include the C-RNTI used in the source cell of the last completed handover before the failure in the ACCESS AND MOBILITY INDICATION message.

Proposal 5: It is proposed to add the condition to send C-RNTI in F1 in stage 2.

According to legacy MRO, for the case that RLF occurs shortly after successful handover, source NG-RAN may not save UE context for the following reason:

1. Successful handover is the common case and usually the number of successful handed over UE is large. NG-RAN may not save UE context for each successful handed over UE.

2. Source NG-RAN stores UE context to wait for handover Report message from target NG-RAN. If not handover Report message received, UE context may have to be stored for 48 hours. For the same reason as above bullet, most of the time source NG-RAN cannot receive handover Report message.

Considering the complexity of saving UE context in source NG-RAN, Mobility Information is introduced in XN interface message e.g., indicating UE type.

Observation 4: For the case that RLF occurs shortly after successful handover, Mobility Information is introduced in XN interface message to avoid UE context saved in source NG-RAN for every successful handover.

For legacy handover, it is CU to perform MRO based on Mobility Information. While for LTM, it is DU to perform MRO. So, Mobility Information should be provided to DU to avoid UE context saved by DU for each successful handover.

For intra-CU inter-DU LTM, source DU could provide Mobility Information to CU via UE CONTEXT MODIFICATION RESPONSE, DU-CU CELL SWITCH NOTIFICATION or UE CONTEXT RELEASE COMPLETE message, etc. Then CU stores Mobility Information and provides Mobility Information back to DU via ACCESS AND MOBILITY INDICATION message.

Proposal 6: It is proposed to introduce Mobility Information in F1 interface message. The TPs for stage 2 and stage 3 are in Annex.

Ping-pong issue

CU could collect UHI and detect ping-pong occurrence during LTM. To solve ping-pong issue, following two solutions are raised in last RAN3 meeting:

- CU can avoid ping-pong without sending UHI to DU by removing the candidate cell, no extra information over F1AP is needed.

- LTM decision is made by DU, should be DU consider how to avoid ping-pong.

In our understanding, for ping-pong issue, for example, ABA handover, handover from A to B should be avoided instead of increasing the handover threshold to make handover harder. Handover threshold as part of mobility configuration is decided by cell coverage and could been optimized according to legacy MRO. Ping-pong issue should not also trigger optimization on handover threshold.

Observation 5: Ping-pong issue should be solved by handover target selection instead of optimization on handover threshold which would conflict with legacy MRO.

Proposal 7: It is proposed to avoid ping-pong issue by removing the candidate cell in CU, and sending UHI to DU is not needed.

  - MRO for CHO with candidate SCGs

    - Analysis for Case 1b), 2b), 3b) and 9b)

The case 1b/2b/3b/9b has been agreed as too late CPC execution as below:

Case 1b), 2b), 3b), 9b): Too late CPC execution

Here is an example of case 1b) and 2b) for which CHO is not met before SCG failure occurs. UE connects with PCell1/PSCellA and is configured with the candidate cell list of CHO with candidate SCGs as indicated in the table below.

| Index | Candidate PCell | MN for candidate PCell | Candidate PSCell |
| --- | --- | --- | --- |
| 1 | PCell2 | 2 | PSCellB |
| 2 | PCell3 | 3 | PSCellC |

Since execution condition of CHO with candidate SCGs is not met, when UE move out of PSCell A, SCG failure occurs.

Figure 1

PCell1 in MN1 receives SCG failure information message from UE. The next suitable PCell should be selected by measure result in SCG failure information message and then detect CHO failure type. Obviously too late handover failure type may be detected and PCell1 in MN1 may perform optimization on CHO.

Proposal 8: For case 1b), 2b), 3b), 9b), source MN optimize CHO base on SCG failure information message.

After PCell1 in MN1 performs optimization on CHO, candidate MN may be informed to perform MRO on CPC which is agreed in last meeting as below:

The serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed (e.g for case 3b).

For example, source MN select the suitable PCell is PCell2 in MN2, so candidate MN2 would be informed to perform the optimization on too late CPC failure type.

In candidate MN, the CPC optimization should be performed based on the suitable PCell.

The following IE in HANDOVER REQUEST ACKNOWLEDGE message could describe the reason why optimization on CPC should be performed based on PCell.

| Conditional Handover Information Acknowledge | O |  |  | Example in figure1 |
| --- | --- | --- | --- | --- |
| >Requested Target Cell ID | M | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | The next suitable PCell:PCell2 |
| >Maximum Number of CHO Preparations | O | 9.2.3.101 |  |  |
| >CHO-CPAC Information | O | 9.2.3.202 |  | The next suitable PSCell:PSCellB. Optimization on CPC is based on PCell2/PSCellB. |

From above IE structure, the optimization on CPC in candidate MN should base on the new selected PCell and PSCell. While the new selected PCell has been decided in source MN, source MN should forward the new selected PCell to candidate MN.

Observation 6: CPC optimization should base on new selected PCell ID.

As for the new selected PSCell, it is also selected based on the measurement result in SCGFailureInformation. Logically either source MN or candidate MN could use the measurement result in SCGFailureInformation to select PSCell. But considering the measurement is configured by source MN, it is better for source MN to use the measurement result.

Observation 7: The measurement is configured by source MN, it is better for source MN to use the measurement result to analyze suitable PSCell.

For case 10), take the following candidate cell list, if the new selected PCell/PSCell is PCell2/PSCellB, the combination of PCell2/PSCellB should be provided to candidate MN, not separate PCell2 and PSCellB.

| Index | Candidate PCell | MN for candidate PCell | Candidate PSCell |
| --- | --- | --- | --- |
| 1 | PCell2 | 2 | PSCellC |
| 2 | PCell3 | 2 | PSCellB |

Observation 8: Source MN may select a pair of PCell/PSCell which is not prepared as candidate pair before.

Proposal 9: It is proposed for source MN to select the next suitable PCell /PSCell pair and forward it to candidate MN.

As for CPC failure type, candidate MN may detect failure type from the next suitable combination of PCell1/PSCellB, but source MN and candidate MN may have different configuration, for example, Tstore_UE_cntxt. So, failure type may be also provided from source MN to candidate MN.

Proposal 10: It is proposed for source MN to send CPC failure type to candidate MN.

As for the message between source MN and candidate MN, the following open issue is captured in last meeting:

Which message is used from the last serving MN to the candidate MN(s):

- Handover Report

- via new message

As UE connects with PCell1 in MN1 when receiving SCG failure information message from UE, the last serving MN in figure 1 is MN1 to transferring above information, we think handover report message is suitable.

Proposal 11: It is proposed to use handover report message to transfer suitable PCell /PSCell pair and CPC failure type from last serving MN to candidate MN.

    - Analysis for case 1a), 2a), 3a), 9a): Too late handover

The case 1a/2a/3a/9a is agreed as too late handover in last meeting.

Case 1a), 2a), 3a), 9a): Too late handover

In case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization.

The solution for case 1a/2a/3a/9a is similar as that for 1b/2b/3b/9b.

UE connects with PCell/PSCell and is configured with the candidate cell list of CHO with candidate SCGs. Since execution condition of CHO with candidate SCGs is not met, when UE move out of PCell coverage, MCG failure occurs.

Source MN as last serving MN receives RLF report and then performs MRO analysis on CHO. Source MN could also provide the new selected combination of PCell/PSCell and failure type to candidate MN for the same reason as for case 1b/2b/3b/9b.

Proposal 12: It is proposed for source MN to perform MRO analysis on CHO failure type and make optimization after receiving RLF report.

Proposal 13: It is proposed to use handover report message to send the new selected combination of PCell/PSCell and failure type from source MN to candidate MN.

The definition of too late handover is an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell, however, last serving MN it may not detect too late handover after receiving RLF report because UE re-establish/re-connect PCell is still last serving PCell but RLF finally due to wrong CPC configuration, for example case 3a). In this case, the CPC configuration also need to be optimized, last serving MN should send “failure event” to candidate NG-RAN node(s).

In case of Too Late Handover or an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a last serving cell when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may inform the failure event to the candidate NG-RAN node(s) controlling the candidate PSCell where the mobility configuration caused the failure.

Editor’s note: FFS on the definition of failure event.

Proposal 14: For Case 1a), 2a), 3a), 9a), in case of UE re re-establish/re-connect PCell to last serving PCell after RLF, which is not belong to too late handover, but CPC configuration also need to be optimized.

  - MRO for Subsequent CPAC

Forwarding mechanism

In last RAN3 meeting, forwarding mechanism has been discussed and the stage2 description still needs further check.

Editor’s note: The forwarding mechanism in the above two paragraphs needs to be further checked.

There are some differences due to the following principles applied to subsequent CPAC captured in TS37.340.

-	For MN initiated subsequent CPAC, the MN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for the initial execution of subsequent CPAC (e.g. CPA or CPC).

-	For SN initiated subsequent CPAC, the source SN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for the initial execution of subsequent CPAC.

-	For both MN and SN initiated inter-SN subsequent CPAC, the candidate SN generates the execution conditions for the following execution of subsequent CPAC when the candidate SN prepares the candidate SCG configuration(s) for candidate PSCell(s). For SN initiated intra-SN subsequent CPAC, the source SN generates the execution conditions for the following execution of subsequent CPAC when the source SN prepares the candidate SCG configuration(s) for candidate PSCell(s).

In case of SN initiated PSCell change, the initiating node may be not the source SN and target SN for current PSCell change when receiving SCGfailureInformation message from UE. For example, SN1 as source SN initiates subsequent CPC, UE perform PSCell change from SN1 to SN2, and then PSCell change from SN2 to SN3. If SCG failure occurs in SN3, SN2 and SN3 as current source SN and target SN may be involved to perform MRO. SN1 as the initiating node which provide the candidate cell list may be also optimized.

Observation 9: For MRO for S-CPAC, the initiating node may be not source SN or target SN in case of SN initiated CPC.

For optimization on candidate cell selection, PSCell candidate list is first selected by initiating node, and then selected by target SN.

For MN initiated S-CPAC, MN and target SN should be involved to optimize candidate cell selection.

For SN initiated S-CPAC, it is the initiating SN and target SN to optimize candidate cell selection.

Proposal 15: The initiating node and target SN is responsible for optimize candidate cell selection.

For optimization on execution conditions, for the initial execution of S-CPAC, MN or SN as the initiating node generate the execution conditions.

For the following execution of subsequent S-CPAC, in case of no intra-SN PSCell change, candidate SN generates the execution conditions. Here the candidate SN refers to the source SN of PSCell change.

In case of intra-SN PSCell change, the source SN (or last serving SN) generates the execution conditions.

Proposal 16: For initial CPC execution, initiating node should perform execution conditions optimization. For following CPC, it is source SN to optimize execution conditions. The source SN is related to the latest PSCell change.

With above understanding, we could conclude that the forwarding mechanism would include two steps:

Step 1: optimization on execution conditions optimization

Step 2: optimization on PSCell candidate list

The related nodes to perform execution conditions optimization and candidate PSCell list selection are listed in following table.

|  | Node for execution conditions optimization | Node for candidate PSCell list selection |
| --- | --- | --- |
| Initial S-CPAC initiated by MN | MN(initiating node) | MN, target SN |
| Following S-CPAC initiated by MN | Source SN | MN, target SN |
| Initial S-CPAC initiated by SN | Initiating SN | Initiating SN, target SN |
| Following S-CPAC initiated by SN | Source SN | Initiating SN, target SN |

The forwarding mechanism captured in TS37.340 in last meeting is as below:

For SN initiated CPC or a following S-CPC initiated by MN, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

According to above table, for SN initiated CPC, it is initiating SN to perform candidate PSCell list selection, not source SN. There are three nodes i.e., source SN, initiating SN and candidate/target SN involved for following SN initiated CPC optimization, but now above text only cover source SN and candidate/target SN, without initiating SN.

Observation 10: There are three nodes i.e., source SN, initiating SN and candidate/target SN involved in following SN initiated CPC optimization

Following S-CPAC initiated by MN, it is MN to perform initial candidate PSCell list selection and candidate or target SN select part of them. There are three node i.e., source SN, MN and candidate/target SN involved subsequent CPC optimization, but now it only cover source and candidate/target SN, without MN.

Observation 11: There are three nodes i.e., source SN, MN and candidate/target SN involved for following S-CPAC optimization initiated by MN.

According to above discussion, we update the forwarding mechanism which includes the above related nodes:

For CPA or MN initiated CPC or initial S-CPAC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation. For the following S-CPC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. If the suitable PSCell is not one of the candidate PSCells provided by the MN at CPAC preparation, MN performs the final MRO related optimisation. Otherwise, MN sends the SCG Failure Information Report message to source SN, source SN performs the final MRO related optimisation.

For SN initiated CPC, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

For SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to initiating SN, and initiating SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the initiating SN, but not one of the candidate PSCells selected by the candidate or target SN, the initiating SN sends the SCG FAILURE TRANSFER message to MN, then MN sends SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, MN sends the SCG Failure Information Report message to the source SN to perform the final MRO related optimisation.

NOTE: FFS on MN how to decide to send SCG Failure Information Report message to candidate/target SN or source SN.

We also try to reuse legacy CPAC forwarding mechanism for S-CPAC as below:

For CPA or MN initiated CPC or initial S-CPAC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report messageto the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation in case of initial S-CPAC initiated by MN and MN initiated CPAC, or MN sends the SCG Failure Information Report message to source SN performs the final MRO related optimisation in case of following CPAC initiated by MN.

For SN initiated CPC or SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to the SN which initiating S-CPAC or CPC, and the initiating SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the initiating SN, but not one of the candidate PSCells selected by the candidate or target SN, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report messageto the candidate or target SN. Otherwise, the initiating SN performs the final MRO related optimisation in case of initial S-CPAC initiated by SN and SN initiated CPC, or MN sends the SCG Failure Information Report message to source SN for final MRO related optimisation in case of following CPAC initiated by SN.

NOTE: FFS on MN how to decide to send SCG Failure Information Report message to candidate/target SN or source SN.

The above two forwarding mechanisms are also provided in annex for discussion.

Proposal 17: It is proposed for RAN3 to discuss above two forwarding mechanisms in TP for TS37.340.

UHI

There is an issue raised in RAN3 meeting as below:

Support UHI and SCG UE History Information transmission during S-CPAC procedure to avoid the ping-pong or unnecessary PSCell change

To solve the issue on ping-pong or unnecessary PSCell change, there are mainly two steps:

The first step is how to record UHI and detect ping-pong occurrence.

The second step is to optimize configuration to solve the issue.

For the first step, UHI could be updated by MN during S-CPAC procedure. The related text in TS37.340 is as below:

The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell.

Since MN is aware of the each PSCell selected during S-CPAC procedure according to above text, MN could correlate PCell and PSCell, i.e. update the correlated UHI which includes PCell and PSCell information.

After the analysis of UHI, MN can detect ping-pong issue occurs during S-CPAC procedure. To solve the issue, i.e. the second step, there may be the following solutions:

Solution 1: MN sends UHI to every candidate SN each time UHI is updated.

Solution 2: MN sends UHI to every candidate SN when ping-pong issue is detected by MN.

Solution 3: MN sends UHI to the candidate SN which ping-pong issue is related to when detecting ping-pong issue by MN.

Solution 4: it is not needed to solve ping-pong issue immediately, i.e. the next time configuring S-CPAC, ping-pong issue will be solved according to the previous collected UHI.

Considering above solutions, we think not every candidate SN should be informed because they may be not involved in ping-pong issue, and it is also not needed to inform SN each time UHI is updated because ping-pong issue may not exist. So, solution 1 and 2 is not preferred.

For solution 3, after detecting ping-pong issue, MN informs the related SN about the updated UHI. The candidate cell add/release may be optimized at once which may lead to many interactions between every candidate cells because the change on each candidate cell should be informed to every other candidate cells in S-CPAC procedure.

For solution 4, considering the complexity to perform the modification of S-CPAC configuration, UHI can be used for the next S-CPAC, i.e. not perform optimization for current S-CPAC. Moreover, most of the times MRO is performed based on the statistics, not one time failure. Therefore, it is not needed to perform MRO in a hurry or each time Ping-Pong issue occurs. Comparing with solution 3, solution 4 also has no specification impact. So, solution 4 is preferred.

Proposal 18: It is proposed to take solution 4 to solve the ping-pong issue for the follow reason:

- Modification of S-CPAC configuration during S-CPAC procedure is complex.

- MRO is based on the statistics. Performing MRO immediately is not required.

- No specification impact.

Information in SCGfailureInformation message

Network could save UE context in MN which may include the initiating node information. But MN may only know the initiating SN node ID and may not know the PSCell ID in the initiating node which is optional present in S-NODE CHANGE REQUIRED message. For the initiating node, it may be not selected as candidate cell and cannot save UE context when UE is not connected to this node. So, it is hard for network to save PSCell information in initiating node in case of SN initiating S-CPAC case.

As we discussed before, the initiating node, specifically, initiating PSCell may be required to perform MRO e.g., candidate PSCell list selection. Therefore, SCGfailureInformation may be enhanced to include source initiating PSCell ID.

Proposal 19: It is proposed to include initiating PSCell ID in SCGfailureInformation message.

Conclusions

MRO for LTM:

Proposal 1: It is proposed to adopt network-based solution to store TA type and LTM candidate cell list.

Proposal 2: If valid TA is provided via LTM Cell Switch Command or measured by UE, UE would perform RACH-less LTM cell switch until T304 expires. There is no fall back mechanism after RACH-less procedure start.

Observation 1: If UE is configured UE-based TA measurement, but UE cannot calculate valid TA, UE would perform RACH-based LTM cell switch.

Proposal 3: For the case of UE-based TA measurement, calculating TA value is UE implementation which is out of the SON scope. Also, the optimization on whether to configure UE-based TA can be achieved by network implementation which has no specification impact.

Proposal 4: The optimization on avoiding network based TA outdates can be performed based on network implementation e.g., shorten TA obtain period.

Observation 2: For the case that RLF occurs shortly after successful handover and "Too Early HO" or "HO to Wrong Cell" failure type is detected, source CU should forward Source C-RNTI and RLF report included in ACCESS AND MOBILITY INDICATION message to source DU.

Proposal 5: It is proposed to add the condition to send C-RNTI in F1 in stage 2.

Observation 4: For the case that RLF occurs shortly after successful handover, Mobility Information is introduced in XN interface message to avoid UE context saved in source NG-RAN for every successful handover.

Proposal 6: It is proposed to introduce Mobility Information in F1 interface message. The TPs for stage 2 and stage 3 are in Annex.

Observation 5: Ping-pong issue should be solved by handover target selection instead of optimization on handover threshold which would conflict with legacy MRO.

Proposal 7: It is proposed to avoid ping-pong issue by removing the candidate cell in CU, and sending UHI to DU is not needed.

MRO for CHO with candidate SCGs:

Proposal 8: For case 1b), 2b), 3b), 9b), source MN optimize CHO base on SCG failure information message.

Observation 6: CPC optimization should base on new selected PCell ID.

Observation 7: The measurement is configured by source MN, it is better for source MN to use the measurement result to analyze suitable PSCell.

Observation 8: Source MN may select a pair of PCell/PSCell which is not prepared as candidate pair before.

Proposal 9: It is proposed for source MN to select the next suitable PCell /PSCell pair and forward it to candidate MN.

Proposal 10: It is proposed for source MN to send CPC failure type to candidate MN.

Proposal 11: It is proposed to use handover report message to transfer suitable PCell /PSCell pair and CPC failure type from last serving MN to candidate MN.

Proposal 12: It is proposed for source MN to perform MRO analysis on CHO failure type and make optimization after receiving RLF report.

Proposal 13: It is proposed to use handover report message to send the new selected combination of PCell/PSCell and failure type from source MN to candidate MN.

Proposal 14: For Case 1a), 2a), 3a), 9a), in case of UE re re-establish/re-connect PCell to last serving PCell after RLF, which is not belong to too late handover, CPC configuration also need to be optimized.

MRO for Subsequent CPAC:

Observation 9: For MRO for S-CPAC, the initiating node may be not source SN or target SN in case of SN initiated CPC.

Proposal 15: The initiating node and target SN is responsible for optimize candidate cell selection.

Proposal 16: For initial CPC execution, initiating node should perform execution conditions optimization. For following CPC, it is source SN to optimize execution conditions. The source SN is related to the latest PSCell change.

Observation 10: There are three nodes i.e., source SN, initiating SN and candidate/target SN involved in following SN initiated CPC optimization

Observation 11: There are three nodes i.e., source SN, MN and candidate/target SN involved in following S-CPAC optimization initiated by MN.

Proposal 17: It is proposed for RAN3 to discuss above two forwarding mechanisms in TP for TS37.340.

Proposal 18: It is proposed to take solution 4 to solve the ping-pong issue for the follow reason:

- Modification of S-CPAC configuration during S-CPAC procedure is complex.

- MRO is based on the statistics. Performing MRO immediately is not required.

- No specification impact.

Proposal 19: It is proposed to include initiating PSCell ID in SCGfailureInformation message.

Reference

[1]

TP for TS 37.340

### 10.18.3	Conditional PSCell addition or change failure

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

Option 1: Add new paragraph CPAC forwarding mechanism for S-CPAC as below:

For CPA or MN initiated CPC or initial S-CPAC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation. For the following S-CPC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. if the suitable PSCell is not one of the candidate PSCells provided by the MN at CPAC preparation, MN performs the final MRO related optimisation. Otherwise, MN sends the SCG Failure Information Report message to source SN, source SN performs the final MRO related optimisation.

For SN initiated CPC, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

For SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to initiating SN, and initiating SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the initiating SN, but not one of the candidate PSCells selected by the candidate or target SN, the initiating SN sends the SCG FAILURE TRANSFER message to MN, then MN sends SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, MN sends the SCG Failure Information Report message to the source SN to perform the final MRO related optimisation.

NOTE: FFS on MN how to decide to send SCG Failure Information Report message to candidate/target SN or source SN.

Option 2: Reusing legacy CPAC forwarding mechanism for S-CPAC as below:

For CPA or MN initiated CPC or S-CPAC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report messageto the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation in case of initial S-CPAC initiated by MN and MN initiated CPAC, or MN sends the SCG Failure Information Report message to source SN performs the final MRO related optimisation in case of following CPAC initiated by MN.

For SN initiated CPC or SN initiated S-CPAC, the MN sends the SCG Failure Information Report message to the SN which initiating S-CPAC or CPC, and the initiating SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the initiating SN, but not one of the candidate PSCells selected by the candidate or target SN, the initiating SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report messageto the candidate or target SN. Otherwise, the initiating SN performs the final MRO related optimisation in case of initial S-CPAC initiated by SN and SN initiated CPC, or MN sends the SCG Failure Information Report message to source SN for final MRO related optimisation in case of following CPAC initiated by SN.

NOTE: FFS on MN how to decide to send SCG Failure Information Report message to candidate/target SN or source SN.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>

TP for TS 38.473

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

- 8.3.3	UE Context Release (gNB-CU initiated)

- 8.3.3.1	General

The purpose of the UE Context Release procedure is to enable the gNB-CU to order the release of the UE-associated logical connection, candidate cells in conditional handover, conditional PSCell addition, or conditional PSCell change or LTM or subsequent CPAC. The procedure uses UE-associated signalling.

- 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE Context Release (gNB-CU initiated) procedure. Successful operation

The gNB-CU initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the gNB-DU.

Upon reception of the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message. If the CG-SDT Kept Indicator IE is contained in the UE CONTEXT RELEASE COMMAND message and set to "true", the gNB-DU shall, if supported, consider that the UE is sent to RRC_INACTIVE state with CG-SDT configuration and store the configured CG-SDT resources, C-RNTI, CG-SDT-CS-RNTI, the CG-SDT related RLC configurations and F1-U connections associated with the SDT bearers while releasing the UE context.

If the old gNB-DU UE F1AP ID IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall additionally release the UE context associated with the old gNB-DU UE F1AP ID.

If the UE CONTEXT RELEASE COMMAND message contains the RRC-Container IE, the gNB-DU shall send the RRC container to the UE via the SRB indicated by the SRB ID IE.

If the UE CONTEXT RELEASE COMMAND message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication or multi-path relay based duplication, if configured, for the SRB for the included RRC-Container IE.

If the Candidate Cells To Be Cancelled List IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall consider that the gNB-CU is cancelling only the conditional handover, conditional PSCell addition, conditional PSCell change, or subsequent CPAC associated to the cells identified by the included NR CGIs and associated to the UE-associated signaling identified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE.

If the Positioning Context Reservation Indication IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall not release the positioning context including the SRS configuration for the UE.

If the LTM Cells To Be Released List IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall, if supported, consider that the gNB-CU is cancelling only the LTM cells identified by the included NR CGIs and associated to the UE-associated signaling identified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE.

If the Recommended SSBs for Paging List IE is included in the UE CONTEXT RELEASE COMPLETE message, the gNB-CU shall, if supported, store it and may use it as assistance information for subsequent paging.

If the DL LBT Failure Information Request IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall, if supported, consider that the gNB-CU requests the reporting of DL LBT Failure information for the analysis of the MRO events of the UE specified in TS 38.300 [6],  and act as specified in TS 38.401 [4].

If the Mobility Information IE is provided in the UE CONTEXT RELEASE COMPLETE message, the gNB-CU shall, if supported, store this information and use it as specified in TS 38.401 [4].

Interactions with UE Context Setup procedure:

The UE Context Release procedure may be performed before the UE Context Setup procedure to release an existing UE-associated logical F1-connection and related resources in the gNB-DU, e.g. when gNB-CU rejects UE access it shall trigger UE Context Release procedure with the cause value of UE rejection.

- 8.3.4	UE Context Modification (gNB-CU initiated)

- 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

- 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

<<<Unchanged part is omitted>>>

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

If the Mobility Information IE is provided in the UE CONTEXT RELEASE COMPLETE message, the gNB-CU shall, if supported, store this information and use it as specified in TS 38.401 [4].

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

- 8.3.9	DU-CU Cell Switch Notification

- 8.3.9.1	General

The purpose of the DU-CU Cell Switch Notification procedure is to enable the gNB-DU to inform the gNB-CU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-DU to the gNB-CU. The procedure uses UE-associated signalling.

- 8.3.9.2	Successful Operation

Figure 8.3.9.2-1: DU-CU Cell Switch Notification procedure. Successful operation.

The gNB-DU initiates the procedure by sending a DU-CU CELL SWITCH NOTIFICATION message.

Upon reception of the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

Interaction with CU-DU Cell Switch Notification procedure:

If the LTM Cell Switch Information IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

If the TA Information List IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message to transfer the TA value(s).

If the Mobility Information IE is provided in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, store this information and use it as specified in TS 38.401 [4].

- 9.2.2.6	UE CONTEXT RELEASE COMPLETE

This message is sent by the gNB-DU to confirm the release of the UE-associated logical F1 connection or candidate cells in conditional handover, conditional PSCell addition, conditional PSCell change or subsequent CPAC.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Recommended SSBs for Paging List | O |  | 9.3.1.297 |  | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) |  | YES | ignore |

- 9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| DU To CU RRC Information | O |  | 9.3.1.26 |  | YES | reject |
| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used in this version of the specification. | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| >>TSC Traffic Characteristics Feedback | O |  | 9.3.1.302 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Status | O |  | 9.3.1.322 |  | YES | ignore |
| DRB Modified List |  | 0..1 |  | The List of DRBs which are successfully modified. | YES | ignore |
| >DRB Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>RLC Status | O |  | 9.3.1.69 | Indicates the RLC has been re-established at the gNB-DU. | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used in this version of the specification. | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| >>TSC Traffic Characteristics Feedback | O |  | 9.3.1.302 |  | YES | ignore |
| >>ECN Marking or Congestion Information Reporting Status | O |  | 9.3.1.322 |  | YES | ignore |
| SRB Failed to be Setup List |  | 0..1 |  | The List of SRBs which are failed to be established. | YES | ignore |
| >SRB Failed to be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to be Setup List |  | 0..1 |  | The List of DRBs which are failed to be setup. | YES | ignore |
| >DRB Failed to be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SCell Failed To Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell Failed to Setup Item |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to be Modified List |  | 0..1 |  | The List of DRBs which are failed to be modified. | YES | ignore |
| >DRB Failed to be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Inactivity Monitoring Response | O |  | ENUMERATED (Not-supported, ...) |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| Associated SCell List | O |  | 9.3.1.77 |  | YES | ignore |
| SRB Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| SRB Modified List |  | 0..1 |  |  | YES | ignore |
| >SRB Modified Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| BH RLC Channel Setup List |  | 0..1 |  | The list of BH RLC channels which are successfully established. | YES | ignore |
| >BH RLC Channel Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| BH RLC Channel Modified List |  | 0..1 |  | The list of BH RLC channels which are successfully modified. | YES | ignore |
| >BH RLC Channel Modified Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| BH RLC Channel Failed to be Setup List |  | 0..1 |  | The list of BH RLC channels whose setup has failed. | YES | ignore |
| >BH RLC Channel Failed to be Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| BH RLC Channel Failed to be Modified List |  | 0..1 |  | The list of BH RLC channels whose modification has failed. | YES | ignore |
| >BH RLC Channel Failed to be Modified Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Setup List |  | 0..1 |  | The List of SL DRBs which are successfully established. | YES | ignore |
| >SL DRB Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Modified List |  | 0..1 |  | The List of SL DRBs which are successfully modified. | YES | ignore |
| >SL DRB Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Failed To Setup List |  | 0..1 |  | The List of SL DRBs which are failed to be setup. | YES | ignore |
| >SL DRB Failed To Setup Item |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Failed To be Modified List |  | 0..1 |  | The List of SL DRBs which are failed to be modified. | YES | ignore |
| >SL DRB Failed To be Modified Item |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>cause | O |  | 9.3.1.2 |  | - |  |
| Requested Target Cell ID | O |  | NR CGI 9.3.1.12 | Special Cell or PSCell ID in the CPAC MCG Information IE indicated in the UE CONTEXT MODIFICATION REQUEST message. | YES | reject |
| SCG Activation Status | O |  | 9.3.1.234 |  | YES | ignore |
| Uu RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  |  |  |
| Uu RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Uu RLC Channel Modified List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| Uu RLC Channel Failed to be Modified List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Failed to be Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| PC5 RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Modified List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| PC5 RLC Channel Failed to be Modified List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SDT Bearer Configuration Info | O |  | 9.3.1.277 |  | YES | ignore |
| UE Multicast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>Multicast F1-U Context Reference CU | M |  | 9.3.2.13 |  | - |  |
| ServingCellMO-encoded-in-CGC List |  | 0..1 |  |  | YES | ignore |
| >ServingCellMO-encoded-in-CGC Item IEs |  | 1 .. <maxNrofBWPs> |  | The servingCellMO which has been encoded in CellGroupConfig IE. | - |  |
| >>servingCellMO | M |  | INTEGER (1..64, ...) |  | - |  |
| >>BWP ID | M |  | INTEGER (0..4) |  | YES | ignore |
| Dedicated SI Delivery Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| Configured BWP List |  | 0..1 |  | This IE is present when the gNB-DU configures at least one BWP with NCD-SSB or without SSB. | YES | ignore |
| >Configured BWP Item IEs |  | 1 .. <maxNrofBWPs> |  |  | EACH | ignore |
| >>BWP-Id | M |  | INTEGER (0..4) | The IE is used to refer to one BWP. | - |  |
| >>BWP Location And Bandwidth | M |  | INTEGER (0..37949) | The IE type range is the same as the locationAndBandwidth IE in BWP IE as specified in TS 38.331 [8]. |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| Mobility Information | O |  | BIT STRING (SIZE (32)) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying or L2 N3C relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N or L2 U2U relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxNrofBWPs | Maximum number of BWPs per serving cell, the maximum value is 8. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |

- 9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| >>Mobility Information | O |  | BIT STRING (SIZE (32)) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>

TP for TS 38.401

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the gNB-DU responsible for the failure.

If the cause for RLF which occurs shortly after successful handover is identified as a "Too Early HO" or a "HO to Wrong Cell", the gNB-CU shall include the C-RNTI used in the source cell of the last completed handover before the failure in the ACCESS AND MOBILITY INDICATION message. If the source gNB-DU controlling that source cell provided the Mobility Information, it is also included in the ACCESS AND MOBILITY INDICATION message. If used, the Mobility Information is prepared at the source gNB-DU of a LTM and may refer to or identify any LTM-related data at this source gNB-DU.

TP for TS 38.300

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

In case of Too Late Handover or an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a last serving cell when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may inform the failure event to the candidate NG-RAN node(s) controlling the candidate PSCell where the mobility configuration caused the failure.

Editor’s note: FFS on the definition of failure event.

For MRO analysis, in case of RLFs or handover failures, the source gNB may take into account the information regarding the LBT failures occurred in UL and DL for a specific UE. For UL, the information may be collected and provided by the UE to the network in the RLF report. For DL, the information may be collected at the source and target gNBs. The target gNB may provide the information to the source gNB via the Access and Mobility Indication procedure over Xn, if requested by the source gNB during the handover preparation.