3GPP TSG-RAN WG3 #127                                     R3-250329

Athens, Greece, 17th – 21st Feb, 2025

Source:	CATT

Title:	Discussion on Evolution of Duplex Operation

Agenda Item:	19.2

Document for:	Discussion and decision

- Introduction

This document discusses Evolution of Duplex Operation base on WID [1].

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

- Information exchange among gNBs, including [RAN3]

- Semi-static cell-specific SBFD time and frequency location configuration

- Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

- Strongest DL beam information

- CLI-mitigation request

- Discussion

  - Semi-static cell-specific SBFD time and frequency location configuration

The following agreements and WA are captured in RAN1:

Time domain:

For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported

- A starting slot index

- A starting symbol index within the starting slot

- An ending slot index

- An ending symbol index within the ending slot

Frequency domain:

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

The design of SBFD time and frequency location configuration should wait for RAN1 and RAN2. If RAN2 reuse container e.g., tdd-UL-DL-ConfigurationCommon for SBFD time configuration, RAN3 can simply reuse TDD UL-DL Configuration Common NR which has been captured in Served Cell Information already. For SBFD frequency location configuration, the SCS-SpecificCarrierList, starting RB and bandwidth of each SBFD for UL and DL respectively need to be introduced which can also wait for RAN1 and RAN2 decision.

For F1AP, the above SBFD time and frequency location configuration can be provided during F1 SETUP REQUEST and GNB-DU CONFIGURATION UPDATE in Served Cell Information IE from DU to CU.

Proposal 1: RAN3 agrees to reuse Served Cell Information IE to transfer SBFD time and frequency location configuration in F1 SETUP REQUEST and F1 GNB-DU CONFIGURATION UPDATE.

  - Measurement resource configuration and Strongest DL beam information

LS [1] received from RAN1 as below

From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported.

Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB.

Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.

For beam pairing, victim gNB measures NZP CSI-RS which is provided by neighbour aggressor cell. After measurement, victim gNB sends measurement result i.e., the strongest DL beam information back to neighbour aggressor cell. According to the strongest DL beam information, neighbour aggressor cell will try to reduce interference by implementation.

It can be found that there are two steps need to be supported

- Step 1: neighbour aggressor cell sends a set of one or more periodic NZP CSI-RS resources to victim cell

- Step 2: victim cell provides strongest DL beam information in the context of NZP-CSI-RS/SSB to neighbour aggressor cell

For step 1, NZP CSI-RS resource defined in TS38.331 is used to configure reference signal Non-Zero-Power (NZP) CSI-RS for measurement. It can be added in Served Cell Information NR.

For CU/DU split, neighbour aggressor gNB-DU sends NZP CSI-RS resources to aggressor gNB-CU for further transfer from neighbour aggressor gNB to victim gNB.

Proposal 2: Add NZP CSI-RS resource Set and NZP CSI-RS resource in Served Cell Information NR in XnAP and F1AP.

For step 2, the strongest DL beam information is defined as CRI and SSB index.

The question is that if the reporting of the strongest DL beam is periodic or one-off or event trigger.

- In case of periodic response, the strongest DL beam information can be added in the RESOURCE STATUS UPDATE base on the request in Report Characteristics from aggressor gNB.

- In case of one-off response, the strongest DL beam information can be added in Xn SETUP RESPONSE and NG-RAN NODE CONFIGURATION UPDATE.

- In case of event trigger, victim gNB reports the strongest DL beam information when the interference over or less the threshold. The threshold is generated by aggressor gNB.

From technically point view, whether perform interference mitigation depends on aggressor gNB. If victim gNB sends strongest DL beam too often which may cause singling overhead and aggressor gNB cannot reduce interference so quick. If victim gNB sends strongest DL beam too lazy, the interference cannot be quick solved by aggressor gNB.

Observation 1: It is up to aggressor gNB decides whether to perform interference mitigation. The aggressor gNB should control how often receiving strongest DL beam from victim to avoid signalling overhead or miss some strongest DL beam information.

For CU/DU split, victim gNB-DU provides strongest DL beam to victim gNB-CU for further transfer from victim gNB to neighbour aggressor gNB. The same question as XnAP also exists in F1AP i.e., whether the reporting of the strongest DL beam from victim gNB-DU is periodic or one-off or event trigger in DU.

Proposal 3: Victim gNB response the strongest DL beam information in both XnAP (from victim gNB to aggressor gNB) and F1AP (from victim DU to victim CU).

Proposal 4: RAN3 further discuss how to response the strongest DL beam in XnAP and F1AP i.e., periodic response or one-off response or response base on event (threshold) trigger.

After neighbour aggressor gNB-CU receiving strongest DL beam information in the context of NZP-CSI-RS/SSB from victim gNB-CU, neighbour aggressor gNB-CU need to send strongest DL beam information to aggressor gNB-DU e.g., for beam adjustment.

The strongest DL beam information can be added in Neighbour Cell Information List in GNB-CU CONFIGURATION UPDATE message.

Proposal 5: Add strongest DL beam information in a Neighbour Cell Information List in GNB-CU CONFIGURATION UPDATE send from aggressor gNB-CU to aggressor gNB-DU.

  - CLI-mitigation request

- Notes: Following are assumed for information exchange among gNBs

  - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3

  - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact

The victim gNB sends the CLI-mitigation handling request aggressor gNB performs the CLI-mitigation. The NG-RAN NODE CONFIGURATION UPDATE can be reused.

For CU/DU split, the CLI-mitigation handling request can be sent from victim gNB-DU to victim gNB-CU first.

For beam paring, the CLI-mitigation request can be considered as a trigger for neighbour aggressor gNB to send NZP CSI-RS to victim gNB to try to reduce CLI but we do not need to specify when and how to use CLI-mitigation request in specification. But it not preclude that the NZP CSI-RS exchange can be happened at any time as long as victim gNB being the measurement.

For beam nulling, the CLI-mitigation request can trigger aggressor gNB starts measuring to reduce CLI base on the NZP CSI-RS provided from victim gNB. If NZP CSI-RS of victim did not send yet, the aggressor can wait for receiving NZP CSI-RS of victim and then start measuring.

Proposal 6: For both beam paring and beam nulling, CLI-mitigation request can be send either before or after receiving/sending NZP CSI-RS. There is no need to add order restrictions to the specification.

Proposal 7: For beam nulling, CLI-mitigation request may trigger aggressor to start measuring on NZP CSI-RS provided by victim gNB. But RAN3 will not specify the behaviours of receiving node (aggressor gNB) in specification.

Proposal 8: RAN3 agree to introduce CLI-mitigation request in both XnAP NG-RAN NODE CONFIGURATION UPDATE and F1AP gNB-DU CONFIGURATION UPDATE.

- Conclusion

Proposal 1: RAN3 agrees to reuse Served Cell Information IE to transfer SBFD time and frequency location configuration in F1 SETUP REQUEST and F1 GNB-DU CONFIGURATION UPDATE.

Proposal 2: Add NZP CSI-RS resource Set and NZP CSI-RS resource in Served Cell Information NR in XnAP and F1AP.

Observation 1: It is up to aggressor gNB decides whether to perform interference mitigation. The aggressor gNB should control how often receiving strongest DL beam from victim to avoid signalling overhead or miss some strongest DL beam information.

Proposal 3: Victim gNB response the strongest DL beam information in both XnAP (from victim gNB to aggressor gNB) and F1AP (from victim DU to victim CU).

Proposal 4: RAN3 further discuss how to response the strongest DL beam in XnAP and F1AP i.e., periodic response or one-off response or response base on event (threshold) trigger.

Proposal 5: Add strongest DL beam information in a Neighbour Cell Information List in GNB-CU CONFIGURATION UPDATE send from aggressor gNB-CU to aggressor gNB-DU.

Proposal 6: For both beam paring and beam nulling, CLI-mitigation request can be send either before or after receiving/sending NZP CSI-RS. There is no need to add order restrictions to the specification.

Proposal 7: For beam nulling, CLI-mitigation request may trigger aggressor to start measuring on NZP CSI-RS provided by victim gNB. But RAN3 will not specify the behaviours of receiving node (aggressor gNB) in specification.

Proposal 8: RAN3 agree to introduce CLI-mitigation request in both XnAP NG-RAN NODE CONFIGURATION UPDATE and F1AP gNB-DU CONFIGURATION UPDATE.

- Reference

[1] R1-2407533, LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation, RAN1.