3GPP TSG-RAN WG3 #127	R3-250332

Athens, Greece, 17th – 21st February 2025

Agenda Item:	8.1

Source:	CATT

Title: 	Discussion on LS for XR

Document for:	Discussion and Approval

- Introduction

This paper discusses four LS received from other groups about UL bit rate control, available data rate and MMSID.

- Discussion

  - R3-250012 LS on XR UL bit rate control

RAN2 has made the following agreement regarding a solution on UL bit rate control

Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

MAC CE is sent from gNB-DU to UE. According to TS 38.473, gNB-DU knows QoS flow to DRB mapping, which is normally used for scheduling. gNB-DU is able to detect congestion on UL per QoS flow and send MAC CE per QoS flow to UE.

Proposal 1: gNB-DU is able to send per QoS flow MAC CE to UE.

  - R3-250018 LS (S2-2411282/R3-244844) on FS_XRM Ph2 from RAN3

For semi-static periodicity traffic, it is feasible for NG-RAN node to provide the available data rate to CN via UP. For dynamic traffic, some companies have concern that the reporting of available data rate may not be accurate.

RAN3 would like SA2 to further clarify the definition of “available data rate”, e.g., it is only below the GFBR, or between GFBR and MFBR, or even above MBFR; whether it should be periodic reporting, on demand reporting, or threshold(s) defined reporting.

Feedback from SA2:

From SA2 perspective, it is expected that the available data rate is properly reflecting the resources that are currently provided by the NG-RAN for the QoS Flow. While it is up to the application layer how to make use of it, we can however assume that the application will typically exchange data with a somewhat lower rate than what is reserved or measured at NG-RAN.

Regarding the definition of “available data rate”, it refers to the data rate that NG-RAN is currently able to provide for a QoS Flow with GFBR. The “available data rate” can be between the GFBR and the MFBR for a QoS Flow with MFBR>GFBR. The “available data rate” can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without AQP. The “available data rate” should however not be above the MFBR (as the NG-RAN is not required to provide more resources that what is required for the MFBR).

Periodic reporting should not be useful for the application layer (as the “available data rate” should often be identical to the GFBR). Threshold based reporting is considered as useful with one or more threshold values provided by the application. When the data rate that NG-RAN is currently able to provide for a QoS Flow crosses one of the thresholds, the NG-RAN is expected to provide the reference to the respective threshold in the UL user plane packets and the application can then adjust its data rate accordingly. The usage of the first packets after a change in the “available data rate” should be sufficient to transfer the information to the UPF. Similar to the logic described in 23.501 clause 5.7.2.4.1a, the NG-RAN is expected to immediately report a change to a lower threshold value for the “available data rate” and a change to a higher threshold value with some hysteresis.

SA2 clarified the definition of “available data rate” is the data rate that NG-RAN is currently able to provide for a QoS Flow with GFBR.

For reporting, SA2 considers it useful to set one or more threshold values provide by application. The NG-RAN will report the reference to the respective threshold of “available data rate” to core network in case the “available data rate” crosses one of the thresholds.

Therefore, RAN3 impact includes introducing thresholds from CN to NG-RAN and providing “available data rate” in NG-U once it crosses one of the thresholds. The detail should be discussed by SA2 first. We can inform them to provide more details to help us to enhance specifications.

Observation 1: RAN3 impact includes introducing thresholds from CN to NG-RAN and providing “available data rate” in NG-U once it crosses one of the thresholds.

Proposal 2: RAN3 reply SA2 that RAN3 will start work about “available data rate” when receiving the details from SA2 e.g., threshold value and“available data rate” format.

  - R3-250027/R3-250032 LS Reply on multi-modality awareness

MMSID:

According to R3-250027 from SA4, it can be found that not all applications require all QoS flows to be granted. If the gNB cannot grant all associated QoS Flows, only the application layer can determine how to handle the case. Therefore, SA4 have negative attitude towards introducing MMSIDs from CN to RAN.

However, in R3-250032, SA think the multi-modality awareness can be achieved in the RAN by the CN providing multimodality information (i.e., the MMSID) to the NG-RAN by N2 SM information. SA2 have a lot of things to be discussed. RAN3 will follow the SA2 decision and updated specification after they have final conclusion.

Proposal 3: It is SA2 to make the final decision about how to provide MMSID to NG-RAN. After receiving LS from SA2, RAN3 will start updating our specification accordingly.

Traffic synchronization (on a per packet basis):

SA4 propose that we already have well-defined mechanisms to support synchronized playout among different media flows which can provide frame level synchronization. There is no evidence to support per packet synchronization

Proposal 4: RAN3 will not start discussing on traffic synchronization unless other group explicitly support it and send us LS.

3 Conclusions

In this contribution the following observations and proposal were derived:

Proposal 1: gNB-DU is able to send per QoS flow MAC CE to UE.

Observation 1: RAN3 impact includes introducing thresholds from CN to NG-RAN and providing “available data rate” in NG-U once it crosses one of the thresholds.

Proposal 2: RAN3 reply SA2 that RAN3 will start work about “available data rate” when receiving the details from SA2 e.g., threshold value and“available data rate” format.

Proposal 3: It is SA2 to make the final decision about how to provide MMSID to NG-RAN. After receiving LS from SA2, RAN3 will start updating our specification accordingly.

Proposal 4: RAN3 will not start discussing on traffic synchronization unless other group explicitly support it and send us LS.