3GPP TSG-RAN WG3 Meeting #127	R3-250348

Athens, Greece, 17th - 21th Feb 2025

Title:	Discussion on Continuous MDT measurement

Source:				China Telecom

Agenda item:		11.4

Document for:		Discussion and Decision

1. Introduction

In RAN3#125-bis and RAN3#126 meetings, we discussed the AI/ML based continuous MDT collection targeting the same UE across RRC states and the following agreements were made.

RAN3#125bis: RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN. RAN3#126: In R19, introducing the Continuous MDT functionality with a solution being applicable to legacy UEs and not only to Rel-19 UEs. Agree to send an LS to SA5 capturing e.g. problem description, solutions requirements, potential solutions (if agreeable). When to send the LS depends on convergence on the content of the LS.

In this document, we continue discussing AI/ML based continuous MDT collection targeting the same UE across RRC states.

2. Discussion

In RAN3#126 meeting, RAN3 agree to send an LS to SA5 capturing e.g. problem description, solutions requirements, potential solutions (if agreeable). When to send the LS depends on convergence on the content of the LS. In this document, we provide our view on the problem description, solution requirements and potential solutions.

The two problems that need to be resolved for Continuous MDT are:

-	Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

-	Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

In RAN3#125bis meeting, RAN3 assumed that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN. Additionally, in RAN3#126 meeting, RAN3 agreed introducing the Continuous MDT functionality with a solution being applicable to legacy UEs and not only to Rel-19 UEs. Based on the agreements and assumptions in the previous meetings, we have following proposal:

Proposal #1: The solution requirements should include:

-	OAM should configure RAN with a Management Based MDT configuration to be used for Continuous MDT.

-	Continuous MDT solutions should avoid impacts on legacy UEs.

To solve the problem, following solutions were derived in the previous meeting excluding solutions with UE impact [2].

- Solution #1: Based on OAM-RAN interaction

  - The OAM configures Management Based MDT to the gNBs taking part in Continuous MDT. The configuration includes specific identifiers (e.g. Trace Record) dedicated to the Continuous MDT process, namely the NG-RAN deduces from such identifier that the Management Based configuration is for Continuous MDT.  The RAN configures parameters such as the Trace Record Session Reference, which are reassigned to the UE (in full or in part) across RRC States and RRC_Connected mobility, namely such parameters are assigned/reassigned to the UE throughput the duration of the Continuous MDT session.

  - Measurement continuity is ensured by

    - Including in the Handover signaling an indication that the UE has to be selected for continuous MDT, e.g. the TR/TRSR assigned at source to the Continuous MDT process for the UE

    - Checking the TR/TRSR in the Logged MDT reports signaled by the UE at RRC_Idle/Inactive to RRC_Connected transitions. TR/TRSR reveal that the UE should be selected for Continuous MDT

  - Trace Correlation is achieved by:

    - Reusing part/all of the identifiers in the Trace Record header, e.g. the TRSR, for the whole duration of the Continuous MDT trace.

- Solution #2: Based on OAM-RAN-AMF interaction.

  - The OAM configures Management Based MDT at a gNB that selects the UE for Continuous MDT. When UE transits to RRC_Idle, a context related to the MDT configuration is stored by the gNB serving the UE to the Core Network. Upon transition from RRC_Idle to RRC_Connected or after successful handover, the AMF indicates to the second gNB serving the UE that the MDT configuration used for the UE, including an indication that the UE should be selected for management-based immediate MDT. In case of handover, the source gNB sends the MDT context related to the data collection to the target gNB during handover preparation.

  - Measurement continuity is ensured by

    - Receiving at the handover target gNB the MDT configuration previously used for Continuous MDT for the UE (signaled from the source gNB to the target gNB)

    - Receiving at the new serving gNB the MDT configuration previously used for Continuous MDT for the UE (signaled from the AMF to the serving gNB)

  - Trace Correlation is achieved by:

    - Create an identifier at the AMF, e.g. generated when the UE registers to the network and UE context is created in the Core Network and maintained in the Core Network for the duration of the UE context.

    - The AMF assigned identifier is signalled to the serving gNB as part of the MDT configuration used for continuous MDT. The serving gNB includes such identifier in the Trace Records signaled to the TCE.

- Solution #3: Based on OAM-RAN-AMF interaction.

  - Introduce a new Job Type in SA5 specifications which allows to configure a MDT session encompassing Immediate MDT and Logged MDT simultaneously, and assigning to this MDT session the same Trace Reference (TR). Then the gNB activates the MDT functionality for the selected UE and assigns a Trace Recording Session Reference (TRSR) to the selected UE. Therefore, in management-based MDT, a single TR could be assigned to an MDT session with a Job Type equal to “Immediate MDT and Logged MDT”, and this configuration is valid for all UEs selected by the NG-RAN. In order to then correlate the Immediate and Logged MDT reports from the same UE (properly anonymized), the TCE could perform such correlation by “collating” those MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE.
The gNB serving a Continuous MDT UE sends parts of the MDT configuration to the AMF, which stores such information. In this manner, once the UE re-establishes/resumes its RRC connection, the (potentially new) serving gNB will be provided by the 5GC/AMF with UE context data including user consent information as well as the stored parts of the MDT configuration

  - Measurement continuity is ensured by

    - The serving gNB receiving from the AMF information concerning the Continuous MDT configuration for the UE (e.g., at least Job Type “Immediate MDT and Logged MDT” and TRSR allocated by the old serving gNB) which were previously stored in the AMF/5GC by the old serving gNB, so that the same UE can be re-selected and re-configured with management-based MDT immediately after it re-establishes/resumes its RRC connection.

    - Source gNB provides the target gNB with information concerning the Continuous MDT configuration (e.g., at least Job Type “Immediate MDT and Logged MDT” and TRSR assigned by the source gNB). The provided information, along with availability of the user consent, can be used by the target gNB in order for the same UE to be re-selected and re-configured with management-based MDT immediately after handover completion.

  - Trace Correlation is achieved by:

    - “collating” those MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE

- Solution #4: Based on OAM-RAN interaction (Trace Correlation without RAN3 Impact).

  - For MDT report

    - Immediate MDT report: gNB sends the immediate MDT Records to TCE, including the TR1, TRSR1 of immediate MDT(legacy behavior), and the assigned TR2,TRSR2 for logged MDT when UE goes to idle.

    - Logged MDT report: gNB sends the received MDT Record to the TCE, including the TR2, TRSR2 of the logged MDT(legacy behavior), and the assigned TR1,TRSR3 for immediate MDT UE will execute during connected state.

  - Measurement continuity is ensured by

    - Similar with solution 2, e.g., OAM/AMF assign an identifier and stored in CN.

  - Trace Correlation is achieved by:

    - Correlation two MDT Record by the “assigned TRx,TRSRx”  in this MDT report.

Regarding Solution #2 and Solution #4, measurement continuity is ensured by OAM/AMF assigning an identifier and stored in CN. In solution #2, trace correlation is achieved based on the identifier stored in CN. In solution #4, trace correlation is achieved based on the TR/TRSR in the MDT report. In these two solutions, the CN is required to consistently assign an identifier for each UE, which may lead to additional burden on the CN or OAM. Therefore, in our opinion, solution #2 and solution #4 need to be down prioritized.

Regarding Solution #1 and Solution #3, management-based MDT for Continuous MDT is configured by OAM to RAN and RAN configures parameters, such as Trace Recording Session Reference (TRSR), which are assigned to the selected UE. These two solutions are the most straightforward solutions. Solution #1 is a simpler solution with minor standard impact. However, in solution #4, new Job Type in SA5 specifications need to be defined, which needs to be agreed by SA5. Since both solutions are related to SA5, we have following proposal:

Proposal 2: RAN3 suggests taking the Solution #1 and Solution #3 as the starting point for continuous MDT solution and down prioritize the Solution #2 and solution #4.

Apart from the continuous MDT measurement indication parameter, the continuous MDT measurement duration is also important. When the UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive and/or when the UE is handed over between gNBs, the gNB should know whether the UE is configured the continuous MDT measurement indication parameter and whether the validity of continuous MDT measurement indication parameter is expired. If the validity of continuous MDT measurement indication parameter is expired, the gNB will not select the UE for continuous management-based immediate MDT even though the UE is configured the continuous MDT measurement indication parameter. Therefore, we have following proposal:

Proposal 3: The Continuous MDT solutions should support the continuous MDT measurement duration. It defines the validity of continuous MDT measurement indication.

Based on the discussion above, we have the following proposal:

Proposal 4: The LS sent to SA5 should include the content in Proposal 1, Proposal 2 and Proposal 3.

3. Conclusion

We have following proposals:

Proposal #1: The solution requirements should include:

-	OAM should configure RAN with a Management Based MDT configuration to be used for Continuous MDT.

-	Continuous MDT solutions should avoid impacts on legacy UEs.

Proposal 2: RAN3 suggests taking the Solution #1 and Solution #3 as the starting point for continuous MDT solution and down prioritize the Solution #2 and solution #4.

Proposal 3: The Continuous MDT solutions should support the continuous MDT measurement duration. It defines the validity of continuous MDT measurement indication.

Proposal 4: The LS sent to SA5 should include the content in Proposal 1, Proposal 2 and Proposal 3.

4. Reference

- TR 38.743, Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (Release 19).

- R3-247790, SoD on CB: # AIRAN5_ContinousMDT.