3GPP TSG-RAN WG3 Meeting #127	   				         R3-250350

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:		14.3

Source:			China Telecom

Title:	Discussion on regenerative NTN payload enhancement

Document for:	 	Discussion & Decision

- Introduction

The Rel-19 NTN enhancements WI contains the following objective[1]:

Support of regenerative payload [RAN3, RAN2, RAN4]

- Specify the support of gNB on board in TS 38.300

- Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In this contribution, we will continue to discuss the potential impact of supporting NTN regeneration payloads from RAN3 perspective.

- Discussion

2.1 NG interface management

SA2 discussed whether the OAM solution would have impacts to the existing procedures related to paging and sequencing of OAM updates and RAN3 updates towards the AMF, e.g. in NGAP Setup/configuration update etc. And in the reply LS, some companies expressed concerns about the frequency of OAM updates and whether there would be conflicts with TAI interactions in existing signaling procedures.

From our point of view, effective signaling design (e.g. valid time windows) can reduce the frequency of OAM updates. Even if this information is transmitted via NGAP instead of OAM, it will still face the same problem. Therefore, the focus is on how to design a reasonable signaling format, rather than using OAM. SA5 has agreed the potential solutions of providing the information of supported TAIs for NTN gNB to AMF in TR 28.874.

On the other hand, with reasonable configuration by network operators, we believe that there will not be any conflict between NGAP and OAM, as TAI's planning and satellite ephemeris information are pre-configured. Besides, RAN3 did not refuse to update TAI via the NGAP configuration update procedures, and the legacy way is always workable.

Reply to SA2 that from the perspective of RAN3, the OAM based solution will not have any unexpected impact on the existing NGAP procedures.

Proposal 1: From the perspective of RAN3, the OAM based solution will not have any unexpected impact on the existing NGAP procedures.

2.2 Location Based CHO

We have enhanced the Xn/NG interface to support time-based CHO/HO. At the final RAN3 meeting of Rel-18, some companies proposed to enhance location-based HO. Supporting location-based HO includes the following benefits:

- Location-based trigger condition generally do not have an "expiration", as the target gNB cannot cancel the configured HO and may face the risk of allocating resources that cannot be released;

- The early data forwarding adopted in NG HO with time-based trigger condition is also applicable to location-based trigger condition in principle;

- Coarse UE location information can assist target satellites for resource scheduling.

The current RAN3 specifications do not describe how to support CHO/HO with location-based trigger conditions in Xn/NG. For the mobility between gNBs, it is proposed that RAN3 consider whether and how to better support location-based CHO/HO via the Xn/NG interfaces. For example, introducing coarse UE location information or location-based trigger conditions or reusing time-based trigger conditions in the HANDOVER REQUEST message.

Proposal 2: It is proposed to study whether and how to support location-based CHO/HO via the Xn/NG interfaces.

2.3 Feeder Link Switch over

In the case of soft feeder link switch over, the current control plane mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management. As for NG-U, When the NTN-GW changes, the NG-U channel may also change. The gNB needs to inform the UPF for the change of TNL address for all connected UEs, this can be performed by the gNB initiating the PDU Session Resource Modify Indication procedure or Path Switch procedure for each UE. The Path switch procedure may need to be implemented via the Handover, which requires the satellite to support additional functions (e.g. the same physical gNB implementations two logical gNBs), with additional impact on CN,e.g. connection with two logical gNBs during Handover and one connection with one logical gNB after Handover. Whether non HO Path switch procedure is supported may depend on implementation.

Proposal 3: It is proposed to discuss how to support the NG-U tunnels update in case of soft feeder link switch without relocation of the AMF.

In case of hard feeder link switch over, a satellite/gNB can only be connected to one NTN-GW at the same time. If 5GC does not know the hard feeder link switch over, the 5GC may continue sending the DL NG-C/U packet to the old NTN-GW. The old NTN-GW has no way to deliver it to the gNB or forward it to the new NTN-GW, which will result in packet loss. Although the existing retransmission mechanism at application layer can work, it is better not to cause interruption for some services requiring service continuity (such as voice services). It is beneficial to inform the 5GC to buffer DL data based on actual situation of the UEs and feeder links, so the 5GC can suspend the DL NG-C/U.

Figure 1: Feeder link switchover without AMF change

- The gNB onboard the satellite connects to 5GC via NTN-GW1. In case of hard feeder link switchover, the gNB may informs AMF about the upcoming hard feeder link switchover. It can be per UE procedures or non-UE associated procedures.

2~4. The AMF informs the corresponding SMF(s) about the upcoming feeder link switchover receive from gNB. Based on feeder link switchover information, the SMF(s) controls the UPF(s) to buffer the DL data of the corresponding PDU sessions. It can be existing per UE procedures or new non-UE associated procedures, depending on further SA2 study.

- The gNB onboard the satellite performs Feeder link switchover to NTN-GW2. After the Feeder link is switched, the gNB is assumed to get a new TNLA.

- The gNB connects to AMF via NTN-GW2 by using the new TNLA as described in proposal 3.

7~8. The gNB needs to inform the UPF for the change of TNL address for all connected UEs, this can be performed by using existing procedures as described in proposal 4 or define new non-UE associated procedures.

Proposal 4: It is proposed to study if any enhancement is required for hard feeder link switch scenarios to reduce interruption and packet loss problem.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: From the perspective of RAN3, the OAM based solution will not have any unexpected impact on the existing NGAP procedures.

Proposal 2: It is proposed to study whether and how to support location-based CHO/HO via the Xn/NG interfaces.

Proposal 3: It is proposed to discuss how to support the NG-U tunnels update in case of soft feeder link switch without relocation of the AMF.

Proposal 4: It is proposed to study if any enhancement is required for hard feeder link switch scenarios to reduce interruption and packet loss problem.

- References

[1]	RP-241667, Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT.