3GPP TSG-RAN WG3 Meeting #127	   				         R3-250351

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:		14.3

Source:			China Telecom

Title:		(TP for TS 38.300) Support of regenerative payload

Document for:	 	Other

- Introduction

The following agreement has been made at previous RAN3 meeting:

The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch.

RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

In this contribution, we further provide some potential stage-2 description updates for regenerative payload based on RAN3 agreements.

- TP to BLCR for TS 38.300

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>

16.14.4	Switchover

16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in transparent or regenerative NTN payloads.

16.14.4.2	Assumptions

A feeder link switchover may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switchover, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switchover, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption may occur during the transition between the feeder links.

For regenerative NTN payload, the current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switchover.

16.14.4.3	Procedures

In transparent NTN payload, the NTN Control function (see Annex B.4) determines the point in time when the feeder link switchover between two gNBs is performed. The transfer of the affected UE(s)' context between the two gNBs at feeder link switchover is performed by means of either NG based or Xn based handover, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

In regenerative NTN payload, the NTN Control function determines the point in time when the feeder link switchover is performed. During the feeder link switchover, AMF is not changed for the UEs.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>