3GPP TSG-RAN WG3 Meeting #127	   				         R3-250353

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:		22.2

Source:			China Telecom

Title:	UE authorization for multi-hop relay

Document for:	 	Discussion, Decision

- Introduction

In the newest WID, the objective of service continuity is as follows [1]:

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

- Discussion

RAN3 has received the LS from SA2 on ProSe authorization information related to Layer-2 multi-hop U2N relay operation. The following authorization statuses are introduced[2].

If the UE is authorised to use 5G ProSe services, then the AMF shall include in a NGAP message sent to NG-RAN:

-	"5G ProSe authorised" information, including one or more of the following:

-	whether the UE is authorized to use 5G ProSe Direct Discovery;

-	whether the UE is authorized to use 5G ProSe Direct Communication;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE;

-	whether the UE is authorized to use multi-path communication via direct Uu path and via 5G ProSe Layer-2 UE-to-Network Relay as a 5G ProSe Layer-2 Remote UE;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-UE Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 End UE;

-	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-Network Relay supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 Intermediate UE-to-Network Relay;

-	whether the UE is authorized to act as a 5G ProSe Layer-2 Remote UE supporting 5G ProSe Layer-2 multi-hop UE-to-Network Relay.

According to LS, the “5G ProSe authorised” information needs to be enhanced to include the authorization information for Layer-2 multi-hop UE-to-Network relay operation. In addition, we introduced authorization information for 5G ProSe Layer-3 UE-to-Network Relay in previous Release, which is also applicable to multi-hop operation.

Proposal 1: NG-RAN receives the multi-hop U2N relay authorization information from the AMF.

- Whether the UE is authorized to act as a 5G ProSe Layer-3 multi-hop UE-to-Network Relay;

- Whether the UE is authorized to act as a 5G ProSe Layer-2 multi-hop UE-to-Network Relay;

- Whether the UE is authorized to act as a 5G ProSe Layer-2 Intermediate UE-to-Network Relay;

- Whether the UE is authorized to act as a 5G ProSe Layer-2 multi-hop Remote UE.

- Conclusions

Based on the above discussion, we propose the following:

Proposal 1: NG-RAN receives the multi-hop U2N relay authorization information from the AMF.

- Whether the UE is authorized to act as a 5G ProSe Layer-3 multi-hop UE-to-Network Relay;

- Whether the UE is authorized to act as a 5G ProSe Layer-2 multi-hop UE-to-Network Relay;

- Whether the UE is authorized to act as a 5G ProSe Layer-2 Intermediate UE-to-Network Relay;

- Whether the UE is authorized to act as a 5G ProSe Layer-2 multi-hop Remote UE.

- References

[1]	RP-242349, Revised WID on NR sidelink multi-hop relay, LG Electronics, InterDigital, FirstNet, 3GPP TSG RAN Meeting #105.

[2]	S2-2500386, Authorization information for Layer-2 multi-hop U2N relaying to NG-RAN.