3GPP TSG-RAN WG3 Meeting #127                                                                    R3-250357

Athens, Greece, Feb 17th–21st, 2025

Source:	CATT

Title:	(TP to BLCR for TS 38.423) Discussion on enhancements for WAB

Agenda Item:	12.2

Document for:	Approval

1 Introduction

In RAN3#126 meeting [1], additional topological enhancements for NR are studied.

In this contribution, we will discuss potential enhancements to WAB and provide the TP to TS 38.423.

2 Discussion

2.1 Xn connection management

2.1.1 Dynamic Xn setup

Based on the WID approved during RAN#105 [3], Xn interface setup for the WAB-gNB with BH-RAN-node or surrounding NG-RAN nodes should be specified, which has not been captured in [3]. Different to normal gNB, Xn setup is dynamic for WAB-gNB, due to the neighbour relationship may change with the movement of WAB-node.

As a legacy mechanism, UEs served by WAB-node can detect the PCI of the neighbour cell of WAB-gNB, and UE’s CGI reporting procedure can be used to derive the RAN-node ID of the neighbour nodes. However, the CGI acquisition is optionally supported by UE. Other method should be studied. Basically, the cell serving the WAB-MT under the BH-RAN-node is probably the nearest neighbour cell of WAB-gNB. As the RAN-node ID of the BH-RAN-node is broadcasted in SIB1, the WAB-gNB can recognize the BH-RAN-node after the WAB-MT obtaining SIB1 of the serving cell. Then, the WAB-gNB obtains IP address of the BH-RAN-node and sets up Xn connection with the BH-RAN-node. IP address of the BH-RAN-node can be obtained by the legacy Xn-C TNL address discovery mechanism. This procedure only impacts stage-2.

Proposal 1-1: WAB-gNB can recognize BH-RAN-node via SIB1 of the cell serving the WAB-MT.

Proposal 1-2: WAB-gNB obtains IP address of the BH-RAN-node via legacy Xn-C TNL address discovery mechanism and sets up Xn connection with it.

If setting up Xn with the BH-RAN-node, the WAB-gNB may discover some neighbour cells’ CGI from the serving cell information delivered by the BH-RAN-node over Xn interface.

There may be other neighbour cells of WAB-gNB, which are supposed in scope of the neighbour cells of the BH-RAN-node. The WAB-gNB can also discover neighbour cells’ CGI from the neighbor cells of the BH-RAN-node. The information of BH-RAN-node’s neighbour cells is sent from BH-RAN-node via Neighbour Information NR IE or Neighbour Information E-UTRA IE in some Xn messages, including Xn SETUP REQUEST/RESPONSE message, NG-RAN NODE CONFIGURATION UPDATE message, and NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

After discovering the neighbour cells’ CGI, the WAB-gNB can add the neighbour cells to the NCRT and look up the TNL address of the neighbour nodes to set up Xn interface. The WAB-gNB can get the IP addresses of the neighbour RAN-nodes using the legacy Xn-C TNL address discovery mechanism. This procedure impacts only stage-2.

Proposal 1-3: If WAB-gNB establishes Xn connection with BH-RAN-node, it discovers neighbour cells through Xn messages sent from the BH-RAN-node.

Proposal 1-4: WAB-gNB looks up the IP address(es) of the neighbour nodes using legacy Xn-C TNL address discovery mechanism and set up Xn connections with surrounding RAN nodes.

2.1.2 Xn setup avoidance

According to the WID approved during RAN#105 [3], setting up Xn between WAB-gNBs should be avoided. Due to the mobility of WAB-node is unpredictable, the WAB-gNBs is recommended not to set up Xn connection with another WAB-gNB in most cases, in order to avoid the frequent Xn setup and removal. From our point of view, the Xn setup avoidance should not be limited to the case between WAB-gNBs as it would be also necessary to avoid Xn connection setup between a WAB-gNB and a NG-RAN node. Although a WAB-gNB is allowed to establish Xn connection with a NG-RAN node, it’s possible that a WAB-gNB and a NG-RAN node would not like to establish Xn with each other.

Observation 1-1: Besides the Xn setup avoidance between WAB-gNBs, avoiding Xn connection setup between a WAB-gNB and a NG-RAN node would also be necessary.

WAB cell specific info (e.g., including a WAB-cell indication) can be introduced to Served Cell Information NR IE which is included in XN SETUP REQUEST message and XN SETUP RESPONSE message. For avoidance of setting up Xn between WAB-gNBs, a WAB-gNB can refuse the Xn setup request from another WAB-gNB. For Xn setup avoidance between a WAB-gNB and a NG-RAN node, the WAB-gNB can refuse the Xn setup request from the NG-RAN node. And if the NG-RAN node is enhanced to support WAB, it can recognize the WAB cell specific info and refuse the Xn setup request from the WAB-gNB.

Observation 1-2: WAB-cell specific information can be added to the Served Cell Information NR IE over Xn interface.

Similar as IAB, the resource configuration for WAB cell, including slot format and symbol type configurations, can be added to Served Cell Information NR IE for the neighbour node to take into account for cross-link interference mitigation. Therefore, the explicit WAB cell indicator may not need to be introduced to the Served Cell Information NR IE.

Proposal 1-5: Resource configuration of WAB cell is introduced to Served Cell Information NR IE to implicitly indicate a WAB cell.

2.1.3 Dynamic Xn removal

Due to the mobility of WAB-node, the NCRT in the WAB-gNB may change more frequently than legacy gNB. The WAB-node would need to set up Xn with new neighbour nodes and release Xn with the original neighbour nodes during its movement. The WAB-node can discover PCI of the new neighbour cell through the UE’s measurement result.

If the WAB-MT connects to a new BH-RAN-node and WAB-gNB establishes Xn with the new BH-RAN-node, the WAB-gNB can derive CGI of the current neighbour cells through serving cell information or neighbour cell information sent by the new BH-RAN-node over Xn. The WAB-gNB may add the new neighbour cells to the neighbour relationship, and set up Xn connection with the new neighbour nodes. Accordingly, the WAB-gNB would have to remove cells from the NCRT dynamically.

The WAB-gNB will consider the cell(s), which is not included in the serving/neighbour cell information of current BH-RAN-node, no longer acting as the neighbouring cells and remove the cell(s) from the NCRT. If there is any NG-RAN node which has already set up Xn connection with the WAB-gNB no longer serves the neighbour cell of the WAB-gNB, the WAB-gNB can initiate Xn removal to the NG-RAN node.

Proposal 1-6: WAB-gNB considers the cell not included in the serving/neighbour cell information of current BH-RAN-node as no longer acting as the neighbour cell.

Proposal 1-7: WAB-gNB removes Xn with the with the NG-RAN nodes which do not serve any neighbour cell of the WAB-gNB.

2.2 Resource multiplexing

Based on the WID approved during RAN#105 [3], radio-resource coordination between access and backhaul links should be specified. According to the TR [4], the resource coordination mechanism introduced for IAB can be considered as the starting point to resource multiplexing for WAB.

Resource multiplexing for IAB is specified in Rel-16/17. The transmission of each IAB-DU’s cell depends on resource configuration or implicit determination to make sure the access link will have no impact to the BH link. The same principle should be applied to WAB, that DL/UL transmission of each WAB-gNB’s cell should be constrained for potential impacts to transmission over the BH link.

Each slot is assigned to be hard, soft, or unavailable for D/U/F symbols for IAB-DU’s cell. When a symbol is configured as hard, the IAB-DU can use it for schedule. When a symbol is configured to be unavailable, the IAB-DU cannot use it except in some special cases. When a symbol is configured as soft, the IAB-DU can conditionally use it in cases where the parent node explicitly indicates the availability to the co-located IAB-MT via DCI format 2_5, or up to IAB-node’s decision.

Similar to IAB, there can be three types for D/U/F symbols in a slot for WAB-gNB’s cell. The hard symbols can be used by WAB cell, the unavailable symbols cannot be used by WAB cell and the soft symbols can be conditionally used by WAB cell depending on whether it has interference to the backhaul link. WAB-gNB can determine whether the soft symbols can be used relying on implementation. Different with IAB, a DCI for indicating the availability of soft symbols is not recommended to be considered for WAB, as the impact to RAN1 spec.

Proposal 2-1: Three types of symbol (i.e., hard, soft and unavailable) can be used for resource multiplex of WAB. WAB-gNB can determine whether the soft symbols are used relying on implementation.

- Resource coordination information

To perform resource multiplexing between access link and backhaul link, resource information for each cell of BH-RAN-node and each cell of WAB-gNB is exchanged between the BH-RAN-node and the WAB-node. WAB-node can send resource configurations of the WAB-node to the BH-RAN-node. The resource configuration of the WAB-gNB’s cell contains the configuration for slot format and symbol type (H/S/NA) of each slot. The slot format indicates the number and position of D/U/F symbols.

The BH-RAN-node can send resource configurations of the cell(s) serving the WAB-MT to the WAB-node. Since BH-RAN-node is legacy NG-RAN node, the resource configuration of BH-RAN-node’s cell only needs to include the slot format to indicate the number and position of D/U/F symbols.

Observation 2-1: The resource configuration of WAB-gNB’s cell(s) including slot format and symbol type of each slot can be exchanged from WAB-gNB to BH-RAN node. The resource configuration including the slot format of WAB-MT’s serving cell(s) can be exchanged from BH-RAN node to WAB-gNB.

The multiplexing capability between WAB-gNB cell and cell(s) serving the co-located WAB-MT should also be sent to the BH-RAN-node. The multiplexing capability indicates whether simultaneous operation of WAB-MT Rx / WAB-gNB Rx, WAB-MT Tx / WAB-gNB Tx, WAB-MT Rx / WAB-gNB Tx, WAB-MT Tx / WAB-gNB Rx is supported. If the simultaneous operation of WAB-MT and WAB-gNB is not supported, time multiplexing between access and BH links should be applied, otherwise, frequency multiplexing can be utilized between access and BH links.

In IAB, the cell specific signal configured for the IAB-DU cell are considered as hard resources of IAB-DU cell, thus the relevant configurations are sent to the IAB-donor CU, including STC configuration, RACH Config Common, CSI-RS Configuration, SR Configuration and PDCCH Configuration SIB1. Similarly, the cell specific signal configured for the WAB-gNB cell should be sent to the BH-RAN node for resource coordination.

Observation 2-2: The multiplexing capability and cell specific signal configuration for WAB-gNB cell(s) should be sent to the BH-RAN node.

Proposal 2-2: The resource coordination information sent from WAB-node to BH-RAN node includes:

- Resource configuration including slot format and symbol type for a WAB-gNB cell,

- Multiplexing capability between a WAB-gNB cell and cells serving the co-located WAB-MT,

- Cell specific signal configuration for a WAB-gNB cell.

Proposal 2-3: The resource coordination information sent from BH-RAN-node to WAB-node includes the resource configuration including slot format of each cell serving the WAB-MT.

- Use Xn or RRC to exchang resource coordination information

The resource coordination information would be exchanged between WAB-node and its BH-RAN-node via RRC or XnAP (If Xn interface between BH-RAN-node and WAB-node is established).

As the solution of using RRC signaling can be done without the Xn connection between WAB-node and BH-RAN-node, this solution is less constraint. However, it has large impact to the RRC specification.

Using XnAP signaling to achieve resource coordination is similar with the mechanism of IAB, but the disadvantage is that it relies on the Xn connection between the BH-RAN-node and WAB-gNB. Further, it requires co-location discovery of WAB-MT and WAB-gNB.

Observation 2-3: If the resource coordination is done by XnAP, the BH-RAN-node needs to discover co-location of WAB-MT and WAB-gNB.

However, considering resource multiplexing is optional and Xn is also optional, using Xn is feasible. Although co-location discovery is required, limited impact is expected for XnAP. In our view, the resource coordination information can be sent between access and backhaul links via XnAP.

Proposal 2-4: Exchanging resource coordination information between BH-RAN-node and WAB-node over Xn.

- Discover co-location of WAB-MT and WAB-gNB

The identity of WAB-MT is used over RRC between the BH-RAN-node and the WAB-node, while the gNB ID of WAB-gNB is used over Xn between the BH-RAN-node and the WAB-node. BH-RAN-node can discover co-location of the WAB-MT and the WAB-gNB by either the gNB ID of WAB-gNB or the identity of WAB-MT.

There are two options for co-location discovery:

Option 1: WAB-MT reporting the identity of WAB-gNB.

The WAB-MT reports the gNB ID of its co-located WAB-gNB to the BH-RAN-node via RRC message, e.g., during phase 1 for WAB-MT setup. The BH-RAN-node can discover the WAB-MT is co-located with the WAB-gNB through the gNB ID, e.g., when the WAB-gNB sets up Xn interface with it during phase 2-3.

Option 2: WAB-gNB reporting the identity of WAB-MT.

The BH-RAN-node allocates C-RNTI in the servicing cell to WAB-MT in phase 1. WAB-gNB sends the C-RNTI and the cell identity of the serving cell of its co-located WAB-MT to the BH-RAN-node using XnAP signaling, after the Xn setup procedure in phase 2-3. The BH-RAN-node can discover co-location of the WAB-MT and the WAB-gNB via the C-RNTI and cell identity.

Comparing the two options, option 2 has an advantage over option 1 on avoiding the impact to RRC. It is proposed to adopt option 2 for co-location discovery.

Proposal 2-5: BH-RAN-node discovers the co-location of WAB-gNB and WAB-MT via the WAB-gNB reporting the identity of the WAB-MT over Xn.

- Signaling design

For resource coordination between WAB-node and BH-RAN node, a new Class 1 XnAP procedure can be introduced.

In IAB, an IAB-node is served as a UE by both F1-terminating donor non-F1-terminating donor, and the resource coordination information for the IAB-node is exchanged via the F1-terminating and non-F1-terminatig donors over Xn, so the IAB Resource Coordination procedure is a UE-associated Xn procedure. Different to IAB, where the resource coordination information for the IAB-node must be transferred via its donor-CUs, the WAB-node can exchange the resource coordination information with the BH-RAN node by itself. Thus, the resource coordination procedure of WAB should be non-UE associated.

Proposal 2-6: A new Class 1 XnAP procedure is introduced for WAB resource coordination, this procedure is a non-UE associated procedure.

Based on above, for the BH-RAN node to discover the co-location of WAB-MT and WAB-gNB, the WAB-MT identity information can be added to the new Xn message, which includes C-RNTI of the WAB-MT as well as identity of the cell serving the WAB-MT. The WAB-node can send the new Xn message with the identity information to the BH-RAN node.

Proposal 2-7: The WAB-MT identity information is added to the new Xn messages.

As discussed above, the following information of each WAB-gNB cell can be added to the new Xn message:

- Resource configuration including slot format and symbol type

- Multiplexing capability

- Cell specific signal configuration

The WAB-node can send above information via the new Xn messages to the BH-RAN node.

Also, the resource configuration of each cell serving the WAB-MT, including slot format, can be added to the new Xn messages, so that the BH-RAN node can send this information to the WAB-node.

Proposal 2-8: The information of each WAB-gNB cell, including the resource configuration, multiplexing capability and cell specific signal configuration, and the resource configuration of each cell serving the WAB-MT are included in the new Xn messages.

3 Conclusion

In the previous sections we made the following proposals:

Xn connection management

Proposal 1-1: WAB-gNB can recognize BH-RAN-node via SIB1 of the cell serving the WAB-MT.

Proposal 1-2: WAB-gNB obtains IP address of the BH-RAN-node via legacy Xn-C TNL address discovery mechanism and sets up Xn connection with it.

Proposal 1-3: If WAB-gNB establishes Xn connection with BH-RAN-node, it discovers neighbour cells through Xn messages sent from the BH-RAN-node.

Proposal 1-4: WAB-gNB looks up the IP address(es) of the neighbour nodes using legacy Xn-C TNL address discovery mechanism and set up Xn connections with surrounding RAN nodes.

Observation 1-1: Besides the Xn setup avoidance between WAB-gNBs, avoiding Xn connection setup between a WAB-gNB and a NG-RAN node would also be necessary.

Observation 1-2: WAB-cell specific information can be added to the Served Cell Information NR IE over Xn interface.

Proposal 1-5: Resource configuration of WAB cell is introduced to Served Cell Information NR IE to implicitly indicate a WAB cell.

Proposal 1-6: WAB-gNB considers the cell not included in the serving/neighbour cell information of current BH-RAN-node as no longer acting as the neighbour cell.

Proposal 1-7: WAB-gNB removes Xn with the with the NG-RAN nodes which do not serve any neighbour cell of the WAB-gNB.

Resource multiplexing

Proposal 2-1: Three types of symbol (i.e., hard, soft and unavailable) can be used for resource multiplex of WAB. WAB-gNB can determine whether the soft symbols are used relying on implementation.

Observation 2-1: The resource configuration of WAB-gNB’s cell(s) including slot format and symbol type of each slot can be exchanged from WAB-gNB to BH-RAN node. The resource configuration including the slot format of WAB-MT’s serving cell(s) can be exchanged from BH-RAN node to WAB-gNB.

Observation 2-2: The multiplexing capability and cell specific resources configuration for WAB-gNB cell(s) should be sent to the BH-RAN node.

Proposal 2-2: The resource coordination information sent from WAB-node to BH-RAN node includes:

- Resource configuration including slot format and symbol type for a WAB-gNB cell,

- Multiplexing capability between a WAB-gNB cell and cells serving the co-located WAB-MT,

- Cell specific signal configuration for a WAB-gNB cell.

Proposal 2-3:  The resource coordination information sent from BH-RAN-node to WAB-node includes slot format for each cell serving the WAB-MT.

Observation 2-3: If the resource coordination is done by XnAP, the BH-RAN-node needs to discover co-location of WAB-MT and WAB-gNB.

Proposal 2-4: Exchanging resource coordination information between BH-RAN-node and WAB-node over Xn.

Proposal 2-5: BH-RAN-node discovers the co-location of WAB-gNB and WAB-MT via the WAB-gNB reporting the identity of the WAB-MT over Xn.

Proposal 2-6: A new Class 2 XnAP procedure is introduced for WAB resource coordination, this procedure is a non-UE associated procedure.

Proposal 2-7: The WAB-MT identity information is added to the new Xn message.

Proposal 2-8: The information of each WAB-gNB cell, including the resource configuration, multiplexing capability and cell specific signal configuration, and the resource configuration of each cell serving the WAB-MT are included in the new Xn messages.

4 Reference

- RAN3#126 agenda, RAN3 Chair

- RP-242359, New WID on additional topological enhancements for NR, NTT DOCOMO, INC., AT&T, RAN#105

- R3-244849, 3GPP TR 38.799, Study on additional topological enhancements for NR, v1.2.0 (2024-08)

5 Annex (TP to BLCR for TS 38.423)

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Start of TP<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

# 8	XnAP procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | XN REMOVAL REQUEST | XN REMOVAL RESPONSE | XN REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| Data Collection Reporting Initiation | DATA COLLECTION REQUEST | DATA COLLECTION RESPONSE | DATA COLLECTION FAILURE |
| WAB Resource Coordination | WAB RESOURCE COORDINATION REQUEST | WAB RESOURCE COORDINATION RESPONSE |  |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

## 8.6	WAB Procedures

### 8.6.1	WAB Resource Coordination

#### 8.6.1.1	General

The purpose of the WAB Resource Coordination procedure is to exchange the semi-static radio resource configuration pertaining to a WAB-node and its BH-RAN node, for the purpose of resource multiplexing between the WAB-MT and the WAB-gNB of the WAB-node. The procedure can be initiated by the WAB-node or the BH-RAN node. The procedure can be used for WAB-node.

The procedure uses non-UE-associated signalling.

#### 8.5.4.2	Successful Operation

Figure 8.5.4.2-1: WAB Resource Coordination triggered by BH-RAN-node/WAB-node, successful operation

The BH-RAN-node/WAB-node initiates the procedure by sending the WAB RESOURCE COORDINATION REQUEST message to the WAB-node/BH-RAN-node. The WAB-node/BH-RAN-node shall respond with the WAB RESOURCE COORDINATION RESPONSE message to the BH-RAN-node/WAB-node.

If the WAB-MT Identity Information IE is included in the WAB RESOURCE COORDINATION REQUEST or in the WAB RESOURCE COORDINATION RESPONSE message, the receiving BH-RAN-node shall consider the information therein for discovering the co-location of a WAB-gNB and a WAB-MT.

If the WAB-gNB Cells List IE and/or BH-RAN Node Cells List IE is included in the WAB RESOURCE COORDINATION REQUEST or in the WAB RESOURCE COORDINATION RESPONSE message, the receiving WAB-node/BH-RAN-node shall take this information into account for resource coordination with the sending BH-RAN-node/WAB-node.

#### 8.5.4.3	Unsuccessful Operation

Not applicable.

#### 8.5.4.4	Abnormal Conditions

Not applicable.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

### 9.1.5	Messages for WAB Procedures

#### 9.1.5.1	WAB RESOURCE COORDINATION REQUEST

This message is sent by a BH-RAN-node/WAB-node to a WAB-node/BH-RAN-node, for the purpose of coordination of the semi-static resources of WAB-node.

Direction: BH-RAN-node  WAB-node, WAB-node  BH-RAN-node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| WAB-MT Identity Information |  | 0..1 |  | This IE contains the identity information of the co-located WAB-MT in case this message is sent from the WAB-gNB to the BH-RAN node. | YES | ignore |
| >C-RNTI | M |  | BIT STRING (SIZE (16)) |  | - |  |
| >NR CGI | M |  | 9.2.2.7 | Cell identity of the cell serving the WAB-MT. | - |  |
| WAB-gNB Cells List |  | 0..1 |  | List of cells served by the WAB-gNB. | YES | reject |
| > WAB-gNB Cells List Item |  | 1 .. <maxnoofServedCellsWAB > |  |  | – |  |
| >>WAB Node cell Information | M |  | WAB Cell Information 9.2.2.xx |  | – |  |
| BH-RAN Node Cells List |  | 0..1 |  | List of cells served by the BH-RAN node. | YES | reject |
| > BH-RAN Node Cells List Item |  | 1 .. < maxnoofServingCells > |  |  | – |  |
| >>BH-RAN Node Cell Information | M |  | Cell Resource Configuration 9.2.2.yy |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedCellsWAB | Maximum number of cells served by a WAB-gNB. Value is 512. |
| maxnoofServingCells | Maximum no. of serving cells for a WAB-MT. Value is 32, as defined by the maxNrofServingCells in TS 38.331 [10]. |

#### 9.1.5.2	WAB RESOURCE COORDINATION RESPONSE

This message is sent by a WAB-node/BH-RAN-node to a BH-RAN-node/WAB-node, in response to an wAB RESOURCE COORDINATION REQUEST message.

Direction: WAB-node  BH-RAN-node, BH-RAN-node  WAB-node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| WAB-MT Identity Information |  | 0..1 |  | This IE contains the identity information of the co-located WAB-MT. It is only present in case this message is sent from the WAB-node to the BH-RAN node. | YES | reject |
| >C-RNTI | M |  | BIT STRING (SIZE (16)) |  | - |  |
| >NR CGI | M |  | 9.2.2.7 | Cell identity of the cell serving the WAB-MT. | - |  |
| WAB-gNB Cells List |  | 0..1 |  | List of cells served by the WAB-gNB. | YES | reject |
| >WAB-gNB Cells List Item |  | 1 .. <maxnoofServedCellsWAB > |  |  | – |  |
| >>WAB Node cell Information | M |  | WAB Cell Information 9.2.2.xx |  | – |  |
| BH-RAN Node Cells List |  | 0..1 |  | List of cells served by the BH-RAN node. | YES | reject |
| > BH-RAN Node Cells List Item |  | 1 .. < maxnoofServingCells > |  |  | – |  |
| >>BH-RAN Node Cell Information | M |  | Cell Resource Configuration 9.2.2.yy |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedCellsWAB | Maximum number of cells served by a WAB-gNB. Value is 512. |
| maxnoofServingCells | Maximum no. of serving cells for a WAB-MT. Value is 32, as defined by the maxNrofServingCells in TS 38.331 [10]. |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

#### 9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID | – |  |
| NR CGI | M |  | 9.2.2.7 |  | – |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code | – |  |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  | – |  |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs contained in the SIB1 message as specified in TS 38.331[10], associated to the NR Cell Identity in the NR CGI IE. | – |  |
| >PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| CHOICE NR-Mode-Info | M |  |  |  | – |  |
| >FDD |  |  |  |  |  |  |
| >>FDD Info |  | 1 |  |  | – |  |
| >>>UL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 | This IE is ignored for NR operating bands for which uplink range of NREF is not defined in section 5.4.2.3 of TS 38.104 [24]. | – |  |
| >>>DL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – |  |
| >>>UL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 | This IE is ignored for NR operating bands for which uplink range of NREF is not defined in section 5.4.2.3 of TS 38.104 [24]. | – |  |
| >>>DL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – |  |
| >>>UL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the UL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>DL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the DL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>gNB-DU Cell Resource Configuration-FDD-UL | O |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains FDD UL resource configuration of gNB-DU’s cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | YES | ignore |
| >>>gNB-DU Cell Resource Configuration-FDD-DL | O |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains FDD DL resource configuration of gNB-DU’s cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | YES | ignore |
| >>>WAB Cell Resource Configuration-FDD-UL | O |  | WAB Cell Resource Configuration 9.2.2.xx+1 | Contains FDD UL resource configuration of WAB’s cell. Only applicable if the gNB is a WAB-gNB. | Yes | ignore |
| >>>WAB Cell Resource Configuration-FDD-DL | O |  | WAB Cell Resource Configuration 9.2.2.xx+1 | Contains FDD DL resource configuration of WAB’s cell. Only applicable if the gNB is a WAB-gNB. | Yes | ignore |
| >TDD |  |  |  |  |  |  |
| >>TDD Info |  | 1 |  |  | – |  |
| >>>Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – |  |
| >>>Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 | This IE is ignored if the Transmission Bandwidth asymmetric IE is present. | – |  |
| >>>Intended TDD DL-UL Configuration NR | O |  | 9.2.2.40 |  | YES | ignore |
| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | Includes the tdd-UL-DL-ConfigurationCommon contained in the SIB1 message as defined in TS 38.331 [10] | YES | ignore |
| >>>Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>gNB-DU Cell Resource Configuration-TDD | O |  | gNB-DU Cell Resource Configuration 9.2.2.95 | Contains TDD resource configuration of gNB-DU’s cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | YES | ignore |
| >>>>WAB Cell Resource Configuration-TDD | O |  | WAB Cell Resource Configuration 9.2.2.xx+1 | Contains TDD resource configuration of WAB’s cell. Only applicable if the gNB is a WAB-gNB. | Yes | ignore |
| >>>Transmission Bandwidth asymmetric |  | 0..1 |  | Indicates the asymmetric UL and DL transmission bandwidth. | YES | ignore |
| >>>>UL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – |  |
| >>>>DL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – |  |
| **************** Unchanged part is omitted **************** |  |  |  |  |  |  |
| Barring Exemption for Emergency Call Information | O |  | ENUMERATED (true, ...) | Corresponds to information provided in the barringExemptEmergencyCall contained in the SIB1 message as defined in 38.331 [10]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by one gNB. Value is 256. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U channel IDs in a cell. Value is 16. |
| maxnoofMTCItems | Maximum no. of measurement timing configurations associated with the neighbour cell. Value is 16. |
| maxnoofCSIRSconfigurations | Maximum number of CSI RS configurations reported in the MTC. Value is 96 |
| maxnoofCSIRSneighbourCells | Maximum number of cells neighbouring a CSI-RS coverage area. Value is 16 |
| maxnoofCSIRSneighbourCellsInMTC | Maximum number of CSI-RS coverage areas neighbouring a specific CSI-RS coverage area. Value is 16 |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Next Change<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

#### 9.2.2.xx	WAB Cell Information

This IE contains WAB node cell resource configuration, cell specific signal/channel configuration and multiplexing info of the cell of a WAB-node or BH-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.2.2.7 |  |
| CHOICE WAB Cell Resource Configuration-Mode-Info | O |  |  |  |
| >TDD |  |  |  |  |
| >>TDD Info |  | 1 |  |  |
| >>>WAB Cell Resource Configuration-TDD | M |  | WAB Cell Resource Configuration 9.2.2.xx+1 | Contains TDD resource configuration of WAB-gNB’s cell. |
| >>>Frequency Info | O |  | NR Frequency Info 9.2.2.19 |  |
| >>>Transmission Bandwidth | O |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >>>Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the Transmission Bandwidth IE shall be ignored. |
| >FDD |  |  |  |  |
| >>FDD Info |  | 1 |  |  |
| >>> WAB Cell Resource Configuration-FDD-UL | M |  | WAB Cell Resource Configuration 9.2.2.xx+1 | Contains FDD UL resource configuration of WAB-gNB’s cell. |
| >>> WAB Cell Resource Configuration-FDD-DL | M |  | WAB-gNB Cell Resource Configuration 9.2.2.xx+1 | Contains FDD DL resource configuration of WAB-gNB’s cell. |
| >>>UL Frequency Info | O |  | NR Frequency Info 9.2.2.19 |  |
| >>>DL Frequency Info | O |  | NR Frequency Info 9.2.2.19 |  |
| >>>UL Transmission Bandwidth | O |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >>>DL Transmission Bandwidth | O |  | NR Transmission Bandwidth 9.2.2.20 |  |
| >>>UL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the UL Transmission Bandwidth IE shall be ignored. |
| >>>DL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the DL Transmission Bandwidth IE shall be ignored. |
| WAB STC Info | O |  | 9.2.2.xx+2 | STC configuration of this WAB-gNB’s cell. |
| RACH Config Common | O |  | OCTET STRING | Includes the rach-ConfigCommon as defined in subclause 6.3.2 of TS 38.331 [10]. |
| CSI-RS Configuration | O |  | OCTET STRING | Includes the NZP-CSI-RS-Resource IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| SR Configuration | O |  | OCTET STRING | Includes the SchedulingRequestResourceConfig IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| PDCCH Configuration SIB1 | O |  | OCTET STRING | Includes the PDCCH-ConfigSIB1 IE as defined in subclause 6.3.2 of TS 38.331 [10]. |
| SCS Common | O |  | OCTET STRING | Includes the subCarrierSpacingCommon as defined in subclause 6.2.2 of TS 38.331 [10]. |
| WAB Multiplexing Info | O |  | 9.2.2.xx+3 | Contains information on multiplexing with cells configured for collocated WAB-MT, if applicable. |

#### 9.2.2.xx+1	WAB Cell Resource Configuration

This IE contains the resource configuration of the cells served by a WAB-gNB, i.e. the TDD/FDD resource parameters for each activated cell (TS 38.213 [40], clause 11.1.1). This IE is only applicable the case of WAB-node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the TDD/FDD slot configuration. |
| DUF Transmission Periodicity | O |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, …) |  |
| DUF Slot Configuration List |  | 0..1 |  |  |
| >DUF Slot Configuration Item |  | 1..<maxnoofDUFSlots> |  |  |
| >>CHOICE DUF Slot Configuration | M |  |  |  |
| >>>Explicit Format |  |  |  |  |
| >>>>Permutation | M |  | ENUMERATED (DFU, UFD, …) |  |
| >>>>Number of Downlink Symbols | O |  | INTEGER (0..14) |  |
| >>>>Number of Uplink Symbols | O |  | INTEGER (0..14) |  |
| >>>Implicit Format |  |  |  |  |
| >>>>DUF Slot Format Index | M |  | INTEGER (0..254) | Index into Table 11.1.1-1 and Table 14-2 in TS 38.213 [40], excluding the last row in Table 14-2. |
| HSNA Transmission Periodicity | M |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160, …) |  |
| HSNA Slot Configuration List |  | 0..1 |  |  |
| >HSNA Slot Configuration Item |  | 1..<maxnoofHSNASlots> |  |  |
| >>HSNA Downlink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for downlink symbols in a slot. |
| >>HSNA Uplink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for uplink symbols in a slot. |
| >>HSNA Flexible | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for flexible symbols in a slot. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDUFSlots | Maximum no. of slots in 10ms. Value is 320. Corresponds to the maxNrofSlots defined in TS 38.331 [10]. |
| maxnoofHSNASlots | Maximum no of "Hard", "Soft" or "Not available" slots in 160ms. Value is 5120. |

#### 9.2.2.xx+2	WAB STC Info

This IE contains cell SSB Transmission Configuration (STC) information of a WAB node or a BH-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| WAB STC-Info List |  | 1 |  |  |
| >WAB STC-Info Item |  | 1 ..<maxnoofWABSTCInfo> |  |  |
| >>SSB Frequency Info | M |  | INTEGER (0.. maxNRARFCN) | The SSB central frequency. |
| >>SSB Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, …) | The SSB subcarrier spacing. |
| >>SSB Transmission Periodicity | M |  | ENUMERATED (sf10, sf20, sf40, sf80, sf160, sf320, sf640, …, sf5) |  |
| >>SSB Transmission Timing Offset | M |  | INTEGER (0.. 127, …) | SSB transmission timing offset in number of half-frames. |
| >>CHOICE SSB Transmission Bitmap | M |  |  | Corresponds to information provided in the SSB-ToMeasure IE defined in TS 38.331 [10]. |
| >>>short bitmap |  |  |  |  |
| >>>>Short Bitmap | M |  | BIT STRING (SIZE (4)) |  |
| >>>medium bitmap |  |  |  |  |
| >>>>Medium Bitmap | M |  | BIT STRING (SIZE (8)) |  |
| >>>long bitmap |  |  |  |  |
| >>>>Long Bitmap | M |  | BIT STRING (SIZE (64)) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofWABSTCInfo | Maximum no. of STC configurations. Value is 5. This includes 1 STC configuration for access and 4 STC configurations for backhaul. |
| maxNRARFCN | Maximum value of NR ARFCNs. Value is 3279165. |

#### 9.2.2.xx+3	WAB Multiplexing Info

This IE contains information about the multiplexing capabilities between the WAB-gNB’s cell and the cells configured on the co-located WAB-MT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| WAB-MT Cell List |  | 1 |  |  |
| >WAB-MT Cell Item |  | 1 .. <maxnoofServingCells> |  |  |
| >>NR Cell Identity | M |  | BIT STRING (SIZE(36)) | Cell identity of a serving cell configured for a co-located WAB-MT. |
| >>WAB-gNB_RX/WAB-MT_RX | M |  | ENUMERATED (supported, not supported , …) | An indication of whether the WAB-node supports simultaneous reception at its gNB and MT side. |
| >> WAB-gNB _TX/ WAB-MT _TX | M |  | ENUMERATED (supported, not supported, …) | An indication of whether the WAB-node supports simultaneous transmission at its gNB and MT side. |
| >> WAB-gNB _TX/ WAB-MT _RX | M |  | ENUMERATED (supported, not supported, …) | An indication of whether the WAB-node supports simultaneous transmission at its gNB and reception at its MT side. |
| >> WAB-gNB _RX/ WAB-MT _TX | M |  | ENUMERATED (supported, not supported, …) | An indication of whether the WAB-node supports simultaneous reception at its gNB and transmission at its MT side. |

| Range bound | Explanation |
| --- | --- |
| maxnoofServingCells | Maximum no. of serving cells for an WAB-MT. Value is 32, as defined by the maxNrofServingCells in TS 38.331 [10]. |

#### 9.2.2.yy	Cell Resource Configuration

This IE contains the resource configuration of a cell, i.e., the TDD/FDD resource parameters for each activated cell (TS 38.213 [40], clause 11.1.1).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the TDD/FDD slot configuration. |
| DUF Transmission Periodicity | O |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, …) |  |
| DUF Slot Configuration List |  | 0..1 |  |  |
| >DUF Slot Configuration Item |  | 1..<maxnoofDUFSlots> |  |  |
| >>CHOICE DUF Slot Configuration | M |  |  |  |
| >>>Explicit Format |  |  |  |  |
| >>>>Permutation | M |  | ENUMERATED (DFU, UFD, …) |  |
| >>>>Number of Downlink Symbols | O |  | INTEGER (0..14) |  |
| >>>>Number of Uplink Symbols | O |  | INTEGER (0..14) |  |
| >>>Implicit Format |  |  |  |  |
| >>>>DUF Slot Format Index | M |  | INTEGER (0..254) | Index into Table 11.1.1-1 and Table 14-2 in TS 38.213 [40], excluding the last row in Table 14-2. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDUFSlots | Maximum no. of slots in 10ms. Value is 320. Corresponds to the maxNrofSlots defined in TS 38.331 [10]. |

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>End of TP<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<